# T800-50-0000 Pinouts

SK6 (Lower)

| Pin | Function | Description |
| --- | --- | --- |
| 1 | 13.8V | 13.8 Volt DC supply |
| 2 | 13.8V | 13.8 Volt DC supply |
| 3 | TX/EX AUX 2 Output | This is a digital output that can be programmed to toggle depending on which channel is selected |
| 4 | TX/EX AUX 1 Output | This is a digital output that can be programmed to toggle depending on which channel is selected |
| 5 | RX CTCSS Disable | This overrides the requirement for a CTCSS tone to open the mute |
| 6 | RX + TX/EX AUX Ground | Digital ground for the Aux outputs and channel select inputs. |
| 7 | RX + TX/EX Channel select 6 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 8 | RX + TX/EX Channel select 4 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 9 | RX + TX/EX Channel select 2 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 10 | RX Aux 1 Output | This is a digital output that can be programmed to toggle depending on which channel is selected |
| 11 | RX Aux 2 Output | This is a digital output that can be programmed to toggle depending on which channel is selected |
| 12 | 0V | Supply Ground |
| 13 | 0V | Supply Ground |
| 14 | 13.8V | 13.8 Volt DC supply |
| 15 | RX Aux 0 Output | This is a digital output that can be programmed to toggle depending on which channel is selected |
| 16 | TX/EX AUX 0 Output | This is a digital output that can be programmed to toggle depending on which channel is selected |
| 17 | TX CTCSS Disable | This overrides the CTCSS tone being generated from the transmitter |
| 18 | RX Inhibit | This will stop the receiver from receiving a signal. It is usually used when a transmitter in the same rack/site is transmitting on the same frequency. In this case you can connect this line to the TX enable line from the Exciter and “Mute” the receiver whenever the transmitter is keyed. |
| 19 | RX+EX/TX Channel Select 7 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 20 | RX+EX/TX Channel Select 5 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 21 | RX+EX/TX Channel Select 3 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 22 | RX+EX/TX Channel Select 1 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 23 | RX+EX/TX Channel Select 8 | There are 8 channel select lines which can be used to select up to 128 channels using a binary sequence – See the service manual for the sequence required to select each channel |
| 24 | RX/TX/EX Secondary Serial Comms | Used to program the radio from PGM 800 |
| 25 | 0V | Supply Ground |

SK7 (Upper)

| Pin | Function | Description |
| --- | --- | --- |
| 1 | RX Line 1 | 600 Ohm balanced line receiver output |
| 2 | RX Line 2 | Normally shorted to RX Line 3 |
| 3 | RX Line 3 | Normally shorted to RX Line 2 |
| 4 | RX Line 4 | 600 Ohm balanced line receiver output |
| 5 | RX sig. Strength (RSSI) | A constantly variable voltage proportional to the received signal strength. See the appropriate manual for exact voltage curves. |
| 6 | RX Audio 1 | Normally used for an unbalanced line output. This is a 1K ohm line output |
| 7 | RX/TX/EX primary Serial Comms | Used to program the radio from PGM 800 |
| 8 | TX/EX Opto Key Neg. | This is an opto-coupled input used to key the TX from a 48v supply. You need to supply enough current to light the LED, typically 20-40 mA. |
| 9 | TX/EX Opto Key Pos. | As for Pin 8. |
| 10 | PA Forward Power Alarm | Open collector output, goes to Gnd when the PA output power drops by 3dB |
| 11 | PA Reverse Power Alarm | Open collector output, goes to Gnd when the PA output has a 3:1 VSWR |
| 12 | PA Forward Power Meter | Provides an output current for a moving coil meter proportional to the forward power of the PA output. |
| 13 | PA Reverse Power Meter | Provides an output current for a moving coil meter proportional to the reverse power on the PA output. |
| 14 | RX Gate | An open collector output which goes to ground when the receiver is receiving a valid signal. |
| 15 | RX Relay Common | The common lead of a relay inside the RX which is operated in conjunction with the Mute. |
| 16 | RX Relay Normally Open | The N/O lead of a relay inside the RX which is operated in conjunction with the Mute. |
| 17 | TX/EX key | Ground this lead to key the transmitter. |
| 18 | TX/EX tone input | This is a flat audio input normally used for external CTCSS tones, but can also be used for FFSK modem tones |
| 19 | RX Speaker | Low Impedance monitor speaker output (1W) |
| 20 | TX Audio 2 | This is the unbalanced input to the transmitter. It is normally shorted to TX Audio 1, and is used when you want to do additional processing on the audio. |
| 21 | TX/EX Enable | This is an open collector output that goes low when the transmitter is keyed. It is normally connected to the PA to enable the alarm circuits. |
| 22 | TX Line 1 | 600 Ohm balanced line Transmitter input |
| 23 | TX Line 2 | Normally shorted to RX Line 3 |
| 24 | TX Line 3 | Normally shorted to RX Line 2 |
| 25 | TX Line 4 | 600 Ohm balanced line Transmitter input |