# Q-MAC Hand Crank Generator (QM8027)

## Description:

The QM8027 is a lightweight, durable hand crank generator primarily designed for charging the 7Ah SLA gel battery supplied with the HF-90 Manpack and Portable Packages.  It has been specifically designed for use in hostile environments where the unit may be subject to shock, vibration, rain, snow and temperature extremes.  The high impact case is hermetically sealed and has folded stainless steel handles and crankshaft.  This creates a virtually indestructible package, which is immersible in water up to 1m.

## Specifications:

### ELECTRICAL

Output: 	30 Watts at 16.0 Volts regulated at 70 RPM.  Unit is reverse polarity protected and suitable for charging batteries at nominal voltage of 12 – 15 Volts.  LED indicator shows charge state.  Ripple is 300 mV nominal, 500 mV peak at 1 Amp load.  Other output voltages are available on special order.

### MECHANICAL

Case: 	High impact material per MIL-M-14.  Dimensions 153H x 108W x 112D mm.  Crank throw is 140 mm.  Color is military green with black handles.  Case bottom is fitted with four threaded holes for fixing to various optional mounts.

Weight:		3.3 kgs.

Environmental:	Ambient Temperature Range: -40C to +55C

Humidity: 95% @ +55C

Immersion: Waterproof to one meter for one hour

Shock: Designed to MIL-STD-810D

### INDICATORS

Humidity:	Indicates watertight integrity of case and possible need for drying the interior.

Status:		Red LED Bright – battery functional or being charged.

Red LED Dim – Battery functional but needs charging.

Red LED Blinking – Battery discharged and needs charging.

Specifications are subject to change without notice.