MFP1200-4:  Silent Sealed & Rugged cots ac- dc power Supply1

Features and Benefits

- Convection Cooled:  Ultra-quiet Operation

- Rain/Dust/Sleet/Sand Resistant:  Quality Performance in Harsh & Hazardous Conditions

- Sealed, Watertight & Condensation-proof:  Reliable & Rugged Operation You Can Trust

- Wide Operating Temperature:  Continuous Operation From -40°C ~ 65°C

- MIL-STD 810D Compliance:  Enhanced Shock and Vibration Protection2

- MIL-STD 461E Compliance:  Protected against EMI/RFI Conditions3

- 95% PFC:  The Only DC-operable or 45Hz ~ 440Hz; 95~250V Input Rugged Power Supply4

Input Specifications

Operating Voltage:  Auto-ranging 95 ~ 250Vac

Input Frequency Range:  DC; 45Hz ~ 440Hz

Input Current:  15A Max

Inrush Current:  In-rush Current Limiter

AC Power Cord:  1 Meter or 2 Meters (Optional)

Efficiency (1.2kW):  86% @ 110Vac

PFC (1.2kW): > 95% @ Full Operating Power

DC Output Selector

Output Specifications

Peak DC Output:  24 ~ 28Vdc @ 1.2kW

Cont. DC Output:  24 ~ 28Vdc @ 750W

Max Normal Output Current:  50Adc @ 1.2kW

PARD (20MHz):  < 50mVp-p/5mVrms

DC Output Connector: MS3470W14-4S

4-pin Dip Switch

Protection

Output Over-voltage:  OVP @ 30Vdc (Red LED indicator)

Output Over-current/Short Circuit Protection:  Self-recovering Current Limiting Protection @ 60Adc (Red LED indicator)

Over-temperature:  OTP Protection at 100°C Internal Temperature (Red LED indicator)

Input Over-current:  In-rush Current Limiter

*1.  Preliminary Specifications and subject to change without notice

*2.  Shock (MIL-STD-810/Method 516) & Vibration (MIL-STD-810/Method 514-3)

*3.  MIL-STD-461 - (CE102, CS101, CS114, RE102, RS103)

*4.  PFC (1.2kW): > 95% @ Full Operating Power

Environmental Specifications

Temperature Normal Operation:  -40°C ~ 65°C

Storage:  -40°C ~ 75°C

Full Power Temperature Operation:  -40°C ~ 50°C

Power De-rating:  -2% per °C from 50°C ~ 65°C

Shock (MIL-STD-810/Method 516)

Vibration (MIL-STD-810/Method 514-3)

Rain (MIL-STD-202 Method 104A, Test Condition A)

Sand & Dust (MIL-STD-202 Method 110A)

Salt Atmosphere

(MIL-STD-202 Method 101E, Test Condition B)

Atmospheric Pressure

Operational:  Air Pressure  -1000 ft ~ +10,000 ft

Non-operational: Air Pressure  -1000 ft ~ + 40,000 ft

Solar Radiation:  1130Watts/m^2

Ozone:  Density up to 220ug/m^3

EMI/RFI:  MIL-STD-461

Front and Rear Panel Description

(CE102, CS101, CS114, RE102, RS103)

Packaging and Cooling

Cooling:  Sealed Convection Cooled

Dimensions:  16” X 7.5” X 3.5”

Weight:  15.3lbs

Safety Specifications

Input to Case Isolation:  1500Vac

Input to Output Isolation:  1500Vac

Output to Case Isolation:  1500Vdc

Mechanical Diagram Dimensions

AMREL/AMERICAN RELIANCE INC.     www.amrel.com    Phone 800 654-9838    E-mail support@amrel.com