Example 802.16 OFDMA Setup Files for

N7615A Signal Studio software (.xml) and

89600S Option B7Y VSA software (.set)

DL-PUSC 26 sym 64QAM.xml

DL-PUSC 26 sym 64QAM.set

This N7615A .xml file sets up a simple downlink TDD waveform at 2.5 GHz with 10 MHz BW, 1024 FFT, and 5 ms frame length with 65% DL.  The frame has one DL-PUSC zone containing 26 symbols of 64QAM modulation (PN9 data).  The 89601A .set file uses the uniform burst analysis feature to demodulate the data burst.

DL-PUSC 10 MHz 3 bursts.xml

DL-PUSC 10 MHz 3 bursts.set

This N7615A .xml file sets up a more complex downlink TDD waveform at 2.5 GHz.  This waveform has 10 MHz BW, 1024 FFT, and 5 ms frame length with 60% DL.  The frame has one DL-PUSC zone containing the FCH and 3 data bursts with different modulation types of QPSK, 16QAM, and 64QAM.  The 89601A .set file uses data burst analysis, enabling the user to measure the bursts either individually or all at the same time.

UL-PUSC 1 burst 16QAM.xml

UL-PUSC 1 burst 16QAM.set

This N7615A .xml file sets up an uplink TDD waveform at 2.5 GHz with 10 MHz BW, 1024 FFT, and 5 ms frame length with 60% DL.  The frame has one UL-PUSC zone containing a single burst with 18 symbols of 16QAM modulation (PN15 data).  The 89601A .set file uses the uniform burst analysis feature to demodulate the data burst.

UL-PUSC 3 bursts.xml

UL-PUSC 3 bursts.set

This N7615A .xml file sets up an uplink TDD waveform at 2.5 GHz with 10 MHz BW, 1024 FFT, and 5 ms frame length with 60% DL.  The frame has one UL-PUSC zone containing 3 bursts of different modulation types: QPSK, 16QAM, and 64QAM.  The 89601A .set file uses data burst analysis, enabling the user to measure the bursts either individually or all at the same time.

WiBro DL 26 symb 64QAM.xml

WiBro DL 26 symb 64QAM.set

This N7615A .xml file sets up a simple downlink TDD waveform at 2.35 GHz with 8.75 MHz BW, 1024 FFT, and 5 ms frame length with 63.952% DL (for DL:UL of 27:15).  The frame has one DL-PUSC zone containing 26 symbols of 64QAM modulation (PN9 data).  The 89601A .set file uses the uniform burst analysis feature to demodulate the data burst.

WiBro DL 3 bursts.xml

WiBro DL 3 bursts.set

This N7615A .xml file sets up a more complex downlink TDD waveform at 2.35 GHz.  This waveform has 8.75 MHz BW, 1024 FFT, and 5 ms frame length with 63.952% DL.  The frame has one DL-PUSC zone containing the FCH, DL-MAP, and 3 data bursts with different modulation types of QPSK, 16QAM, and 64QAM.  The 89601A .set file uses data burst analysis, enabling the user to measure the bursts either individually or all at the same time.

WiBro UL 1 burst 16QAM.xml

WiBro UL 1 burst 16QAM.set

This N7615A .xml file sets up an uplink TDD waveform at 2.35 GHz with 8.75 MHz BW, 1024 FFT, and 5 ms frame length with 63.952% DL.  The frame has one UL-PUSC zone containing a single burst with 15 symbols of 16QAM modulation (PN15 data).  The 89601A .set file uses the uniform burst analysis feature to demodulate the data burst.

WiBro UL 3 bursts.xml

WiBro UL 3 bursts.set

This N7615A .xml file sets up an uplink TDD waveform at 2.35 GHz with 8.75 MHz BW, 1024 FFT, and 5 ms frame length with 63.952% DL.  The frame has one UL-PUSC zone containing 3 bursts of different modulation types: QPSK, 16QAM, and 64QAM.  The 89601A .set file uses data burst analysis, enabling the user to measure the bursts either individually or all at the same time.

Recommended Equipment Setup

- Connect 10 MHz reference output from E4438C to External Reference input in PSA

- Connect EVENT 1 output from E4438C to External Trigger input in PSA

- Connect RF output from E4438C to RF input on PSA

- Refer to the installation information for Signal Studio and the 89601A VSA software to set up the LAN or GPIB connections to both instruments.

- To use the N7615A setup files, start the Signal Studio software, then go to File, Open, and open the .xml file.

- To use the 89601A setup file, start the Vector Signal Analyzer software, then go to File, Recall, Recall Setup and select the .set file.  Note you may also need to configure the VSA software or the PSA to use the proper external trigger input (front or rear) and external reference.