HV-F31F(Sample) Register Map(IIDC Ver.1.30)

- Status and control register for feature

3-1. BRIGHTNESS		[Address：F0F0-0800h]

Value:820000□□h		□□：00h　～ FFh

3-2. SHARPNESS		[Address：F0F0-0808h]

Value:820000□□h		□□：00h　～ FFh

3-3. WHITE BALANCE	[Address：F0F0-080Ch]

━━R gain/ Bgain Manual Adjust）━━

Value:820△△0□□h		△△： 00h　～ FFh(B gain)

□□： 00h　～ FFh(R gain)

━━ One Push (AWB)━━

Value:86000000h

━━ AUTO━━

Value:83000000h

3-4. GAIN			[Address：F0F0-0820h]

━━Manual Adjust）━━

Value:82000□□□h		□□□：000h　～ 0C0h (04/04/06訂正)

━━ AUTO━━

Value:83000000h

3-5. SHUTTER		[Address：F0F0-081Ch]

━━Manual Adjust）━━

Value:82000□□□h		□□□：7C4h　～ B27h

━━ AUTO━━

Value:83000000h

3-6. AUTO EXPOSURE	[Address：F0F0-0804h]

━━Manual Adjust）━━

Value:820000□□h		□□：00h　～ FFh

3-6. SATURATION		[Address：F0F0-0814h]

━━Manual Adjust）━━

Value:820000□□h		□□：00h　～ FFh

3-7. GAMMA			[Address：F0F0-0818h]

━━OFF━━

Value:80000000h

━━ON1━━

Value:82000000h

━━ON2━━

Value:82000001h

━━ON3━━

Value:82000002h

3-7. TRIGGER		[Address：F0F0-0830h]

━━OFF━━

Value:80000000h

━━ MODE 0 ━━

Value:82000000h

━━ MODE 1━━

Value:82000001h

3-7. FLASH			[Address：F200-0018h] (Advanced Command)

━━OFF━━

Value:80000000h

━━ON━━

Value:8200△□□□h	  △： 0　 NARROW,  1 MIDDLE,  2 WIDE

□□：000h　～ FFFh　( START)

3-8. INITIALIZZE		[Address：F0F0-0000h]

Value:80000000h

3-9. MEMORY SAVE		[Address：F0F0-0620h]

Value:10000000h

[Reference Data ]

━━━ Shutter ━━

(1) SHUTTER OFF( 1/30sec)

Command：80000000h

(2) AUTO

Command：83000***h     (***:Unfixed)

(3) Manual Adjust( ４sec ～ 1/100,000sec))

Command：82000nnnh     (nnn  Setting range：7C4ｈ～B27h,

Standard Setup 800h：≒1/30sec )

[The calculation method]

( shutter speed　≦ 1/30 )

①Exposure time → Value
nnnh = 800h + log0.99(30×”Shutter Speed”)

②Value→Exposure time
Shutter Speed[sec] = (1/30) × 0.99(nnn―800h)

Example 1.　 Exposure time = 1/100sec

Value:nnn=800h+ log0.99(30×(1/100))

＝800h+ log(30×(1/100)) / log0.99
≒ 878h

Example 2.　 Value:nnn = 878h

Exposure time = (1/30) × 0.99(878h－800h) 　≒ 1/100(sec)

( shutter speed > 1/30)

①Exposure time → Value
nnnh = 800h - (15×”Shutter Speed”)

②Value→Exposure time
Shutter Speed[sec] = (800h – nnn) / 15

Example　 Exposure time = 1/7.5 sec

Value:nnn=800h- (15×(1/7.5)) = 7FE