AWS3274: 3274 Control Unit Emulator

===================================

AWS3274 allows TCP/IP connectable 3270 emulators to be attached as 3274

control unit display devices such as 3277, 3278, and 3279.

AWS3274 uses a TCP/IP telnet server to connect 3270 emulators such as

xhcon, x3270, PC3270, etc.  This allows 3270 sessions to be connected

from any TCP/IP client on the LAN.  The port number 3046 is used to

connect these sessions to AWS3274 on the R/390 System.

NOTE: Even though AWS3274 communicates with client sessions via TCP/IP,

to S/370 or S/390 hosts, AWS3274 devices appear as non-SNA DFT

sessions.

REMOTE SESSIONS:

================

For remote sessions, use the IP address or name of the R/390's AIX Host:

Example: x3270 -ext -sk -graphics -port 3046 9.117.118.45 &

Example: x3270 -ext -sk -graphics -port 3046 R390001 &

LOCAL SESSIONS:

===============

Use the Loopback address (usually localhost or 127.0.0.1) for local

sessions:

Example: x3270 -ext -sk -graphics -port 3046 localhost &

Example: x3270 -ext -sk -graphics -port 3046 127.0.0.1 &

NOTE: Local sessions are started automatically during "IPL P/390".

DEVICE TYPE:

============

Generally, the 3270 emulator characteristics determine the true device

type.  Valid device types are 3277, 3278, and 3279.

MAX DEVICES SUPPORTED

=====================

At release r390.obj 4.3.x.x  (or with the latest maintenance applied), the

maximum number of AWS3274 devices supported is 200.

CONFIGURING a 3274 CONTROL UNIT

===============================

To configure a 3274, go to the Configurator F2 "UPDATE SYSTEM DEVICES" menu

and enter the real address of a 3277, 3278, or 3279 device in your system I/O

configuration.   Enter a device type (although the 3270 emulator

will actually determine this) and enter the manager code for AWS3274.

Example:

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>200   >3278                         >3   >

MGR Codes: 3=AWS3274

Optionally, enter a valid /R parameter in the FN/P field (only if the

intended client supports TN3270E. See "TN3270E Support" for details).

Devices that do not use the /R parameter are assigned to sessions

sequentially, starting with the lowest address first.  Thus if you have

AWS3274 device address 200 and 201 defined, the first session connecting

will get address 200 and the next 201.  This can cause clients to be

assigned a different device address each time they disconnect and re-connect.

This can be avoided using TN3270E clients and the /R parameter.

TN3270E Support:

================

For those emulators that support TN3270E Protocol, AWS3274 can be set up to

allow them to request (and reserve) specific AWS3274 devices.  This can be

used to 'associate' specific AWS3274 devices with specific client sessions.

Once a client session is established, if it supports TN3270E protocol, along

with a device type (e.g. IBM-3278-2-E), the client can optionally send a

"device name"; this device name can be either a AWS3274 device address

(e.g. 0200) or a 'group name'.

Example: For versions of PCOMM that support TN3270E (v4.2 or greater), on

the 'Advanced Telnet3270' panel (the one where you specify the port

address), there's an additional "LU Name" field; this is where you

would specify the AWS3274 device address or groupname you want to

associate with the session).

To set up a AWS3274 device to support TN3270E clients, change the FN/P field

to one of the following values:

/R : Only TN3270E clients can request this device; the

device will be assigned to the first client that

requests this device by address (ie: the client

connects with the desired device address as the

"device name" which is ussually called the

Device ID or LU Name in the emulator definition.

/R=groupname : Only TN3270E clients can request this device;

more than one device can have the same groupname;

The device will be assigned to the first client

that asks for it by name.

AWS3274 devices with a /R parameter will NOT be assigned to a client

unless that client supports the TN3270E protocol.

Example:  You are running two VSE systems under VM.  You have some

terminal address dedicated to each VSE system, and some for CMS.  You

also want to insure there is a terminal available for the system

programmer.  In the configurator, use the /R parameter as follows:

For addresses dedicated to the first VSE system, specify "/R=VSE1"

For addresses dedicated to the 2nd VSE system, specify "/R=VSE2"

For addresses for CMS users, specify "/R=CMS"

For the address reserved for the sysprog, specify "/R=mysecret"

Now users can request connections for specific session types by coding

"vse1", "vse2", "cms" or "mysecret" in the device-id/luname field

of their emulator.  Note that case is ignored in the groupname, and that

using the /R field as a "password" is not foolproof -- no special effort

is made to hide this value or protect it during transmission.

Example:

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>600   >3278                         >3   > /R=VSE1

>601   >3278                         >3   > /R=VSE1

>700   >3278                         >3   > /R=VSE2

>701   >3278                         >3   > /R=VSE2

>800   >3278                         >3   > /R=CMS

>900   >3278                         >3   > /R

In this example, the client that wants to connect as address 0900 must

specify 0900 as the "device name" (which is called the "LU Name" on PCOMM

sessions and "Device ID" on other TN3270E emulators.)