AWS3215: LINE-MODE CONSOLE EMULATOR ON LOCAL X-WINDOW

======================================================

The 3215 sessions are typewriter/keyboard sessions.  These sessions are most

suitable for virtual machines that need a console to monitor them, but do not

require much input.  They are more suited to this activity than 3270 sessions

are because 3215 sessions automatically scroll off old data and do not require

the "MORE..." or "HOLDING" indicators that 3270 sessions do.

DEVICE TYPES

============

3215: Use device type "3215" for defining an I/O attached line mode console.

HDWR: If supported by the P/390E ESA microcode, AWS3215 can also be used as a

"Hardware Message Console" aka "Operating System Messages" aka "Operator

Message and Response Console". The human factors of this console are identical

to the I/O attached 3215, however the interface to the S/390 CP is handled

though "Read/Write Event" SCLP instructions and external interrupts rather

than by I/O instructions and I/O interrupts. Only One Operator Message Console

per R/390E system is allowed at a special device address FFFF and with device

type "HDWR".  In summary, 4 conditions are needed to use this device:

1) R/390E microcode must support "Read/Write Event" architecture

This support was released at the same time as the AWS3115 support

and will be automatically applied by the "apply4310" script used

when applying maintenance fixes for R/390E system only.

2) Device address must be FFFF

3) Device type must be HDWR

4) R/390E must be IPL'ed in ESA mode (HDWR not supported in 370 mode)

NOTE: This was added for VIF (Virtual Image Facility) and "Linux for S/390"

Operating Systems but is works with all IBM Operating Systems.

CONFIGURING a 3215

===================

To configure a 3215 session, go to the Configurator F2 "UPDATE SYSTEM DEVICES"

menu and enter the real address for a 3215 device in the Addr field, the MGR

code for AWS3215 in the Mgr field, and the title you want to display for the

session in the FN/P field.  Here is an example entry:

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>009   >3215                         >5   >System Operator

FEATURES AND CUSTOMIZING

========================

Several options make 3215 sessions easy-to-use:

1.  Save size/location

The initial size and location can be changed by modifying the resource file

/usr/lib/X11/app-defaults/Aws3215, or at anytime by using the mouse.

2.  Scrolling

The last 280 lines of input/output are saved.  The user can scroll forward and

backward through this data using the pageup/pagedown keys, or by using the

vertical slider on the right.

3.  Colors

The colors can be changed by modifying the resource file

/usr/lib/X11/app-defaults/Aws3215.

4.  Command Retrieve Keys

The UP and DOWN cursor keys are command retrieve keys.  They allow users to

move forward and backward through the last 12 commands entered.  F12

also acts as a retrieve key.

5.  Mouse Support

You can use the mouse to transfer commands or output data from the output area

to the input line.  One click retrieves the line so you can modify it.  A double

click processes the command.

6.  Pausing Output

To make VM pause so you can look at the data, press ENTER.  Press ENTER again

to resume output.

7.  Run with SET AUTOREAD OFF

If you are not interested in the details here, put SET AUTOREAD OFF in the CMS

PROFILE of any virtual machine you want to run from a 3215 session.

If you've run with a real 2741 or 3215, you know how awkward they were to use.

The user was faced with a difficult choice:  either run with AUTOREAD ON, which

prevented messages, etc. from appearing while CMS waited for a command to be

typed, or run with AUTOREAD OFF, which meant the user had to press ATTN before

entering every command.

We solve this problem by allowing you to enter a command whenever you want.

ATTN is automatically generated when you press ENTER.  CMS automatically does a

SET AUTOREAD ON for 3215/2741 sessions, and a SET AUTOREAD OFF for 3270

sessions.  To get the most effective use of your 3215 emulation, put SET

AUTOREAD OFF in your PROFILE EXEC or in your system SYSPROF EXEC.

8.  Fonts

The font can be changed by modifying the resource file

/usr/lib/X11/app-defaults/Aws3215.