## Planning for Software Installation
and Configuration

## Overview

Before you install and configure the 8235-I40 DIALs Switch software, use the worksheet below to gather important site information.

## Site Installation and Configuration Worksheet

The worksheet below lists the information you need to perform software installation and configuration on the I40 DIALs Switch.

| Category | Item | Information |
| --- | --- | --- |
| Site information | Site name |  |
|  | Site address |  |
|  | Primary contact name |  |
|  | Primary contact telephone number |  |
|  | Primary contact fax number |  |
|  | Primary contact cell telephone number |  |
|  | Primary contact email address |  |
|  | Secondary contact name |  |
|  | Secondary contact telephone number |  |
|  | Secondary contact fax number |  |
|  | Secondary contact email address |  |

| Category | Item | Information |  |  |
| --- | --- | --- | --- | --- |
| Line provisioning | Service provider contact name |  |  |  |
|  | Service provider telephone number |  |  |  |
|  | T1/E1/PRI telephone numbers | For T1 Service |  | For ISDN PRI Service |
|  | Number of bearer channels/timeslots on each line |  |  |  |
|  | Number of digits from the telephone company switch provided to the T1/E1/PRI lines |  |  |  |
|  | Analog modem telephone number for remote testing |  |  |  |
|  | Do you have Internet access? |  |  |  |
|  | Are the PRI/T1 lines installed at this time? |  |  |  |
|  | If the PRI/T1 lines are not installed what date will they be installed. |  |  |  |
| Security | Type of third-party security: SecureID, Digital Pathways, TACACS, TACACS+, Radius |  |  |  |
|  | IP address of IBM User List server |  |  |  |
|  | IP address of security server |  |  |  |
|  | UDP port of security server |  |  |  |
|  | Radius version |  |  |  |
|  | Novell NetWare Bindery server name |  |  |  |
| General configuration | Is the NetWare Bindery dial-in group defined on the server? |  |  |  |
|  | Do you have PC or UNIX management software? |  |  |  |
|  | IP address to be used |  |  |  |
|  | Subnet Mask to be used |  |  |  |
|  | Default Router on the network |  |  |  |
|  | Are you using DHCP? |  |  |  |
|  | DHCP server address |  |  |  |
|  | IPX frame type: 802.3, Ethernet II, SNAP, or 802.2 |  |  |  |
|  | For AppleTalk networks, do you want the 8235-I40 DIALs Switch set up in end node mode, conforming router mode, or seed router mode? |  |  |  |

| Category | Item | Information |
| --- | --- | --- |
| BootP/TFTP Configuration | What operating system is your BootP and TFTP server? |  |
|  | IP address of the BOOTP Server |  |
|  | Subnet mask |  |
|  | Do you currently use TFTP and BootP? |  |
|  | Is TFTP configured and working? |  |
|  | Is BootP configured and working? |  |
|  | Is the BootP server the same as the TFTP server? |  |
|  | Does the BootP server and the TFTP server reside on the same LAN segment as the 8235-I40 DIALs Switch ? |  |
|  | Do the routers on the network support UDP helpers for BootP? |  |
| Client workstations | Do you have a BRI line installed for testing? |  |
|  | What type of BRI line provisioning do you have (one or two B-channels; data, voice, or both)? |  |
|  | What protocols on the dial-in client do you use? Circle the protocols that apply. | IPX IP Netbeui/LLC ARA 1.0 ARA 2.0 |
|  | Client workstation OS version |  |
|  | ARA or MACPPP/MAC SLIP version |  |

# Ordering Lines

Use the information in this section when you call your service provider to order lines for a T1 service, an ISDN PRI service, for your 8235-I40 DIALs Switch.

## What Information Does My Service Provider Need From Me?

The table below contains information that your service provider needs from you to provision the lines for your 8235-I40 DIALs Switch.

| Needed Information | Available Options |  |
| --- | --- | --- |
| Number of lines | Your required number of lines. |  |
| Service type | Your required network service or services for the lines. |  |
| Telephone numbers | Your required number of telephone numbers for the lines. |  |
| Hunt groups | Your required hunt groups. Recommended option: IBM recommends that you use a uniform (random) call distribution hunt group. |  |
|  | For T1/ PRI | For E1/PRI |
| Framing type | No framing T1 with Super Frame (SF) (for a D4 channel bank) T1 with Extended Super Frame (ESF) T1 with Extended Super Frame (ESF) SLC96 T1 with Extended Super Frame (ESF) ZBTSI Recommended option: IBM recommends T1 with Extended Super Frame (ESF). | No framing E1 with CRC-4 multiframing with the Si bit set for Far End Block Error (FEBE) checking E1 with CRC-4 multiframing and the Si bit set to 1 E1 with basic framing without CRC-4 and the Si bit set to 1 Recommended option: IBM recommends CRC-4 Multiframing with the Si bit set for Far End Block Error (FEBE) checking. |

| Needed Information | Available Options |  |
| --- | --- | --- |
|  | For T1/PRI | For E1/PRI |
| Coding | Alternate Mark Inversion (AMI) Jammed Bit Zero Suppression (JBZS) Zero Byte Time Slot Interchange (ZBTSI) Bipolar 7 Zero Substitution (B7ZS) Bipolar 8 Zero Substitution (B8ZS) Recommended option: IBM recommends B8ZS for T1 service. You must use B8ZS for ISDN PRI service. | Alternate Mark Inversion (AMI) Jammed Bit Zero Suppression (JBZS) High Density Bipolar 3 (HDB3) Recommended option: IBM recommends HDB3. |
| Dial type | Dual Tone Multi-Frequency (DTMF) Pulse Dial | Not applicable |

## What Information Do I Need From My Service Provider?

The table below contains the information that you need from your service provider to set up your 8235-I40 DIALs Switch. When your I40 arrives, use this information during software configuration.

| Needed Information | Available Options |  |
| --- | --- | --- |
| Telephone numbers | The required telephone numbers for your lines. |  |
| Telephone number digits | The number of telephone number digits transmitted from your service provider’s switch to the DIALs Switch. |  |
| Circuit ID numbers | The circuit ID number assigned by your service provider for each line you order. Write these numbers down for troubleshooting purposes. |  |
|  | For T1/PRI | For E1/PRI |
| Line build-out (LBO) | Line build-out enables the DIALs Switch to compensate for the loss in decibels on the line connecting the DIALs Switch to the line from your service provider. If you have an internal CSU, your service provider needs to specify the loss in decibels based on the distance to the first repeater. If you have an external CSU, such as a DSX-1 interface, your service provider needs to specify the length in feet to the DSX-1 cross-connect point. | Not applicable |
| Switch type | AT&T 4ESS AT&T 5ESS Northern Telecom DMS-100 Northern Telecom DMS-250 Ericsson MD-110 for T1 Ericsson MD-110 for E1 Siemens EWSD NTT Other switch types that conform to the CCITT standards |  |

| Needed Information | Available Options |  |
| --- | --- | --- |
|  | Switch Type | Variant |
| Switch type variant | AT&T 4ESS | AT&T Custom Bellcore National ISDN-2 CCITT Q.933 CCITT T.123 |
|  | AT&T 5ESS | AT&T Custom Bellcore National ISDN-2 CCITT Q.933 CCITT T.123 |
|  | Northern Telecom DMS-100 | Northern Telecom Custom Bellcore National ISDN-2 |
|  | Northern Telecom DMS-250 | Northern Telecom Custom Bellcore National ISDN-2 |
|  | Ericsson MD-110 for T1 | Bellcore National ISDN-2 General CCITT CCITT Q.933 CCITT T.123 |
|  | Ericsson MD-110 for E1 | Net-5 1TR6 PRI VN3 General CCITT CCITT Q.933 CCITT T.123 |
|  | Siemens EWSD | Net-5 1TR6 PRI VN3 General CCITT CCITT Q.933 CCITT T.123 |
|  | NTT | JATE General CCITT |
|  | Other | Net-5 1TR6 PRI VN3 TS014 General CCITT CCITT Q.933 CCITT T.123 |

| Needed Information | Available Options |  |
| --- | --- | --- |
|  | For T1/PRI | For E1/PRI |
| Signaling type | No signaling Note: Choose this option only if you have a digital leased line. ISDN Primary Rate Interface (PRI) Robbed Bit Wink Start Robbed Bit Immediate Start Robbed Bit Ground Start Robbed Bit Loop Start Robbed Bit Delay Dial Recommended option: IBM recommends that you ask your service provider for the signaling type available in your area. | No signaling Note: Choose this option only if you have a digital leased line. ISDN Primary Rate Interface (PRI) Recommended option: You must use ISDN PRI signaling. |
| Line termination | Not applicable | The resistance on the E1 line for termination can be: 75 Ohms for coaxial cable 120 Ohms for twisted pair |

Note

If the customer has to reschedule the actual install at the customers insistence, the ability to re-schedule an early return to site will depend on the engineer’s commitment to other customers.

Declaration

To the best of my knowledge all the information given on this pre-installation checklist is accurate.

Printed Name............................................................

Signed Name.............................................................

Date..............\...........................\.....................