IBM INSTALLATION INSTRUCTIONS

8260 NWAYS(*) MULTIPROTOCOL SWITCHING HUB

INSTALLATION INSTRUCTIONS

FOR

ETHERNET FLEXIBLE CONCENTRATION (ETHERFLEX) MODULE

V1.01 MICROCODE UPGRADE FIELD B/M

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     1 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     2 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

CONTENTS

________

TRADEMARKS AND SERVICE MARKS   . . . . . . . . . . . . . . . . . . . . . .  5

BEFORE INSTALLATION  . . . . . . . . . . . . . . . . . . . . . . . . . . .  7

Purpose  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  7

Machines Affected  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  7

Requirements   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  7

For Out-of-Band download method.   . . . . . . . . . . . . . . . . . . .  7

For Inband download method.  . . . . . . . . . . . . . . . . . . . . . .  8

INSTALLATION   . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   11

Distribution Diskette  . . . . . . . . . . . . . . . . . . . . . . . . .   11

Distribution Diskette for EtherFlex Module   . . . . . . . . . . . . .   11

Safety   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   11

References   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   11

UPGRADING THE ETHERFLEX MODULE OPERATIONAL CODE  . . . . . . . . . . . .   13

Download Out-of-Band   . . . . . . . . . . . . . . . . . . . . . . . . .   13

General Scenario   . . . . . . . . . . . . . . . . . . . . . . . . . .   13

Scenario for UCDK Users  . . . . . . . . . . . . . . . . . . . . . . .   13

Download Inband  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   14

AFTER INSTALLATION   . . . . . . . . . . . . . . . . . . . . . . . . . .   15

Labelling  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   15

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     3 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     4 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

TRADEMARKS AND SERVICE MARKS

____________________________

The following terms, denoted by an asterisk (*), used in this Installation

Instructions, are trademarks or service marks of the IBM Corporation in the

United States or other countries.

OS/2                          AIX

IBM                           Nways

The following terms, denoted by a double asterisk (**), used in this Instal-

lation Instructions, are trademarks of other companies:

ProComm             Data Storm Technologies Corporation

Windows             Microsoft Corporation

Softerm Custom      Softronics, Inc.

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     5 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     6 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

BEFORE INSTALLATION

___________________

PURPOSE

_______

The purpose of this document is to provide the instructions to update the

software microcode (flash EEPROM) of the 8260 Nways(*) Multiprotocol

Switching Hub EtherFlex Module using material provided in the field B/M P/N

29H4289.

MACHINES AFFECTED

_________________

This installation instruction, part number P/N 29H4289 EC level E28017,

applies to 8260 Nways(*) Multiprotocol Switching Hub feature code 1004,

REQUIREMENTS

____________

FOR OUT-OF-BAND DOWNLOAD METHOD.

Updating software microcode using the download Out-of-Band method requires a

workstation with emulated ASCII terminal and the support of XMODEM protocol

for file transfer.  The workstation is connected to the console port of the

management module either locally using a null modem, or remotely.

ASCII terminal emulation and XMODEM protocol are offered by many products

such as:

o   DOS platform

-   FTTERM

-   ProComm Plus for DOS

-   ProComm(**) for DOS

NOTE:  This product is the one IBM provides in its Universal Code

Download Kit (UCDK) part number 80G3150. Refer to Installation

____________

Instructions for IBM Universal Code Download Kit, part number

________________________________________________

80G3152.

o   Windows(**) Platform

-   ProComm Plus for Windows

-   Windows Terminal function

o   OS/2(*) Platform

-   Softerm Custom(**)

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     7 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

o   AIX(*) Platform

-   Asynchronous Terminal Emulation (ATE)

The above list is not exhaustive and whatever the product one may use, the

emulated ASCII terminal line settings should be set to the factory defaults

for the first connection:

o   Baud rate: 9600 bps

o   Data bits: 8

o   Parity: None

o   Stop bits: 1

The terminal parameters should be setup as follows:

o   Terminal emulation: VT100 (preferred)

o   Duplex: Full

o   Flow Control: None

FOR INBAND DOWNLOAD METHOD.

The DMM provides an inband download feature that allows you to update your

DMM or modules operational or boot EPROMs using TFTP (Trivial File Transfer

Protocol).

You must check the following prerequisites prior to initializing the down-

load.

o   Have a TFTP server on the network to perform inband downloads.

o   Verify IP connectivity from the TFTP server to the DMM

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     8 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

+---  IMPORTANT ------------------------------------------------------------+

!                                                                           !

! o   If your TFTP server is running in a DOS environment                   !

!      Check that the current directory of your PC is the one containing    !

!     the binary file you want to download when you start the server.       !

!                                                                           !

! o   If your TFTP server is running in a OS/2 environment                  !

!      If the TFTP server has been started in foreground session check that !

!     you gave access restricted to the directory where the binary file     !

!     resides (in opposition with the TFTP server started by INETD super    !

!     server).                                                              !

!                                                                           !

! o   If your TFTP server is running in a AIX environment                   !

!      ensure the binary file may be read by everyone . If a file           !

!     "tftpaccess.ctl" exists in /etc be sure that at least one "allow"     !

!     line is provided to allow access to the path where the file resides.  !

!                                                                           !

! o   For more on that see below the details in the section Inband Down-    !

!     load.                                                                 !

!                                                                           !

+---------------------------------------------------------------------------+

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                     9 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    10 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

INSTALLATION

____________

The installation of new version of software microcode requires the following

tasks to be performed.

o   Use the distribution diskette to copy the files on the hard disk on the

workstation (DOS or AIX).

o   Upgrade the module using one of the download methods available.

DISTRIBUTION DISKETTE

_____________________

There is one diskette provided (P/N 29H4294) with the Field B/M.

DISTRIBUTION DISKETTE FOR ETHERFLEX MODULE

The diskette, part number P/N 29H4294, contains:

o   The operational microcode (ETHFV101.OP) of the EtherFlex Module.

o   A ProComm DOS command file (ETHFOPER.CMD).

o   The boot microcode (ETHFV100. BT) of the EtherFlex Module.

o   A ProComm DOS command file (ETHFBOOT.CMD).

o   These Instructions in ASCII format INSTALL.DOC

SAFETY

______

Since this update is to be performed with machine powered on, review the

SAFETY NOTICES delivered with the shipping group.

REFERENCES

__________

Installation Instructions for IBM Universal Code Download Kit, part

_____________________________________________________________

number 80G3152.

8260 DMM Commands Guide, form number SA33-0275-03

_______________________

8260 DMM User's Guide, form number SA33-0259-04

_____________________

ProComm Reference Manual.

________________________

8260 Ethernet Flexible Concentration Module Release Note PN P/N 29H4261.

________________________________________________________

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    11 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    12 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

UPGRADING THE ETHERFLEX MODULE OPERATIONAL CODE

_______________________________________________

DOWNLOAD OUT-OF-BAND

____________________

GENERAL SCENARIO

1.  Establish the connection between your Workstation and the management

module.  Refer to the appropriate Installation and Operations Guide, or

_________________________________

User's Guide.

____________

2.  Start the ASCII Terminal Emulator

3.  Start the download out-of-band procedure that pertains to the Management

module the terminal is connected to.

4.  When the management module is expecting the file to be sent, start the

send file procedure .

SCENARIO FOR UCDK USERS

1.  Establish the connection between your DOS station and the management

module.  Refer to the appropriate Installation and Operations Guide, or

_________________________________

User's Guide.

____________

2.  Start the ProComm software

3.  After the terminal screen is displayed, press the "Alt-B" keys to specify

the diskette unit where the distribution diskette will be mounted (in the

"PATH" input field).

4.  Insert the IBM Distribution Diskette P/N 29H4294

5.  Start the download out-of-band procedure that pertains to the Management

module the terminal is connected to i.e.

o   Enter the command DOWNLOAD OUT_OF_BAND MODULE <SLOT.SUBSLOT> OPERA-

TIONAL to initiate the download of operational code to the EtherFlex

Module specified by slot and subslot information.

o   When the management module is expecting the file to be sent (you may

see the "Please initiate file transfer" prompt message):

a.  Press the "Alt-F5" key

b.  YOU MUST SELECT THE ETHFOPER.CMD FILE  in order to initiate the

download of the Oper code version V1.01 on the EtherFlex Module

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    13 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

c.  File transfer progress will be indicated on the terminal until

completion.

DOWNLOAD INBAND

_______________

The DMM provides an inband download feature that allows you to update 8260

modules using TFTP (Trivial File Transfer Protocol).

Use the following procedure to perform an inband download .

1.  Verify that the TFTP server is able to PING the DMM.

2.  Be sure to have running TFTP Daemon on your server.

3.  Copy the operational code files on a directory of your TFTP server.

4.  Configure the TFTP parameters in the DMM by issuing the following manage-

ment commands as shown:

a.  > set tftp file_name <name of the file> (you may need a path)

b.  > set tftp server_ip_address <ip address of your TFTP server>

c.  > clear tftp results

d.  > save tftp

NOTE:

This can be accomplished either by TELNETing to the DMM or by

directly attaching to the DMM front panel serial port via and

RS-232 connection.

5.  Issue the SHOW TFTP command to verify the TFTP parameters previously set.

> SHOW TFTP

6.  Issue the appropriate INBAND DOWNLOAD command as shown below to begin the

download.

DOWNLOAD INBAND MODULE <SLOT.SUBSLOT> OPERATIONAL to initiate the down-

load of operational code to the module specified by slot and subslot

information.

7.  Once the download process begins, do no press any keys or interfere with

the process . You will have messages prompted to you .

8.  Verify that the code is now the updated version with

SHOW DEVICE command.

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    14 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

AFTER INSTALLATION

__________________

LABELLING

_________

After the upgrade of the microcode , you may change from current P/Ns to the

new P/Ns as follow:

o   New Part Number for EtherFlex Module is P/N 29H4286

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    15 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------

IBM INSTALLATION INSTRUCTIONS

END OF DOCUMENT

-----------------------------------------------------------------------------

8260                       P/N 29H4289     E28017                    16 of 16

Install Inst                                                     June 7, 1996

-----------------------------------------------------------------------------