# Release Notes

3.22 		1-14-99

Here is the information on the changes between 3.22 and 3.10,

The following problems are corrected:

1. Fixes memory Leak involving SNMP request.

2. Fixes problem with STAP (Spanning Tree Protocol) not working correctly with VLTs (Virtual Lane Trunk).

3. VLT's count all frames longer than 1514 bytes as errors, due to frames on a VLT having an additional 4 bytes of VLT data.

4. Receive Errors on ATM exceeding 100%.

5.  When STAP Blocks on a port, addresses stored in the switch

Database against that port is not always deleted.

6. Downloading the ATM software from AIX TFTP server fails -

illegal operation'

7. Unit responding to ARP requests on VLans other than VLAN 1.

8. ATM equipped units may reset with the following fault

codes : 0080085B, 008085D, 00802102,00802103