IBM Corporation

IBM Nways RouteSwitch Release Notes

Version 3.4.7

- Rev. M

September 15, 1999

Nways RouteSwitch Release Notes

Version 3.4.7

- IBM Networking Home Page:

- http://www.networking.ibm.com/support

PTF		APAR

8274 ROUTESWITCH					NP01056 	NA05687

8274 ADVANCED ROUTING				NP01057 	NA05688

8274 ROUTECELL					NP01058 	NA05689

8274 LANE SERVICES MODULE				NP01059 	NA05690

8277 ROUTESWITCH					NP01060		NA05691

8277 ADVANCED ROUTING				NP01061		NA05692

8277 LANE SERVICES MODULE				NP01062		NA05693

8274 GRS ROUTESWITCH					NP01063 	NA05694

8274 GRS ADVANCED ROUTING				NP01064 	NA05695

8274 GRS LANE SERVICES MODULE			NP01065 	NA05696

IMPORTANT INFORMATION FOR IBM CUSTOMERS:

Before reading the remainder of this document, please read the following:

Notice: This document may contain information about features, models, and/or functions, not currently offered for sale by IBM.  The inclusion of this information should NOT be interpreted as indicative that these features/models/functions currently exist in any IBM product or that IBM plans to offer them in the future.

Table of Contents

1	Introduction	8

2	Technical Support Guidelines	8

3	Release 3.4 IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Hardware and Feature Set Supported	8

3.1	New Hardware in Release 3.4.7	8

3.1.1	IBM 8274 Model GRS Nways LAN RouteSwitch:	8

3.2	New Software Major Features	8

4	Unsupported Features in this release (3.4.7)	8

4.1	Unsupported Features by platform	8

4.2	8273 RouteSwitch	9

4.3	SNMP Agent	9

5	General Instructions and Notes for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch	9

5.1	General	9

5.2	Start-Up	9

5.3	ATM	10

5.4	Autoencapsulation	11

5.5	Nways Route Tracker Manager VLANs	11

5.6	Bridging	11

5.7	Commands	11

5.8	FCSM Switching Module	11

5.9	Frame Relay	12

5.10	Gigabit Ethernet Switching Module (GSX)	12

5.11	Group Mobility	12

5.12	MPM Redundancy	12

5.13	IBM 8274 Model GRS	12

5.14	Routing	12

5.15	System Requirements	12

5.16	Token Ring	13

5.17	RouteVision	13

6	Additional Feature Information for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch,  and IBM 8277 Nways Ethernet RouteSwitch	14

6.1	ATM LANE	14

6.1.1	LAN Emulated Client Start–up and Back-off Timers	14

6.1.2	LANE Service Module	15

6.2	ATM Memory	15

6.3	Content Addressable Memory (CAM): Advanced Uses (IBM 8274 Nways LAN RouteSwitch Only)	15

6.4	CSM/FCSMI Hot Swap Instructions	16

6.5	CSM-PNNI MTU Size Configuration Guidelines	16

6.6	Duplicate MAC Support on IBM 8274 Model GRS Nways LAN RouteSwitch and IBM 8274 Nways LAN RouteSwitch	17

6.7	E1 Framing types supported and how to Configure 31 Timeslots on a WAN E1 Port	17

6.8	ESI for an ASM Port	17

6.9	Optimized Ports on ESM-100C-12, ESM-100F-8, ESM-C-32, and IBM 8277 Nways Ethernet RouteSwitchs	18

6.10	Functional Address Support for IBM Spanning Tree on IBM 8274 Model GRS Nways LAN RouteSwitch and IBM 8274 Nways LAN RouteSwitch	18

6.11	Group Mobility	19

6.11.1	GMAPLS Command	19

6.11.2	Upgrading to Group Mobility II	19

6.12	Hardware Tagging	19

6.12.1	Support:	19

6.12.2	User Interface:	19

6.12.3	Limitations:	19

6.13	Hot Swapping ASM Modules	20

6.14	HRE Notes	20

6.15	HRE-X with IPX Routing	20

6.16	IPX Performance Updates	20

6.16.1	IPX Extended RIP and SAP Packets	20

6.16.2	IPX Default Route	20

6.17	MPM Upgrade	21

6.18	MPX on-board Ethernet Ports	21

6.18.1	Configuring an IP Address on the MPX on-board Ethernet Module	21

6.19	IBM 8277 Nways Ethernet RouteSwitch and Mammoth based ESM/ESX Port Partitioning	21

6.20	IBM 8277 Nways Ethernet RouteSwitch 3032 and IBM 8277 Nways Ethernet RouteSwitch 3032E	21

6.21	Simm Flash Module (IBM 8274 Nways LAN RouteSwitch)	21

6.22	PNNI - Configuring Node-Specific Parameters	22

6.23	PNNI and Redundant MPM's	22

6.24	Port Mapping	22

6.25	Static Route Configuration and GateD	22

7	FLASH Requirements for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch	23

8	Multiple Image Requirements	25

9	ATM Services	26

10	Example of IBM 8274 Model GRS Nways LAN RouteSwitch FLASH Requirements for Release 3.4.7	27

11	Examples of IBM 8274 Nways LAN RouteSwitch FLASH Requirements for Release 3.4.7	28

12	Examples of IBM 8277 Nways Ethernet RouteSwitch FLASH Requirements for Release 3.4.7	29

13	Release 3.4.7 IBM 8277 Nways Ethernet RouteSwitch Images Required	29

14	IBM Operating System (IOS) Version 3.4.5 Problem Resolutions	30

14.1	IOS Problems fixed in this release	30

14.2	IOS Problems no longer an issue in this release	31

15	Known IBM 8274 Model GRS Nways LAN RouteSwtich, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Problems / 
Limitations	32

15.1	Known Advanced Routing, GateD & Routing Problems/Limitations	32

15.2	Known ATM Problems/Limitations	34

15.3	Known Authenticated Services Problems/Limitations	35

15.3.1	Client	35

15.3.2	Switch	35

15.4	Known Bridging Problems/Limitations	35

15.5	Known CSM/FCSM Problems/Limitations	36

15.6	Known Duplicated MAC/Source Learning Problems/Limitations	37

15.7	Known Ethernet (ESM/ESX) Problems/Limitations	37

15.8	Known FDDI Problems/Limitations	38

15.9	Known Firewall Problems/Limitations	38

15.9.1	Firewall Server	38

15.9.2	Firewall Switch	38

15.10	Known Gigabit Ethernet (GSX) Problems/Limitations	38

15.11	Known Group Mobility Problems/Limitations	38

15.12	Known Hot Swap Problems/Limitations	39

15.13	Known HRE Problems/Limitations	39

15.14	Known LSM Problems/Limitations	39

15.15	Known Mammoth Problems/Limitations	39

15.16	Known MPM Redundancy Problems/Limitations	40

15.17	Known MPX Problems/Limitations	40

15.18	Known IBM 8277 Nways Ethernet RouteSwitch Problems/Limitations	40

15.19	Known PNNI Problems/Limitations	41

15.20	Known Port Manager Problems/Limitations	42

15.21	Known Port Mirroring Problems/Limitations	42

15.22	Known Port Monitoring Problems/Limitations	42

15.23	Known SNMP Problems/Limitations	42

15.24	Known System Problems/Limitations	44

15.25	Known Token Ring (TSM/TSX) Problems/Limitations	44

15.26	Known Translation Problems/Limitations	45

15.27	Known User Interface and General Problems/Limitations	45

15.28	Known Wide Area Switching Modules Problems/Limitations	46

16	IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrices	47

16.1	IBM 8274 Model GRS Nways LAN RouteSwitch Hardware/Software Release Compatibility Matrix	47

16.2	IBM 8274 Nways LAN RouteSwitch Hardware/Software Release Compatibility Matrix	48

16.3	IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrix	55

17	Existing Software & Hardware Features	57

17.1	Software Feature Matrix  (X = Supported)	57

17.2	Existing and Supported Hardware	59

17.2.1	IBM 8274 Model GRS	59

17.2.2	IBM 8274 Nways LAN RouteSwitch	59

17.2.3	IBM 8277 Nways Ethernet RouteSwitch	61

17.3	Appendix A - Authenticated VLANs	62

17.3.1	Server	62

17.3.2	AMC Server Setup	62

17.3.3	Troubleshooting	62

17.3.4	Helpful Hints and other Notable Information	63

Revision History

| Release | Revision | Date | Description of Changes |
| --- | --- | --- | --- |
| 3.4.1 | A | 10/22/98 | Early Availability Release - Supports IBM 8274 Model GRS Nways LAN RouteSwitch Only |
| 3.4.1 | B | 11/6/98 | Early Availability Release - Updated Known/Fixed Problems in Release 3.2.6 |
| 3.4.2 | C | 12/10/98 | General Availability Release – Support for OSR, IBM 8274 Nways LAN RouteSwitch, IBM 8277 Nways Ethernet RouteSwitch. No Support for 8273 RouteSwitch. |
| 3.4.2 | D | 12/15/98 | Documentation Change Only |
| 3.4.3 | E | 2/26/99 | Internal Release Only |
| 3.4.4 | F | 3/11/99 | Maintenance Release - Fix Outstanding Problem Reports |
| 3.4.4 | G | 3/17/99 | Documentation Change Only |
| 3.4.4 | H | 3/23/99 | Documentation Change Only |
| 3.4.5 | J | 4/8/99 | Maintenance Release - Fix Outstanding Problem Reports |
| 3.4.5 | J1 | 7/15/99 | Documentation Change Only |
| 3.4.6 | K | 6/18/99 | Beta Release - New Feature: IPMS |
| 3.4.7 | L | 9/2/99 | Maintenance Release |
| 3.4.7 | M | 9/8/99 | Documentation Change Only |

# Introduction

These release notes cover the basic feature set supported, known limitations, bugs and known work-arounds for IBM Operating System Version 3.4.7 software.  Please read this document in its entirety as it contains important operational aspects that may impact your network.

# Technical Support Guidelines

This section provides contacts for help if you have questions about the IBM RouteSwitch products or if a RouteSwitch product is not working correctly.  It also explains how to access the IBM electronic sites to obtain the latest version of microcode and release notes.

- Electronic Support
This section explains how to access the IBM electronic site to obtain the latest version of microcode, drivers, and software by using the Internet World Wide Web or FTP.

WWW:	 http://www.networking.ibm.com/ 
This is the IBM Networking home page.  From here, you can access product announcements, publications, and other information regarding hardware and software updates, and a technical support information database.  The direct path to the support area is:

http://www.networking.ibm.com/support

Voice Support

IBM Network Hardware Support: 1-800-IBM-SERV.  Follow the menu prompts for Network Hardware.

# Release 3.4 IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Hardware and Feature Set Supported

The following hardware and software features are supported subject to the restrictions in this note:

Notice: This document may contain information about features, models, and/or functions, not currently offered for sale by IBM.  The inclusion of this information should NOT be interpreted as indicative that these features/models/functions currently exist in any IBM product or that IBM plans to offer them in the future.

## New Hardware in Release 3.4.7

### IBM 8274 Model GRS Nways LAN RouteSwitch:

- ESX-FM-24	24-port Ethernet Switching Module with MT-RJ connectors. (Multimode)

- ESX-FS-24	24-port Ethernet Switching Module with MT-RJ connectors. (Single mode)

## New Software Major Features

The following matrix lists only new software features.  Many of these features have been previously released for IBM 8274 Nways LAN RouteSwitch and IBM 8277 Nways Ethernet RouteSwitch.  See Section 17.1 for Existing Software Features.

|  | IBM 8274 Model GRS Nways LAN RouteSwitch | IBM 8274 Nways LAN RouteSwitch | IBM 8277 Nways Ethernet RouteSwitch |
| --- | --- | --- | --- |
| 802.1Q on 8277 Model 424 (8277-42G) and RSESM2 |  |  | X |

| # Unsupported Features in this release (3.4.7) ## Unsupported Features by platform "X" denotes features not supported in platform indicated. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Feature | IBM 8274 Model GRS Nways LAN RouteSwitch | IBM 8274 Nways LAN RouteSwitch | IBM 8277 Nways Ethernet RouteSwitch | Comments |
| 802.1Q is not supported over RouteChannel | X | X | X | Support is not planned. |
| Autonegotiation on IBM 8277 Nways Ethernet RouteSwitch gigabit uplink |  |  | X | Support is not planned. |
| Data direct VCs over Group Mobility and Token Ring LECs |  | X |  | Group Mobility is not compatible with Token Ring Lane and Source Routing. |
| Group Mobility over Token Ring Lane | X | X |  | Group Mobility is not compatible with Token Ring Lane |
| HotSwap is not supported on ASM |  | X |  | Support is planned for Release 4.1. |
| HotSwap is not supported on CSM/FCSM-II |  | X |  | Support is planned for Release 4.1. |
| IPX routing across the TSX 16M in full duplex mode. | X |  |  | Support is planned. |
| IPX routing in source-routed networks with or without HRE-X installed. | X |  |  | Support is planned for Release 4.1. |
| Non-Ethernet Router Port Encapsulation |  |  | X | The RS-1032, RS-4016 and RS-4024 Ethernet switches do not support Token Ring, Source Routing or FDDI encapsulation on the router port. |
| RouteChannel |  | X |  | Support is not planned. |
| RS-1032 & RS-4016 |  |  | X | Support is not planned. |
| Port Mirroring on 802.1q ports and Gigabit ports. | X | X | X | Support is not planned. |
| Port Monitoring on Gigabit ports. | X | X | X | Support is not planned. |
| Statistics on 802.1q ports. |  |  | X | Support is planned. |
| FCSM-II and HRE installed together in an 8274 Switch running token ring traffic. |  | X |  | Alternative is FCSM-I with HRE. |

| ## 8273 RouteSwitch |  |
| --- | --- |
| 8273 RouteSwitch is no longer supported effective Release 3.4.2. Please refer to Release 3.2.8. |  |
| ## SNMP Agent |  |
|  | - 802.1q Tagging |
|  | - ATM Traffic Shaping |
|  | - CD6 Port Modes |
|  | - Differentiation of 10BaseFL adapter Boards (ESM-AB-FL, ESM-AB-FL-S, ESM-AB-FL-SH) on ESM-U modules. |
|  | - Ethernet 10/100 (Speed, Modes) |
|  | - LANE QoS |
|  | - LSM |
|  | - Port Mapping |
|  | - Port Monitoring |
|  | - Token Ring Fiber |
|  | - VP Tunneling |

# General Instructions and Notes for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch

## General - The very first packet received by a switch with RouteChannel enabled is sent out on all RouteChannel ports. - Each NI is shipped with a fixed number of MAC addresses. One MAC address is used for each LAN port or ATM service. When all MAC Addresses are used, the following error message is displayed “pmPortCreate(): couldn't allocate a mac address". - Statistics on 802.1q ports are not recorded on IBM 8277 Nways Ethernet RouteSwitchs - Due to the nature of RouteTracker VLANs and the multicast router code, we cannot support multicast routing with Nways Route Tracker Manager VLANs. Since a port can be located in multiple VLANs, it is impossible for the multicast router to be certain of the source network of the traffic stream. This restriction will be lifted with the release of IP Multicast Switching which is currently scheduled for release 4.1.

## Start-Up

- If you are upgrading from any IBM 8274 Nways LAN RouteSwitch software to Version 3.4.7, download the 3.4.7 software and reboot the IBM 8274 Nways LAN RouteSwitch. (Refer to Chapter 6 of the IBM 8274 Nways LAN RouteSwitch User’s Guide for details on software downloading and configuration.)

- To perform an IBM 8274 Nways LAN RouteSwitch (version 3.4.7) installation with floppy diskette(s), which requires a PC, follow these instructions: Insert the first floppy into the PC floppy drive, CD to the directory where software installation is to occur and type X:install (X = the drive letter for installation). Instructions are provided for the remainder of the installation. Warning: File names handled by Windows95 32-bit application programs change the first letter to uppercase, which will not be recognized by IBM 8274 Nways LAN RouteSwitch operating system. All filenames must be lowercase. - Use ProComm 3.0 or any program that makes the target file names in all lowercases. - If using other communication programs, make sure to rename all file names with lowercases before downloading them.

- IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Passwords: The following default users are defined: diag, admin, user. The default password for all users is "switch".

- Upon initial start-up (or after removal of mpm.cnf file) of a system with large numbers of physical ports, primarily the 9 slot chassis, the message: “File system compaction in progress…..” may be seen. This is not a problem, unless it repeats indefinitely. This message may be seen from time to time if network management functions are occurring. Excessive repetitions may indicate a problem and IBM Technical support should be contacted.

- Can not use NetWare login with authentication using XVSS client.

- To configure more than the default of 32 virtual router ports, the user should add the following command to his mpm.cmd file after the cmInit command: numVirtualRouterPorts=N where N stands for the number of Virtual Router Ports. Please note that the virtual router ports come from the same pool of ports as all other virtual ports (bridge, trunk, Lane, etc.). This means that the more virtual router ports he uses the fewer virtual ports that will be available. There are a total of 254 virtual ports available.

- When downloading files using ftp, FAT filenames are changed to uppercase which makes files unreadable. The user must change filenames to lowercase. The following versions do not display this problem: WFWG of ftp version 32b, ChameleonNFS, and SUN’s PCNFS.

- Use of the “break” key on a terminal/PC will change the baud rate of the serial port connected to the IBM 8274 Nways LAN RouteSwitch. The baud rate will cycle to the next highest baud rate. When it gets to 38,400, it will cycle back to 1200 baud. The cycle is as follows: 9600  19200  38400  1200  9600.

- All parameters added to the configuration file MUST be placed after the line containing cmInit, and the switch must then be rebooted in order for the changes to take effect unless explicitly specified otherwise.

- Caution: DO NOT power down the IBM 8274 Nways LAN RouteSwitch during boot-up process. Wait for the Login Prompt before powering down. If the Login prompt does not appear after approx. 6 minutes, then power down and restart.

- Caution: DO NOT power down during system file compaction. "File System Compaction in progress" is displayed on the console when compaction is in progress. Powering down during file compaction will cause corruption of flash memory.

## ATM

- The ATM Forum IISP Standard does not support a dynamic routing protocol for ATM NSAP addresses. Consequently, when connecting CSMs together via IISP in a LANE environment, the path to the LANE well known address (wka = 47.00.79.00.00.00.00.00.00.00.00.00.00.00.A0.3E.00.00.01.00) needs to be statically configured on all CSMs not directly connected to the IBM8210 (or any LANE server). The CSMs which have the directly connected server will learn the correct path via the ILMI address registration process. You may abbreviate this to a shorter prefix and if a LANE Server becomes available, PNNI will favor that over the shorter prefix. Additionally, addresses required for all ATM server connectivity must be statically configured. This includes any LES/BUS requirements. The prpadd and pradd commands are used to add these routes. Note: PNNI does not require that these addresses be statically added.

- To ensure proper operation, an ASM port should be connected to CSM ports and not back to back with another ASM port. Back to Back connection of ASMs is supported only when PVC services are used on a physical port configured to be in PVC mode. In otherwords, a physical port comes up as SVC by default. If the user wants to connect ASMs back to back, it is the user's responsibility to use the map command and change the physical port to PVC type.

- This version of software supports ATM SVCs on up to 10 ASM ports per switch. Up to 16 LANE LECs are also supported per port and a maximum of 64 LECs per IBM 8274 Nways LAN RouteSwitch has been tested. Only one service (either ELAN or PTOP) is supported per group on an IBM 8277 Nways Ethernet RouteSwitch ATM uplink.

- When changing your UNI port to PNNI or IISP and vice versa, you must change PNNI port to UNI and reboot the switch to have the changes become effective.

## Autoencapsulation

- In order to use Autoencapsulation feature, the user must create a dummy Nways Route Tracker ManagerVLAN.

- Autoencapsulation will not work properly if the IPX server is down when the IPX client boots up.

## Nways Route Tracker Manager VLANs

- When upgrading from version 2.0 or 2.1 software to versions 3.1 or higher with Nways Route Tracker Manager VLANs configured with router ports, user must first upgrade the software, remove those Nways Route Tracker Manager VLANs, reboot the switch and reconfigure those Nways Route Tracker Manager VLANs.

- When a destination node is moved to a different location under Nways Route Tracker Manager VLAN, the source does not learn the new location until the destination node sends a packet.

- SNA traffic on Token Ring and Nways Route Tracker Manager VLANs: If the user wants to configure Nways Route Tracker Manager VLANs for SNA traffic based on DSAP/SSAP protocol rules, it is recommended that the user also configures port rules for ports that connect the IBM 8274 Nways LAN RouteSwitch to the server. The reason being that the servers normally will not generate traffic unless a client talks to it. But before the client initiates the SNA session, it sends out the “Duplicate MAC test” frames which carry the SNA DSAP/SSAP values. As a result, the client gets assigned to the SNA VLAN and subsequent SNA traffic will not go out on the server port as that port is not still part of the SNA VLAN.

## Bridging

- If a port is set to bridge mode, and a PC is directly connected to the IBM 8274 Nways LAN RouteSwitch, the IBM 8274 Nways LAN RouteSwitch Spanning Tree will reconfigure when installed or rebooted (this is generally not a problem). However if a network layer protocol is started at boot time (via the autoexec.bat) of the PC and is dependent upon a connection (such as an IPX connection), then the network layer process will not be able to start until the Spanning Tree process has completed. In the event that the IPX client is unable to attach to its server the following steps may be used to solve the problem: - If only 1 client is attached, the port type can be changed to optimize device switching using the modvp command. This configuration will turn off Spanning Tree. - If the system constraints allow, the Spanning Tree forwarding timer can be changed to a lower value. This should not be done without careful consideration for the network needs of Spanning Tree. - If necessary, either the netx or VLM commands can be re-entered. In some versions of NETX, the driver is loaded regardless of whether a connection is made. When this occurs, the driver must be unloaded then restarted. NETX version 3.32 and VLM version 1.20 were used in the IBM testing and do not require an unload between commands.

- A modification to the aging timer for transparent bridging will only take place on newly learned entries. Any entries learned on the previous timer will time-out on that value. Once they time-out, and the entry is re-learned, the correct aging timer will be used.

- When changing the encapsulation type (e.g., 802.3 to Ethernet II), you must reset the physical port either by pulling the physical cable out and reinserting it or using the User Interface commands to disable and then enable the port.

- Each Group within the IBM 8274 Nways LAN RouteSwitch executes the IEEE 802.1d Spanning Tree algorithm. Spanning Tree defines its bridge ID as the lowest MAC address of all the virtual ports attached to the given Group. If a virtual port is moved from “Group A” to “Group B”, then a Spanning Tree reconfiguration may result. If ‘Group A’s” bridge ID is the MAC address of that virtual port, then its bridge ID must change. This will require a Spanning Tree reconfiguration. If the virtual port added to “Group B” has a MAC address lower than all other virtual ports attached to “Group B”, a Spanning Tree reconfiguration must occur. If the user performs such port moves via a Telnet session, they will experience a temporary lockout (up to 30 seconds) in their session, if Spanning Tree reconfigures on the Group supporting their Telnet session.

- Spanning Tree must be configured OFF through the stc command in order for the Enhanced Diagnostics program to function properly. This applies to version 3.0.2 and later. The switch defaults to Spanning Tree ON. Set it to OFF through the stc command, save the change, and power cycle. If the problem persists, remove cfg/cnf files, and repeat the steps turning the Spanning Tree off.

- Spanning Tree on the IBM 8277 Nways Ethernet RouteSwitch is supported only if the ports are in "bridged mode". Spanning Tree will not work if the ports are in "AutoSW" mode.

- To make sure that the proper negotiation occurs for the switch to become the Spanning Tree root bridge, always set the priority of this switch accordingly. Do not rely on the MAC address to determine which switch becomes the root bridge.

## Commands

- The camcfg command is not supported on IBM 8277 Nways Ethernet RouteSwitch and IBM 8274 Model GRS.

- With an HRE, one must not use the chngmac command if there is a TSM-CD-6, TSM-CD-16 or FCSM board present.

## FCSM Switching Module

- FCSM-II requires MPM-1G revision B16 or higher which includes the following part numbers: 050143-06 (MPM-1G), 050143-16 (MPM-1G 16MB), 050143-17 (MPM-1G 64MB), 050143-18 (MPM-1G 32MB), 050143-19 (MPM-1G).

## Frame Relay

- Multicast routing is not supported over frame relay connections that have multiple virtual circuits. In order to use Multicast routing over a frame relay connection only one virtual circuit for the connection must be made.

## Gigabit Ethernet Switching Module (GSX)

- When the cable is pulled multiple times, the GSX may reset. This occurs because an error is detected by the software which in turn resets the chip.

## Group Mobility

- If a group is declared to be a Default Group for Frame Relay Virtual Circuit creation, then it cannot be deleted, even if it is not currently being used. Rmgp will display the message “Cannot delete. Group has active entries.” Work-around: use the frm slot/port to remove the group as a Default and then rmgp. (PR 4549).

- The maximum number of groups supported for ATM Trunking is 48 or the number of available MAC Addresses for that ATM port, whichever is less.

## MPM Redundancy

- In a redundant MPM configuration, do not remove the primary MPM without performing a renounce command first. Then extract the MPM.

## IBM 8274 Model GRS

- IBM 8274 Model GRS Nways LAN RouteSwitch does not support any module in slot 1 other than an MPX.

## Routing

- Virtual router port does not support FDDI-raw frame. When virtual router ports are configured as Ethernet 802.3 and FDDI port as FDDI-raw, FDDI-raw packets received on the other side of router are not routed.

- Multicast Routing is not supported over ATM CIP connections in Versions 3.0 and greater. If this option is selected, the traffic will not be sent to these connections.

- In a topology where a loop exists, do not configure static routes on each of the switches (in the loop) such that each switch points to another switch (in the same loop).

- There is a label “use_mrouted” which must be set to a non-zero value for mrouted to load and run. Originally the presence of the file “mrd.img” was sufficient to activate multicast routing functionality. Now the file must be present, and the “use_mrouted” flag must be set. This flag can be set by inserting the line: "use mrouted=1" into the mpm command file before the line which invokes cmInit.

- There is a restriction on the trunking protocol which does not allow specific routed frames to get to FDDI trunks or ATM trunks when coming from TR LANE side.

- Systems using the Advanced IP Routing software on media types with MTUs greater than 4K (Token Ring, ATM, etc) must set the “gated_recv_mtu” flag in the mpm.cmd file to indicate the MTU. The syntax is “gated_recv_mtu = X” where X is the MTU size in bytes. This line MUST be placed after the line containing cmInit, and the switch must then be rebooted in order for the change to be effective. It is advisable to add 200 extra bytes to the MTU size so that the receive buffers are guaranteed to be large enough. There are no restrictions on the values of this parameter.

## System Requirements

- IBM 8274 Model GRS Nways LAN RouteSwitch Version 3.4.7 requires a minimum of 32 MB SIMM and 8 MB Flash. (See Section 10)

- IBM 8274 Nways LAN RouteSwitch Version 3.4.7 requires a minimum of 16 MB SIMM and 4 MB Flash. (See Section 11-12)

- IBM 8277 Nways Ethernet RouteSwitch Version 3.4.7 requires a minimum of 16 MB SIMM and 4 MB Flash. 32 MB DRAM is strongly recommended for installations running heavy ATM or routed traffic or those with unique configurations.

- IBM 8277 Nways Ethernet RouteSwitch 2032, 3032 and 5024 require 32 megabytes of DRAM when running ATM with over 6 services. The current memory size can be found through the systat command. To upgrade the DRAM, order RSTK-32MDRAM-UPGR, an IBM 8277 Nways Ethernet RouteSwitch 32 Megabyte Upgrade Kit.

- Diagnostic image, "p4diag.img", is dynamically loaded upon login as "diag". In order to run diagnostics on IBM 8277 Nways Ethernet RouteSwitch with a non-Ethernet uplink, it is recommended that the user install 32MB DRAM.

- The IBM 8274 Nways LAN RouteSwitch supports a maximum of 16,384 MAC addresses.

- Please see the Hardware Compatibility Matrix in section 17.2 to verify that your platform is compatible with IOS software version 3.4.7.

## Token Ring

- Caution should be used when issuing the mrrtab command in the user interface. There is a known problem that occurs when there are large numbers of routes defined in the system (>3000 routes). Use the mrrnum command to display the number of known routes in the system. Use of the mrrtab with large quantities of routes may cause the switch to hang, or crash due to lack of memory.

- If TSM ports are configured to support either source routing (SR) or source route transparent (SRT) operation, the port should be connected to Ring In or Ring Out of the concentrator, but not to both. This is for IBM Fiber RI/RO support only.

## RouteVision

- All network management applications must load all IBM Version 3.4.7 MIBs.

- XVSS client does not work over Token Ring.

# Additional Feature Information for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch

## ATM LANE

| ### LAN Emulated Client Start–up and Back-off Timers |  |  |
| --- | --- | --- |
| The IBM 8274 Nways LAN RouteSwitch LEC provides the capability of backing-off if the LEC notices calls being released or when multiple LECs on the same IBM 8274 Nways LAN RouteSwitch are being enabled at the same time (e.g. physical cable is disconnected/reconnected). The amount (time in ticks) and nature (fixed/random) of the back-off period can be controlled using the variables described in the table below. It should be noted that the adjustment of these values needs to be done only if the ATM Network (which the LECs are connected) is not capable of processing the total number of calls that could be generated by all the LECs connected to the ATM network. This scenario is most likely to occur under the following two situations: - During a global restart of all the LECs due to power on or power failure. - During a global attempt to join an ELAN when a central LANE server resource such as LECs/LES/BUS is disconnected/reconnected or some failure. Each LEC needs to setup a call to the LEC/ LES and BUS before it stops retrying. If any fails, the LEC backs off and continues the process of attempting to join the ELAN. The default values are chosen so that calls to the LECs/LES/BUS are individually throttled by a time period of 16.66 – 266.56 milliseconds. |  |  |
| For 3.4.7, the default timers with comments are as follows; |  |  |
| Default Timers | Description | Value |
| atmlec_randomize_throttle = 1 | This flag determines if the procedure uses random back-off or fixed back-off. set = 1 for random back-off and =0 for fixed back-off. | 0 or 1 |
| atmlec_backoff = 500 | The % back-off in both random and fixed can be controlled using the flag atmlec_backoff, by default + 500 (for 500%). | > = 100 |
| atmlec_lecs_throttle_ticks = 16 | All values for ticks should be a power of two (example 8, 16, 32, 64). Each tick is 16.66 milliseconds. | Power of 2 and > = 2 |
| atmlec_les_throttle_ticks = 16 | All values for ticks should be a power of two (example 8, 16, 32, 64). Each tick is 16.66 milliseconds. | Power of 2 and > = 2 |
| atmlec_bus_throttle_ticks = 16 | All values for ticks should be a power of two (example 8, 16, 32, 64). Each tick is 16.66 milliseconds. | Power of 2 and > = 2 |
| atmlec_lecs_retry = 3 | Number of retries before restarting the LEC | > = 3 |
| atmlec_les_retry = 3 | Number of retries before restarting the LEC | > = 3 |
| atmlec_bus_retry = 3 | Number of retries before restarting the LEC | > = 3 |

To modify these default timers, the mpm.cmd file needs to be modified. An example mpm.cmd file with modified timer values is provided below; cmDoDump=1 atmlec_lecs_throttle_ticks=64 atmlec_les_throttle_ticks=64 atmlec_bus_throttle_ticks=64 atmlec_lecs_retry=7 atmlec_les_retry=7 atmlec_bus_retry=7 cmInit

### LANE Service Module

The following details the memory used in the LANE Service Module (LSM) assuming one LEC and one LES BUS (one ELAN) is configured with the ELAN_NAME policy: LSM code image (un-compressed): 500,000 bytes LECS runtime memory 5,200 bytes LES-BUS runtime memory 4,300 bytes Message queue for AM 504,000 bytes Message queue for Signaling 335,000 bytes Message queue for Timer 2,000 bytes Message queue for VSE 1,200 bytes Total memory used: 1,351,700 bytes * Each LEC will consume 4,618 bytes

If you have 60 LECs joined in the same ELAN then it will be 60*4,618=277,080 bytes plus the 1,351,700 above which will come up with 1,628,780 bytes usage. If there are 60 LECs and 4 ELANs, divided into two ELANs you need an additional 4,300 bytes for the second ELAN.

A single LSM will support up to 300 LAN Emulation Clients (LECs) and 10 ELANs (LES/BUS) which is an average of 30 LECs per ELAN. The BUS forwarding rate measured on SAHI is 50,000 packets per second.

The LECS Policy Value of ESI/Selector under the ATM Address Prefix is not supported in release 3.4.7. Any user input will have no effect in filtering, any LECS Configuration Request will pass this criteria if it is selected.

LSM LES/BUS redundancy cannot be configured on the same switch.

## ATM Memory

- IBM 8277 Nways Ethernet RouteSwitchs 2032, 3032 and 5024 now require 32 megabytes of DRAM when running ATM. The current memory size can be found through the systat command. To upgrade the DRAM, order an IBM 8277 Nways Ethernet RouteSwitch 32Megbyte Upgrade Kit or RSTK-32MDRAM-UPGR.

## Content Addressable Memory (CAM): Advanced Uses (IBM 8274 Nways LAN RouteSwitch Only)

- This overview assumes the reader has a basic knowledge of the CAMs function in an IBM 8274 Nways LAN RouteSwitch. It is intended to address advanced uses of the CAM, such as restrictions, configuration, and the page architecture of CAM. It also provides several examples to illustrate how the CAM is allocated to switching modules.

- The Goal of CAM

- Without any user configuration, every switching module, (regardless of it’s the number of CAMs present by it,) should come up upon a reset or after a reboot. There are restrictions to CAM usage. The main restriction is that only 16K of CAM is available in the entire switch due to limited VRAM memory. However, when the number of CAMs exceeds the maximum allowed, the user should be able to configure the proper CAM usage on each board (via the camcfg command) to get as close to the maximum limitation as possible.

- CAM's Page Architecture

- Structurally, the switch's CAMs consists of 16 x 1K pages of memory numbered from 0 to 15. A page ID identifies each CAM page and is also used to differentiate the CAM allocated to each slot.

- Switching modules with 2K or 4K of CAM must start CAM allocation on an even page (i.e., page ID 0, 2, 4, etc,). In addition, if multiple CAM page IDs are needed for a 2K or 4K module, then those page IDs must be contiguous.

- Boards with 1K of CAM are first initialized as page ID 15, then page ID 14, then page ID 13, etc. Boards with 2K or 4K of CAM are first initialized on page ID 0. So 1K CAM boards start at the bottom of the CAM page structure and 2K and 4K boards start at the top of the CAM page structure. A 4K CAM module in slot 2 may use Page 0 (an even page) through Page 3, and a 1K CAM module in slot 3 may use Page 15. Modules cannot share MAC entries because each MAC entry is unique in the switch.

- The MPM and Page 0

- Since the CAM database is only used for MAC learning, only 128 entries are needed by the MPM to store router port MACs. Page ID 0 is always used by the MPM CAM. The extra 896 entries in Page 0 that the MPM does not need can be used by modules in other slots. Any board using Page 0 may have 128 entries deducted from its CAM usage because those entries are used by the MPM.

- Since the MPM uses 128 entries of the total 16K available in the switch, the maximum number of entries that can be learned by switching modules is actually 16226 instead of 16786.

- CAM Initialization

- A successful initialization of CAM is important. If CAM initialization on a board fails, then that board will not come up properly. With no user-configuration of CAM, the switch software tries to initialize and set up all CAM available on each module. As long as there is available memory, this is possible.

- If there is not enough memory for the entire CAM or the configured amount of CAM on a module, the switch will attempt to find room for 64 contiguous CAM entries. This minimum amount of CAM is enough to initialize the CAM and allow the board to come up.

- If 64 contiguous entries are not available, switch software will take 64 entries way from another board, which will usually be the board in the last slot. In this case, the switch will reinitialize the CAM so the board can come up. A warning message similar to the following will be displayed to notify the user during the reboot or reset:

- slot x: Cam usage was reduced to y so it can be properly initialized!

- The user can see the CAM usage by each module through the camstat UI command.

During a reboot, the switch does not know the number of CAMs on a board until the Chassis Manager software initializes the board. Also, boards do not necessarily come up in sequential order (e.g., slot 8 may come up before slot 7). Therefore, CAM page assignments may be altered after hot swapping multiple boards, and hot swapping may also affect the CAM usage by each board.

- Restrictions for 2K and 4K CAM Boards

- The restrictions for 2K and 4K CAM boards (i.e., CAMs must start on an even page and those pages must be contiguous) can mean that every CAM page ID may not be used.

## CSM/FCSMI Hot Swap Instructions

Hot Swap only works when an FCSMI is in a slot before a CSM. The FCSMI can be installed in a chassis slot except slot 1. The FCSMI should be inserted before any CSMs (i.e. the FCSMI must have a lower slot number than any CSM in the same chassis).

The following instructions are recommended when performing hot swap with CSM and FCSMI modules: - The swap command should be on when pulling or inserting CSM modules - Before pulling CSM module, disconnect all CSM ports on that CSM module - Wait for initialization to complete and for OK2 LED to return to blinking green - Insert CSM module - Reconnect any CSM ports

Currently, if an FCSMI is pulled and reinserted, the LECs attached (externally) to that chassis might not rejoin the ELAN and stay in the initial state (PR 10780). This is an external ASM LEC problem. One of the following workarounds must be used to bring the attached external ASM LEC back up: - Disconnect the LECs before hot swapping the FCSMI or - Use the mas command to bring the LEC back up after hot swapping the FCSMI.

Hot Swap for CSM/FCSM2 is not supported.

## CSM-PNNI MTU Size Configuration Guidelines

This section is being provided as additional design guidelines for building a core ATM-based network running PNNI when using the IBM 8274 Nways LAN RouteSwitch with CSMs.

The design guidelines take into consideration that the PNNI frame sizes used have a direct impact on the ability to have a certain number of CSM (Cell Switching Module) ports operational in one IBM 8274 Nways LAN RouteSwitch. PNNI frame sizes will vary depending on what type of information is being sent between PNNI nodes. It ranges from 16 bytes to 8192 byte sizes. The most frequently sent message is the “hello” message – it consists roughly of about 100 bytes. However, database summaries and PTSEs (PNNI Topology State Elements) that are exchanged between switches can run from 64 to 8K frame sizes. As the support for frame sizes become larger, the total number of CSM ports you can have on a switch becomes smaller.

The chart above illustrates that if you need to support PNNI frames of up to 8192 bytes, the total number of CSM ports that you can have in the chassis is limited to 21 CSM ports.

It is also recommended that the maximum number of PNNI neighbors per PNNI node should be less than, or equal to, 30 neighbors. Having more than 30 neighbors will trigger higher CPU utilization on the MPM, unless the Hello timers are re-configured [using the pmcfg command] with higher values. The frame sizes are configurable [using the command MAP slot # / 2 on the FCSM] and these are our frame size recommendations: - Small to Medium networks (up to 20 PNNI nodes ) : 2048 to 6144 bytes - Large size networks (more than 20 nodes): 8192 bytes

By default, IBM switches will not originate frames larger than 2K bytes. In an all-IBM network re-configuration of the MTUs is not required. However, if you plan to have a multi-vendor ATM network, it is recommended that you use the 8K frame size, and also limit the number of CSM port connections to less than or equal to 21 per IBM 8274 Nways LAN RouteSwitch. As an example, if the PNNI node is required to support up to 8K frame sizes, you can configure up to 21 CSM ports running PNNI.

[Note: this recommendation applies only when the FCSM-I module is used; this is not applicable to the FCSM-II modules].

## Duplicate MAC Support on IBM 8274 Model GRS Nways LAN RouteSwitch and IBM 8274 Nways LAN RouteSwitch

In some network configurations, the IBM 8274 Model GRS Nways LAN RouteSwitch and IBM 8274 Nways LAN RouteSwitch must interoperate with servers and routers that carry the same MAC address on multiple interfaces. Switch software supports these duplicate MAC addresses. Chapter 11 of your IBM 8274 Nways LAN RouteSwitch Users' Manual describes this feature. Support for duplicate MACs can be limited by the lack of internal memory and CAM space. However, since there are usually a small number of servers or routers in a given network, this limitation usually is not a problem. It is not recommended that you bridge multiple VLAN Groups together as this causes most MAC addresses learned to become duplicate entries.

## E1 Framing types supported and how to Configure 31 Timeslots on a WAN E1 Port

On WSM E1 ports, the unframed format isn't supported since WSMs only support standard E1 framing for PPP or Frame Relay (the “unframed” format is only supported for unstructured Circuit Emulation T1 or E1 ports). For WSM E1 ports, it is mandatory that the frame type be set to one of the standard E1 Framing types (E1, E1-CRC, E1-MF, E1-MF-CRC) with the temod command. Most E1 services only allow a maximum of 30 usable timeslots since timeslot 0 is always used for Frame Synchronization (which is why you can't use unframed for Frame Relay or PPP ports since you must specify how timeslot 0 is used) and timeslot 16 is usually used for multiframe sequencing. The WSM can support 31 timeslots for cases where timeslot 16 isn't used for multiframe control. When you configure the timeslots for a WSM E1 port, you specify a starting timeslot followed by a number of timeslots by using the wpmodify (WAN Port Modify) command. Normally, the WSM will use a default configuration which skips timeslot 16 automatically. In this way, it will select the E1 frame to generate E1 timeslot 0 (the “synchonization” timeslot), but leave timeslot 16 (the “multiframe control” timeslot) free. The WAN port configuration software when configured for 31 timeslots will then use all timeslots from 1 to 31 to give you a full E1 where timeslot 16 is also used for data. Again, this should only be done for facilities that do not require E1 Multi-Frame. For those types of E1 lines, they can support a maximum of 30 timeslots. Only those E1 lines that don't require E1 multiframe can be configured in the method described below. So, to configure a WAN E1 port for 31 timeslots, follow the steps below: - Enter temod <slot>/<port> at the system prompt, where <slot> is the slot number of the module with the E1 port and <port> is the port number of the E1 port. For example, to configure WSM E1 port 4/2, enter temod 4/2. - Enter 2=4 at the prompt to set the frame type to E1 or enter 2=5 at the prompt to set the frame type to E1-CRC. - Enter save at the prompt to save your settings. - Enter wpmodify <slot>/<port> or wpm <slot>/<port> at the system prompt, where <slot> is the slot number of the module with the E1 port and <port> is the port number of the E1 port. For example, to configure WSM E1 port 4/2, enter wpm 4/2. (Note: wpm is the abbreviated form of wpmodify.) - Enter 3=1 to set the starting timeslot to 1. - Enter 4=31 to set the number of timeslots to 31. Enter save at the prompt to save your settings.

## ESI for an ASM Port

A change was made in release 3.4 and later to correctly assign an ESI for an ASM port. The ESI assigned to each ASM port uses the base MAC address for that board and the physical port number. This will uniquely assign ESI's for port 1 and port 2 of the ASM board based on the base MAC address of that board. This method creates a problem if the ASM board is moved between different slots, as the ESI's will be changed because the physical port number has changed.

In release 3.4, the method of assigning an ESI is changed slightly. The base MAC address is still used, but the physical port number is no longer used, instead the interface number is used. For a 2 port ASM board, there is interface 1 and interface 2. With this change, even when an ASM board is moved between different slots, the interface number remains the same and thus the ESI is maintained.

With this change, there might be some backward compatible issue with pre-3.4 release software upgrading to 4.0 software when the ESI of the ASM ports is changed. This will only affect the user using SVC based PTOP and Trunking service since the remote station has stored the old ESI in their system. Users with LANE and any PVC based ATM services will not be affected.

## Optimized Ports on ESM-100C-12, ESM-100F-8, ESM-C-32, and IBM 8277 Nways Ethernet RouteSwitchs

The ESM-100C-12, ESM-100F-8 AND ESM-C-32 and IBM 8277 Nways Ethernet RouteSwitchs will flood packets with unknown destination addresses on ports configured for optimized device mode. To prevent this from occurring, the following can be entered in the mpm.cmd file: MamOptSwtchPorts=1

NOTE: If the port is set to optimized and has not learned a MAC address, it will flood these packets out regardless if the above command is used.

## Functional Address Support for IBM Spanning Tree on IBM 8274 Model GRS Nways LAN RouteSwitch and IBM 8274 Nways LAN RouteSwitch

- The user can enable on a per Group basis the IBM Spanning Tree algorithm. This capability is provided by one of the menu items under the stc [Group ID] command. When the user selects IBM Spanning Tree, the IBM 8274 Nways LAN RouteSwitch automatically sets the default values for maximum age, forward delay, and hello time (6, 4, and 2 seconds, respectively) as seen in the subsequent menu items. Unlike the IEEE Spanning Tree algorithm, in the IBM Spanning Tree algorithm these values are fixed. However, for consistency in menu displays, IBM chose to not only display these values as in the IEEE Spanning Tree case but to enable the user to change them. In fact, the user should not change these values.

- The default Spanning Tree algorithm for each Group is IEEE. If the user wishes to change Spanning Tree parameters on a per Group basis through use of the stc [Group ID] command, at least 1 virtual port needs to be attached to the Group. An error message is displayed to this effect if the user selects the stc [Group ID] command and there are no virtual ports attached to the specified Group.

- When the user selects IBM Spanning Tree, additional default settings are carried out as follows:

- all virtual ports attached to the Group in question whose physical port is either 4Mb or 16MB token ring are initialized to use Functional rather than Group addresses;

- all virtual ports attached to the Group in question whose physical port is neither 4Mb nor 16Mb are set to manual forwarding mode; and,

- as additional virtual ports are attached to the given Group, rules a) and b) are applied.

Virtual ports that are in manual forwarding state means that they are not participating in any flavor of Spanning Tree (neither IEEE nor IBM). Since this state occurs automatically as a result of rule b), IBM decided to forward any IEEE Spanning Tree frame received on such a port to all other virtual ports in the Group that are in manual forwarding state. This is to prevent a loop in the topology as a result of applying rule b) mechanically.

- For a Group configured for IBM Spanning Tree, the user can override the default settings at his or her discretion. However, as a general rule, overriding the manual forwarding state to enable IEEE Spanning Tree on that port only is not recommended. The user is now running a “hybrid” of IBM and IEEE Spanning Tree within the same Group. This may have some unintended consequences, and, although not forbidden, is not recommended.

- IBM SRT bridges send, on token ring, an IEEE-style BPDU with an STE RIF (0xC200). We do not support this mode of operation. The IBM 8274 Nways LAN RouteSwitch will neither process nor forward these frames.

- IBM Spanning Tree uses the following 48-bit Functional Address (FA) given in canonical form to transmit BPDUs: hexadecimal 0300 0000 8000. IBM has discovered that IBM clients and servers use this address to transmit data.

- With the current architecture of the IBM 8274 Nways LAN RouteSwitch, we cannot support the same FA for data and Spanning Tree. The FA for IBM Spanning Tree is programmed into our MPM CAM; therefore, all data frames with this FA are claimed by the MPM. This prevents the data from passing through our switch. However, we provide the following workarounds:

- There exists a mpm.cmd settable global variable faBpGrpDisable such that when set to 1, Spanning Tree will not program the FA into the MPM CAM. This works for those environments that are not running IBM Spanning Tree. The setting of the variable faBpGrpDisable should be placed BEFORE the cmInit command in file mpm.cmd.

- There exists an mpm.cmd settable global variable faBpGrpOverride which allows the user to set the lower 32-bits of the FA to some value other than hexadecimal 0000 8000 which is the lower 32 bits of the IBM Spanning Tree FA. The upper 16 bits are always hexadecimal 0300. This works for those environments whose applications use FA hexadecimal 0300 0000 8000, who need IBM Spanning Tree, and whose switches are all IBM or third-party bridges / switches whereby their IBM Spanning Tree FA is configurable. The setting of the variable faBpGrpOverride should be placed BEFORE the cmInit command in file mpm.cmd.

## Group Mobility

### GMAPLS Command

Use the gmapls command to view the contents of the GMAP database.

GMAP Table

==========

MAC Address Group Protocol Src Switch ID Flags Timeout(sec)

------------- ----- -------- ------------- ----------- ------------

000039:7A690C 7 800 0020DA:999660 00:00:00:00 716

9 28137 0020DA:999660 00:00:00:00 716

0000C0:21A5B8 9 28137 0020DA:999660 00:00:00:0A 716

The fields on this screen are defined as follows:

MAC Address - the MAC address of the end-station for this entry.

Group - one of the groups to which the MAC address belongs.

Protocol - the protocol associated with this group on the switch from which the information was received.

Src Switch ID - the switch from which this information was received.

Flags - flags that go with the entry. The upper two bytes are not used. The third byte holds the RouteTracker flags associated with this entry on the source switch. The fourth byte holds the router flags associated with this entry on the source switch.

Timeout - the number of seconds remaining until this entry is deleted (unless another message is received that refreshes the entry).

### Upgrading to Group Mobility II

- This section only applies if the user is currently using Group Mobility in 3.1 and the user wants to upgrade to 3.4 using Group Mobility II.

Follow these instructions: Remove the VLANs that have been configured for Group Mobility in 3.1. - Take "group_mobility=1" out of the mpm.cmd file. When loading 3.4: Create the mobile groups desired and reconfigure the policies for those groups. Perform the gmcfg command to enable Group Mobility on the switch. It is not required to do rm mpm.cnf on the UI shell when loading 3.4.

## Hardware Tagging

### Support:

GSX-FM2 and GSX-FM4

### User Interface:

There are three User Interface commands to configure, delete and view hardware tagging.

Following is a brief description: addtggp : Add X802.1q group(s) to a port deltggp : Delete X802.1q group(s) to a port vitggp : Display X802.1q group(s) from a port(s)

### Limitations:

Please note tagged or untagged refers to a port that is configured with or without hardware 802.1q tagging respectively.

- Mismatch configuration of groups on hardware X802.1q is not supported. If 10 groups are configured at one end of the link, the same number of groups must be configured on the remote side of the link, with the same set of group Ids.

- The order or sequence in which the user adds or deletes group(s) from local and remote switches is very important to the functionality of hardware tagging. For example, lets take the example of a user who wants to trunk 5 groups with X802.1q tagging enabled. The user adds group 1,2,3,4,5 on the local switch then decides to add the same 5 groups on the remote end of the link. It is imperative that the groups are added in the same sequence; 1,2,3,4,5 on the local switch and remote end. The same applies to deleting groups.

- A group with X802.1q tagging enabled on one switch port can not be configured for tagging unless the group is disabled or moved from that port. For example, if the user wants to add tagged group 2 to port 2/1, they must verify that port 2/1 does not have group 2 already assigned (vi 2/1 will display the untagged group membership). Use addvp to move the untagged group from the port.

- The GSX-2 and GSX-4 support a proprietary tagging method called X802.1q. This is an implementation of an early draft of the pre-standard IEEE 802.1q. It is only interoperable between IBM Gigabit Ethernet interfaces and not with equipment supporting the final draft of the IEEE 802.1q standard.

- User must rebuild all 802.1Q assignments or incorrect slot/group assignments may occur. Split single mobile group ports out to (2) 802.1Q.

- Flooding may not be performed correctly based upon group mobility. To avoid this problem add dupMacFlood=1 to the mp4.cmd and mpx.cmd files.

## Hot Swapping ASM Modules

- The instructions for hot swapping modules on pages 3-6 to 3-8 in the IBM 8274 Nways LAN RouteSwitch User Manual do not apply to ASM modules. If you use the reset command with an ASM module, the module will not come back up. To hot swap ASM modules, follow all steps in the instructions except those requiring the reset command (i.e., steps 2 and 7).

## HRE Notes

- Large Routed Frames over IBM Proprietary FDDI Trunking:

If a FDDI port is configured for IBM Proprietary FDDI Trunking, large routed frames received from that port, will not be correctly routed by the HRE. Data corruption will occur if the frame is larger than 1856 bytes, including the MAC header. Since Ethernet frames are smaller than 1856 bytes, they will not be affected by this limitation. Only large frames such as those generated by FDDI and Token Ring will be affected.

- HRE must be in slot 1:

- If an MPM has an HRE installed, the MPM must be installed in slot 1. Slot 1 is optimized for high performance routing.

- Redundant HRE is not currently supported.

## HRE-X with IPX Routing

- The default value of the hrXnative flag is "1" which should be used when similar encapsulation ports are used. - To enable routing for different encapsulations on the router port (e.g. Vlan2:1 IPX=802.3 and Vlan3:1 IPX=Ethernet-II), set the hrXnative flag to 0 ("hreXnative = 0) in the cmd file before cmInit.

## IPX Performance Updates

There are two additional commands in the IPX submenu; one for enabling/disabling extended RIP and SAP packets (ipxext) and another for configuring/disabling an IPX default route (ipxdrt).

### IPX Extended RIP and SAP Packets

Larger RIP and SAP packets may be transmitted so that congestion in the network is reduced. To enable/disable larger packets, use the following UI command syntax:

ipxext [on ¦ off]

For example, enter ipxext on to enable the feature.

To display the current status of the feature, enter the command alone:

ipxext

When the feature is disabled, the following message displays: IPX extended RIPs and SAPs off. When the feature is enabled, the following message displays: IPX extended RIPs and SAPs on.

### IPX Default Route

- A default IPX route may be configured for packets destined for networks unknown to the switch. If RIP messages are disabled, packets can still be forwarded to a router that knows where to send them.

To configure a default route, use the following UI command syntax:

ipxdrt [network ID [MAC address]] ¦ [off]

If the network ID indicates a direct network on this switch, the MAC address (IPX node address) must be supplied for the next router. You can enter the MAC address in the command line, or the system will prompt you for it. For example, enter ipxdrt 222 00:20:da:99:88:77 to configure a default route to a direct network.

To view the status of the default route, enter the following:

ipxdrt

The default route network ID and MAC address display.

To disable the default route, enter the following:

ipxdrt off

## MPM Upgrade

- Release 3.0 to 3.3 requires a minimum of 16 Mbytes of DRAM memory. To upgrade memory, existing units in the field should purchase an MPM-UPGR-16MB upgrade kit. The MPM-UPGR-16MB upgrade kit expands the Management Processor Module DRAM memory to 16 Mbytes. This includes a single SIMM with 16 Mbyte of DRAM memory and installation instructions.

- Release 3.4 or greater requires a minimum of 32Mbytes of DRAM memory. To upgrade memory, existing units in the field should purchase an MPM-UPGR-56MD-12MF upgrade kit. The MPM-UPGR-56MD-12MF upgrade kit is a single SIMM with 56 Mbytes of DRAM and 8 Mbytes of flash memory. The MPM-UPGR-56MD-12MF also comes with installation instructions

- IBM memory is required for all MPM upgrades.

## MPX on-board Ethernet Ports

### Configuring an IP Address on the MPX on-board Ethernet Module

- Before accessing the IBM 8274 Model GRS through the MPX on-board Ethernet port, you must assign an IP address to it first. It can be done only through the Boot prompt. Note the Boot prompt is case sensitive. - 1. When the Boot prompt displays as shown below: - [Boot]: - enter a lowercase c to begin configuring parameters. - A prompt similar to the following displays. - '.' = clear field; '-' = go to previous field; ^D = quit - Boot device : ffs - 2. Press the <Return> key to accept the defaults for all values until you encounter the ethernet address prompt, as shown below: - Eth IP addr[:mask] : - 3. Enter an IP address for the Ethernet plug-in module in dotted decimal or hexadecimal notation. As an option, you can also enter an IP address mask in hexadecimal notation. If no mask is provided, the switch will try to determine the mask using Internet Control Message Protocol (ICMP) requests. - 4. Several more configurable fields will be displayed. Enter control-D. - 5. When the Boot prompt displays again, as shown below: - [Boot]: - enter an uppercase S to save the Ethernet plug-in module’s new IP address.

## IBM 8277 Nways Ethernet RouteSwitch and Mammoth based ESM/ESX Port Partitioning

- Mammoth 10BaseT and 10/100BaseT boards now detect certain cabling errors and/or physical media misconfigurations which could lead to many retries and/or the reception of many spurious frames which may impact performance of devices attached to the mammoth board.

- When such a condition is detected, the IBM 8274 Nways LAN RouteSwitch/IBM 8277 Nways Ethernet RouteSwitch system will now "partition" such a port. The port is marked with an Operational state of "Inactive" in the vi menu, even though there may be a cable attached to the port, and the link light is on. If a cable drop is detected, the system will remove the partitioned state, and bring the port back up as normal once link is detected.

- If the original cabling problem has not been corrected, the link may become "partitioned" again. If the problem has been corrected, normal operation will be enabled.

## IBM 8277 Nways Ethernet RouteSwitch 3032 and IBM 8277 Nways Ethernet RouteSwitch 3032E

- When the IBM 8277 Nways Ethernet RouteSwitch 3032 is still initializing, the normal "login" prompt will appear prior to the switch going through its full bootup process. If you attempt to login to the switch prior to the sequence in which the OK1 and OK2 LEDS are blinking, then slot 5 or expansion port 2 will fail to initialize. - A slot command will show slot 5 as "unknown". In order to avoid this failure, allow the RS-3032 to go through its full bootup cycle before commencing to login.

## Simm Flash Module (IBM 8274 Nways LAN RouteSwitch)

The following files MUST be in /flash: The boot file - mpm.img See below. The startup script - mpm.cmd See below.

Redundant MPM issues with 32MB/56MB SIMM/flash: A file in /flash can only be transferred to /flash on another MPM. A file in /simm can only be transferred to /simm on another MPM. This applies to imgsync, mpmstore, mpmreplace, mpmload and mpmget.

## PNNI - Configuring Node-Specific Parameters

If a pncfg command has been issued on a 3.1.X switch, it will store the user's node id/address configured. If the administrator did not modify the node id or address, pncfg will store the factory default address.

If the user then upgrades to 3.4.7, the node ID will be the same as it was in 3.4 and prior builds. Once the user upgrades their 3.1.X switch to 3.4.7, they need to issue the pncfg command and change it to the new prefix. When PNNI configuration has been changed using the pncfg, a flag is set which will tell PNNI whether it should use the 3.4.7 factory defaults or the user configured prefix.

After upgrading to 3.4.7, the user must verify that the address summary matches the node ID (in the first 13 bytes).

## PNNI and Redundant MPM's

When the node ID is unspecified, the node ID changes when the secondary MPM takes over. To prevent this failover problem, one must specify the default, save it to the configuration file, and then perform an image sync as follows: - Execute pncfg on the primary and set items 1,2,3, and 6 to the defaults (from unspecified …) - Save the data and force the changes to take immediate effect - Execute a configsync. (immediately …) (optional): - Execute renounce (to verify that the values are on the redundant MPM) - Login to the (now) primary - Execute pncfg and verify items 1,2 3, and 6 …

## Port Mapping

- If the user executes the pmap command and exits without configuring any ports for port mapping, port mapping will be turned on in the box. This will interfere with normal operation and might cause functional problems. If the user does not intend to configure port mapping, do not execute the pmap command.

## Static Route Configuration and GateD

Due to differences in the way that routing information is maintained by GateD and the IP protocol stack (kernel), mismatches in routing tables can appear if the RISR/AISR commands are used to configure static routes while GateD is running.

Whereas the kernel IP routing table houses any and all routes (whether currently reachable or not), GateD will not allow for the configuration of "unreachable" static routes. Another way of stating the latter point is to say that GateD will not add a static route to its routing table unless the interface for the route's "next hop" is operational.

On systems in which all interfaces are always "up", this difference in behavior is not apparent because both the kernel and GateD accept routes for which the "next hop" is via an operational interface. On systems which have one or more "down" interfaces, confusion arises due to the difference in the way that routing tables are maintained by both the kernel and GateD.

For example, when a static route to a non-operational interface is "added" with the AISR command, the kernel accepts the route unconditionally and adds it to the kernel forwarding table. After the route is added, it will show up in the output of the IPR command. In this same situation, GateD refuses to accept the route because there is no operational interface for the route's "next hop". The route will not appear in the output of the GATED ROUTES command precisely because GateD refuses to accept the "non-operational" route.

On systems which have "down" interfaces, mismatches might appear in the kernel and GateD routing tables whenever "static" routes have been configured with the AISR and/or RISR commands. (The kernel routing table is displayed with the IPR command, while the GateD routing table is displayed with the GA ROUTES command.)

When GateD is running, this de-synchronization problem can be avoided and proper routing operation ensured only if use of the AISR/RISR commands is discontinued in favor of GateD's own "static" route configuration mechanism, which is controlled by statements in the gated.conf file. Rather than using AISR to create static routes, users should insert "static" statements in the gated.conf file--this will ensure that GateD correctly receives knowledge of static routes.

When GateD is running, the "correct" way to add or remove static routes is to edit the gated.conf file and then issue the GATED RECONFIG command which will cause GateD to re-read the gated.conf file and learn of the static route changes. [Note that GateD will complain about unreachable static routes (e.g. routes for which there is no active interface) which are configured in the gated.conf file. This stresses the importance of adding static routes that are dependent only on operational ("up") interfaces.]

# FLASH Requirements for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch

Version 3.4.7 requires 32 Mbytes of DRAM and 8 Mbytes of Flash memory for IBM 8274 Model GRS Nways LAN RouteSwitch and 16 Mbytes of DRAM and 4 Mbytes of Flash memory for IBM 8277 Nways Ethernet RouteSwitch and IBM 8274 Nways LAN RouteSwitch. Configuration information is stored in FLASH in addition to the compressed file images.  See the following table for the flash memory required for each module.

| Filename | Image Size (Bytes) | Module Type |
| --- | --- | --- |
| mpm.img | 1,704,665 | MPM, MPM II or MPM-1G image. |
| mpm.cmd | 38 Approximately | MPM, MPM II or MPM-1G command file containing system commands for booting. |
| mpm.cfg | 1k | MPM, MPM II or MPM-1G configuration file fixes at 1K to allow the chassis bootstrap to initialize. |
| mpm.cnf | 32k minimum- 512k maximum | MPM, MPM II, or MPM-1G configuration file (size indicated is the default or minimum) size. As configurations are added, the size will increase. |
| tmpmpm.cnf | 32k minimum- 512k maximum | Temporary shadow file of the MPM configuration file during expansion. It remains the same size as the mpm.cnf. |
| diag.img | 241,730 | Off-line Enhanced Diagnostics image for the MPM. |
| dni.img | 18,767 | Off-line Enhanced Diagnostics image for the NI. |
| dmesm.img | 26,052 | The NI loadable diagnostic image used for stress test for IBM 8274 Nways LAN RouteSwitch Mammoth based ethernet modules. |
| gated.img | 502,011 | Advanced Routing – Dynamically loadable GateD image for OSPF, and RIP II. |
| isdn.img | 202,730 | Dynamically loadable module which contains the ISDN signaling code, and the WSM-BRI daughtercard code. (Daughtercard code takes up flash space only) Needed only if WSM-BRI is present. |
| mrd.img | 70,595 | Advanced Routing – Dynamically loadable image for Multicast Routing function. |
| t1e1drv.img | 75,109 | Image for the following switching modules: CSM-AB-E1, CSM-AB-CE-E1, CSM-AB-CE-T1, WSM-FT1, WSM-FE1. |
| asm.img | 843,574 | ASM-155-F, ASM-155-C, ASM-DS3, ASM-E3, ASM2-155-F, ASM2-622-RFM/RFS, ASX switching module loadable image. Includes MPM code to support modules, signaling stack and ATM Services (see Section 9). |
| asmce.img | 47,486 | Image for the following switching modules: ASM-CE |
| asmcedrv.img | 156,532 | Image for the following switching modules: ASM-CE, ASM2-CE, CSM-CE. |
| cell.img | 854,297 | CSM – Dynamically loadable image for the MPM to manage all types of CSM. |
| ds3e3drv.img | 53,984 | Image for the following switching modules: ASM-DS3, ASM-E3, ASM-CE-DS3, ASM-CE-E3, ASM2-DS3, ASM2-E3, CSM-AB-DS3, CSM-AB-E3. |
| mesm.img | 25,950 | Image for the following switching modules: ESM-100C-12, ESM-100F-8, and ESM-C-32, GSM-FS/FM. |
| e12.img | 27,633 | Image for the following switching modules: ESM-C-12, ESM-T-12, ESM-F-8. |
| esm.img | 27,219 | Image for the following switching modules: ESM-8, ESM-U. |
| fesm.img | 31,934 | Image for the following switching modules: ESM-100-C with HSM motherboard. |
| fesm2.img | 31,901 | Image for the following switching modules: ESM-100-C with HSM2 motherboard. |
| fsm.img | 291,754 | Image for the following switching modules: FSM-M, FSM-S, FSM-C, FSM-M-C. Includes MPM code to support modules. |
| tsm.img | 192,394 | Image for the following switching modules: TSM-C-6, TSM-F-6. |
| mtsm.img | 218,218 | Image for the following switching module: TSM-CD-16W-4C |
| tsm.pga | 74,410 | Includes trbyr6.pga, tryrr.pga, trlobe.pga, trstatn.pga, trsyr6.pga, trsyrr.pga, trxyln3.pga, and tryln6.pga. |
| wsm.img | 425,922 | Includes WSM Driver, PPP Code (Telenetworks), Frame Relay, LMI, and WSM daughtercard (Daughtercard takes up flash space only.). |

| 7 FLASH Requirements for IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch (Cont'd) |  |  |
| --- | --- | --- |
| Filename | Image Size (Bytes) | Module Type |
| fwd.img | 138,113 | Firewall image for IBM 8274 Model GRS Nways LAN RouteSwitch and IBM 8274 Nways LAN RouteSwitch |
| fwd.img | 137,784 | Firewall image for IBM 8277 Nways Ethernet RouteSwitch |
| lsm.img | 175,862 | LANE Service Module image |
| mp4.cfg | 1k | MP4 configuration file fixes at 1K to allow the chassis bootstrap to initialize. |
| mp4.cnf | 32k minimum 512K maximum | MP4 configuration file (size indicated is the default (or minimum) size. As configurations are added the size will increase. |
| mp4.img | 1,644,228 | Management Processor Image for the IBM 8277 Nways Ethernet RouteSwitch 4024 & 5024. |
| mp4.cmd | 18 | Management Processor Image command file containing system commands for booting for the IBM 8277 Nways Ethernet RouteSwitch 2032, 3032, 5024. |
| ae4.img | 825,948 | Image for the following subswitch modules: ATM and Ethernet for the IBM 8277 Nways Ethernet RouteSwitch 2032, 3032 & 5024. |
| em4.img | 28,547 | Image for the Ethernet subswitch module for all IBM 8277 Nways Ethernet RouteSwitch units. |
| p4diag.img | 139,698 | Image for Enhanced Diagnostics for the IBM 8277 Nways Ethernet RouteSwitch 4024 & 5024. |
| dem4.img | 28,658 | Substitute ethernet driver for IBM 8277 Nways Ethernet RouteSwitch used for stress test. |
| ws4.img | 495,066 | Includes WSM Driver, Frame Relay, LMI, and WSM daughtercard (Daughtercard takes up flash space only.). |
| mpx.img | 1,742,635 | MPX image. |
| mpx.cmd | 38 Approximately | MPX command file containing system commands for booting. |
| diagx.img | 251,422 | Off-line Enhanced Diagnostics image for the MPX. |
| esx.img | 26,702 | Image for the following switching modules: ESX-100C-12W, ESX-100C-32W, GSX-FM -2W, GSX-FS-2W |
| desx.img | 26,792 | NI loadable diagnostic image used for stress test and fabric test for IBM 8274 Model GRS Mammoth based ethernet modules. |
| tsx.img | 218,218 | Image for the following switching modules: TSX-C-32W, TSX-CD-16W |

- Note:  	Release 3.0 and higher require 4 Mbytes of FLASH for the IBM 8277 Nways Ethernet RouteSwitch and IBM 8274 Nways LAN RouteSwitch.

Due to the increasing number of image files and varying sizes of the image files, the user has an enormous amount of flexibility in configuring an IBM 8274 Nways LAN RouteSwitch.  The table above shows the images with a brief description of what each image is used for. All images will not fit into 4 MB of FLASH, however in most cases not all images are required.  Also the FLASH is used to store configuration information so additional space will be required.   It is highly recommended that users remove all images not required for operation on a given switch (make sure you back them up on another device e.g. PC or Unix endstation). Once they are backed up remove (rm command) any Network Interface Module files that are not used to conserve flash memory usage.   For example, if the token ring module (TSM) is not installed in the IBM 8274 Nways LAN RouteSwitch, the tsm.img and tsm.pga files may be removed.

IMPORTANT: The chassis requires a minimum amount of space available in FLASH for booting. When all images are loaded in FLASH, there needs to be at least .5 MB free. If your images exceed this amount, you MUST delete some images from FLASH.  Also if you are running with any configuration other than the default, you will need additional available FLASH.

# Multiple Image Requirements

Multiple images are required for some modules.  The WSM modules and some of the ASM and CSM modules require more than one image. Below is a table showing which board types require more than one.  Note that all modules require the mpm.img image.

| #### IBM 8274 Nways LAN RouteSwitch |  |  |  |  |
| --- | --- | --- | --- | --- |
| Board Type | Images (besides mpm.img) |  |  |  |
| CSM-AB-DS1 | cell.img | asm.img | t1e1drv.img |  |
| CSM-AB-E1 | cell.img | asm.img | t1e1drv.img |  |
| CSM-AB-CE-T1 | cell.img | asm.img | t1e1drv.img | asmcedrv.img |
| CSM-AB-CE-E1 | cell.img | asm.img | t1e1drv.img | asmcedrv.img |
| CSM-AB-DS3 | cell.img | asm.img | ds3e3drv.img |  |
| CSM-AB-E3 | cell.img | asm.img | ds3e3drv.img |  |
| CSM-ABB-155x | cell.img | asm.img |  |  |
| ASM-CE-T1/OC3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
| ASM-CE-E1/OC3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
| ASM-CE-T1/DS3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
| ASM-CE-E1/E3 | t1e1drv.img | asmce.img | asm.img | asmcedrv.img |
| WSM | wsm.img |  |  |  |
| WSM-BRI | wsm.img | isdn.img | isdn.img |  |
| WSM-FT1 | wsm.img | t1e1drv.img | t1e1drv.img |  |
| WSM-FE1 | wsm.img | t1e1drv.img | t1e1drv.img |  |

| #### IBM 8277 Nways Ethernet RouteSwitch |  |  |  |  |
| --- | --- | --- | --- | --- |
| Board Type | Images (besides mpm.img) |  |  |  |
| RSASM2-155 | ae4.img | em4.img |  |  |
| RSASM2-622 | ae4.img | em4.img |  |  |
| RSESM2 | em4.img |  |  |  |
| RSESM2 | em4.img |  |  |  |
| RSWSM-BRI | w4.img | isdn.img | em4.img |  |
| RSWSM-FE1 | wsm.img | t1e1drv.img | em4.img |  |
| RSWSM-FT1 | wsm.img | t1e1drv.img | em4.img |  |

# ATM Services

| ATM Services | ASM (Midway) | ASM2 (SAHI) | ASX (SAHI) |
| --- | --- | --- | --- |
| LEC | X | X | X |
| CIP | X | X | X |
| X-LANE |  |  |  |
| 1483 | X | X | X |
| PRIV | X |  |  |
| Trunking |  |  |  |
| 1483 | X | X | X |
| PRIV | X | X | X |
| PTOP |  |  |  |
| 1483 | X | X | X |
| PRIV | X | X | X |

# Example of IBM 8274 Model GRS Nways LAN RouteSwitch FLASH Requirements for Release 3.4.7

| Images |  |  |  | Grand Total Flash Size |
| --- | --- | --- | --- | --- |
| Files | Basic Images |  |  |  |
|  | BOOTROM |  |  | 512,000 |
|  | FFS Spare |  |  | 512,000 |
| 1 | MPX Image |  |  | 1,742,635 |
| 2, 3 | MPX Config (Potentially 2 files) |  |  | 65,536 |
|  | Potential Config information |  |  | 200,000 |
| 4 | MPX Log |  |  | 20,000 |
| 5 | MPX Misc |  |  | 4,096 |
|  |  |  | Subtotal | 3,056,267 |
| Network Interface Images |  |  |  |  |
| 6 |  | asm.img | ASX | 843,574 |
| 7 |  | esx.img | ESX 12 & 32 port | 26,702 |
| 8 |  | tsx.img | TSX (MPX & NI) | 208,750 |
|  |  |  | Subtotal | 1,079,026 |
| Dynamically Loadable Images |  |  |  |  |
| 9 |  | diagx.img* | Diagnostic MPX image | 251,422 |
| 10 |  | desx.img* | Diagnostic ESX image | 26,792 |
| 11 |  | fwd.img* | Firewall image | 138,113 |
|  |  |  | Subtotal | 416,327 |
|  |  |  | Total | 4,551,620 |
|  |  | 8 MB = 8,388,608 |  | 8,388,608 |
|  |  |  |  | -3,836,988 |

*When utilizing these dynamically loadable images, we recommend 32MB DRAM.

WARNING:

You will have to remove unused network interface images from flash to make room for loadable images if the amount of remaining flash is less than 512,288 bytes.

# Examples of IBM 8274 Nways LAN RouteSwitch FLASH Requirements for Release 3.4.7

| Images |  |  | Grand Total Flash Size | ATM Edge Flash Size | ATM Edge w/ Adv. Rtg Flash Size | Ethernet / FDDI Flash Size | Ethernet / FDDI w/Adv. Rtg. Flash Size | Cell Flash Size | Cell with LSM Flash Size | Wide Area Edge Flash Size | Wide Area Edge w/Adv. Rtg. Flash Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Files | Basic Images |  |  |  |  |  |  |  |  |  |  |
|  | BOOTROM |  | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 |
|  | FFS Spare |  | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 | 262,144 |
| 1 | MPM Image |  | 1,704,665 | 1,704,665 | 1,704,665 | 1,704,665 | 1,704,665 | 1,704,665 | 1,704,665 | 1,704,665 | 1,704,665 |
| 2, 3 | MPM Config (Potentially 2 files) |  | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 | 65,536 |
|  | Potential Config information |  | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 | 200,000 |
| 4 | MPM Log |  | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 | 12,048 |
| 5 | MPM Misc |  | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 | 4,096 |
|  |  | Subtotal | 2,510,633 | 2,510,633 | 2,510,633 | 2,510,633 | 2,510,633 | 2,510,633 | 2,510,633 | 2,510,633 | 2,510,633 |
| Network Interface Images |  |  |  |  |  |  |  |  |  |  |  |
| 6 | asm.img | ASM Driver | 843,574 | 843,574 | 843,574 |  |  | 843,574 | 843,574 |  |  |
| 7 | asmce.img | ASM CE | 47,486 |  |  |  |  |  |  |  |  |
| 8 | asmcedrv.img | ASM CE Driver | 156,532 |  |  |  |  |  |  |  |  |
| 9 | e12.img | ESM 12-port | 27,633 |  |  |  |  |  |  |  |  |
| 10 | esm.img | ESM 8-port | 27,219 |  |  |  |  |  |  |  |  |
| 11 | fesm.img | FAST ESM (HSM1) | 31,934 |  |  |  |  |  |  |  |  |
| 12 | fesm2.img | FAST ESM (HSM2) | 31,901 |  |  |  |  |  |  |  |  |
| 13 | fsm.img | FSM (MPM & NI) | 291,754 | 291,754 | 291,754 | 291,754 | 291,754 |  |  | 291,754 | 291,754 |
| 14 | mesm.img | Mammoth based ESM | 25,950 | 25,950 | 25,950 | 25,950 | 25,950 |  |  | 25,950 | 25,950 |
| 15 | mtsm.img | TSM (MPM & NI) | 218,218 | 218,218 | 218,218 |  |  |  |  | 218,218 | 218,218 |
| 16 | tsm.img | TSM (MPM & NI) | 192,394 |  |  |  |  |  |  |  |  |
| 17 | t1e1drv.img | T1/E1 NI driver | 75,109 |  |  |  |  |  |  |  |  |
| 18 | ds3edrv.img | DS3/E3 NI driver | 53,984 |  |  |  |  |  |  |  |  |
|  |  | Subtotal | 2,023,688 | 1,379,496 | 1,379,496 | 317,704 | 317,704 | 843,574 | 843,574 | 535,922 | 769,473 |
| Dynamically Loadable Images |  |  |  |  |  |  |  |  |  |  |  |
| 19 | cell.img* | OmniCell image | 854,297 |  |  |  |  | 854,297 | 854,297 |  |  |
| 20 | diag.img* | Diagnostic MPM image | 241,730 |  |  |  |  |  |  |  |  |
| 21 | dni.img | Diagnostic NI image | 18,767 |  |  |  |  |  |  |  |  |
| 22 | dmesm.img | Diagnostic MESM image | 26,052 |  |  |  |  |  |  |  |  |
| 23 | wsm.img | FR NI image | 425,922 |  |  |  |  |  |  | 425,922 | 425,922 |
| 24 | isdn.img | ISDN BRI | 202,730 |  |  |  |  |  |  |  |  |
| 25 | gated.img | Advanced Routing | 502,011 |  | 502,011 |  | 502,011 |  |  |  | 502,011 |
| 26 | mrd.img | IP Multicast Routing | 70,595 |  |  |  |  |  |  |  |  |
| 27 | lsm.img* | LANE Server | 175,862 |  |  |  |  |  | 175,862 |  |  |
| 28 | fwd.img* | Firewalls | 138,113 |  | 138,113 |  | 138,113 |  |  |  | 138,113 |
|  |  | Subtotal | 2,656,079 | 0 | 640,124 | 0 | 640,124 | 854,297 | 1,030,159 | 425,922 | 1,066,046 |
|  |  | Total | 7,190,400 | 3,890,129 | 4,530,253 | 2,828,337 | 3,468,461 | 4,208,504 | 4,384,366 | 3,472,477 | 4,346,152 |
|  | 4 MB = 4,194,304 |  | ---- | 4,194,304 | ---- | 4,194,304 | ---- | ---- | ---- | 4,194,304 | ---- |
|  | 8 MB = 8,388,608 |  | 8,388,608 | ---- | 8,388,608 | ---- | 8,388,608 | 8,388,608 | 8,388,608 | ---- | 8,388,608 |
|  |  |  | -1,198,208 | -304,175 | -3,858,355 | -1,365,967 | -4,920,147 | -4,180,104 | -4,004,242 | -721,827 | -4,042,456 |

* When utilizing these dynamically loadable images, we recommend 32MB DRAM and 8 MB FLASH.

WARNING:

You will have to remove unused network interface images from flash to make room for loadable images if the amount of remaining flash is less than 512,288 bytes.

# Examples of IBM 8277 Nways Ethernet RouteSwitch FLASH Requirements for Release 3.4.7

| Images |  |  |  | IBM 8277 Nways Ethernet RouteSwitch Grand Total Flash Size | ATM Uplinks Flash Size | Fast Ethernet Uplinks Flash Size | Wide Area Edge Flash Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Files | Basic Images |  |  |  |  |  |  |
|  | BOOTROM |  |  | 524,288 | 524,288 | 524,288 | 524,288 |
|  | FFS Spare |  |  | 262,144 | 262,144 | 262,144 | 262,144 |
| 1 | MP4 Image |  |  | 1,644,228 | 1,644,228 | 1,644,228 | 1,644,228 |
| 2, 3 | MPM Config (potentially 2 files) |  |  | 65,536 | 65,536 | 65,536 | 65,536 |
|  | Potential Config information |  |  | 200,000 | 200,000 | 200,000 | 200,000 |
| 4 | MPM Log |  |  | 12,048 | 12,048 | 12,048 | 12,048 |
| 5 | MPM Misc |  |  | 4,096 | 4,096 | 4,096 | 4,096 |
|  |  | Subtotal |  | 2,712,340 | 2,712,340 | 2,712,340 | 2,712,340 |
|  | Network Interface Images |  |  |  |  |  |  |
| 6 | ae4.img* | ATM & Ethernet Driver |  | 825,948 | 825,948 |  |  |
| 7 | em4.img | Ethernet Driver |  | 28,547 | 28,547 | 28,547 | 28,547 |
| 8 | ws4.img | WSM Driver |  | 495,066 |  |  | 495,066 |
|  |  | Subtotal |  | 1,349,561 | 854,495 | 28,547 | 523,613 |
|  | Advance Feature Images |  |  |  |  |  |  |
| 9 | p4diag.img* | Diagnostic MPM image |  | 139,698 |  |  |  |
| 10 | dem4.img | Diagnostic Ethernet Driver |  | 28,658 |  |  |  |
| 11 | gated.img* | Advanced Routing |  | 502,011 |  |  |  |
| 12 | mrd.img* | IP Multicast Routing |  | 70,595 |  |  |  |
| 13 | isdn.img | ISN BRI |  | 202,730 |  |  |  |
| 14 | t1e1drv.img | T1/E1 NI driver |  | 75,109 |  |  |  |
| 15 | fwd.img* | Firewall image |  | 137,784 |  |  |  |
|  |  | Subtotal |  | 1,016,887 | 0 | 0 | 0 |
|  | Total |  |  | 5,078,788 | 3,566,835 | 2,740,887 | 3,235,953 |
|  | 4 MB = 4,194,304 |  |  | 4,194,304 | 4,194,304 | 4,194,304 | 4,194,304 |
|  |  |  |  | 884,484 | -627,469 | -1,453,417 | -958,351 |

*These images are dynamically loadable images.  When running lsm.img and fwd.img, it is recommended that the user install 32MB DRAM.

# Release 3.4.7 IBM 8277 Nways Ethernet RouteSwitch Images Required

|  | RS-5024/3032/2032 | OSASM | RSESM2 | RSWSM-BRI-SC-1 | RSWSM-FE1/FT1-SC-1 |
| --- | --- | --- | --- | --- | --- |
| mp4.cmd | YES | YES | YES | YES | YES |
| em4.img | YES | YES | YES | YES | YES |
| mp4.img | YES | YES | YES | YES | YES |
| ae4.img | YES | YES | NO | NO | NO |
| p4diag.img | YES | YES | YES | YES | YES |
| dem4.img | YES | NO | YES | NO | NO |
| ws4.img | YES | NO | NO | YES | YES |
| isdn.img | YES | NO | NO | YES | NO |
| t1e1drv.img | NO | NO | NO | NO | YES |

# IBM Operating System (IOS) Version 3.4.5 Problem Resolutions

## IOS Problems fixed in this release

- ESX ports should default to 10/Half duplex when autonegotiation fails. (PR 21307)

- Occasionally, the VC is not created for PNNI after a reboot from IBM 8274 Nways LAN RouteSwitch. (PR 21466)

- Dead primary MPM-1G console port in redundant configuration. (PR 22324)

- Source-Route IPX routing data corruption on Token-Ring, FDDI & ATM (IBM 8274 Nways LAN RouteSwitch and IBM 8274 Model GRS Nways LAN RouteSwitch). (PR 22325)

- modvp on an ASM/FCSM does not cycle through all the virtual ports. (PR 22730)

- Static MACs not retained after reboot. (PR 22941)

- Redundancy with MSS 2.2 and LSM on IBM 8274 Nways LAN RouteSwitch causes crash on the IBM 8274 Nways LAN RouteSwitch. (PR 23036)

- Under heavy traffic, big gap between LE_ARP requests for an unknown Destination Address. (PR 23580)

- Route Cache is not flushed on disabling an RouteTracker VLAN. (PR 23639)

- Generated errors are not enough to partition the port on an RS-5024. (PR 23684)

- FCSM-II initialized while running unicast and multicast traffic. (PR 23706)

- Fastpath routing fails when a Bigfoot Token Ring port is set to FDx mode. (PR 23763)

- Switch locks up if Traffic Descriptor values are set to a large number. (PR 23779)

- NI cash on a WSM board followed by tWsmdDrv crash takes down the PPP connection. (PR 23842)

- ICMP redirect on Frame Relay WAN causes routing problem. (PR 23956)

- Token Ring client sometimes can not insert into the ring. (PR 23972)

- ASM-E3, change of signalling back to default after reboot. (PR 24002)

- MAC address rules in Group Mobility do not work. (PR 24010)

- Recieving error message S_objLib_OBJ_ID_ERROR during attempted telnet. (PR 24013)

- Topology change fluctuation on Mammoth-based Token Ring modules. (PR 24106)

- Need temporary board reset when port partitions and fldQ still stuck. (PR 24112)

- Memory leak when performing NI source routing on TSX and ASX. (PR 24150)

- Buffer leak in SAHI reset logic. (PR 24167)

- Running a second pmon session on a port crashes the switch. (PR 24186)

- IOP has channels in memory but not in hardware. (PR 24194)

- IOP does not send data to the SAR. (PR 24197)

- ATM cell discards in source routed Token Ring environment. (PR 24210)

- Using OSPF when calculating more than 500 route system locks up. (PR 24225)

- csmsigTask.#.dmp was created when switch crashed at csmsigDestroyCall & csmsigmemPoolAlloc (PR 24269)

- Mirroring an 802.1q port to a non-q port stops the traffic for both ports. (PR 24290)

- Add SAP filter to allow for separation of traffic. (PR 24293)

- Source Route Explorer frames are sent out to one port only on a Mammoth based Token Ring.(PR 24307)

- IPX routing with IBM 8274 Model GRS Nways LAN RouteSwitch and HRE-X does not work. (PR 24327)

- Token Ring routed packets age-out when pings come from external device. (PR 24338)

- On TSM-16 and TSX-16 with virtual ring configured, duplicate MAC test on OS2 fails. (PR 24390)

- GSX-FM-4 non-802.1Q tagged ports do not pass traffic if one port is 802.1Q tagged. (PR 24391)

- Dump routine does not work on Mammoth based Token-Ring modules. (PR 24405)

- Port partition feature required on Token Ring ports since bad ports can cause crashes. (PR 24406)

- Switch stopped routing for no apparent reason gave screen dump. (PR 24412)

- When a CSM-U submod1 fails and is replaced, a reboot is required for VCs to return to submod1. (PR 24417)

- PCAM test is failing good boards because delay is too short. (PR 24444)

- tTsmDrv task crashes from facility to catch accessing low memory. (PR 24459)

- mboxSend semTake failed when pinging module message upon bootup on TSM-C-6. (PR 24488)

- Links on the TSM-CD6 and fiber board go up and down. (PR 24496)

- No_Directed_Broadcast does not work. (PR 24498)

- Unicast frame with RIF forwarded to SR-only port that does not match RIF info. (PR 24499)

- FDDI trunk ports will not forward if other trunk services are not terminated. (PR 24500)

- FDDI crash causing entire switch to crash. (PR 24587)

- Need to mask the second digit of part number in ESX-100FM-12 and ESX-100C-12. (PR 24656)

- High utilization of threXrc_timer around 75 - 98%. (PR 24661)

- OSPF not running with FCSM-I redundancy. (PR 24714)

- Incorrect mib values returned in ifTable for GSX and ESX modules. (PR 24722)

- IBM 8274 Model GRS Nways LAN RouteSwitch runs out of memory by 2 Mb per day and finally crashed after 5 days. (PR 24744)

- Problem with structured Circuit Emulation. (PR 24822)

- PTOP Admin/Oper Status - SNMP does not disable PTOP Service. (PR 24868)

- CPU utilization goes up when SNMP queries iproute and ipNetToMedia tables. (PR 24869)

- Primary MPM is reset by Secondary MPM after imgsync and reset. (PR 24875)

- Mammoth Token Ring cards violate the Token Ring standard for beaconing. (PR 25024)

- WSM IBM 8274 Nways LAN RouteSwitch & IBM 8277 Nways Ethernet RouteSwitch fails with PPP compression, line errors and heavy load. (PR 25055)

- Flood limit credit discards may cause the flood queue to become stuck. (PR 25063)

- On a Mammoth TSM board, no dump file was created when the flood queue became stuck. (PR 25064)

- Unable to pass a single structured channel between IBM and Cisco. (PR 25085)

- TSX-C-32 modules do not pass a Source Routed SABME Poll. (PR 25155)

- Can not set cas on a T1 using an ASM-CE interface. (PR 25223)

- IBM 8274 Model GRS Nways LAN RouteSwitch is leaking approximately 1 mb of memory a day. (PR 25249)

- atmLecConfigTable and atmLecstatusTable time out during SNMP GETS. (PR 25332)

- TSX-CD-16 ports intermittently deinsert from the ring for several seconds. (PR 25344)

- Switch crashed with tClearRouteCache and threXstats suspended. (PR 25356)

- Source Routed attached PC looses connectivity when an IBM 8274 Nways LAN RouteSwitch sends an arp without rif. (PR 25408)

- HRE-X crashed on tClearRouteCache & threXstats. (PR 25434)

- SNMP Agent crashes on GetEmptyAtmLecTables. (PR 25451)

- IP ping fails directly connected two switches, both ports are forwarding. (PR 25461)

- VseReceive crashed with a "memory address not aligned" exception. (PR 25598)

## IOS Problems no longer an issue in this release

- When configuring an IPX router port for source routing, an IPX ping across a trunk port will not work. (PR 2332/11277/12922/13090)

- IBM 8274 Nways LAN RouteSwitch switches RIP II multicast packets instead of consuming them entirely. This causes a slight increase in traffic on the network. No longer recreatable in the current release. (PR 7448)

- IP Routing Table displays a strange route while the GateD routing table does not. (PR 12854)

- If the mpm.cmd file is removed and a renounce command is performed on the primary mpm, the new primary mpm issues command sls to check on the new secondary mpm. The terminal may hang for about 3 minutes, and then the switch reboots. (PR 13612/21333)

- User's UNI Autoconfiguration is not working with CSM. When the user NIC, which has a function UniAutoConfig, is connected to one port of CSM, the switch did not reply with the right parameters. When the user NIC, which has the function of UNI Autoconfiguration, tries to configure UNI signaling version with the switch, the user asks about the atmfLayerTable parameters setting by the switch. For this inquiry, the switch responds with "No Such Name". Therefore, the connection is failed. (PR 13811)

- Terminating a port monitoring session via pmd after killing the user with write/read privilege causes Switch to crash. (PR 13892/20904)

- Transparent Bridge needs duplicate MAC aging timer. (PR 14101)

- Unable to delete LANE service. Works as designed. (PR 14523)

- Ethernet attached IPX endstations that are switched to a Token Ring attached server configured for SNAP encapsulation will not work correctly. This will occur for IPX endstations attached to the ESM-C12 or ESM-100C-8. (PR 14650/22180)

- When the switch routes IPX traffic between Ethernet stations configured for Ethernet RAW and Token Ring stations configured for SNAP across an ATM LANE backbone (configured for Ethernet LEC), there is a problem with the construction of the frames. (PR 14736/22182)

- If user configures PVCs and SVCs using a CIP service on a back to back configuration between an ASM module and ASM2 module, the PVC CIP will not pass any traffic. No longer recreatable in the current release. (PR 14811/14818)

- No second CIP setup message transmitted after the first fails. (PR 20652)

- RIP frames contain bogus data in fields that 'should be' zero fields for RIP V.1 frames. No longer recreatable in the current release. (PR 20876)

- Mammoth modules are intermittently change MAC address after 3 consecutive runts. (PR 21014)

- FCSM-II with TSM-CD-16 causes switch to crash. (PR 21431)

- Ethernet link may not come up with some NIC cards and less than 15 meters of cable. (RS-5024 and RS-4016 only). (PR 21472/21897)

- If the modvp command is used to change the virtual port's MAC address to the last ATM service on a physical port, the chassis manager will receive a board down event and the ATM port will disappear. User will have to reboot the switch in order to see the ATM port. This will only happen if modvp is down to the last ATM service on any physical port. (PR 21497)

- GSX may crash after a hot swap. (PR 21674)

- Spanning tree may fail when connecting 14 groups between 2 boxes on the GSX-4. (PR 21756)

- ESM-100FM-8 port will stay in the partition state. (PR 21977)

- ATM fabric reinitializes after removal of secondary MPM. (PR 22009)

- UI shows "inactv" on some 802.1Q ports after user creates Q ports on GSX-2. No longer recreatable in the current release. (PR 22108)

- Flood queue may not dequeue correctly when deleting IEEE Q groups on the GSX-2. (PR 22145)

- IPX routing may fail when one of the router ports is set to either FDDI or Token Ring on IBM 8274 Model GRS Nways LAN RouteSwitch (PR 22183)

- Reset does not work on GSX-FM-4W. (PR 22031)

- SNMP information for "if speed & desc" are not correct. (PR 22214)

- SNA packet corruption under heavy Token Ring NI load. (PR 22358)

- PTSEs are not re-originated after pncfg, modify ATM address and save. No longer recreatable in the current release. (PR 22466)

- UI always shows 100base-f ports as 10mbit speed. (PR 22502)

- ATM switch hangs and outputs "xMSGQ" messages. (PR 22519)

- 64 bytes tagged 802.1Q packets not forwarded by Omni. (PR 22654)

- Port-monitor display may show incorrect timestamp for Rel.Time, Delta Time and Abs. Time. (PR 22711)

- Race condition in accepting telnet connections locks UI. (PR 22729)

- If more then one service is created for a group on an ASM or FCSM, the modvp command will only allow you to modify the last one created. (PR 22730)

- When the number of VC's exceeds the SAR limit, invalid VC's are created. (PR 22747)

- Changing autonegotiate mode on gigabit ports may cause the port to stop passing traffic. (8277 Model 424 (8277-42G) and RS-GSM only) (PR 22764)

- Routing with Source Routing Token Ring encapsulation causes slow path packet duplication. (PR 22771)

- Traffic stops if Port Monitoring is turned on then off on an 802.1q port. (PR 22834)

- SSCOP may go up and down on the CSM wth ESM-100C-32W installed. (PR 22895)

- Receive a bad semaphore message when terminating pmon with the pmon command. (PR 22924)

- TSX port will stay in Auto mode after changing it to either 16M or 4M. (PR 22939)

- Static MACs may not be retained after reboot. (PR 22941)

- Gated routes command will hang when multiple OSPF areas defined in GATED.CONF. (PR 22958)

- Cisco LEC may cause problem with transmitting over either of the data direct VC's. (PR 22984)

- Unable to configure SCR lower than 535kbit when setting up bandwidth groups. (PR 23111)

- Point to Multipoint VC's on a leaf virtual circuit are bidirectional instead of unidirectional. (PR 23160)

- Routing between token ring and ethernet broken for clients behind Source Routing bridge. (PR 23161)

- Fwt and fwtvl only show mac addresses for first instance specifying slot and port. (PR 23172)

- Modvp descriptions longer than 30 characters causes corruption on the display. (PR 23211)

- Vrp using token ring source routing encapsulation is unable to reach an unresolved source routing bridge destination on a TSM-CD-16. (PR 23231)

- When moving stations within a mobile groups, ports may get partitioned. (PR 23414)

- ESM-100C-FD2 ports lockup as traffic is being received but not transmitted. (PR 23334)

- The switch crashes with ESX-FM-24 and ESX-100-C12. (PR 23472)

- ATM statistics per Vpi/Vci do display the RX cells for both RX and TX. (PR 23503)

- Port stays partitioned even after the port speed mismatch has been corrected. (PR 23529)

- PNNI peers disconnect when processing IP multicast packets. (PR 23567)

- FDDI trunking causes high MPM utilization due to source learning problem. (PR 23719)

- Group information of dlcis gets lost on reboot. (PR 24221)

- Frames are being tagged with the wrong vlan ID. (PR 24296)

- CSM-U submod1 fails and is replaced. Reboot is required for VCs to return to submod1. (PR 24417)

- Incorrect mib values returned in ifTable for GSX and ESX modules. (PR 24722)

- Dump file will be created when a TSX flood queue gets stuck. (PR 25064)

- WSM running frame relay received large number of inbound errors and left a dump file. (PR 25488)

# Known IBM 8274 Model GRS Nways LAN RouteSwtich, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Problems/Limitations

The problems listed here include problems known at the time of the product’s release.  Any problems not discussed in this section should be brought to the attention of the IBM Technical Support organization as soon as possible.

## Known Advanced Routing, GateD & Routing Problems/Limitations

- Problem Report 4122 The total datagrams received and total PDU’s requested for transmit statistics displayed in the ips command can be incorrect. The switch uses TCP packets to enforce paging for the user interface displays, which causes these statistics to increase even when no external IP traffic is being received or transmitted. Work-around: Turn off the switch “more” function, which allows multiple page user interface output to be displayed one page at a time. This is accomplished by issuing the nomore UI command.

- Problem Report 5611 The exact parameter of the gated import filtering statement does not work properly. If exact is configured then no routes will pass the specified filter. In addition, the refine parameter of the import filtering statement does not work properly. Refine does not allow subnet routes to pass the filter. Host routes are allowed as specified. Work-around: NONE

- Problem Report 6060 - OSPF does not pass on the route tag in its external LS advertisement. Work-around: NONE

Problem Report 6534 When Frame Relay is used with OSPF and more than two switches are interconnected on the same Frame Relay service with different DLCI's but the same IP network address, and the switches are not fully meshed, then routing connectivity among all the switches is not maintained. This is a byproduct of the way OSPF and IP routing operates. Work-around: This configuration cannot be expected to work in normal IP routing environments. This requires changes to the way routing works in a non-meshed frame relay network. Those changes have been made in the RIP routing supported by the base IBM 8274 Nways LAN RouteSwitch software. If this configuration is required, use the IBM 8274 Nways LAN RouteSwitch RIP implementation, not the Advanced Routing package.

Problem Report 6947, 6943 IPX clients were unable to connect to any IPX server when the router port encapsulation was set to SR_FDDI_LLC or SR_FDDI_SNAP and the client and server did not have source routing enabled. - Work-around: Do not set router port encapsulation to either SR_FDDI_LLC or SR_FDDI_SNAP if source routing is not configured on the bridge ports.

Problem Report 7063 ipxping does not work over FDDI trunking with TR_SNAP & TR_LLC router port encapsulation. - Work-around: ipxping does work using TR_SNAP & TR_LLC router port encapsulations over other media types.

Problem Report 9801 OSPF neighbor list is not displayed with nonbroadcast interfaces. Work-around: NONE

Problem Report 10012 Gated RIP v1 and v2 never remove peers from "rip peers" table. - Work-around: NONE

Problem Report 10380/12016 Routing Tables are not updated with PPP connection to CISCO OSPF. Work-around: Configure nonbroadcast media type for your PPP interfaces.

Problem Report 11560 OSPF ASE routes show up as STATIC with IPR command. Work-around: NONE

Problem Report 14365 A problem can occur where an ISDN interface is unable to receive a call, due to a problem negotiating layer 2. If the interface is configured with outgoing calls, making a call will clear the condition. If the interface is configured with incoming calls only, then it is necessary to reboot the switch to clear the condition. The problem occurs at initialization, so that once the port comes up, the condition will not occur. Workaround: Configure both switches for both incoming and outgoing calls.

Problem Report 14505 There is a slight chance that IP Multicast Routing can crash in the vseReceive Task if modifying anything on a group while it is running a large amount of multicast traffic. After any group modifications IP Multicast Routing automatically restarts itself to pick up any possible changes. There is a very small timing window where it can crash. - Workaround: When modifying a group while running the MRD (IP Multicast Routing) image, issue an "mrstop" (to stop multicast routing). Then modify and save your group. Then issue an mrstart command (to start multicast routing).

Problem Report 14741/22432 RIP route metrics are not updated in the kernal routing table when the gated.conf configuration file is updated with a new RIP metric and gated is reconfigured. Work-around: Execute a ripflush command after the gated reconfiguration is completed.

Problem Report 20050 - Laptop clients using the Xircom or Megahertz ethernet PCMCIA cards and running solely the IPX stack with the framing type set to autonegotiation might experience intermittent connectivity issues connecting to IPX servers. Work-around: Connect the station to a hub. The hub connected to the ethernet port fixes the problem.

- Problem Report 20909 After a reboot, RIP does not advertise internal router ports. Work-around: Turn rip off on each routing port that does not work. Then turn each of them on. They will function from that point on until the next switch reboots.

- Problem Report 21673 Cisco does not recognize oan inverse arp from a Frame Relay IBM box translated through an FRF.8 Frame Relay to an ATM gateway. . Work-around: NONE.

Problem Report 21981 If Gated's RIP2 receives a routing update with a next hop address of 0.0.0.0, it does not properly interpret the route. The RFC states that the routes should be installed in the routing table using the send of the RIP update as the next hop gateway. Gated drops the route. Work-around: Make sure the router sending the RIP update inserts its own address as the next hop gateway in its routing updates instead of 0.0.0.0.

Problem Report 23853 Routing frames larger than1500 byes from FDDI to Ethernet can cause corruption. If packets larger than 1500 bytes are routed between FDDI and ethernet using software routing and the router port encapsulation is Ethernet II then file corruption will occur. Work-around: NONE.

Problem Report 23920 When the PVC's are configured to use compression, the switch may show frame relay discard errors. Work-around: Turn compression off.

Problem Report 24096 IPX source routed packets originated on Token Ring clients or servers and sent over FDDI do not get cached properly in the system. This causes one direction of the client-sever communication to be routed using the software "slowpath" which will produce slow response and transfer times. Work-around: NONE.

Problem Report 24650 Having CPU utilization at 100% prevents insertion of route cache entries. Work-around: Reduce the CPU utilization..

Problem Report 25120 IP routing instance fails if only port in a VLAN is disconnected and reconnected. Work-around: modvl vlan and save will clear the issue until the switch reboots..

Problem Report 25268 If a static route is configured where the next hop router is reachable over one of the switch's IP interfaces and the IP address of that interface is changed using modvl, and if the removal of the static route is attempted using risr, the risr will fail. Work-around: Remove the static route before changing the IP address of the group or vlan.

Problem Report 25363 IP Multicast routing is showing a slow performance over ATM. Work-around: NONE.

Problem Report 25454 IP routing does not operate if an IP Group/VLAN has only 802.1q tagged ports in it. Work-around: Place another active port of any type (other than 802.1q) in the same group as the 802.1q tagged port.

Problem Report 25563 MPX IPX fastpath routing will not let old IPX clients connect. Work-around: NONE.

Problem Report 25613 Multicast frames are truncated when forwarded twice over ATM Work-around: NONE.

## Known ATM Problems/Limitations

- Problem Report 4730 Peak cell rates do not get initialized on an ATM-DS3 module if ATM-OC3 module was previously in the same slot. Work-around: NONE

Problem Report 12557 Under certain load/traffic conditions, the IBM 8274 Nways LAN RouteSwitch telnet client may drop the connection. Work-around: NONE

Problem Report 12688 mvc does not modify the existing connection but instead creates another one. Work-around: NONE

Problem Report 13066 ATM-DS3 and E3 modules may cause a memory protection error and lock up after sublayer changes. Work-around: NONE If memory protection error occurs, user has to reboot the switch.

- Problem Report 13392 With a large number of LECs, the switchover between primary and secondary is slow. Work-around: NONE

- Problem Report 13651 Calling parameters, range and keyLen, of cmHashListFindIndex got a bogus number "0". These zero values caused "ZERO DEVIDE" trap. Work-around: Turn on ERRCHK

- Problem Report 13942 When a virtual connection is created with an out of range VCI, an entry is created in the ATM connections table anyway. Work-around: Do not create a virtual connection with an out of range VCI.

- Problem Report 14279 Under very rare circumstances, the cas (create ATM service) will fail with the error message of "Creating service failed". The state of the service shows "disabled". Work-around: Delete the service and re-create it again.

- Problem Report 21742 Under overload conditions, ASM2 may stop transmitting data. Work-around: Reset the board and do not continuously overload the bandwidth group.

- Problem Report 22162 When two switches with redundant port ASXs connected back to back on both the physical ports (A & B) are rebooted together, then one switch will come up on Port A and the other switch on Port B. Work-around: To avoid this condition, reboot these back to back switches one at a time waiting until that switch comes up.

Problem Report 22597 Under a heavy continuous load, signaling goes down triggering the LEC to reset. If the heavy continuous load persists, while the LEC recovers, the recovered LEC will not be able to resolve the MAC address related to the load. This problem persists even if the load is stopped. Work-around: NONE

Problem Report 24715 If PhyB is active before the reset or hot swap of a redundant port, ASM-PhyA never becomes active when PhyB becomes inactive. Work-around: Before one resets or hotswaps a redundant ASM port, make sure PhyB is inactive.

Problem Report 24932 When an SSCOP down/up event occurs rapidly on an ATM link where auto-activated lecs are configured, the following message may scroll on the screen. "Service already running on port slot/port". Work-around: Use static lecs or keep SSCOP up.

Problem Report 24985 ASM's and FCSM-I's may get in a situation where the CAM fails. Work-around: Reset the ASM/FCSM module.

Problem Report 25318 In structured circuit emulation, alarms (OOF, OOMF, RDI, multifram RDI) are not propagated to the far-end. Work-around: NONE.

Problem Report 25574 atmLecActualLanName in IBM-atm.mib is showing empty when the elan name is "default" Work-around: NONE.

## Known Authenticated Services Problems/Limitations

### Client

Problem Report 8321 If a user is defined with a third party authentication mechanism such as SecureID and it is not in place, the telnet client hangs. The AVLAN debug message on the switch shows "Trying to send to client SecureID request failed". - Work-around: Verify all third party authentication mechanisms are available and reachable.

Problem Report 14081 When someone kills a session running the telnet client on a switch where both switches have the WRITE privilege, the session is cancelled but the WRITE privilege is not cancelled for both switches. Work-around: Kill the server end of the telnet session on the second switch or eventually the WRITE privilege will timeout.

### Switch

Problem Report 9177 Switch remains in connected state long after AMC goes down. If only one switch is connected to the AMC, it will "unconnect". If there is more than one switch connected to the AMC, then only one switch "unconnects" and the other switches stay "connected". Work-around: NONE

Problem Report 10126 When a switch is configured to run VLAN Authentication only, the EMC Firewall can still get control of the switch and download rules. Work-around: NONE

Problem Report 14801 User can not use Telnet to perform authentication over Token Ring. Work-around: NONE

## Known Bridging Problems/Limitations

- Problem Report 5854 Modifying the Spanning Tree forward delay parameter using the stc command may cause the forwarding table entries not to be flushed and relearned properly. Work-around: NONE If you change the forward delay, it is recommended that the switch be rebooted.

Problem Report 23899 Can not delete policy rules and VLANs using the rdat command Work-around: NONE.

Problem Report 24258 Camstat shows the maximum number of Cam's available as a negative number and usage as "0". Work-around: NONE.

Problem Report 24683 The viqs -usage command is incorrect. If you do a viqs <return>, the usage comes back with the wrong syntax for this command: "Usage: viqstats [slot/port] [grp-id]". There is no viqstats command, and the "slot/port" is not optional. It should read: "Usage: viqs slot/port [grp-id]". Work-around: Type the vig command instead of the viqstas.

Problem Report 25337 Connection of RS-4024 to 8274-900 over FE causes high utilization and source learning Work-around: NONE.

Problem Report 25370 If 30 characters are exceeded along with spaces when adding a port description, the data within the various fields gets corrupted. Work-around: Do not exceed the 30 character limit for description

Problem Report 25444 Static Macs move from assigned groups to another group upon reboot. Work-around: NONE.

Problem Report 25651 Both tagged and untagged frames are found on the default group if the user assigns zero on a default group. Work-around: NONE.

## Known CSM/FCSM Problems/Limitations

Problem Report 6721 Switch needs to be rebooted after changing the signalling version on CSM port(s). - Work-around: NONE

Problem Report 6901 When selecting the max vci of 255, the vci is not included and system creates a vpc. - Work-around: Do not select maximum vci of 255.

- Problem Report 7380 - Vvc and vcs do not work for individual connections on an FCSM board. - Work-around: List all connections.

- Problem Report 7869 - The calculation of PCR for CBR with CSMs with values less than 1K are incorrect. - Work-around: NONE

Problem Report 9664 If a large value for VPI/VCI is entered in scvc for a second try after an out of range value for VPI/VCI, vpi/vci configuration could show the status "OTHER". - Work-around: NONE

Problem Report 10225 User cannot disable a Soft PVC configuration. - Work-around: Delete Soft PVC

Problem Report 10477 VP tunnels on CSM port of FCSM is not supported. The system will allow the user to configure VP tunnels, however the following error message will appear: "Invalid param - programmer error." - Work-around: Do not try to configure VP tunnels on CSM port of FCSM.

Problem Report 11034 Receive the following message when modifying soft pvc connections: "Any additions and deletions of Multicast parties are permanent, canceling the menu will not cancel multicast changes! " - Work-around: NONE, Display Only

Problem Report 11238 Users should not create PVC connections with the end point that is the same as an existing pvc. - Work-around: Check if PVC exists before creating connections.

Problem Report 13001 The vap command shows incorrect number of configured vc on FCSM-II. - Work-around: NONE (Cosmetic does not affect operation.)

Problem Report 13285 PVPs do not work if MAX VPI bits are not consistent between incoming and outgoing ports Work-around: Make MAX VPI bits the same for incoming and outgoing ports with the map command.

Problem Report 14390 Local PNNI advertises the Admin Weight metric correctly, but MAX CR and AV CR are advertised incorrectly, they are displayed as zero. Work-around: NONE This occurs for static routes configured locally with a non-default Admin weight. Remote Nodes ignore the RAIG for the admin weights thus leading to load balancing by remote nodes on the local ports. To coerce remote nodes to prefer certain local links above other local links, configure longer prefixes for the local static routes on the links preferred.

Problem Report 24633 Ocassionally, SSCOP on FCSM-II bounces after a cold reboot with 6 ESM-100C32 modules installed in the system. Work-around: Add the following two flags in the mpm.cmd file before the cmInit statement: csm_reset_count=2 and csm_reset_ticks=2000.

Problem Report 24639 Reset or hot swap does not work reliably on the FCSM-II Work-around: Power off the switch before changing FCSM-II modules.

Problem Report 24641 CSM driver task priority is very high (90) and non-preemptive. During initialization it hogs up CPU cycles and in some configurations, could prevent other modules from coming up. This happens only when WSM boards are used in conjunction with CSM boards in a fully loaded chassis. Work-around: NONE

Problem Report 25258 Multicast based protocols do not work after FCSM fail over. An IBM 8274 Nways LAN RouteSwitch can be configured with two FCSM modules (not FCSM II). This is called FCSM redundancy. FCSM failover happens when the secondary FCSM takes over operation after the primary FCSM fails. The following protocols may not work after the FCSM failover event due to the reason that the multicast addresses are not properly initialized: 1. GateD/RouterDiscovery. 2. GateD/OSPF. 3. GateD/RIPv2. 4. IPX. 5. VAP. Work-around: NONE

## Known Duplicated MAC/Source Learning Problems/Limitations

Problem Report 13772 When the Filtering Database is updated after the CAM is programmed by Source Learning, this can result in a frame or two being lost as follows. The CAM is programmed with an address. A frame comes in with that source address and the 'sci' for that CAM entry is put into the Vbus header. In the meantime, the Filtering Database entry for that 'sci' is not yet fully updated. The frame is discarded on outbound. Work-around: NONE

Problem Report 13868 “last seen” field of macinfo sometimes does not get updated in IBM 8274 Model GRS. Work-around: NONE

## Known Ethernet (ESM/ESX) Problems/Limitations

- Problem Report 12452 When IBM 8274 Model GRS Nways LAN RouteSwitch Mammoth ethernet ports receive misaligned frames, the transmitting port will generate CRC errors. Work-around: NONE

Problem Report 13869 Statistics information is swapped between multicast received and broadcast received on ESMs. Work-around: NONE

Problem Report 14255 With Auto-negotiation enabled between two IBM 8274 Model GRS’s, the link comes up but occasionally goes back and re-negotiates unnecessarily. Work-around: NONE

Problem Report 23530 Autonegotiation is not working on some ports. Inactive ports are sporadically set to 10/Half preventing autonegotiation. Work-around: NONE

Problem Report 24702 ESM-C-12 modules dropping multicast frames Work-around: Use Mammoth based ESM modules.

Problem Report 25231 Duplicate frames seen on mirroring port when mirrored port is attached to hub. Work-around: NONE

## Known FDDI Problems/Limitations

- Problem Report 2600 Cannot change FDDI connection rejection policy via user interface. This works as designed. Work-around: Use IBM Nways RouteSwitch Network Manager

- Problem Report 10759 FDDI client failed to get IP address from DHCP Server. Work-around: NONE

## Known Firewall Problems/Limitations

### Firewall Server

Problem Report 11572 When several switches are connected to the NT Management Server, the fw.exe aborts when trying to start the Log view. - Work-around: Try to initialize again.

Problem Report 11946 EMC aborts with an access violation when one deactivates the Firewall service while the status window is up and running, and then activates the service again. - Work-around: Bring the status window down before reactivating the EMC.

### Firewall Switch

- Problem Report 7027 SNMP Fetch must be done twice sometimes on the Check Point Management Station (EMC) when configuring the Switch network object’s interfaces. - Work-around: NONE

Problem Report 9322 Firewall display on switch says connected while EMC is down. - Work-around: NONE

Problem Report 9876 Changing secondary EMC setting on switch requires reboot. Work-around: NONE

Problem Report 23688 Firewall not filtering some IP packets over ATM CIP. Some IP sessions originated from the ATM Classical IP client are not filtered properly. Work-around: NONE

## Known Gigabit Ethernet (GSX) Problems/Limitations

Problem Report 13661 Auto-negotiation UI for GSX displays an in-active port as 1000-full. Work-around: NONE

- Problem Report 14105 Transmit & receive LED on a GSX module occasionally does not turn on under Traffic if the cable is unplugged and plugged back in. Work-around: Plug in again.

- Problem Report 14117 The flood limits setting using the modvp command does not work on the GSX-FM-2W. Work-around: NONE

Problem Report 14248 GSX drops every other packet upon a cable pull/plug. Work-around: Wait at least 5 seconds after the cable is pulled before the cable is plugged back in.

Problem Report 14712 Occasionally, ping timeouts are observed over the gigabit link. Work-around: If ping timeouts or performance degradation is observed over the gigabit link, unplug the cable for approximately 5 seconds and plug in.

Problem Report 14793 Duplicate MACs will not be processed correctly on GSX-2 boards running X802.1q if they are connected to ports 2 and 4 of a GSX-FM/FS-4 running X802.1q Work-around: Use ports 1 and 3 on the GSX-FM/FS-4.

## Known Group Mobility Problems/Limitations

Problem Report 12178 When group-mobility is enabled and the router is configured, the dynamic LANE does not automatically activate for that router port. Work-around: NONE

- Problem Report 13309 Port rules do not work after switch is rebooted. Work-around: NONE

- Problem Report 20785 When adding a static MAC to a mobile group, the station with the MAC address does not receive frames until after the station transmits a frame. Work-around: NONE

Problem Report 22410 Mobile ports to Static may fail until switch is rebooted if the attached PC's IP address is changed. Work-around: NONE

## Known Hot Swap Problems/Limitations

Problem Report 12547 Hot Swapping Mammoth modules with non-Mammoth modules when RouteChannel is enabled, will fail.. Work-around: Delete RouteChannel prior to changing the non-Mammoth module with a Mammoth module..

Problem Report 13012 Hot swapping CSM-A25 while pinging across two CSM-155 modules will result in a ping time out. Work-around: NONE

Problem Report 13533 When two switches are connected and a hot swap of an ASM occurs, the data direct is lost. Work-around: Reboot remote switch.

Problem Report 13659 After a TSM-CD-6 is hot swapped, the board will restart itself every time IPX is started. Work-around: NONE

Problem Report 13661 Resetting of IBM 8274 Model GRS Nways LAN RouteSwitch modules occasionally causes system problems such as VSE queue filling up on GSX, only first ports initializing in ESX12 or rmonMgr crash on ESX-32. Work-around: NONE

Problem Report 13840 Need to reboot MPM to enable LEC after hot swap of FCSM. Work-around: Use the renounce command or reboot switch.

## Known HRE Problems/Limitations

Problem Report 6121 HRE is unable to route IP FDDI data through FDDI trunking. Work-around: NONE

## Known LSM Problems/Limitations

Problem Report 9180 ESI/Sel type policy under ATM prefix policy does not work. Work-around: NONE

Problem Report 10389 Occasionally the counter that displays the number of proxy and non-proxy LEC's joined to a particular ELAN does not reset. Work-around: NONE

Problem Report 12246 When LSM is configured on a Midway ASM module, the number of LECs which can be supported is limited depending on the size of Tx/Rx SAR Seg size. If the number of LECs exceeds the number that port can support, then some of the LECs will be stuck in the LECs configuration state and the VCCs can not be released. Work-around: Configure the port SAR segment (seg) size to be a smaller number (follow the instructions in the manual). When there are more than 232, VCCs will be set up in the same ASM port (this number is calculated based on a 2Mb port).

Problem Report 24674/25207/25654/25691 When a large number of LECs switch from primary MSS to secondary LSM, certain LECs never join their respective ELANs. Work-around: NONE

Problem Report 25687 LSM does not flush the redundancy VCI even though it is in an idle state. Work-around: NONE, Display Only

## Known Mammoth Problems/Limitations

- Problem Report 12452 When IBM 8274 Nways LAN RouteSwitch, IBM 8274 Model GRS or IBM 8277 Nways Ethernet RouteSwitch Mammoth ethernet ports receive misaligned frames, the transmitting port will generate CRC errors. Work-around: NONE

Problem Report 13696 Fully loaded Chassis with ESM-C-32 will return pmHndlCrsp() messages. Switch will come up after awhile. Work-around: NONE

Problem Report 14494 All Mammoth based boards (except MPX) allocate 8K for each multi-buffer. These modules will only forward frames/packets of less than 8K in size. Work-around: NONE

## Known MPM Redundancy Problems/Limitations

Problem Report 12490 mpmstore only works with full path name to the file. Work-around: Use the full path name.

Problem Report 12829 When the secondary MPM becomes primary, the MPM menu still displays the local MPM as secondary. Work-around: NONE

Problem Report 12969 When the secondary fails to come up due to missing mpm.img, the primary MPM may display the error message "cmMasterStates(1): START state, without Watchdog" then cannot log in. Work-around: NONE

Problem Report 13088 Command mpmreplace gated.img will display error message "there is no file gated.img on this MPM" when the file is in /simm directory. Work-around: Enter the command mpmreplace /simm/gated.img.

Problem Report 13173 When hot swapping FCSM, the MPM can become too busy, and will miss sending a heartbeat message to secondary MPM. This may cause secondary MPM to try to take over as primary. Work-around: NONE

Problem Report 13674 The configsync command does not operate correctly after enabling the syncctl command. Work-around: Manually do a configsync command. Afterwards, the configsyc command starts to sync on intervals.

Problem Report 13720 Sometimes, the syncctl command does not sync the files completely right after booting up. Work-around: Make sure the syncctl command is off. Manually do the configsync and imgsync commands.

Problem Report 13733 imgsync repeats that there is one more file to sync when files are updated. It should report that all files are currently synchronized. Work-around: NONE

Problem Report 25068 An MPM that will not boot up as a standalone MPM will boot up and appear to be functional and fully redundant as a secondary MPM. If the primary MPM fails, the secondary MPM will not takeover. Work-around: NONE.

## Known MPX Problems/Limitations

Problem Report 14202 When using the MPX on-board ethernet port the following error message may be displayed on the management console: "(tNetTask): pcnGetFullRMD(): interface down! flag=0x20". This does not seem to affect connectivity. Work-around: NONE

## Known IBM 8277 Nways Ethernet RouteSwitch Problems/Limitations

Problem Report 10827 If an IBM 8277 Nways Ethernet RouteSwitch is rebooted without an "mp4.img" file, the IBM 8277 Nways Ethernet RouteSwitch reboots infinitely. Work-around: When it displays a countdown timer (2 . . 1 . . 0), press any key. The IBM 8277 Nways Ethernet RouteSwitch will return to the "boot" prompt. See User Manuals for boot procedure.

Problem Report 14383 RMON probes readable only on directly connected network Work-around: NONE

Problem Report 14441 The IBM 8277 Nways Ethernet RouteSwitch4024 can route about 2000 packets per second. If an input stream for routing is much greater than this amount for hours without any relief, the IBM 8277 Nways Ethernet RouteSwitch may stop routing any packet. Work-around: The IBM 8277 Nways Ethernet RouteSwitch 4024 will recover after the routed traffic is lowered or reboot the IBM 8277 Nways Ethernet RouteSwitch 4024.

Problem Report 14591 An IBM 8277 Nways Ethernet RouteSwitch with a T1/E1 card in it (even if not being used) seems to interfere with the passing of an IGMP membership report packet being switched through the unit. The mrd.img is not resident on the IBM 8277 Nways Ethernet RouteSwitch, but is on the next switch across an Ethernet connection. This works fine without the T1 in the unit, but if the T1 or E1 uplink is present, it does not work. Work-around: Load mrd.img on IBM 8277 Nways Ethernet RouteSwitch with T1/E1 (even though it is switching). Enable IP Multicast Routing by putting the command use_mrouted=1 in the mp4.cmd file, before the line with cmInit. This way the IGMP packets from the PC are correctly read and interpreted on a bridged environment.

Problem Report 14774/14854/21973 Port translation does not occur on data from the router port. Work-around: Port translation is not supported on pure ethernet switches (IBM 8277 Nways Ethernet RouteSwitch 1032, 4016 and 4024 only). The encapsulation of the router port should be set according to the devices attached or vice versa.

Problem Report 14775 RouteChannel can not be created on ports 3/13 to 3/24 (IBM 8277 Nways Ethernet RouteSwitch 4024 Only). Only the first 12 ports in each slot may be used in an RouteChannel to allow at least 12 ports for local use. Works as designed. Work-around: Only use ports 3/1-3/12 for RouteChannel.

Problem Report 14795 A problem with the NI occurs when there is a routing situation with an alternate path where both Switches are placing calls to each other and there is also a common path via Ethernet. This is not a typical customer configuration, and if there is an ethernet path, it is unlikely to have and/or need an ISDN connection between the two switches. Work-around: Do not configure a parallel path between the switches for both ISDN and Ethernet.

Problem Report 21411 IBM 8277 Nways Ethernet RouteSwitch 5024 configuration files may become corrupted after a reboot. Work-around: Delete the static IP route whose destination IP address is 0.0.0.0. Create the same static IP route, and reboot (optional)

Problem Report 21875 When using the camstat command, CAM usage may show wrong count for actual usage on IBM 8277 Nways Ethernet RouteSwitch 4024's. Work-Around: NONE, Display only.

Problem Report 21338 IBM 8277 Nways Ethernet RouteSwitch with WAN/BRI does not send IP RIP updates. Work-around: Issue the ripflush command from the UI.

Problem Report 23819 When sending a 8192 byte NFS Packet/UDP from ethernet across ATM-622 onto an IBM 8277 Nways Ethernet RouteSwitch 5024 with an ASM-622 Uplink, the UDP portion of the packet will report a checksum error indicating data corruption. Work-around: Enable 32bit mode operation. This is a dshell flag.

Problem Report 23923 The chnli command can display the wrong number of MAC addresses learned Work-around: NONE.

Problem Report 23989 The gmstat and gmcfg commands allow the user to turn off group mobility while 802.1Q ports are still active. Work-around: Delete 802.1Q groups prior to turning off group mobility.

## Known PNNI Problems/Limitations

Problem Report 14390 Local PNNI advertises the Admin Weight metric correctly, but MaxCR and AvCR are advertised incorrectly, they are displayed as zero. Work-around: NONE This occurs for static routes configured locally with a non-default Admin weight. Remote Nodes ignore the RAIG for the admin weights thus leading to load balancing by remote nodes on the local ports. To coerce remote nodes to prefer certain local links above other local links, configure longer prefixes for the local static routes on the links preferred.

Problem Report 23542 VV - PNNI cells may get tagged even in the absence of any data traffic. Work-around: NONE

Problem Report 24866 When an IISP link's operational status is enabled but SSCOP is down, PNNI continues to route calls to that port. Work-around: Disconnect the link to the port.

## Known Port Manager Problems/Limitations

- Problem Report 7646 (Replaces PR 7541) Power off an IBM 8274 Nways LAN RouteSwitch and changed board types, for example take out a Token Ring CD6 Board and replace it with an E12 board. When the system comes up, other boards contained in the chassis may have problems initializing. This is indicated by the system message: Problem slot #, <board type> not responding to INIT in state VSE_UP The other boards usually do complete initialization as the system re-attempts the initialization. However, occasionally, the initialization fails in all attempts and the board needed to be manually reset. Work-around: Manually reset.

Problem Report 13029 Maxing out available ATM port MAC addresses will result in DETACHED ports. However, after removing all the services, those DETACHED ports still appear. Can not remove the groups belonging to those detached ports. Must remove configuration file to clean up. Work-around: Avoid creating more ATM services than are available on that board (typically 16).

## Known Port Mirroring Problems/Limitations

Problem Report 9901 RMON with port mirroring over Mammoth does not show RMON statistics. - Work-around: NONE

## Known Port Monitoring Problems/Limitations

Problem Report 22230 The pmstat command shows percent ram available as 128% Work-around: NONE, Display only.

Problem Report 25752 Port monitoring on the Gigabit uplink port hangs the UI. Work-around: NONE.

## Known SNMP Problems/Limitations

- Problem Report 4335 No mib support for atmAddArpEntry for CIP services. - Work-around: NONE

Problem Report 5042/10164/11087 SNMP ifSpeed info is not correct for ESM-100C-12, ESM-100F-8, ESM-100-C2 & ESM-100-F2. Work-around: NONE

Problem Report 8188 Entries in the bridge forwarding database returned by the SNMP agent should be in numeric order. Work-around: NONE All entries are returned.

Problem Report 10709 Four IBM supplied MIB files (1850v1, nhrp, pnhrp, softpvc) will not compile with some off-the shelf compilers. The SNMPv2 IMPORT statements must be commented out if this is the case. Work-around: NONE

Problem Report 11733 All non-direct routes are incorrectly displayed as RIP routes. Work-around: NONE

Problem Report 11807 phyPortTable statistics of ATM ports are not correct. Work-around: NONE

Problem Report 11817 When lowering the ATM port values for buffer size and segment Size, set the buffer size before the segment size. Work-around: NONE

Problem Report 11904 The IPX Routing table does not show entries for VLANs. Work-around: NONE

Problem Report 11905 Can not set ifAdminStatus. Work-around: Use IBM-vport AdminStatus instead.

Problem Report 11975 IfDescr for ESM-C-16 are not correct. Work-around: NONE

Problem Report 12064 The ifTable shows LSM virtual ports as UNKNOWN type. Work-around: NONE

Problem Report 12139 The Source Routing type is not allowed to be Set to a value other than the default (ARE) on the creation of an IPX Router. It is necessary to Modify the Router to change the SR Type and to set the value in SNMP. Work-around: NONE

Problem Report 12189 IP multicast routing MIB does not include IGMP version, it always defaults to Version 2. Work-around: NONE

Problem Report 12352 LECS entries for Auto Activated LANE service for mobile groups are not included in SNMP. Work-around: NONE Problem Report 12453 Console debug message appears after creating a LANE Service using a TR LEC via SNMP. Work-around: NONE

Problem Report 12897/22598 PhyPort table has dummy values for ATM. Work-around: NONE

Problem Report 13053 AtmxVccOperStats will occasionally return invalid value 0. Work-around: NONE

Problem Report 13063 ATM SVC PTOP Service virtual ports show 0 for vporlfIndex while the interfaces table showed 3001 and 3002. Work-around: NONE

Problem Report 13137 Enable/Disable VLAN Cluster Service via SNMP wipes out connections. Work-around: Use UI instead of IBM Nways RouteSwitch Network Manager or HOPV to disable/enable a vlan cluster service.

Problem Report 13185 AtmxServiceType returns 8 for LSM, but it is not defined in MIB. Work-around: NONE

Problem Report 13188 Some of the entries in the vBrdgTpFwdTable only have the MAC address information and all other information such as slot/port, status, etc is set to 0 or invalid. Work-around: NONE

Problem Report 13237 Can not set the vLanStpStatus and vLanBrdgExtendAgeingTime variables when creating new Groups. It can only be set when modifying existing Groups. Work-around: NONE

Problem Report 13590 SNMP cannot access the router port on a VLAN which only has IPX enabled. Work-around: Enable IP on the VLAN.

Problem Report 13754 SNMP information for ifDesc and ifSpeed are not correct. Unable to modify ifAdminStatus. Work-around: NONE

Problem Report 13758 ESM-AB-FL card on MPX is not recognized. Work-around: NONE

Problem Report 13759 Gigabit speed is reported as 10MB on port interfaced in chassis properties. Work-around: NONE, Display only

Problem Report 14092 GETNEXT of vatIPRouting Table and vatIPXRouting Table goes into an infinite loop. Work-around: NONE

Problem Report 14093 Cell statistics stop accumulating when they reach 2 billion. Work-around: NONE

Problem Report 14094 SVC CIP Service will not allow the Address Index to be changed. Work-around: NONE

Problem Report 14095 Sets on Advanced E3 parameters fail. Work-around: NONE

Problem Report 14099 Authentication Protocols can not be changed once Group is created. Work-around: NONE

Problem Report 22483 Admin Status of CSM ports cannot be set (disabled) by UI and SNMP. Work-around: NONE. Works as designed.

Problem Report 24531 The agent does not return the next lexicographical node when a SNMP GetNext request is made on non-existent row index. Work-around: NONE.

Problem Report 24640 Variable atmxVclOperStatus has a localdown value even though traffic passes through the circuit. Work-around: NONE.

Problem Report 24903 FCSM II service creation by SNMP creates invalid connections. Work-around: NONE.

Problem Report 25590 Query atmLecStatusTable, atmLecActualLanName has a 0 at end of name. Work-around: NONE.

Problem Report 25592 Query atmLecStatusTable, atmLecActualLanName is balnk for disabled Service. Work-around: Enable the service.

Problem Report 25593 Query atmLecConfigTable returns no entry for Auto Activated LECs. Work-around: NONE.

## Known System Problems/Limitations

- Problem Report 7203 - Switch may not reboot properly after primary MPM-1G is removed from slot 2. Work-around: NONE

Problem Report 13640 If the default IP is not set in boot strap, SNMP agent will not initialize and eventually will cause system to crash MPX only with memPartAllo error. Work-around: NONE

Problem Report 22223 Can not disable the switch from creating the MPM.LOG. Work-around: NONE

Problem Report 22691 If the /simm memory is filled with many files, the imgcl never finishes. Work-Around: Use the newfs command to delete all the files in the file-system. Otherwise, one must wait roughly 10-30 seconds per file.

Problem Report 23841 Entering the traceroute command may cause switch to reboot. Work-around: NONE

Problem Report 25604 The imgcl command takes a very long time to remove the images. Work-around: NONE

## Known Token Ring (TSM/TSX) Problems/Limitations

Problem Report 8139 If excessive errors are injected to a Token Ring port, after a while, the port might appear to be hung. The port LED will still be flashing, and the vi command would show the port in a forwarding state. Even though, the cable was disconnected from the port. Work-around: Do not inject too many errors to a port. Also, if a port seems to be hung, a port reset should bring it back to normal operation.

Problem Report 8753 IBM 8274 Nways LAN RouteSwitch participates as an active monitor even when not configured to participate as an active monitor. Work-around: NONE

Problem Report 9993 IBM 8274 Nways LAN RouteSwitch Token Ring fiber interfaces using 802.5j does not interoperate with other vendors 802.5j equipment. Work-around: NONE

Problem Report 13456/22355 The Smartbits Token Ring card is not truly token ring compliant equipment. Therefore, after a power cycle or a reboot on the IBM 8274 Nways LAN RouteSwitch S/R, the Smartbits Token Ring card connected to a TSX port does not insert into the ring reliably. Work-around: Cycle the power on the Smartbits equipment after the IBM 8274 Model GRS Nways LAN RouteSwitch has come up completely.

Problem Report 21492 Packets seen on mirroring token ring port, when mirroring is disabled. Work-around: Delete mpm.cnf file and reboot.

Problem Report 22380 A system that uses too many ports may not boot or be resettable. Work-around: Do not load a chassis full with 32-port boards. A maximum of (7) 32-port boards is supported at this time.

Problem Report 22863 Token Ring port will not bring up ring until it sees another MAC address. The ring will never display a status of 'up' unless there is a Token Ring device connected to the Token Ring module for any switch running 3.2 or above with Tokin Ring modules. Work-around: NONE

Problem Report 23266 Need to configure the port twice before it gets saved on a TSM-CD-16. Work-around: Configure the port twice in a row to get it saved. Confirm it with the tpvc command.

## Known Translation Problems/Limitations

Problem Report 10788 IPX translations - unable to use Token Ring router encapsulations with ETH media. Work-around: NONE

## Known User Interface and General Problems/Limitations

- Problem Report 1969 The command srs, which displays source routing parameters, shows counters for duplicate LAN ID’s and duplicate segments which increase when there is an external loop. Work-around: NONE (Visual only).

Problem Report 6261 Packet discards due to insufficient buffers are not properly displayed in the vs command. Work-around: NONE

Problem Report 13052 Debug message "ui_do_atm: sel"0x12" appears after SEL change in cas SVC. Work-around: NONE (Ignore message).

Problem Report 13069 The cas command does not allow set up of a VLAN cluster if the connection type was changed to PVC from SVC. Work-around: NONE

Problem Report 13119 Unknown VPI/Unknown VCI fields in the vlrs command do not display last unknown VPI/VCI per port basis. Work-around: NONE

Problem Report 13182 vvc on an individual slot/port vpi/vci on OC-12 port show two entries on UI. Work-around: NONE

Problem Report 21948 - High CPU utilization and the following error message displayed after rebooting an invalid PPP configuration: "pmHndlCrsp - CRSP with Null wait list (NAck received!) S-objlib-OBJ-IP". Work-around: Correct configuration for valid PPP peer information so port does not continually bring the PPP session up and down.

Problem Report 22060 - Occasionally, all slots do not come up on a 9 slot chassis (WSM or BRI). Work-around: Remove and re-insert the WSM card that didn't load or reset system.

Problem Report 25399 - Dropped telnet session corrupted semiphore which cripples some switch routines Work-around: NONE.

## Known Wide Area Switching Modules Problems/Limitations

Problem Report 25480 WSM will not support broadcast frames for more than 32 DLCIs in one bridging group Work-around: NONE

# IBM 8274 Model GRS Nways LAN RouteSwitch, IBM 8274 Nways LAN RouteSwitch, and IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrices

| ## IBM 8274 Model GRS Nways LAN RouteSwitch Hardware/Software Release Compatibility Matrix |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Software Release |  |  |  |  |  |  |
|  | Product | 3.4.1 | 3.4.2 | 3.4.3 | 3.4.4 | 3.4.5 | 3.4.6 | 3.4.7 |
|  | MPX (P/N 050193-06) |  |  |  |  |  |  |  |
|  | Rev A (1.00) Initial Release | x | x | x | x | x | x | x |
|  | Rev A1 (1.01) - Rev. A15 (1.15) | x | x | x | x | x | x | x |
|  | HSX (P/N 050224-06) |  |  |  |  |  |  |  |
|  | Rev A (1.00) Initial Release | x | x | x | x | x | x | x |
|  | Rev A1 (1.01) - Rev. A4 (1.04) | x | x | x | x | x | x | x |
|  | HRE-X Hardware Routing Engine (P/N 050247-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  |  |  |  | x | x | x |
|  | ESX-100C-12W (P/N 050222-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release | x | x | x | x | x | x | x |
|  | Rev. A (1.01) - Rev. A7 (1.07) | x | x | x | x | x | x | x |
|  | ESX-100C-32W (P/N 050233-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A12 (1.12) Initial Release |  |  | x | x | x | x | x |
|  | Rev A13 (1.13) - Rev. A16 (1.16) |  |  | x | x | x | x | x |
|  | ESX-100C-32W (P/N 050233-08) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A3 (1.03) Initial Release |  |  | x | x | x | x | x |
|  | ESX-100FM-12W (P/N 050244-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  |  |  | x | x | x | x |
|  | ESX-100FM-12W (P/N 050244-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B2 (2.02) Initial Release |  |  |  |  |  |  | x |
|  | ESX-100FS-12W (P/N 050244-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B2 (2.02) Initial Release |  |  |  |  |  |  | x |
|  | GSX-FM-2W (P/N 050237-26) Multimode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) Initial Release |  |  | x | x | x | x | x |
|  | Rev A2 (1.02) - A5 (1.05) |  |  | x | x | x | x | x |
|  | GSX-FM-2W (P/N 050237-31) Multimode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A3 (1.03) Initial Release |  |  | x | x | x | x | x |
|  | GSX-FM-2W (P/N 050237-33) Multimode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A2 (1.02) Initial Release |  |  | x | x | x | x | x |
|  | GSX-FS-2W (P/N 050237-28) Single mode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) Initial Release |  |  | x | x | x | x | x |
|  | Rev. A2 (1.02) - A5 (1.05) |  |  | x | x | x | x | x |
|  | GSX-FS-2W (P/N 050237-30) Single mode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) Initial Release |  |  | x | x | x | x | x |
|  | GSX-FS-2W (P/N 050237-32) Single mode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) Initial Release |  |  | x | x | x | x | x |
|  | GSX-FM-4W (P/N 050215-26) Multimode |  |  |  | Discontinued |  |  |  |
|  | Rev. B (2.00) Initial Release | x | x | x | x | x | x | x |
|  | Rev B1 (2.01) - Rev. B2 (2.02) | x | x | x | x | x | x | x |
|  | GSX-FM-4W (P/N 050215-29) Multimode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B5 (2.05) Initial Release | x | x | x | x | x | x | x |
|  | GSX-FM-2W (P/N 050215-46) Multimode |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C4 (3.04) Initial Release | x | x | x | x | x | x | x |
|  | GSX-FS-4W (P/N 050215-28) Single mode |  |  |  | Discontinued |  |  |  |
|  | Rev. B (2.00) Initial Release | x | x | x | x | x | x | x |
|  | Rev B1 (2.01) - Rev. B2 (2.02) | x | x | x | x | x | x | x |
|  | GSX-FS-4W (P/N 050215-30) Single mode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B5 (2.05) Initial Release | x | x | x | x | x | x | x |
|  | GSX-FS-4W (P/N 050215-48) Single mode |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C4 (3.04) Initial Release |  |  | x | x | x | x | x |
|  | TSX-C-16-EXP (P/N 050221-08) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) Initial Release | x | x | x | x | x | x | x |
|  | Rev. A2 (1.02) - Rev. A4 (1.04) | x | x | x | x | x | x | x |
|  | TSX-C-32W (P/N 050220-08) |  |  |  |  |  |  |  |
|  | Rev. A11 (1.11) Initial Release | x | x | x | x | x | x | x |
|  | Rev. A12 (1.12) - Rev. A22 (1.22) | x | x | x | x | x | x | x |
|  | TSX-CD-16W (P/N 050220-06) |  |  |  |  |  |  |  |
|  | Rev. A9 (1.09) Initial Release | x | x | x | x | x | x | x |
|  | Rev. A10 (1.10) - Rev. A19 (1.19) | x | x | x | x | x | x | x |

| ## IBM 8274 Nways LAN RouteSwitch Hardware/Software Release Compatibility Matrix |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Software Release |  |  |  |  |  |  |
|  | Product | 3.4.1 | 3.4.2 | 3.4.3 | 3.4.4 | 3.4.5 | 3.4.6 | 3.4.7 |
|  | MPM (P/N 050026-00) Requires 16 Meg for 3.0 |  |  |  |  |  |  |  |
|  | Rev. C (2.10) |  | x | x | x | x | x | x |
|  | Rev. D (2.11) |  | x | x | x | x | x | x |
|  | Rev. E (2.12) |  | x | x | x | x | x | x |
|  | Rev. F (2.13) |  | x | x | x | x | x | x |
|  | Rev. G (2.14) |  | x | x | x | x | x | x |
|  | Rev. H (2.15) |  | x | x | x | x | x | x |
|  | Rev. I (2.16) |  | x | x | x | x | x | x |
|  | Rev. J (2.17) |  | x | x | x | x | x | x |
|  | Rev. K (2.18) |  | x | x | x | x | x | x |
|  | Rev. L (2.19) - Rev. L8 (2.27) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | MPM (P/N 050026-01) Requires 16 Meg for 3.0 |  |  |  |  |  |  |  |
|  | Rev. C (2.10) |  | x | x | x | x | x | x |
|  | Rev. D (2.11) |  | x | x | x | x | x | x |
|  | Rev. E (2.12) |  | x | x | x | x | x | x |
|  | Rev. F (2.13) |  | x | x | x | x | x | x |
|  | Rev. G (2.14) |  | x | x | x | x | x | x |
|  | Rev. H (2.15) |  | x | x | x | x | x | x |
|  | Rev. I (2.16) |  | x | x | x | x | x | x |
|  | Rev. J (2.17) |  | x | x | x | x | x | x |
|  | Rev. K (2.18) |  | x | x | x | x | x | x |
|  | Rev. L (2.19) - Rev. L10 (2.29) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | MPM-1G (P/N 050143-06) |  |  |  |  |  |  |  |
|  | Rev. A1 (1.01) - Rev. A29 (1.29) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | MPM-1G (P/N 050143-19) |  |  |  |  |  |  |  |
|  | Rev. B1 (2.01) - Rev. B15 (2.15) |  | x | x | x | x | x | x |
|  | Rev. B16 (2.16) - Rev. B30 (2.30) |  | x***** | x***** | x***** | x***** | x***** | x***** |
|  | MPM-1G - 16MB (P/N 050143-16) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B15 (2.15) |  | x | x | x | x | x | x |
|  | Rev. B16 (2.16) - Rev. B30 (2.30) |  | x***** | x***** | x***** | x***** | x***** | x***** |
|  | MPM-1G - 32MB (P/N 050143-18) |  |  |  |  |  |  |  |
|  | Rev. B5 (2.05) - Rev. B15 (2.15) |  | x | x | x | x | x | x |
|  | Rev. B16 (2.16)- Rev. B30 (2.30) |  | x***** | x***** | x***** | x***** | x***** | x***** |
|  | MPM-1G - 64MB (P/N 050143-17) |  |  |  |  |  |  |  |
|  | Rev. B1 (2.01) - Rev. B15 (2.15) |  | x | x | x | x | x | x |
|  | Rev. B16 (2.16) - Rev. B30 (2.30) |  | x***** | x***** | x***** | x***** | x***** | x***** |
|  | MPM-II (P/N 050120-06) Requires 16 Meg for 3.0 |  |  |  |  |  |  |  |
|  | Rev. C1 (2.02) - Rev. C5 (2.06) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. C4 (3.05) - Rev. C25 (3.26) |  | x | x | x | x | x | x |
|  | MPM 11-16GB (P/N 050120-13) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A10 (1.10) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. C17 (1.17) - Rev. C25 (3.26) |  | x | x | x | x | x | x |
|  | HRE Hardware Routing Engine (P/N 050125-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | HRE Hardware Routing Engine (P/N 050125-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B7 (2.07) |  | x | x | x | x | x | x |
|  | HRE Plus (P/N 050125-08) |  |  |  |  |  |  |  |
|  | Rev. A4 (1.04) - Rev. A9 (1.09) |  | x | x | x | x | x | x |
|  | HRE Plus (P/N 050125-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B11 (2.11) |  | x | x | x | x | x | x |
|  | HSM (P/N 050031-06) |  |  |  |  |  |  |  |
|  | Rev. E (2.05) “ “ |  | x | x | x | x | x | x |
|  | Rev. F (2.06) “ “ |  | x | x | x | x | x | x |
|  | Rev. G (2.07) “ “ |  | x | x | x | x | x | x |
|  | Rev. H (2.08) “ “ |  | x | x | x | x | x | x |
|  | Rev. J (2.09) “ “ |  | x | x | x | x | x | x |
|  | Rev. K (3.10) |  | x** | x** | x** | x** | x** | x** |
|  | Rev. L (3.11) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. M (4.12) - Rev. M5 (4.17) |  | x | x | x | x | x | x |
|  | HSM2 (P/N 050110-06) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. C1 (3.01) |  | x | x | x | x | x | x |
|  | Rev. C2 (3.02) - Rev. C3 (3.03) |  | x* | x* | x* | x* | x* | x* |
|  | Rev. D (4.00) |  | x | x | x | x | x | x |
|  | Rev. D1 (4.01) - Rev. D10 (4.10) |  | x* | x* | x* | x* | x* | x* |
|  | HSM2 4K (P/N 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ATM OC3 Daughter Card (P/N 050044-00) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. A (1.01) |  | x | x | x | x | x | x |
|  | Rev. B (2.00) - Rev. B2 (2.02) |  | x | x | x | x | x | x |
|  | ATM OC3 Daughter Card (P/N 050116-26) Multimode, ½ MB SRAM |  |  |  |  |  |  |  |
|  | Rev. A (1.01) - Rev. A6 (1.07) |  | x | x | x | x | x | x |
|  | ATM OC3 Daughter Card (P/N 050116-24) Multimode, 2 MB SRAM |  |  |  |  |  |  |  |
|  | Rev. A (1.01) - Rev. A7 (1.08) |  | x | x | x | x | x | x |
|  | ATM OC3 Daughter Card (P/N 050116-25) Single mode, 2 MB SRAM |  |  |  |  |  |  |  |
|  | Rev. A (1.01) - Rev. A7 (1.08) |  | x | x | x | x | x | x |
|  | ATM OC3 Daughter Card (P/N 050116-27) Single mode, ½ MB SRAM |  |  |  |  |  |  |  |
|  | Rev. A (1.01) - Rev. A6 (1.07) |  | x | x | x | x | x | x |
|  | ATM DS3 Daughter Card (P/N 050104-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) |  | x | x | x | x | x | x |
|  | Rev. B (2.00) - Rev. B4 (1.04) |  | x | x | x | x | x | x |
|  | ATM E3 Daughter Card (P/N 050104-13) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | ATM-UTP Daughter Card (P/N 050115-06) one UTP |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B6 (2.06) |  | x | x | x | x | x | x |
|  | ATM2-155-FM (P/N 050146-06) Multimode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A11 (1.11) |  | x | x | x | x | x | x |
|  | ATM2-155-FS (P/N 050146-13) Single mode |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A11 (1.11) |  | x | x | x | x | x | x |
|  | ATM2-155-FM (P/N 050175-32) Multimode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B5 (2.05) |  | x | x | x | x | x | x |
|  | ATM2-155-FS (P/N 050175-33) Single mode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B5 (2.05) |  | x | x | x | x | x | x |
|  | ATM2-155-FSH (P/N 050175-34) Single mode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B5 (2.05) |  | x | x | x | x | x | x |
|  | ASM-155FSH-1EW-4C (PN 050116-28) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | ASM-CE-155FM-2S2EW (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-155FM-2S2EW-4C (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-FM-2S2TW (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-155FM-2S2TW-4C (PN 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ASM-CE-155FS-2S2EW (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-155FS-2S2EW-4C (PN 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ASM-CE-155FS-2S2TW (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-155FS-2S2TW-4C (PN 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ASM-CE-DS3-2S2TW (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-DS3-2S2TW-4C (PN 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ASM-CE-E3-2S2EW (PN 050110-06) |  |  |  |  |  |  |  |
|  | Rev. D4 (4.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. D5 (4.05) - Rev. D10 (4.10) |  | x | x | x | x | x | x |
|  | ASM-CE-E3-2S2EW-4C (PN 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ASM-CE-155FM-2S2EW-4C (PN 050168-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A8 (1.08) |  | x | x | x | x | x | x |
|  | ATM2-622RFM (P/N 050178-46) Multimode |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C3 (3.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C4 (3.04) - Rev. C7 (3.07) |  | x | x | x | x | x | x |
|  | ATM2-622RFS (P/N 050178-48) Single mode |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C3 (3.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C4 (3.04) - Rev. C7 (3.07) |  | x | x | x | x | x | x |
|  | CDDI Daughter Card (P/N 050073-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.01) |  | x | x | x | x | x | x |
|  | Rev. B (1.02) |  | x | x | x | x | x | x |
|  | Rev. C (1.03) |  | x | x | x | x | x | x |
|  | Rev. D (1.04) |  | x | x | x | x | x | x |
|  | Rev. E (1.05) - Rev. E3 (1.08) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | CSM-155-6M2S (P/N 050113-15) |  |  |  |  |  |  |  |
|  | Rev. F (6.00) - Rev. F7 (7.07) |  | x | x | x | x | x | x |
|  | CSM-155-6M2S (P/N 050113-25) |  |  |  |  |  |  |  |
|  | Rev. G1 (7.01) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155-6M2S (P/N 050113-70) |  |  |  |  |  |  |  |
|  | Rev. H1 (8.01) - Rev. H5 (8.05) |  | x | x | x | x | x | x |
|  | CSM-155-6M2S (P/N 050113-85) |  |  |  |  |  |  |  |
|  | Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G6 (7.06) - Rev. G9 (7.09) |  | x | x | x | x | x | x |
|  | CSM-155-6M2SL (P/N 050113-27) |  |  |  |  |  |  |  |
|  | Rev. F5 (6.05) |  | x | x | x | x | x | x |
|  | Rev. G1 (7.01) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155-6M2SL (P/N 050113-72) |  |  |  |  |  |  |  |
|  | Rev. H (8.00) - Rev. H5 (8.05) |  | x | x | x | x | x | x |
|  | CSM-155-6M2SL (P/N 050113-87) |  |  |  |  |  |  |  |
|  | Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G5 (7.05) - Rev. G9 (7.09) |  | x | x | x | x | x | x |
|  | CSM-155-8 (P/N 050113-14) |  |  |  |  |  |  |  |
|  | Rev. F (6.00) - Rev. F8 (6.08) |  | x | x | x | x | x | x |
|  | CSM-155-8 (P/N 050113-19) |  |  |  |  |  |  |  |
|  | Rev. G (7.00) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155-8 (P/N 050113-24) |  |  |  |  |  |  |  |
|  | Rev. G (7.00) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155-8 (P/N 050113-69) |  |  |  |  |  |  |  |
|  | Rev. H1 (8.01) - Rev. H5 (8.05) |  | x | x | x | x | x | x |
|  | CSM-155-8 (P/N 050113-75) |  |  |  |  |  |  |  |
|  | Rev. H (8.00) - Rev. H1 (8.01) |  | x | x | x | x | x | x |
|  | CSM-155-8 (P/N 050113-84) |  |  |  |  |  |  |  |
|  | Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G5 (7.05) - Rev. G9 (7.09) |  | x | x | x | x | x | x |
|  | CSM-155-8S (P/N 050113-08) |  |  |  |  |  |  |  |
|  | Rev. F (6.00) - Rev. F7 (6.07) |  | x | x | x | x | x | x |
|  | CSM-155-8S (P/N 050113-18) |  |  |  |  |  |  |  |
|  | Rev. G1 (7.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G2 (7.02) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155-8S (P/N 050113-66) |  |  |  |  |  |  |  |
|  | Rev. H1 (8.01) - Rev. H5 (8.05) |  | x | x | x | x | x | x |
|  | CSM-155-8S (P/N 050113-73) |  |  |  |  |  |  |  |
|  | Rev. H (8.00) - Rev. H1 (8.01) |  | x | x | x | x | x | x |
|  | CSM-155-8S (P/N 050113-78) |  |  |  |  |  |  |  |
|  | Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G5 (7.05) - Rev. G9 (7.09) |  | x | x | x | x | x | x |
|  | CSM-155C-8 (P/N 050113-68) |  |  |  |  |  |  |  |
|  | Rev. H1 (8.01) - Rev. H5 (8.05) |  | x | x | x | x | x | x |
|  | CSM-155C-8 (P/N 050113-74) |  |  |  |  |  |  |  |
|  | Rev. H (8.00) - Rev. H1 (8.01) |  | x | x | x | x | x | x |
|  | CSM-155C-8 (P/N 050113-83) |  |  |  |  |  |  |  |
|  | Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G6 (7.06) - Rev. G9 (7.09) |  | x | x | x | x | x | x |
|  | CSM-155C-8W (P/N 050113-23) |  |  |  |  |  |  |  |
|  | Rev. G (7.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G1 (7.01) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155FSL-8 (P/N 050113-26) |  |  |  |  |  |  |  |
|  | Rev. G (7.00) - Rev. G8 (7.08) |  | x | x | x | x | x | x |
|  | CSM-155FSL-8 (P/N 050113-71) |  |  |  |  |  |  |  |
|  | Rev. H (8.00) - Rev. H5 (8.05) |  | x | x | x | x | x | x |
|  | CSM-155FSL-8 (P/N 050113-86) |  |  |  |  |  |  |  |
|  | Rev. G4 (7.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. G5 (7.05) - Rev. G9 (7.09) |  | x | x | x | x | x | x |
|  | CSM-622-2E (P/N 050133-13 ) |  |  |  |  |  |  |  |
|  | Rev. A1 (1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A2 (1.02) - Rev. A15 (1.15) |  | x | x | x | x | x | x |
|  | CSM-622-2E (P/N 050133-33 ) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) |  | x | x | x | x | x | x |
|  | CSM-622-2E (P/N 050133-53 ) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. C (3.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C1 (3.01) - Rev. C3 (3.03) |  | x | x | x | x | x | x |
|  | CSM-622-2E (P/N 050133-73 ) |  |  |  |  |  |  |  |
|  | Rev. C1 (3.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C2 (3.02) - Rev. C11 (3.11) |  | x | x | x | x | x | x |
|  | CSM-622-2E (P/N 050133-90) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1(1.01) Initial Release |  | x | x | x | x | x | x |
|  | CSM-622-2E (P/N 050133-93) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) |  | x | x | x | x | x | x |
|  | Rev. A8 (1.08) - Rev. A17 (1.17) |  | x | x | x | x | x | x |
|  | CSM-622-2SE (P/N 050133-06) |  |  |  |  |  |  |  |
|  | Rev. A1 (1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A2 (1.02) - Rev. A15 (1.15) |  | x | x | x | x | x | x |
|  | CSM-622-2SE (P/N 050133-26 ) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) |  | x | x | x | x | x | x |
|  | CSM-622-2SE (P/N 050133-46 ) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. C (3.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C1 (3.01) - Rev. C3 (3.03) |  | x | x | x | x | x | x |
|  | CSM-622-2SE (P/N 050133-66 ) |  |  |  |  |  |  |  |
|  | Rev. C1 (3.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C2 (3.02) - Rev. C11 (3.11) |  | x | x | x | x | x | x |
|  | CSM-622-2SE (P/N 050133-86) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) |  | x | x | x | x | x | x |
|  | Rev. A8 (1.08) - Rev. A17 (1.17) |  | x | x | x | x | x | x |
|  | CSM-622-2SE (P/N 050133-89) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1(1.01) Initial Release |  | x | x | x | x | x | x |
|  | CSM-622FSH-2EW (P/N 050133-74) |  |  |  |  |  |  |  |
|  | Rev. A13 (1.13) - Rev. A16 (1.16) Initial Release |  |  | x | x | x | x | x |
|  | Rev. A17 (1.17) |  |  | x | x | x | x | x |
|  | CSM-A25-12 (P/N 050134-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | CSM-A25-12 (P/N 050134-66) |  |  |  |  |  |  |  |
|  | Rev. A (1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.02) - Rev. A3 (1.04) |  | x | x | x | x | x | x |
|  | CSM-A25-12 (P/N 050134-68) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | CSM-A25-24 (P/N 050134-13) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | CSM-A25-24 (P/N 050134-73) |  |  |  |  |  |  |  |
|  | Rev. A1(1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A2 (1.03) - Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | CSM-A25-24 (P/N 050134-69) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | CSM-AB-155C-2W (P/N 050164-10) |  |  |  |  |  |  |  |
|  | Rev. A4 (1.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A5 (1.05) |  | x | x | x | x | x | x |
|  | CSM-AB-155FM-2W (P/N 050164-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A5 (1.05) |  | x | x | x | x | x | x |
|  | CSM-AB-155FS-2W (P/N 050164-08) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A5 (1.05) |  | x | x | x | x | x | x |
|  | CSM-AB-155FSH-2W (P/N 050164-09) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A5 (1.05) |  | x | x | x | x | x | x |
|  | CSM-AB-CE-E1-4W (P/N 050187-08) |  |  |  |  |  |  |  |
|  | Rev. A6 (1.06) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A7 (1.07) - Rev. A10 (11.0) |  | x | x | x | x | x | x |
|  | CSM-AB-CE-E1-4W (P/N 050187-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B3 (2.03) Initial Release |  |  | x | x | x | x | x |
|  | CSM-AB-CE-T1-4W (P/N 050187-06) |  |  |  |  |  |  |  |
|  | Rev. A5 (1.05) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A6 (1.06) - Rev. A9 (1.09) |  | x | x | x | x | x | x |
|  | CSM-AB-CE-T1-4W (P/N 050187-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B5 (2.05) |  | x | x | x | x | x | x |
|  | CSM-AB-CM (P/N 050214-09) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) |  | x | x | x | x | x | x |
|  | CSM-AB-DS1-4W (P/N 050179-06) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A4 (1.04) - Rev. A6 (1.06) |  | x | x | x | x | x | x |
|  | CSM-AB-DS1-4W (P/N 050179-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | CSM-AB-DS3-2W (P/N 050169-26) |  |  |  |  |  |  |  |
|  | Rev. b (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. b1 (2.01) |  | x | x | x | x | x | x |
|  | CSM-AB-E1-4W (P/N 050179-13) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A4 (1.04) - Rev. A5 (1.05) |  | x | x | x | x | x | x |
|  | CSM-AB-E1-4W (P/N 050179-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | CSM-AB-E3-2W (P/N 050169-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  |  | x | x | x | x | x |
|  | CSM-UW (P/N 050157-86) |  |  |  |  |  |  |  |
|  | Rev. b1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B2 (2.02) - Rev. B7 (2.07) |  | x | x | x | x | x | x |
|  | FCSM-OC3 (P/N 050129-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | Rev. B (2.00) - Rev. B10 (2.10) |  | x | x | x | x | x | x |
|  | FCSM-OC3-4C (P/N 050129-68) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | FCSM-OC3-4C (P/N 050129-73) |  |  |  |  |  |  |  |
|  | Rev. B1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B2 (2.02) - Rev. B10 (2.10) |  | x | x | x | x | x | x |
|  | FCSM-OC12 (P/N 050181-06) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A4 (1.04) |  | x | x | x | x | x | x |
|  | FCSM-OC12 (P/N 050181-66) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A4 (1.04) - Rev. A14 (1.14) |  | x | x | x | x | x | x |
|  | ESM-AB-AFD (P/N 050144-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | ESM-AB-AT (P/N 050079-06) |  |  |  |  |  |  |  |
|  | Rev. B - Rev. B1 |  | x | x | x | x | x | x |
|  | ESM-AB-B (P/N 050097-06) |  |  |  |  |  |  |  |
|  | Rev. B |  | x | x | x | x | x | x |
|  | Rev. C - Rev. C4 |  | x | x | x | x | x | x |
|  | ESM-AB-FL (P/N 050085-06) |  |  |  |  |  |  |  |
|  | Rev. B |  | x | x | x | x | x | x |
|  | Rev. C |  | x | x | x | x | x | x |
|  | Rev. D |  | x | x | x | x | x | x |
|  | Rev. E - Rev. E5 |  | x | x | x | x | x | x |
|  | ESM-AB-FL-S (P/N 050123-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A5 (1.05) |  | x | x | x | x | x | x |
|  | ESM-AB-FL-SH (P/N 050123-13) |  |  |  |  |  |  |  |
|  | Rev. A1 (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A2 (1.01) - Rev. A3 (1.02) |  | x | x | x | x | x | x |
|  | ESM-AB-T (P/N 050091-06) |  |  |  |  |  |  |  |
|  | Rev. A - Rev. A4 |  | x | x | x | x | x | x |
|  | ESM-100C-12 (P/N 050158-06) |  |  |  |  |  |  |  |
|  | Rev. B2 (2.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B3 (2.03) - Rev. B6 (2.06) |  | x | x | x | x | x | x |
|  | Rev. B7 (2.07) - Rev. B15 (2.15) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | ESM-100C-12 (P/N 050158-17) |  |  |  |  |  |  |  |
|  | Rev. C6 (3.06) - Rev. C17 (3.17) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | ESM-100C-12 (P/N 050158-26) |  |  |  |  |  |  |  |
|  | Rev. D (4.00) - Rev. D7 (4.07) | x**** | x**** | x**** | x**** | x**** | x**** | x**** |
|  | ESM-100C-32W, ESM-100C-32W-2C (P/N 050262-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) Initial Release |  |  | x**** | x**** | x**** | x**** | x**** |
|  | ESM-100C-32W, ESM-100C-32W-2C (P/N 050262-08) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  |  | x**** | x**** | x**** | x**** | x**** |
|  | Rev. A1 (1.01) - Rev. A3 (1.04) |  | x | x | x | x | x | x |
|  | ESM-100C-FD (P/N 050122-06) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B5 (2.05) |  | x | x | x | x | x | x |
|  | ESM-100C-FD, AE (P/N 050122-15) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B4 (2.04) |  | x | x | x | x | x | x |
|  | ESM-100FM-8 (P/N 050159-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A4 (1.04) |  | x | x | x | x | x | x |
|  | Rev. A5 (1.05) - Rev. A13 (1.13) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | ESM-100FM-8 (P/N 050159-18) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B7 (2.07) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | ESM-100FM-FD (P/N 050122-13) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B7 (2.07) |  | x | x | x | x | x | x |
|  | ESM-100FM-FD (P/N 050122-17) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A2 (1.02) | x | x | x | x | x | x | x |
|  | Rev. B3 (2.03) | x | x | x | x | x | x | x |
|  | Rev. B6 (2.06) - Rev. B7 (2.07) | x | x | x | x | x | x | x |
|  | ESM-100FS-8W-4C (P/N 050159-16) |  |  |  |  |  |  |  |
|  | Rev. B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B5 (2.05) - Rev. B7 (2.07) |  | x | x | x | x | x | x |
|  | ESM-100FS-FD (P/N 050122-16) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A2(1.02) |  | x | x | x | x | x | x |
|  | Rev. B3 (2.03) |  | x | x | x | x | x | x |
|  | Rev. B6 (2.06) - Rev. B7 (2.07) |  | x | x | x | x | x | x |
|  | ESM-100FS-FD-1 (P/N 050122-14) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B7 (2.07) |  | x | x | x | x | x | x |
|  | ESM-C-8 (P/N 050014-00) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. D (1.09) |  | x | x | x | x | x | x |
|  | Rev. E (1.10) |  | x | x | x | x | x | x |
|  | Rev. F (1.11) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. F1 (1.12) - Rev. F5 (1.16) |  | x* | x* | x* | x* | x* | x* |
|  | ESM-C-12 (P/N 050112-06) |  |  |  |  |  |  |  |
|  | Rev. D (4.00) - Rev. D3 (4.03) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. D4 (4.04) |  | x* | x* | x* | x* | x* | x* |
|  | Rev. E (5.00) |  | x | x | x | x | x | x |
|  | Rev. E1 (5.01) - Rev. E11 (5.11) |  | x* | x* | x* | x* | x* | x* |
|  | ESM-C-16W (P/N 050154-17) |  |  |  |  |  |  |  |
|  | Rev. C6 (3.06) - Rev. C12 (3.12) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C13 (3.13) - Rev. C17 (3.17) |  | x | x | x | x | x | x |
|  | ESM-C-16W (P/N 050154-18) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C4 (3.04) |  | x | x | x | x | x | x |
|  | Rev. C5 (3.05) - Rev. C12 (3.12) |  | x | x | x | x | x | x |
|  | ESM-C-16W (P/N 050154-28) |  |  |  |  |  |  |  |
|  | Rev. D (4.00) - Rev. D13 (4.13) |  | x | x | x | x | x | x |
|  | ESM-C-16WC (P/N 050154-29) |  |  |  |  |  |  |  |
|  | Rev. D (4.00) - Rev. D13 (4.13) |  | x | x | x | x | x | x |
|  | ESM-C-32 (P/N 050154-06) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B6 (2.06) |  | x | x | x | x | x | x |
|  | Rev. B7 (2.07) - Rev. B13 (2.13) |  | x | x | x | x | x | x |
|  | ESM-C-32 (P/N 050154-16) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C1 (3.01) - Rev. C3 (3.03) |  | x | x | x | x | x | x |
|  | Rev. C4 (3.04) - Rev. C11 (3.11) |  | x | x | x | x | x | x |
|  | ESM-C-32 (P/N 050154-26) |  |  |  |  |  |  |  |
|  | Rev. D (4.00) - Rev. D11 (4.11) |  | x | x | x | x | x | x |
|  | ESM-F-8 (P/N 050118-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A4(1.04) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. A5 (1.05) |  | x* | x* | x* | x* | x* | x* |
|  | Rev. B (2.00) - Rev. B13 (2.13) |  | x* | x* | x* | x* | x* | x* |
|  | ESM-FM-8-EXP (P/N 050199-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A2(1.02) Initial Release |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | ESM-FM-8W-2C (P/N 050197-06) |  |  |  |  |  |  |  |
|  | Rev. A1 (1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A6 (1.06) |  | x | x | x | x | x | x |
|  | ESM-FM-16W-2C (P/N 050197-08) |  |  |  |  |  |  |  |
|  | Rev. A1 (1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | ESM-T-12 (P/N 050140-06) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B1 (2.01) |  | x | x | x | x | x | x |
|  | Rev. B2 (2.02) - Rev. B10 (2.10) |  | x* | x* | x* | x* | x* | x* |
|  | ESM-T-24W (P/N 050189-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  |  | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A3 (1.03) |  |  | x | x | x | x | x |
|  | ESM-U-6 (P/N 050067-06) |  |  |  |  |  |  |  |
|  | Rev. C (1.03) |  | x | x | x | x | x | x |
|  | Rev. D (1.04) |  | x*** | x*** | x*** | x*** | x*** | x*** |
|  | Rev. E (1.05) |  | x | x | x | x | x | x |
|  | Rev. F (2.00) - Rev. F1 (2.01) |  | x | x | x | x | x | x |
|  | Rev. F2 (2.02) - Rev. F10 (2.10) |  | x* | x* | x* | x* | x* | x* |
|  | ESM-U-6-4C (P/N 050067-08) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B7 (2.07) |  | x | x | x | x | x | x |
|  | FDDI/2-S-ATM Cat 2 Laser (P/N 050108-15) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A1 (1.01) |  | x | x | x | x | x | x |
|  | Rev. C3 (3.00) - Rev. C5 (3.05) |  | x | x | x | x | x | x |
|  | FESM 100BaseT Daughter Card (P/N 050109-06) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C16 (3.16) |  | x | x | x | x | x | x |
|  | FESM AE Daughter Card (P/N 050109-13) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | FESM II 100BTX (P/N 050139-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A10 (1.10) |  | x | x | x | x | x | x |
|  | FESM II 100BTX (P/N 050139-13) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | FSM Daughter Card (P/N 050037-06) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. C (1.03) |  | x | x | x | x | x | x |
|  | Rev. D (1.04) |  | x | x | x | x | x | x |
|  | Rev. E (1.05) - Rev. E2 (1.02) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | FSM Daughter Card (P/N 050108-06) Multimode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C (2.03) - Rev. C5 (2.05) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | Rev. C3 (3.03) - Rev. C5 (3.05) |  | x | x | x | x | x | x |
|  | FSM Daughter Card (P/N 050108-13) Single mode |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C (2.03) - Rev. C5 (2.05) |  | x**** | x**** | x**** | x**** | x**** | x**** |
|  | Rev. C3 (3.03) - Rev. C5 (3.05) |  | x | x | x | x | x | x |
|  | FSM Daughter Card (P/N 050108-26) Multimode |  |  |  |  |  |  |  |
|  | Rev. D (4.00) Initial Release |  |  |  |  |  |  | x |
|  | FSM Daughter Card (P/N 050108-28) mode |  |  |  |  |  |  |  |
|  | Rev. D (4.00) Initial Release |  |  |  |  |  |  | x |
|  | FSM Daughter Card (P/N 050108-28) Single mode long reach |  |  |  |  |  |  |  |
|  | Rev. D (4.00) Initial Release |  |  |  |  |  |  | x |
|  | GSM-FM-2 (P/N 050261-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B1 (2.01) Initial Release |  |  |  | x | x | x | x |
|  | Rev. B2 (2.02) - Rev. B4 (2.04) |  |  |  | x | x | x | x |
|  | GSM-FM-2 (P/N 050261-29) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B2 (2.02) Initial Release |  |  |  | x | x | x | x |
|  | GSM-FS-2 (P/N 050261-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B1 (2.01) Initial Release |  |  |  | x | x | x | x |
|  | Rev. B2 (2.02) - Rev. B4 (2.04) |  |  |  | x | x | x | x |
|  | GSM-FS-2 (P/N 050261-30) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B2 (2.02) Initial Release |  |  |  | x | x | x | x |
|  | TSM Copper (P/N 050049-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.01) |  | x | x | x | x | x | x |
|  | Rev. B (1.02) |  | x | x | x | x | x | x |
|  | Rev. C (1.03) |  | x | x | x | x | x | x |
|  | Rev. D (1.04) - Rev. D7 (4.07) |  | x | x | x | x | x | x |
|  | TSM Fiber (P/N 050111-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.01) |  | x | x | x | x | x | x |
|  | Rev. B (2.00) - Rev. B8 (2.08) |  | x | x | x | x | x | x |
|  | TSM Fiber (P/N 050111-26) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C2 (3.02) |  | x | x | x | x | x | x |
|  | TSM Dual Mode (P/N 050121-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | TSM Dual Mode (P/N 050121-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B3 (2.03) |  | x | x | x | x | x | x |
|  | TSM-CD-16W (P/N 050236-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A4 (1.04) |  | x | x | x | x | x | x |
|  | WSM-SC-4/8 WAN (P/N 050117-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A3 (1.103) |  |  |  |  |  |  |  |
|  | Rev. A4 (1.04) - Rev. A13 (1.13) |  | x | x | x | x | x | x |
|  | WSM-S-2 WAN (P/N 050117-13) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) - Rev. A10 (1.10) |  | x | x | x | x | x | x |
|  | WSM-BRI-SC-1W, WSM-BRI-SC-2W (P/N 050160-06) |  |  |  |  |  |  |  |
|  | Rev. A1(1.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A2 (1.02) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | WSM-FE1-SC-1W, WSM-FE1-SC-2W (P/N 050148-08) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) - Rev. A5 (1.05) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A6 (1.06) - Rev. A9 (1.09) |  | x | x | x | x | x | x |
|  | WSM-FT1-SC-1W, WSM-FT1-SC-2W (P/N 050148-06) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A4 (1.04) - Rev. A9 (1.09) |  | x | x | x | x | x | x |

| ## IBM 8277 Nways Ethernet RouteSwitch Hardware/Software Release Compatibility Matrix |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Software Release |  |  |  |  |  |  |
|  | Product | 3.4.1 | 3.4.2 | 3.4.3 | 3.4.4 | 3.4.5 | 3.4.6 | 3.4.7 |
|  | RS-2032, RS-2032-4C (P/N 050180-08) |  |  |  |  |  |  |  |
|  | Rev. B7 (2.07) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B8 (2.08) - Rev. B25 (2.25) |  | x | x | x | x | x | x |
|  | RS-3032, RS-3032-8C (P/N 050180-06) |  |  |  |  |  |  |  |
|  | Rev. B6 (2.06) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B7 (2.07) - Rev. B22 (2.22) |  | x | x | x | x | x | x |
|  | RS-3032E (P/N 050186-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B2 (2.02) |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42C) (P/N 050251-28) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B8 (2.08) |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42C) (P/N 050251-31) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42C)F (P/N 050251-29) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B8 (2.08) |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42C)F (P/N 050251-32) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B5 (2.05) Initial Release |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42F) (P/N 050251-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B1 (2.01) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B8 (2.08) |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42F) (P/N 050251-30) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - B4 (2.04) Initial Release |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42G) (P/N 050253-26) | DISCONTINUED |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B2 (2.02) |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42G) (P/N 050251-46) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - C8 (3.08) Initial Release |  | x | x | x | x | x | x |
|  | 8277 Model 424 (8277-42G) (P/N 050251-48) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - C2 (3.02) Initial Release |  | x | x | x | x | x | x |
|  | RS-5024, RS-5024-8C (P/N 050196-06) |  |  |  |  |  |  |  |
|  | Rev. B3 (2.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B4 (2.04) - Rev. B24 (2.24) |  | x | x | x | x | x | x |
|  | RS-5024, RS-5024-8C (P/N 050196-26) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - Rev. C1 (3.01) Initial Release |  | x | x | x | x | x | x |
|  | RSASM2-155FM-1 (P/N 050175-12) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A6 (1.06) |  | x | x | x | x | x | x |
|  | RSASM2-155FS-1 (P/N 050175-13) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A6 (1.06) |  | x | x | x | x | x | x |
|  | RSASM2-155FSH-1 (P/N 050175-14) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) - Rev. A06 (1.06) |  | x | x | x | x | x | x |
|  | RSASM2-155RFM-1 (P/N 050175-09) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A4 (1.04) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | RSASM2-155RFM-1 (P/N 050175-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B1 (2.01) - Rev. B5 (1.05) |  | x | x | x | x | x | x |
|  | RSASM2-155RFS-1 (P/N 050175-10) |  |  |  |  |  |  |  |
|  | Rev. A3 (1.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A4 (1.04) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | RSASM2-622RFM-1 (P/N 050178-46) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - C3 (3.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C4 (3.04) - C7 (3.07) |  | x | x | x | x | x | x |
|  | RSASM2-622RFS-1 (P/N 050178-48) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) - C3 (3.03) Initial Release |  | x | x | x | x | x | x |
|  | Rev. C4 (3.04) - C7 (3.07) |  | x | x | x | x | x | x |
|  | RSESM-100C-4 (P/N 050171-06) |  |  |  |  |  |  |  |
|  | Rev. A2 (1.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A3 (1.03) |  | x | x | x | x | x | x |
|  | RSESM2-100FM-2 (P/N 050170-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A2 (1.02) |  | x | x | x | x | x | x |
|  | RSESM2-100FS-2 (P/N 050170-08) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A2 (1.02) |  | x | x | x | x | x | x |
|  | RSESM2-FM-2 (P/N 050206-06) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (1.01) - Rev. A2 (1.02) |  | x | x | x | x | x | x |
|  | RSESM2-FM-2 (P/N 050206-26) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B2 (2.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B3 (2.03) |  | x | x | x | x | x | x |
|  | RSESM2-FM-2 (P/N 050206-46) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) Initial Release |  | x | x | x | x | x | x |
|  | RSESM2-FS-2 (P/N 050206-09) |  |  |  |  |  |  |  |
|  | Rev. A (1.00) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A1 (3.01) - Rev. A2 (1.02) |  | x | x | x | x | x | x |
|  | RSESM2-FS-2 (P/N 050206-29) |  |  |  |  |  |  |  |
|  | Rev. B (2.00) - Rev. B2 (2.02) Initial Release |  | x | x | x | x | x | x |
|  | Rev. B3 (2.03) |  | x | x | x | x | x | x |
|  | RSESM2-FS-2 (P/N 050206-48) |  |  |  |  |  |  |  |
|  | Rev. C (3.00) Initial Release |  | x | x | x | x | x | x |
|  | RSWSM-BRI-SC-1 (P/N 050160-06) |  |  |  |  |  |  |  |
|  | Rev. A4 (1.04) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A5 (3.05) - Rev. A7 (1.07) |  | x | x | x | x | x | x |
|  | RSWSM-FE1-SC-1 (P/N 050148-08) |  |  |  |  |  |  |  |
|  | Rev. A5 (1.05) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A6 (1.06) - Rev. A9 (1.09) |  | x | x | x | x | x | x |
|  | RSWSM-FT1-SC-1 (P/N 050148-06) |  |  |  |  |  |  |  |
|  | Rev. A5 (1.05) Initial Release |  | x | x | x | x | x | x |
|  | Rev. A6 (1.06) - Rev. A9 (1.09) |  | x | x | x | x | x | x |

*	This revision and above is required to support the 960 VBUS speed.

**	All FSM-M-2 modules require REV K or greater HSM or any released HSM2.

*** 	This revision and above is required for MPM redundancy.

****	This revision and above is required for Hot Swap.

*****		This revision and above is required for FCSM-II.

| # Existing Software & Hardware Features |  |  |  |
| --- | --- | --- | --- |
| ## Software Feature Matrix (X = Supported) |  |  |  |
| Software Feature | IBM 8274 Model GRS Nways LAN RouteSwitch | IBM 8274 Nways LAN RouteSwitch | IBM 8277 Nways Ethernet RouteSwitch |
| 802.1q Trunking on 10/100 | X | X | X |
| 802.5 LAN Emulation Client |  | X | X |
| Adaptive MAC Learning Algorithm |  | X | X |
| Advanced Routing including OSPF and RIPII | X | X | X |
| ASM2 Traffic Shaping |  | X | X |
| ASM2 Trunking | X | X |  |
| ATM LSM UI Enhancements | X | X |  |
| ATM Services - Ethernet LAN Emulation Client - Omni point to point bridging - RFC 1483 (LLC Encapsulation over ATM) - RFC 1577 (Classical IP over ATM) VLAN Trunking over ATM X-Lane with PVC’s | X | X | X |
| ATM Traffic Shaping | X | X | X |
| ATM UNI 3.0/3.1 |  | X | X |
| Auto-Switch |  | X | X |
| Nways Route Tracker Manager | X | X | X |
| Banyan Vines Translations | X | X | X |
| Bi-directional Traffic Parameters |  | X | X |
| Binding VLANs | X | X | X |
| Default Bridge Mode Change | X | X | X |
| DHCP BootP Relay/Translations | X | X | X |
| DHCP VLANs | X | X | X |
| Distributed CAM Utilization |  | X |  |
| Domain Bridging |  | X | X |
| Dual Homing on FDDI |  | X |  |
| Duplicate MAC Support | X | X | X |
| Dynamic CAC |  | X |  |
| Dynamic LANE |  | X | X |
| E.164 address support |  | X | X |
| Enhanced Diagnostics | X | X | X |
| Ethernet RMON (Groups: Ethernet Statistics, History Control, Ethernet History, Alarm and Event) | X | X | X |
| Fast Path Routing across ASX CIP | X | X |  |
| FCSM-I with Redundancy |  | X |  |
| FCSM-IIW supports hardware Hot Swap as well as UI reset of Hot Swap |  | X |  |
| Flood Limits Override | X | X | X |
| Flooding Limits | X | X | X |
| Frame Relay |  | X | X |
| Gigabit Hardware tagging (802.1q and x802.1q) | X |  |  |
| Group Mobility & Group Mobility II | X | X | X |
| Group Mobility support for Source-routing |  | X | X |
| Hot Swap of like modules | X | X |  |
| HRE Layer 3 frame forwarding ASIC for IP, IPX route forwarding | X | X | X |
| IBM RI/RO Format interoperability | X | X |  |
| IBM Token Ring Support Functional Address | X | X |  |
| ICMP Router Discovery (part of Advanced Routing) | X | X | X |
| IEEE 802.10 VLAN Tagging over FDDI |  | X | X |
| IP Firewall | X | X | X |
| IP Multicast Routing | X | X | X |
| IP Routing | X | X | X |
| IPX Any-to-Any Switching | X | X | X |
| IPX Client Network Policy refinement | X | X | X |
| IPX Raw over FDDI |  | X | X |
| IPX RIP/SAP Filtering | X | X | X |
| IPX Routing | X | X | X |
| LANE NARP Enhancement | X | X |  |
| LANE Service Module (LSM) |  | X |  |
| MPM Port Monitoring |  | X |  |
| MPM Redundancy |  | X |  |
| MPX Port Monitoring | X |  |  |
| MPX Redundancy | X |  |  |
| Multicast Virtual LAN’s | X | X | X |
| Multiple Users’ Sessions (Simultaneous Telnet) | X | X | X |
| RouteChannel | X | X | X |
| Optional Token Ring Copy Bit Setting |  | X | X |
| OSPF on RS-5024 |  |  | X |
| Port Forwarding Policy | X | X | X |
| Port Mapping |  | X | X |
| Port Mirroring (10BaseT, IBM 8277 Nways Ethernet RouteSwitch & ESM-C-32 Only) | X | X | X |
| Port Monitoring | X |  | X |
| PPP support |  | X | X |
| PVCs and SVCs |  | X | X |
| Router VLAN Management | X | X | X |
| Signed Counters | X | X |  |
| SNMP Trap Support | X | X | X |
| Source Route Bridging | X | X |  |
| Soft PVCs |  | X | X |
| STM-1 Coded in accordance with ITU Ch. 4 of I.432 (Geneva July 1995) |  | X | X |
| SVC/PVC Recovery Improvements |  | X | X |
| Token Ring Port Mirroring & Monitoring | X | X |  |
| Token Ring Source Route RIF Stripping | X | X |  |
| Token Ring, Source Route or FDDI encapsulations |  | X |  |
| UDP Relay | X | X | X |
| UDP Relay is supported with Nways Route Tracker Manager Vlans |  | X |  |
| User Authenticated VLANs | X | X | X |
| Virtual port increase to 32 |  | X | X |
| Virtual Rings and Virtual Rings over Trunks (backbones) | X | X |  |
| VP Tunnelling |  | X | X |
| VLAN Trunking over FDDI |  | X |  |
| VPI specification selection in the PRPADD command |  | X |  |
| WAN Routing Enhancements IP RIP Filters, IPX Watchdog Spoofing, NetWare Serialization Packet Filters, SPX Keep-alive Spoofing, Triggered IPX |  | X | X |
| X-LANE |  | X | X |
| X802.1q on GSX | X |  |  |

## Existing and Supported Hardware

| ### IBM 8274 Model GRS |  |  |
| --- | --- | --- |
|  | - ASX-155FM/FS-1W | - 1-port ATM OC-3 uplink module with SC connectors. (Multimode or Single mode). |
|  | - ASX-155RFM/RFW-1W | - 2-port ATM OC-3 dual port uplink module with an SC connector. |
|  | - ESX-100C-12W | 12-port 10/100BaseTX Ethernet Backbone Switching Module with RJ-45 connectors. |
|  | - ESX-100C-32W | 32-port 10/100BaseTX Ethernet Desktop Switching Module with RJ-45 connectors. |
|  | - ESX-100FM-12W | 12-port 100BaseFX switching ports with MT-RJ connectors. (Multimode) |
|  | - ESX-FM-24 | 24-port Ethernet Switching Module with MT-RJ connectors. (Multimode) |
|  | - GSX-FM/FS-2W | 2-port Gigabit Ethernet Backbone Switching Module with SC connectors (Multimode or Single mode). |
|  | - GSX-FM/FS-4W | 4-port Gigabit Ethernet Backbone Switching Module with SC connectors (Multimode or Single mode). |
|  | - HRE-X Layer-3 | Switching daughter-card. |
|  | - MESX-AB-FL | Adapter board for MPX out of band Ethernet port. One 10BaseFL (Multimode) ST port. |
|  | - MESX-AB-T | Adapter board for MPX out of band Ethernet port. One 10BaseT, RJ-45 port. |
|  | - MPX | Management Processor Module for IBM 8274 Model GRS Nways LAN RouteSwitch. |
|  | - MTRJ-SC-FM/FS-5M | MT-RJ to SC Fiber Jumper Cable, 5 meters. (Multimode or Single mode). |
|  | - TSX-C-32W | 32-port 4/16Mbs Token Ring Switching Module with RJ-45 connectors supporting lobe mode only. |
|  | - TSX-CD-16W | 16-port 4/16Mbs Token Ring Switching Module with RJ-45 connectors supporting lobe and station mode. |
|  | - GRS 9 | 9-slot chassis, 650W AC Power supply. |
|  | - GRS 5PS9-650 | Redundant or spare 650W AC Power supply for the GRS 9 |

| ### IBM 8274 Nways LAN RouteSwitch |  |  |
| --- | --- | --- |
|  | - 8274-W33 Frame/Cell Bus. | - 3-slot, Wide IBM 8274 Nways LAN RouteSwitch with built-in 150W power supply, supports Frame/Cell Bus. |
|  | - 8274-W33 Frame/Cell Bus.-48VI | - 3-slot, Wide IBM 8274 Nways LAN RouteSwitch with -48V DC power supply, supports Frame/Cell Bus. |
|  | - 8274-500 / 8274-500e / 5x | Chassis |
|  | - 8274-W53 | 5-slot, Wide IBM 8274 Nways LAN RouteSwitch with built-in 250W power supply, supports Frame/Cell Bus. |
|  | - 8274-W53-48V | 5-slot, Wide IBM 8274 Nways LAN RouteSwitch with -48V DC power supply, supports Frame/Cell Bus. |
|  | - 8274-900 / 8274-900e / 9x | Chassis |
|  | - 8274-900wx | 9-slot, Wide IBM 8274 Nways LAN RouteSwitch with built-in 500W power supply, supports Frame/Cell Bus. |
|  | - 8274-900wx-48V | - 9-slot, Wide IBM 8274 Nways LAN RouteSwitch with -48V DC power supply, supports Frame/Cell Bus. |
|  | - 8274-PS5 / 8274-PS9 | 8274-500 / 8274-900 Power Supply |
|  | - 8274-PS5-250 | 8274-500, 8274-W53 250W Power Supply |
|  | - 8274-PS5-DC250 | 250W, -48V DC power supply for 8274-W53 |
|  | - 8274-PS5-DC250P | 250W, -48V DC power supply for 8274-W53 |
|  | - 8274-PS9-500P | 8274-900x 500W Power Supply |
|  | - 8274-PS9-DC500 | 500W, -48V DC power supply for 8274-900wx |
|  | - MPM-1G / MPM-1GW | MPM 16/32/64 Mbytes DRAM, 4 Mbytes Flash, HRE socket, 960M VBUS for narrow or wide chassis |
|  | - MPM II | 16/32/64Mbytes |
|  | - HRE-PLUS | Hardware Routing Engine |
|  | - ASM-155C-1E/2E | 1/2-port ATM Switching Module, 155Mbps UTP with RJ-45 connectors. |
|  | - ASM-155FSH-1EW/2EW-4C | 1/2-port Wide ATM Switching Module, OC-3, long reach single mode fiber with 4K CAM |
|  | - ASM-CE-155FM-2S2EW-4C | 2-port E1/2 serial circuit emulation + 155 Mbps ATM uplink with 4K CAM (Multimode) |
|  | - ASM-CE-155FS-2S2EW-4C | 2-port E1/2 serial circuit emulation + 155 Mbps ATM uplink with4K CAM. (Single mode) |
|  | - ASM-CE-155FM-2S2TW-4C | 2-port T1/2 serial circuit emulation + 155 Mbps ATM uplink with 4K CAM. (Multimode) |
|  | - ASM-CE-155FS-2S2TW-4C | 2-port T1/2 serial circuit emulation + 155 Mbps ATM uplink with 4K CAM. (Single mode) |
|  | - ASM-CE-DS3-2S2TW-4C | 2-port T1/2 serial circuit emulation + 45 Mbps DS-3 ATM uplink with 4K CAM |
|  | - ASM-CE-E3-2S2EW-4C | 2-port E1/2 serial circuit emulation + 34 Mbps E3 ATM uplink with 1K CAM |
|  | - ASM-DS3-1/2 | 1/2-port ATM Switching Module with 500Kbytes SRAM, 45 Mbps D3, and BNC connectors |
|  | - ASM-E3-1/2 | 1/2-port ATM Switching Module with 500Kbytes SRAM, 34 Mbps E3, and BNC connectors |
|  | - ASM2-155FM-1/2 | 1/2-port ATM Switching Module--SAHI (Multimode) |
|  | - ASM2-155FS-1/2 | 1/2-port ATM Switching Module-SAHI (Single mode) |
|  | - ASM2-622RFM-1W-4C | 1-port ATM Switching Module, 622Mbps port with 4K CAM. (Multimode) |
|  | - ASM2-622RFS-1W-4C | 1- port ATM Switching Module, 622Mbps port with 4K CAM. (Single mode) |
|  | - BPS-250 | 250W, AC Backup Power System for 8274-W33 Frame/Cell Bus. |
|  | - BPS-250-DC | 250W, DC Backup Power System for 8274-W33 Frame/Cell Bus. |
|  | - BPS-AC-PS-250 | Redundant or Spare 250W AC power supply for the AC Backup Power System (BPS-250). |
|  | - BPS-DC-PS-250 | Redundant or Spare 250W DC power supply for the DC Backup Power System (BPS-250). |
|  | - CSM-AB-CM | Internal clocking 8khz to 19.44Mhz |
|  | - CSM-155-8 or 8W | 8-port, 155Mbps OC3c/STM-1 CSM for a narrow or wide chassis. (Multimode) |
|  | - CSM-155-8S or 8SW | 8-port, 155Mbps OC3c/STM-1 CSM for a narrow or wide chassis. (Single mode) |
|  | - CSM-155-6M2SL | 6-port Multimode and 2-port single mode long reach 155 Mbps SONET ST3c/STM-1 |
|  | - CSM-155-6M2S | 6-port Multimode and 2-port single mode 155 Mbps SONET ST3c/STM-1 |
|  | - CSM-155C-8 | 8-port 155 Mbps OC-3/STM-1 copper module |
|  | - CSM-155C-8W | 8-port 155 Mbps OC-3/STM-1 copper module |
|  | - CSM-155FSL-8 | 8-port 155 Mbps OC-3/STM-1 single mode long reach |
|  | - CSM-622-2E | 2-port 622 Mbps OC12c/STM-4c, Cell Switching Module with extended memory. (Multimode) |
|  | - CSM-622-2SE | 2-port 622 Mbps OC12c/STM-4c, Cell Switching Module with extended memory. (Single mode) |
|  | - CSM-622FSH-2EW | 2-port Cell Switching Module, 622Mbps SONET/SDH OC12C/STM-4c. |
|  | - CSM-A25-12 | 12-port ATM25 module with RJ-45 connectors for a narrow or wide chassis |
|  | - CSM-A25-24 | 24-port ATM25 module, RJ-45 connectors for wide chassis |
|  | - CSM-AB-155FM/FS-2W | 2-port 155Mbps OC-3/STM-1 adapter for CSM-U (Single mode or Multimode) |
|  | - CSM-AB-155FSH-2W | 2-port OC3 adapter board, long reach single mode fiber for CSM-UW |
|  | - CSM-AB-155C-2W | 2-port 155Mbps OC-3/STM-1 adapter for CSM-U. (Single mode) |
|  | - CSM-AB-DS1-4W | 4-port DS-1 adapter for CSM-U |
|  | - CSM-AB-DS3-2W | 4-port 45Mbps DS-3 adapter for CSM-U |
|  | - CSM-AB-E1-4W | 4-port E1 adapter for CSM-UW (RJ-45). Memory supports 2K VCs/port & 4,096 cell buffers/port. |
|  | - CSM-AB-E3-2W | 2-port 34Mbps E3 adapter for CSM-UW (BNC). Memory supports 4K VCs/port & 8,192 cell buffers/port. |
|  | - CSM-AB-CE-E1/T1-4W | 4-port E1 or T1 AL-1 circuit emulation module for CSM, adapter for CSM-U |
|  | - CSM-UW | Universal Cell Switching Module. 3 NNI adapter board slots available |
|  | - FCSM | Frame to Cell Switching Module (Narrow and wide module) |
|  | - FCSM-I-4C-CSM-3.4 | Frame Cell Switching Module with 4K CAM. |
|  | - FCSM-II-4C-CSM-3.4 | Frame Cell Switching Module with high capacity internal link and 4K CAM. |
|  | - ESM-100C-FD-1/2 | 1/2-port full duplex 100BaseTx, Ethernet Switching Module (Multimode) |
|  | - ESM-100-FM/FS-FD-1/2 | 1/2- port full duplex 100BaseFx Ethernet Switching Module (Multimode/Single Mode) |
|  | - ESM-100C-5 | 4-port shared 100BaseT + 1 full duplex 100BaseTx Ethernet Switching Module |
|  | - ESM-100C-12 | 12-port 10/100 BaseT Ethernet Switching Module with RJ45 ports |
|  | - ESM-100C-32W | 32-port 10/100Base Fast Ethernet Desktop Switching Module with RJ-45 connectors. |
|  | - ESM-100C-32W-4C | 32-port 10/100Base Fast Ethernet Desktop Switching Module with RJ-45 connectors and 4K CAMs. |
|  | - ESM-100CFM-5 | 4-port shared 100BaseTx + 1 full duplex 100BaseFx Ethernet Switching Module (Multimode) |
|  | - ESM-100CFS-5 | 4-port shared 100BaseTx + 1 full duplex 100BaseFx Ethernet Switching Module (Single mode) |
|  | - ESM-100FM-8 | 8-port 100BaseFx full duplex Ethernet Switching Module with 8 SC pair (Multimode) |
|  | - ESM-100FS-8W-4C | 8-port 100BaseFx Ethernet Switch Module with CAM support for 4096 MAC addresses (Single mode) |
|  | - ESM-AB-AFD | Adapter board for Ethernet Universal Switching Module |
|  | - ESM-AB-AT | 10BaseT RJ-45 and AUI Adapter Card for ESM-U-6 |
|  | - ESM-AB-B | 10Base2 Adapter Card for ESM-U6 |
|  | - ESM-AB-FL | 10BaseFL Adapter Card for ESM-U-6 |
|  | - ESM-AB-FL-S | Single mode 10BaseFL Adapter Card for ESM-U-6 |
|  | - ESM-AB-FL-SH | Single mode (long reach) 10BaseFL Adapter Card for ESM-U-6 |
|  | - ESM-AB-T | 10BaseT RJ-45 Adapter Card for ESM-U-6 |
|  | - ESM-C-8 | 8-port 10BaseT RJ-45 Ethernet Switching Module |
|  | - ESM-C-12 | 12-port 10BaseT RJ-45 Ethernet Switching Module |
|  | - ESM-C-16W/32W | 16/32-port 10BaseT Ethernet Switching Module |
|  | - ESM-F-8 | 8-port 10BaseFL 8 port Ethernet Switching Module |
|  | - ESM-FM-8W-2C/16W-2C | 8/16 dual ST connectors to 10BaseFL hubport or device with cam support for 2048 MAC addresses (Multimode) |
|  | - ESM-T-12 | 12-port 10BaseT Ethernet Switching Module with 50 Pin Telco Connectors. |
|  | - ESM-T-24 | 24-port 10BaseT signals on two RJ21 telco connectors and CAM support for 1024 MAC addresses. |
|  | - ESM-T-24W | 24-port 10BaseT signals with two RJ21 telco connectors. |
|  | - ESM-U-6 | 6-port Ethernet Universal Switching Module |
|  | - FSM-C-4C/8C | 4/8-CDDI port FDDI Switching Module on one ring |
|  | - FSM-M-CC | 1-DAS port FDDI/CDDI Switching Module with MIC connectors. (Multimode) |
|  | - FSM-M/S-1C/2C | 1/2-port FDDI Switching Module on one ring (Multimode or Single mode). |
|  | - FSM-M/S-4C/8C | 4/8-CDDI port FDDI Switching Module (Multimode or Single mode). |
|  | - FSM-SH-1/2 | 1/2-port FDDI Switching Module with SC connectors. (Single mode) (category 2)) |
|  | - MPM-1GW-56MD-12MF | MPM 56 Mbytes DRAM, 12 Mbytes Flash, HRE socket, 640Mbps or 960Mbps Vbus support. |
|  | - MPM-UPGR-56MD-12MF | Upgrade Kit to expand the MPM DRAM memory to 56 MB & Flash to 12MB. Includes a single SIMM with 56MB of DRAM and 8MB of flash. |
|  | - TSM-CD-6 | 6-port Token Ring Dual Mode Module. |
|  | - TSM-CD-16W-4C | 16-port Token Ring Switching Module with RJ-45 ports. |
|  | - TSM-F-6 | 6-port Token Ring Switching Module configurable as a Lobe, Station or RI/RO port. |
|  | - WSM-BRI-SC-1W/2W | 1/2-serial port supporting PPP or Frame Relay, and 1-port BRI (U and S/T) supporting PPP. |
|  | - WSM-FT1/FE1-SC-1W | 2-port (1 serial and 1 Fractional T1 or E1) (with integral DSU/CSU) with compression |
|  | - WSM-FT1/FE1-SC-2W | 3-port (2 serial and 1 Fractional T1 or E1) (integral DSU/CSU) with compression |
|  | - WSM-S-2 | 2-port WAN Switching Module with a Universal Serial External Interface. Supports the five serial cable types (V.35, RS-232, X.21, RS-530, and RS449) for wide chassis without compression. |
|  | - WSM-SC-4 or 4W | 4-port WAN Switching Module with a Universal Serial External Interface. Supports the five serial cable types (V.35, RS-232, X.21, RS-530, and RS449) for narrow & wide chassis with compression. |
|  | - WSM-SC-8 or 8W | 8-port WAN Switching Module with a Universal Serial External Interface. Supports the five serial cable types (V.35, RS-232, X.21, RS-530, and RS449) for narrow & wide chassis |

| ### IBM 8277 Nways Ethernet RouteSwitch |  |  |
| --- | --- | --- |
|  | - RS-2032/2032-4C | 32-port 10BaseT switch, 1 uplink slot for a fast Ethernet or ATM sub-module with 2K/4K CAM |
|  | - RS-3032/3032-8C/3032E | 32 to 96 port 10BaseT switch, 1 uplink slot with 3K/8K CAM |
|  | - 8277 Model 424 (8277-42C) | 24-port 10/100BaseT switch with 2 fixed 100BaseTx uplinks |
|  | - 8277 Model 424 (8277-42C)F | 24-port 10/100BaseT switch with 2 fixed 100BaseFx uplinks |
|  | - 8277 Model 424 (8277-42F) | 24-port 100BaseT switch with 1-port 100BaseTx and 1-port 100BaseFx uplinks |
|  | - 8277 Model 424 (8277-42G) | 24-port 10/100BaseT switch with 2 Gigabit Ethernet 1000BaseFx (Multimode) uplinks. |
|  | - RS-5024/5024-8C | 24-port 10/100 BaseT switch, 1 uplink slot with 3K/8K CAM |
|  | - RSASM2-155FM-1 | 1-port, ATM Uplink Module (Multimode) |
|  | - RSASM2-155FS-1 | 1-port, ATM Uplink Module (Single mode) |
|  | - RSASM2-155FSH-1 | 1-port, ATM Uplink Module (Single mode long reach) |
|  | - RSASM2-155RFM-1 | 1-port, ATM Uplink Modules with physical port redundancy. (Multimode) |
|  | - RSASM2-155RFS-1 | 1-port, ATM Uplink Modules with physical port redundancy. (Single mode) |
|  | - RSASM2-622RFM-1 | 4-port (2 T1/E1 and 2 serial ports) ATM DS3 uplink w/4 CES ports. (Multimode) |
|  | - RSASM2-622RFS-1 | 4-port (2 T1/E1 and 2 serial ports) ATM DS3 uplink w/4 CES ports. (Single mode) |
|  | - RSESM-100C-4 | 4-port, 100BaseTx sub-module, 100 Mbps with RJ45 connectors. |
|  | - RSESM2-100FM/FS-2 | 2-port, 100BaseFx sub-module, 100 Mbps with SC connectors. (Multimode or Single mode) |
|  | - RSESM2-FM-2 | 2-port, 1000 Base SX IBM 8277 Nways Ethernet RouteSwitch Gigabit Uplink with SC connectors. (Multimode) |
|  | - RSESM2-FS-2 | 2-port, 1000 Base SX IBM 8277 Nways Ethernet RouteSwitch Gigabit Uplink with SC connectors. (Single mode) |
|  | - RSWSM-BRI-SC-1 | 1 Serial Port and 1 backup ISDN port, frame relay, PPP with compression. |
|  | - RSWSM-FE1/FT1-SC-1 | 1 fractional E1/T1 and 1 Serial Port, frame relay, PPP with compression. |

- The following include modules contain either HSM or HSM2 boards.

| - ASM-155C-1/2 | 1/2-port, 155 Mbps UTP 5 with RJ45 connectors ½ Mbyte SRAM. |
| --- | --- |
| - ASM-155C-1/2E | 1/2-port, 155 Mbps UTP 5, RJ45 connectors 2 Mbytes |
| - ASM-155FM-1/2 | 1/2-port, ATM Switching Module ½ and 2 Mbyte SRAM. (Multimode) |
| - ASM-155FS-1/2 | 1/2-port, ATM Switching Module with 2 Mbyte SRAM. (Single mode) |
| - ASM-DS3-1/2 | ATM ½ Mbyte w/BNC connectors |
| - ESM-100C-4/8 | 4/8-port, 100BaseT Ethernet Switching Module |
| - FSM-M-1/2 | 1/2-port FDDI Module Multimode |
| - FSM-C-4/8 | 4/8-port CDDI Module |
| - FSM-M-C | FDDI/CDDI Module |
| - FSM-S-1/2 | 1/2-port FDDI Single mode sub-module |
| - TSM-C-6 | 6-port Token Ring Switching Module (4Mbps or 16 Mbps) |
| - TSM-CD-6 | 6-port Token Ring Switching Module (4 Mbps or 16 Mbps) |
| - TSM-F-6 | 6-port Token Ring Switching Module (4 or 16 Mbps Rev B4 and above, 16 Mbps only for revisions) |

## Appendix A - Authenticated VLANs

- In order to use Authenticated VLANs, you must purchase the Authenticated Management Console: AMC-CP-1, AMC-CP-5, AMC-CP-U.

### Server

- The authentication server or AMC runs on an NT server or NT workstation 4.0. The AMC software is produced by Check Point Software Technologies Ltd.

### AMC Server Setup

- After starting the installation of the AMC, you will be asked about licenses. The license that is already installed may have already expired.

- You can order a 30-day evaluation license from IBM (see price list) or the actual product which will include a permanent license for the AMC. In order to obtain a 30-day evaluation license, use the key provided by IBM and log on to Check Point’s license web page. You must select the option to get a license for the AMC. The license you receive must include the word ‘vlan’ in the features. You cannot use a FireWall-1 evaluation license for the AMC. Always license the AMC as a management server. When asked about FireWall-1 version, respond with “3.0”.

- After adding the new license, add an administrator. Next, you will be asked to add GUI clients. GUI clients are not required if you only manage the AMC from the NT system on which the AMC is installed. If you wish to add a GUI client, add the name of the NT system on which you installed the GUI client portion of the AMC. Verify that the name of the NT system running the GUI client software exists in the host file of the NT system running the AMC.

- During the setup you will be asked to add Remote Switches. A remote switch is an authentication agent which is the switch running the user authentication software. Add each switch (using the name that is configured in the NT system's HOSTS file for that switch) that will be in the user authentication configuration. Upon adding the remote switch, you will be prompted for a password. This password is the S/KEY password you will be prompted for when you configure the user authentication software on the switch. After adding a remote switch in the AMC configuration for each of the switches involved in the user authentication configuration, the installation is complete.

- The following information is in the Authentication Management Console Administrator Guide: User Authentication VLAN Server Software.

### Troubleshooting

- If the SNMP Get button does not work, the NT system running the AMC cannot see the switch. Verify that the correct IP address is configured for the switch and verify that you can ping the switch from the NT system running the AMC.

- If the switch continues to report the log authentication failed, you are experiencing a problem with S/KEY password negotiation between the authentication agent and the server. This occurs when the authentication server (AMC) and the switch cannot sync up their connection using the configured S/KEY password. Go into the Configuration dialog and enter a new password for the remote modules corresponding to each of the switches. Verify that each of the switch names used in the configuration of each Remote Module and for each Switch Network Object is the same as the name that is found in the HOSTS file on the NT system running the AMC. Then restart the server. On the switch, run the fwconfig command again and change the S/KEY password to the new one just set on the AMC. Occasionally, it takes a couple of tries, but this will enable the AMC and the switch to sync up the S/KEY passwords.

- If telnet authentication does not work at all, make sure your telnet port is configured as a port accepting telnet authentication requests (using the avlports and avlsports commands). Verify that you are telnetting to the correct address and port. For example, your authenticated network is group 2 configured with an IP router port of 10.0.0.100 in the 10.0.0.0 network. The client is configured with the address of 10.0.0.1 and is attached to port 2/12 that is configured as a port in group 3, the client group with no IP router port. The port 2/12 is configured as a telnet authentication port and is displayed when the avlsports command is executed. The default telnet authentication address is 10.0.0.253. You would telnet from your client to 10.0.0.253, port 259.

- The AV-Client used for authentication is not supported with Token Ring endstation. Token Ring clients can use the Telnet method of authentication. (Except with release 3.4.7.) Please see the User Manual for detailed steps.

- The clients are able to authenticate correctly and seem to function correctly for a short period of time, but do not remain in the authenticated group. The moving of clients out of the authenticated group can be caused by two things: one, the regular timeout was reached, or two, group mobility is not enabled on the switch. Use the gmcfg command.

- Under RADIUS server configuration, the version pull-down menu only lists RADIUS version 1.0. This does not imply that only version 1.0 works. Check Point has successfully tested with many versions of RADIUS from various vendors with no problems. The AMC functions with RADIUS challenge and response methods implemented by various RADIUS server vendors including RACAL.

- Using the mag command, one can add a port binding rule to an authenticated group. Then one can add, modify or remove port binding rules from the group using the modatvl command. The mag command cannot be used again to add a new port binding rule to the same group, one must use modatvl. The mag command simply allows you to add the first port binding rule to an authenticated group that previously had no port binding rules.

- When starting the Policy Editor, you cannot connect to the server. After the timeout, it says that it cannot connect to the server yet in the services dialog, it shows that the firewall daemon is running. If you see this, verify that the name you entered for the AMC server matches the name of your NT system.

- You can authenticate, but you still do not belong to the authenticated group you are configured into. Double check that you used the correct license. Licenses obtained from the Check Point WEB page are only usable on the AMC if the features of the license include "vlanl", "vlan5" or "vlans". If you use a Firewall-1 license, the AMC will appear to work but will not grant the user the actual authentication into an authenticated group configured on the switch.

### Helpful Hints and other Notable Information

- Use the macinfo command to verify that the MAC address of the client belongs to the correct group.

2. Turning off Spanning Tree on the client ports eliminates the wait for the port to become active. Setting the port to Optimized will eliminate this delay BUT be aware that setting the port to optimized will eliminate the ability to time the MAC addresses out. In order to maintain the timeout on the ports, Spanning Tree must be running on the port. If timeouts are required and you do not want to run Spanning Tree, contact customer support for details on how to configure timeouts on optimized ports.

- Changing group configuration on the switch causes all of the groups to be flushed. The result is that all authenticated clients are flushed back to their original groups requiring re-authentication. Always remember to update the switches configuration on the AMC when changes in group configuration have been made.

- At this time, you cannot use a secondary authentication server. The software allows you to configure it but the functionality is not yet supported.

- Make sure DNS is working correctly on the NT system running the AMC. If NT tries to use DNS and cannot reach the DNS server, the installation will take a very long time to finish. Disabling DNS completely during installation will allow the installation to proceed much faster.

- Verify that the NT system's name and the name used in the AMC's configuration for the NT system (the workstation running the AMC) match. These two names must be the same and the name must be in the hosts file or the Policy Editor application gets confused.