CHAPTER 1

GENERAL

1.1	Introduction

This manual contains instructions for the operator of the SPIDER manpack radio transceiver.

The SPIDER manpack described in this manual is a portable VHF transmitter-receiver used to provide two-way voice communication in tactical military radio networks. When using a second transceiver retransmission is possible.

The battery compartment of the transceiver contains alternatively dry cells, or a rechargeable battery block.

An optional Crypto/Data unit may be integrated with the set, providing facilities for high-speed data transfer, encryption of voice or low-speed data,   pre-coded messages and selective calling. This manual, however, refers only to the SPIDER manpack without Crypto/Data unit.

1.2	Technical data

Frequency range	:	30.000 ... 107.975 MHz.

Number of channels	:	3120 in steps of 25 kHz.

Number of preset channels:	9 user programmable channels.

Modulation system	:	FM (frequency modulation).

Power supply	:	10 alkaline batteries type R14 (to be replaced) or battery block of NiCad cells (rechargeable).

Battery life (NiCad)	:	approx. 10 hours.

Transmitter power	:	four user selectable levels:

.	low power

.	medium power

.	high power

.	burn-through power

Ambient temperature range

Operating	:	-30 ... +65°C

Storage	:	-40 ... +70°C

Dimensions	:	240 x 175 x 66 mm

Weight	:	approx. 2.6 kg (transceiver without accessories)

approx. 5 kg

(complete set)

1.3	Component parts

(See diagram 1)

The SPIDER manpack transceiver is composed of the following parts.

1. 	Transceiver

2.	Carrying harness

3.	Antenna matching unit

4.	Antenna

5.	Handset and/or headset

6a.	Rechargeable battery block or

6b.	Battery holder with 10 dry cells type R14

7.	Instruction card.