CHAPTER 3

PREPARING FOR USE

3.1	General

This chapter provides the instructions for installation and for programming the preset channels.

3.2	Installation

(See diagram 3)

1.	Open the battery compartment to check if a battery is present. If not install the battery. For details see para. 5.2.

2.	Mount the antenna matching unit on the coaxial antenna connector.

3.	Mount the antenna on top of the antenna matching unit.

4.	Connect a handset and/or a headset to the 6-pins and/or the 10-pins audio connector of the set.

5.	Program the preset channels, if this has not been done earlier. See para. 3.3.

6.	Install the transceiver on the carrying harness and fasten it with the strap.

3.3	Channel presetting

For each channel to be programmed the following preset data must be used:

-	Frequency value.

-	Power level for transmission.

-	Squelch facility on/off.

Act as follows:

1.		Switch on the set by turning the volume control switch to any position except OFF or Z.

2.		An audio tone is heard until the transceiver is ready for use (approx. 4 seconds). The display shows the channel and frequency last used.

NOTE:	If the audio tone continues and/or NOGO appears on the display, or if the set is completely "dead", replace the battery. If this doesn't help, the transceiver is defect.

3.		Select the channel to be preset by pressing the key CH, followed by one of the keys 0...8. If a handset with channel selector switch is used first turn this switch to channel 0.

4.		Enter the desired frequency by pressing the keys F, FREQ and the necessary numerical keys.

Four or five digits have to be entered from left to right, e.g. when 98.200 MHz is required, key in 9-8-2-0. The last digit will be completed by the set.

5.		Confirm by pressing key ENT. An audio tone will be heard during approx. 1 second and the display shows the selected frequency.

The frequency must be in the range 30.000...107.975 MHz. A non-valid input will be ignored.

ATTENTION: when entering a frequency or other data, the time between two key strokes must be less than 10 seconds.

If not the input disappears again and you will have to start anew.

6.		Enter the desired power level by pressing the keys F, PWR and one of the keys 1...3.

1  =  low power

2  =  medium power

3  =  high power

Confirm by pressing key ENT. Select low power for a range of less than 1 km, medium power for a range of up to 2 km, high power for longer distances.

Note: the ranges given are valid for flat, open country. In densely wooded areas or towns, or if the transmission path is obstructed by e.g. a hill, the range may be much less.

7.		If squelch is desired press keys F, SQ and ENT. The display shows "SQ".

8.		If no squelch is desired press keys F, SQ, SQ and ENT. The display doesn't show "SQ".

9.		If more channels are to be preset, repeat item 3...8 for the relevant channel number(s).

NOTE:	The preset data are retained indefinitely if the set is switched off and/or the battery is removed. Only a zeroise operation (see para. 4.8) will erase the preset data. It is not necessary to erase the preset data when one or more channels are to be preset to a new frequency.

NOTE:	Some channels may be suppressed. See ANNEX for a list of suppressed channels.