CHAPTER 5

OPERATORS MAINTENANCE

5.1	General

The operators maintenance is mainly preventive maintenance, aimed at keeping the set in good condition and so preventing defects.

Replacing the battery is also part of the operators maintenance.

5.2	Replacing the battery

(See diagram 4)

1.	The hinged cover of the battery compartment is closed by means of two clamps at one side. Each clamp consists of an inner part (A) and an outer part (B) as shown in diagram 4.

2.	To loosen the clamps, clip-down (B) (arrow indication) and clip-off (A). Keep the cover tightly pressed down if necessary.

3.	When the cover is open, the rechargeable battery block (C) or the battery holder with the 10 dry cells (D) can be removed. See note on diagram 4.

4.	Replace the battery block by a freshly charged one or replace the 10 dry cells by new ones.

5.	Place the battery block or battery holder in the battery compartment.

6.	Close the cover by first hooking the inner part (A) of the clamps over the lug and then pressing down the outer part (B).

7.	If a rechargeable battery block is used, hand in the discharged battery for recharging.

5.3	Preventive maintenance

Preventive maintenance of the SPIDER manpack consists of two parts:

1.	Cleaning the set, as described in para. 5.4.

2.	A routine test procedure, as described in para. 5.5.

When starting the preventive maintenance, always check first that all parts of the set (see para. 1.3) are present and free from damage.

5.4	Cleaning the transceiver

The cleaning procedure comprises the following items:

1.	Remove dust and loose dirt with a clean soft cloth or with a soft brush. For dirt that is difficult to remove damp the cloth with water.

2.	To remove any moisture use a dry clean cloth.

3.	To remove grease, oil or such-like from the equipment use a cloth moistened with cleaning spirits.

WARNING: Cleaning spirits are highly inflammable!

4.	Remove dust and dirt from plugs and receptacles, taking care not to damage the pins.

5.5	Routine test procedure

A routine test must be performed in the following events:

-	after the transceiver is installed and is ready for operation.

-	periodically for preventive maintenance.

-	when the operation of the transceiver or the connected equipment is not satisfactory.

To check the audio and transceiver functions, a handset (or similar accessory) should be connected to the set and a second radio station (on the same channel) must be available for call procedures.

No special tools are required for the routine test.

The routine test procedure is arranged in the following table, for the operating instructions see chapter 4.

Actions to be carried-out by the operator are given in steps with the correct results in the second column. In case a malfunction should be detected, the corrective actions to be taken are mentioned in the next paragraph (Trouble-shooting).

┌────┬─────────────────────────┬────────────────────┐

│STEP│         ACTION          │      RESULT        │

├────┼─────────────────────────┼────────────────────┤

│  1 │Switch-on the set and    │Last (or default)   │

│    │turn the rotary switch to│selected channel    │

│    │1.                       │number and operatio-│

│    │Wait till the continuous │nal settings will be│

│    │tone has stopped.        │displayed.          │

│    │                         │                    │

│  2 │Select successively the  │The operational     │

│    │preset channels to be    │frequency and pre-  │

│    │used and check their     │settings are shown  │

│    │operational settings and │on the display for  │

│    │radio frequency.         │the selected        │

│    │                         │channel.            │

│    │                         │                    │

│  3 │Select the channel (or   │The radio frequency │

│    │frequency) on which      │for this channel is │

│    │another radio station is │displayed with the  │

│    │available.               │operational         │

│    │                         │settings.           │

│    │                         │                    │

└────┴─────────────────────────┴────────────────────┘

┌────┬─────────────────────────┬────────────────────┐

│STEP│         ACTION          │      RESULT        │

├────┼─────────────────────────┼────────────────────┤

│  4 │Set-up a call to the     │After a response    │

│    │other radio station, if  │from the other      │

│    │necessary increase the   │station the         │

│    │power level.             │connection is       │

│    │Check meanwhile the      │established.        │

│    │connected audio accessory│                    │

│    │(adjust the volume level │                    │

│    │if required).            │                    │

│    │                         │                    │

│  5 │Check if radio traffic on│The SQ field is     │

│    │this channel is possible.│displayed when the  │

│    │Check the squelch        │squelch function is │

│    │function.                │enabled.            │

│    │                         │                    │

│  6 │Check the correct        │Radio traffic is    │

│    │operation of the set in  │possible.           │

│    │both directions and      │                    │

│    │finish the call.         │                    │

│    │                         │                    │

│  7 │If required, repeat the  │See step 3...6      │

│    │call procedure for a     │mentioned before.   │

│    │specific station to check│                    │

│    │the radio link involved. │                    │

│    │                         │                    │

│  8 │Select a free channel    │The given digits    │

│    │and enter some (valid)   │will be displayed   │

│    │frequency values to check│(from left to right)│

│    │the keyboard input.      │when keyed-in.      │

│    │                         │                    │

│  9 │Check the display        │The brightness level│

│    │illumination by using the│of the display      │

│    │LAMP function:           │illumination is     │

│    │0    = switched-off      │adjustable.         │

│    │1...3= switched-on       │                    │

│    │(3   = max. brightness). │                    │

│    │                         │                    │

│ 10 │When the transceiver has │                    │

│    │passed the routine test, │                    │

│    │no further action is     │                    │

│    │needed.                  │                    │

└────┴─────────────────────────┴────────────────────┘

5.6	Trouble-shooting

The following trouble-shooting procedures are given for the operator in case a malfunction should occur at start-up or during operation of the radio set.

Procedures are given for the following events.

a.	Transceiver doesnot work at all

-------------------------------

(i.e. no continuous tone and display contents after switching-on):

-	Check the batteries. See par. 5.2 for replacing the batteries.

-	Check the internal supply by means of the display illumination (use LAMP-function).

-	Try switching the set off and then on again.

No result: higher level maintenance necessary.

b.	No audio signal

---------------

-	Replace the handset or headset by a spare one.

No result: higher level maintenance necessary.

c.	No radio traffic possible

-------------------------

-	Check if the connectors are free from dirt, dust or moisture. Clean if necessary.

-	Check the antenna.

-	Check the correct functioning of the PTT button.

-	Check the radio frequency to be used for the relevant channel.

-	The station being called is not operational on the required channel or frequency used. If possible, try to obtain a connection via another channel or to another station.

No result: higher level maintenance necessary.