CHAPTER 1

GENERAL

1.1	Introduction.

This manual contains instructions for the operator of the low power vehicular set, based on the SPIDER manpack radio transceiver. Only the aspects typical for the vehicular installation are described in this manual, for operation of the radio transceiver consult the operators manual of the SPIDER manpack transceiver or the instruction card issued with the manpack transceiver.

For operation as a vehicular radio set the SPIDER manpack is removed from its carrying harness and fastened in a vehicular mounting. The whip antenna and antenna matching unit are replaced by a broadband vehicular antenna, connected via a coaxial cable. Power is obtained from the vehicular battery via a small power supply unit.

1.2	Technical data.

For the technical data of the radio transceiver consult the manual of the SPIDER manpack.

Power supply	:	vehicular battery 24V DC nominal, limits 22 ... 33V DC

Current consumption	:	200 mA or less when receiving.

Approx. 2A when transmitting at maximum power

Output voltage	:	14V DC nominal.

power supply unit		Limits 13,5 ... 14,5 DC

Max. output current	:	2A

Antenna frequency range	:	30 ... 108 MHz

Antenna impedance	:	50 ohms nominal.

Standing wave ratio 4 : 1 max.

Ambient temperature range

Operating	:	 -30 ... +65°C

Storage	:	 -40 ... +70°C

Dimensions

Mounting	:	233 x 190 x 66 mm

Power supply unit	:	190 x 112 x 90 mm

1.3	Component parts

(see diagram 1)

The following parts are used to convert the SPIDER manpack into a vehicular radio set.

1.	Vehicular mounting.

2.	Power supply unit.

3.	Broadband antenna.

4.	Power cable (24V).

5.	Supply cable (PSU - SPIDER)

6.	Antenna cable.