CHAPTER 3

OPERATION

3.1	General

This chapter only describes the operation typical for the vehicular set. The operation of the radio transceiver itself is covered by the operators manual of the SPIDER manpack transceiver. Condensed operating instructions are available on the instruction card issued with the set.

3.2	Mounting and removing the set

(see diagram 3)

When mounting the manpack transceiver in the vehicle, act as follows:

1.	Remove the antenna and the antenna matching unit from the set.

2.	Loosen the strap and take the transceiver from the carrying harness.

3.	Place the transceiver in the vehicular mounting and fasten it with the strap. Note that if the vehicular mounting is installed in a vertical position, the transceiver must be mounted reversed, to keep the display and the keyboard legible.

4.	Connect the following cables:

*	Coaxial antenna cable to the antenna connector of the transceiver.

*	Power supply cable to the 10-pole connector at the side of the transceiver.

*	Handset and/or headset to the audio connector(s) on the front of the transceiver.

5.	Mount the antenna rods of the vehicular antenna on top of the antenna base, if this has not been done before.

6.	The set is now ready for switching on.

For removing the set act as follows:

1.	Switch off the transceiver and the power supply unit.

2.	Disconnect the power supply cable and the antenna cable.

3.	Loosen the strap and take the transceiver from the vehicular mounting.

4.	Open the battery compartment to check if a battery is present. If not install the battery. For details see the operators manual of the SPIDER manpack.

5.	Mount the antenna matching unit on the coaxial antenna connector.

6.	Mount the antenna on top of the antenna matching unit.

7.	Connect a handset and/or a headset to the audio connector(s) of the set, if this has not been done before.

8.	Install the transceiver on the carrying harness and fasten it with the strap.

3.3	Switching -on and -off

For switching -on act as follows:

1.	Set the switch on the power supply unit to

2.	Turn the volume control switch on the transceiver to one of the positions W or 1...6.

For switching -off act as follows:

1.	Turn the volume control switch on the transceiver to OFF.

2.	Set the switch on the power supply unit to

Notes:

1.	With the power supply unit switched off and the transceiver switched on, the set will operate on the manpack batteries, provided that these have not been removed.

2.	Charging of the manpack battery block by means of the power supply unit is not possible. For charging the battery block must be removed from the battery compartment and a separate battery charger must be used.