CHAPTER 4

OPERATORS MAINTENANCE

4.1	General

The operators maintenance is mainly preventive maintenance, aimed at keeping the set in good condition and so preventing defects.

Preventive maintenance consists of two parts:

1.	Cleaning the set, as described in para. 4.2.

2.	A routine test procedure, as described in

para. 4.3.

When starting the preventive maintenance, always check first that all parts of the set (see para. 1.3) are present and free from damage.

4.2	Cleaning the set

The cleaning procedure comprises the following items:

1.	Remove dust and loose dirt with a clean soft cloth or with a soft brush. For dirt that is difficult to remove damp the cloth with water. Remove the transceiver from its mounting for cleaning.

2.	To remove any moisture use a dry clean cloth.

3.	To remove grease, oil or such-like from the equipment use a cloth moistened with cleaning spirits.

WARNING: Cleaning spirits are highly inflammable!

4.	Remove dust and dirt from plugs and receptacles, taking care not to damage the pins.

4.3	Routine test procedure

A routine test must be performed in the following events:

-	After the transceiver is installed and is ready for use.

-	Periodically for preventive maintenance.

-	When the operation of the transceiver or the connected equipment is not satisfactory.

For the routine test use the procedure described for the manpack, para. 5.5 of the operators manual of the SPIDER manpack transceiver. To make sure that the vehicular power supply unit is included in the test, remove the batteries from the battery compartment of the transceiver before starting the test.

4.4	Trouble shooting

The following trouble-shooting procedures are given for the operator in case a malfunction should occur at start-up or during operation of the radio set.

a.	Transceiver doesn't work at all

(i.e. no continuous tone and display contents

after switching on):

-	Disconnect the power supply unit and install the battery pack. Repeat the test, using the manpack configuration.

-	If the set is now ok, there is a defect in the power supply unit or the associated cables. If the set still doesn't work, the transceiver itself is probably defective.

-	Check the battery by means of the display illumination (use LAMP-function).

-	Try switching the set off and then on again.

No result: higher level maintenance necessary.

b.	No audio signal, but display contents visible

-	Replace the handset or headset by a spare one.

No result: higher level maintenance necessary.

c.	No radio traffic possible

-	Check if the connectors are free from dirt, dust or moisture. Clean if necessary.

-	Check the antenna.

-	Check the correct functioning of the PTT button.

-	Check the radio frequency to be used for the relevant channel.

-	The station being called is not operational on the required channel or frequency used. If possible, try to obtain a connection via another channel or to another station.

No result: higher level maintenance necessary.