CHAPTER 1

GENERAL

1.1	Introduction

This manual provides instructions for field maintenance of the SPIDER manpack radio transceiver. The SPIDER manpack described in this manual is a portable VHF transmitter-receiver used to provide two-way voice communication in tactical military radio networks.

The battery compartment of the transceiver contains alternatively dry cells or a rechargeable battery block.

In combination with an external power supply unit the transceiver can also be used as a low power vehicular radio set.

An optional Crypto/Data unit may be integrated with the set, providing facilities for high speed data transfer, encryption of the voice or low-speed data, pre-coded messages and selective calling.

This manual, however, refers only to the SPIDER manpack without Crypto/Data unit.

1.2	Technical data

1.2.1	General

Frequency range	:	30.000...107.975 MHz

Channel spacing	:	25 kHz

Number of channels	:	3120

Number of preset channels	:	9

Modulation mode	:	frequency modulation

Frequency deviation	:	5 kHz nominal

Speech frequency band	:	300...3400 Hz

Squelch mode	:	150 Hz tone squelch

Supply voltage	:	nominal 12V DC

(min. 9V DC, max.

15V DC)

Ambient temperature range

Operating	:	-30...+65°C

Storage	:	-40...+70°C

Dimensions	:	240 x 175 x 66 mm

Weight	:	approx. 2.6 kg

(transceiver without accessories)

approx. 5 kg

(complete manpack)

1.2.2	Receiver

Sensitivity	:	0.4 μV for 10 dB SINAD

AF output	:	max. 2V RMS in a 500 ohms load

Image rejection	:	60 dB or better

Selectivity	:	60 dB or more suppression of a signal ± 25 kHz from the receiver frequency

Current consumption	:	approx. 140 mA

1.2.3	Transmitter

RF output power	:	four user selectable levels

20 mW   ( 7...19 dBm)

200 mW  (20...28 dBm)

2 W     (31...35 dBm)

5 W     (33...39 dBm)

Load impedance	:	50 ohms

Max. standing wave ratio	: 4:1

Harmonic suppression	:	50 dB or better

(60 ... 400 MHz)

Current consumption	:	approx. 1.2A for 2W output power

approx. 2A for 5W

output power

1.2.4	Interfaces

The SPIDER manpack provides interface connecting points as listed below for the following connectors:

a. Antenna connector.

b. Audio 6-pole connector.

c. Audio 10-pole connector.

d. Supply/Peripheral connector.

The connector points may be used for signal transfer, control, selection and supply.

a.	Antenna connector

50 Ohms coax = Antenna in/output for the radio

frequency range 30 - 108 MHz

b.	Audio 6-pole connector

A =	Common ground

B =	Telephone output: max. 2V RMS in 500 Ohms

C =	Transmit contact (PTT input).

D =	Microphone input: - whisper  0.5 -  50 mV RMS

- normal     2 - 200 mV RMS

- retransmission 500 mV RMS *)

E =	Squelch contact (open collector output).

*)	On 10-pole connector only

F =	Channel preset selection: an external connected resistor (e.g. in the hand-set) may define the preset channel to be selected:

- 47   kOhms (2%)		= channel 1

- 18   kOhms		= channel 2

- 10   kOhms		= channel 3

-  6.8 kOhms		= channel 4

-  4.7 kOhms		= channel 5

-  3.3 kOhms		= channel 6

-  2.2 kOhms		= channel 7

-  1.5 kOhms		= channel 8

- no resistor (open)	= channel selection via

keyboard.

c.	Audio 10-pole connector *)

A =	Common ground.

B =	Telephone output (see b.)

C =	Transmit contact.

D =	Microphone input (see b.).

E =	Squelch contact.

F =	"NOGO" information (active low).

G =	not used.

H =	Peripheral address: an external (built-in) resistor may define the address as implemented for the peripheral equipment to be connected:

- 47   kOhms (2%)		= Retransmission cable.

- 18   kOhms		= Message Exchange

Device.

-  6.8 kOhms		= Intercom system.

-  3.3 kOhms		= Fill gun device.

- no resistor (open)	= Hand- or head-set.

J =	+12 Volt supply to be used for peripheral equipment.

K =	"Data mode" information (active low).

*)	The V24/28 data-interface is provided when the Crypto/Data unit is available and the data mode is selected.

d.	Supply/Peripheral connector

A = External supply ground  ───┐   Internally

B = External supply input      │   connected if

(+10...15 Volt)            ├── external supply

│   is present with

│   the correct

───┘   polarity.

C =	Battery recharge current input (max. 100 mA).

D = Data signal ground  ───┐    Data interface

E = Data signal output     ├──  (V24/28, 2400 Bd)

F = Data signal input   ───┘    for control of

peripheral equip-

ment.

G = +5 Volt supply for peripheral use.

H =	Peripheral address, to be defined by an external resistor included in the peripheral connection.

-  47 kOhm (2%)		= Remote control.

- 6.8 kOhms		= External equipment.

- 2.2 kOhms		= Test equipment.

- no resistor (open)	= Supply cable.

J = Common ground.

1.2.5	Power supply

Internal *)	:	10 batteries type R14, Nicad (rechargeable) or Alkaline (to be replaced).

External	:	10...15V DC, max. 2A to be supplied via the rear connector.

*)	Re-charge facility by connecting a battery charger via the rear connector, the battery charge current should be max. 100 mA.

1.3	Component parts

The SPIDER manpack transceiver is composed of the following parts.

Unit				Signaal Code nr.

Transceiver			9556 304 14800

Carrying harness			9556 304 20600

Antenna matching unit			9556 304 20400

Antenna (long)			9556 004 14100

Handset			9556 004 08200

Rechargeable battery block		9556 304 20700

Instruction card			HGT5-2528e

Optionals			Signaal Code nr.

Battery holder for 10 dry cells	9556 304 20300

Handset with channel selector switch	9556 004 08300

Headset			9556 304 27400

Retransmission cable			9556 820 60000

Antenna (short)			3522 023 17370

Replaceable printed circuit boards	Signaal Code nr.

Transmitter - receiver PCB		9556 820 27500

Exciter - Power supply PCB		9556 820 27600

LF - control PCB			9556 820 27700

Reference circuit PCB			9556 820 33100

Front panel PCB			9556 820 27400

1.4	Differences among models

No different versions of the SPIDER manpack described in this manual exist. The SPIDER manpack with an integrated Data/Crypto unit is described in a separate manual.