CHAPTER 3

FIELD MAINTENANCE

3.1	General

This chapter provides instructions for testing, trouble-shooting and simple repair work on the SPIDER manpack transceiver.

Two separate steps in field maintenance can be distinguished:

a)	Testing and repair in the field. This is described in para. 3.4 and 3.5.

b)	Testing and repair in the (mobile) workshop. This is described in para. 3.6, 3.7 and 3.8.

3.2	Responsibilities

1.	Checking that operator maintenance has been carried out properly and if necessary giving advice for improving it.

2.	Inspecting the equipment on its operational condition, using the available test equipment and methods.

3.	If the equipment is not operational, localizing the defect down to a defective main component, such as a handset, headset, antenna tuning unit or transceiver.

4.	Replacing the defective main component by a non-defective one from the available stock.

5.	Taking the defective main component to the workshop and repairing it, in as far as the field maintenance engineer is authorised to do so.

6.	Sending defective main components that cannot be repaired locally, or defective PCB's to the depot maintenance workshop for repair.

WARNING: Do not remove the covers of the transceiver and replace PCB's in the field! This must be done in a properly equipped workshop, located either in a building or in a shelter mounted on a truck. The PCB's can be damaged easily either by rough handling or by electrostatic discharge.

3.3	Test equipment

To test the radio sets of the SPIDER family a special test box is used. This test box offers the following facilities:

.	Checking DC voltages (battery or power supply voltage).

.	Checking receiver sensitivity.

.	Checking transmitter output.

.	Checking antenna matching.

For this purpose the following circuits are incorporated in the test box:

a.	A signal generator capable of producing a signal on the frequencies:

32.000 MHz

38.400 MHz

44.800 MHz

51.200 MHz

57.600 MHz

64.000 MHz

70.400 MHz

76.800 MHz

83.200 MHz

89.600 MHz

96.000 MHz

102.400 MHz

These signals are so weak that they can only be detected by a receiver of sufficient sensitivity.

b.	A measuring circuit for DC voltages.

This circuit uses a row of light emitting diodes as indicator instead of a moving coil instrument, as the latter is too fragile for use in the field. The circuit is used in all tests.

c.	An audio detector used to measure the receiver noise output during the sensitivity test.

d.	A reflectometer circuit used to measure the transmitter output power and the reflected RF power caused by antenna mismatch.

e.	A 50-ohm dummy load used to replace the antenna during transmitter output power measurements and for "off the air" testing.

f.	An alarm LED to indicate a wrong connection during the transmitter output lower test.

g.	A range switch to select the various tests.

h.	Connectors and cables to connect the test box to the equipment under test.

j.	A dry or rechargeable 12 Volts battery with a stabilizer circuit for the power supply of the testbox.

The testbox has the following controls, connectors and cables (see diagram 8):

| Control or Connector | Symbol | Explanation |
| --- | --- | --- |
| Range switch | 1 OFF 2 ──┐ 3 │ │ │ │ │ │ │ │ │ │ 4 │ │ │ ├──R 5 │ │ │ │ │ │ │ │ 6 │ │ │ 7 │ ──┘ ──┐ 8 │ │ 9 │ ├──T 10 │ │ │ │ 11 │ ──┘ 12 Ω | Used to select the various tests. - Testbox switched off, (Nicad battery can be charged,if used.) - Battery check of test box. - Test of power supply voltage of the trans- ceiver (battery test in portable set, stabilizer circuit in vehicular sets). DC voltage measurements possible, using cable 04. Voltage range 10..30 VDC - Indication of AF output of receiver under test. Part of receiver sensi- tivity test. - A weak RF signal is con- nected to the receiver under test. This results in a reduction of the noise output of the receiver, as measured in position 4 of the switch. - No signal to receiver. Test of the squelch cir- cuit (de-activated). - Weak tone modulated signal to receiver. Test of the squelch cir- cuit (activated). - Transmitter output test (max. 5W). - Test of antenna matching (using low power). - Transmitter output test (high power, up to 45W not for portable transceiver). - Test of antenna matching (using high power). - Resistance measurement, using the (optional) adaptor unit. |
| Coax. Connector | RF TO REC. | Output of the signal generator of the testbox. To be connected to the antenna connector of the set under test during the receiver sensitivity test, using cable 01 |
| Coax. Connector Coax. Connector | IN ──┐ │ │ │ ├─TX │POWER │ │ │ OUT──┘ | Input of the reflecto- meter circuit. To be con- nected to the antenna connector of the set under test during the transmitter output test and the antenna matching test, using cable 01 Output of the reflecto- meter circuit. To be con- nected to the 50-ohm load during the transmitter output test and with the antenna during the antenna matching test. |
| Coax. Connector | 50 Ω LOAD | Input of the 50-ohm dummy load used during the transmitter output test. Use the short coaxial cable 02 to connect "50 Ω LOAD" with "TX POWER OUT". |
| Audio Connector | HANDSET | The telephone handset of the set under test may be connected here. During the receiver test noise is heard. The handset is not necessary for testing. |
| 10-Pole Connector | SPIDER | This connector is connec- ted to the 10-pole audio connector of the trans- ceiver under test, using cable 03. When using cable 04 voltage checks are possible. |
| Alarm LED | CHANGE CABLE 01 | If the transmitter is switched on (test 8... 11) and no cable is con- nected to "TX POWER IN" this LED lights. |
| Indicator LED's | 1...16 | These LED's are used in all tests as an indicator of the result obtained. Basically the LED's are part of a voltmeter circuit. Each LED corresponds to a certain voltage, LED "16" with the highest voltage, LED "1" with the lowest. Two adjoining LED's light for "in between" voltages. |
| Cable | Symbol | Explanation |
| Long coax. cable | 01 | Used to connect the antenna connector of the set under test with either "RF TO REC" or "TX POWER IN". |
| Short coax. cable | 02 | Used to connect "TX POWER OUT" with "50 Ω LOAD". |
| Audio cable | 03 | Used to connect the 10- pole connector of the transceiver under test with connector "SPIDER". |
| Cable with test probe | 04 | Used for voltage measurements. Range 10...30 V DC. Connect the probe to +, the earth wire to -. RF voltage checks are also possible. |
| Adaptor unit (optio- nal), with test leads | 05 | To be mounted on con- nector "SPIDER". Offers additional voltage and resistance ranges. See manual of test box for details. |

3.4	Test Procedure Manpack

The procedure is given in the form of tables. The first column gives the sequence number of each test to be performed.

The second column - operation - states what operations are to be performed.

The third column - normal result - states the result the operation should have if all is well.

The fourth column - wrong - indicates the item number of the trouble shooting procedure (paragraph 3.5) that should be consulted to eliminate the fault.

Before starting the test, check that the transceiver is switched off.

Connect the testbox as shown in diagram 9 fig. a):

.	"RF TO REC." via cable "01" to antenna connector of the transceiver.

.	"SPIDER" via cable "03" to 10-pole audio connector of the transceiver.

.	Handset to connector "HANDSET" of the testbox.

.	Range switch of the testbox to position "1".

| No. | Operation | Normal result | Wrong |
| --- | --- | --- | --- |
| 1 | Set range switch of the testbox to "2". | LED 12...16 lights. | Para. 3.5 item 1 |
| 2 | Set volume control switch of the trans- ceiver to "3". Set range switch of the testbox to "3". | Alarm tone from handset (max. 4 seconds), fol- lowed by noise. LED 2...16 lights. | Para. 3.5 item 2 |
| 3 | Select channel 0 of the trans- ceiver and set the frequency to 32.000 MHz, no squelch. | Alarm tone from handset (max. 4 seconds), fol- lowed by noise. | Para. 3.5 item 2 |
| 4 | Set range switch of the testbox to "4". Adjust volume control switch of the trans- ceiver so that one or two of the LED's 10...15 lights. | Noise from handset adjust- able with volume control switch. | Para. 3.5 item 3 |
| 5 | Set range switch of the testbox to "5". | Less noise from handset. LED 1...7 lights. The number of the LED that lights must be less than half the number of the LED in test 4. | Para 3.5 item 4 |
| 6 | Repeat test no. 4 and 5 for the following fre- quencies: 38.400 44.800 51.200 57.600 64.000 70.400 76.800 83.200 89.600 102.400 | Same as test no. 5. Note: after a frequency change the alarm tone is heard in the handset. Wait till this has stopped (max. 4 seconds). | Para. 3.5 item 4 |
| 7 | Set range switch of the testbox to "6". Switch on the squelch of the transceiver. | No noise from handset. LED 16 lights. | Para 3.5 item 5 |
| 8 | Set range switch of the testbox to "7" | LED 1...4 lights. Note: this test on 102.400 MHz only! | Para 3.5 item 6 |
| 9 | Disconnect cable "01" from "RF TO REC." and connect the cable to " TX POWER IN". Connect "TX POWER OUT" with "50Ω LOAD", using cable "02". See dia- gram 9 fig. b. The transceiver is still tuned to 102.400. Set the trans- ceiver for high power (PWR 3). |  |  |
| 10 | Set range switch of the testbox to "8". | LED 5...16 lights. | Para 3.5 item 7 |
| 11 | Repeat test no. 10 for the fol- lowing frequen- cies: 90.400 80.400 70.400 60.400 50.400 40.400 30.400 (Other frequen- cies may be used if desired) | LED 5...16 lights. Note: after a frequency change the transceiver may need up to 4 seconds to readjust all tuned circuits. During this time the alarm tone is heard in the handset and LED 1 lights. | Para 3.5 item 7 |
| 12 | Remove cable "02". Mount the antenna matching unit (AMU) plus antenna on plug "TX POWER OUT". See diagram 9, fig. C Transceiver frequency is still 30.400 MHz | LED 5...16 lights. (Note which one) | Para. 3.5 item 7 |
| 13 | Set range switch of the testbox to "9" | LED 1...16 lights. (The same, or lower number as during test 12, never higher). | Para 3.5 item 8 |
| 14 | Repeat test 12 (range switch to "8") and 13 for the fol- lowing frequen- cies: 70.400 102.400 | See test 12 and 13. | Para 3.5 item 8 |
| 15 | Set the range switch of the test box to "1". Set the volume control of the trans- ceiver to "OFF" Remove all test cables. |  |  |

Note:	switch positions "10" and "11" of the test box are not used when testing this set. Position "12" is for fault tracing only.

3.5	Trouble shooting in the field.

The trouble shooting procedure in this paragraph should not be used on its own. Always start with the test procedure described in paragraph 3.4. When during this test procedure a deviation occurs, refer to the indicated item of this paragraph.

1.	LED 1...11 lights (or none): battery of test box flat. Before starting the test recharge or replace the battery.

2.	LED 1 lights: battery exhausted. Replace or recharge. Lamp 2, 3 or 4 lights: battery almost exhausted. Alarm tone does not stop and/or NOGO indicated by display: replace transceiver.

3.	Not possible to adjust the volume control so that one of the LED's 10...15 lights, or incorrect operation of the volume control: replace transceiver.

Note:	the LED's indicate the noise output of the receiver. For this reason the LED's may flicker, or adjacent LED's may light now and then. This is not a fault.

4.	LED 8...15 lights: insufficient sensitivity of receiver. Replace transceiver.

5.	Incorrect operation of squelch: replace transceiver.

6.	Incorrect operation of squelch circuit: check if frequency of 102.400 MHz is selected. Still wrong: replace transceiver.

7.	LED 1...4 lights: transmitter output too low or zero. Check battery. Battery O.K.: check if PWR 3 is selected. Still wrong: replace transceiver.

8.	Wrong LED lights: check the antenna. Antenna O.K.: replace AMU.

3.6	Workshop equipment.

Repair of the SPIDER manpack transceiver at field maintenance level is done by localizing the fault down to a PCB, and then replacing this defective PCB by a good one. As the PCB's can be easily damaged both by rough handling and by electrostatic discharge (ESD), this must be done in a properly equipped workshop. Such a workshop can be located in a building, but also in a shelter mounted on a truck. The following equipment should be available:

1.	A workbench suitable for working on ESD sensitive components. Such a workbench should have an anti-static or conductive surface. Persons at the work- bench should be earthed via a wriststrap and a resistor. All equipment should be earthed.

2.	An adjustable stabilized power supply capable of supplying 0...35V DC by up to 4A.

3.	A universal volt-ampere - ohm meter.

4.	An oscilloscope (DC to 15 MHz).

5. 	A frequency counter (up to 110 MHz).

6.	An RF voltmeter (1 MHz....110 MHz, 10 mV....10V high input impedance)

7.	SPIDER test box.

3.7	Test procedure transceiver

For testing the transceiver in the workshop, the SPIDER test box is used. As this test box gives mainly a GO-NO GO indication of the receiver and the transmitter, additional tests are necessary for locating a defective PCB. This is described in para. 3.8. The procedure is given in the form of a flow-chart. Numbers between brackets ( ) refer to the list of test equipment in para. 3.6. During the test the battery of the transceiver is removed and power supply (2) is used, adjusted to 14V and connected to pins B (+) and A (-) of the external supply connector.

For the measurements indicated in the flow-chart the 	following tolerances are permitted:

Power supply voltages : ±   5%

Signal voltages       : ±  20%

Frequencies           : ±   0,01%

For locating the test points see diagrams 12....14.

The following abbreviations are used:

EXC/SMPS         = Exciter-power supply PCB

LF/CTRL          = LF-control PCB

REF.CIRC         = Reference circuit PCB

RX/TX	           = Transmitter-receiver PCB

Front		         = Front panel PCB

Depot		         = Repair in depot maintenance 	  	                               workshop

3.8	Trouble shooting in the workshop

1

2

3

4

5

6

3.9	Replacing printed circuit boards

Before replacing a PCB, always remove the battery from its compartment and switch off the external power supply (if used)!

The PCB's can be easily damaged both by rough handling and by electrostatic discharge (ESD). Always work in a properly equipped workshop (see para. 3.6). Note that most PCB's have components on both sides. For transport, e.g. to the depot maintenance workshop, the PCB's must be carefully packaged, using conductive or anti-static material.

3.9.1	Transmitter - receiver PCB

(see diagrams 10 and 12)

For replacing the transmitter - receiver PCB, act as follows.

1.	Remove the upper cover of the transceiver, by loosening the four captive screws (1).

2.	Remove the coaxial connector X36 from its socket X27 (2) near the front panel.

3.	Remove the six screws M3 (3).

4.	Remove the four screw M2.5 (4).

5.	The PCB can now be pulled vertically from the transceiver frame, using the cords supplied for this purpose.

6.	Lower the replacement PCB carefully into the transceiver frame. Guiding pins are used to insure correct positioning. Make sure that all connector pins are properly engaged before pressing down firmly.

7.	Fasten the screws (3) and (4). Do not forget the washers. Tighten down firmly, but not excessively.

8.	Reconnect coaxial connector X36.

9.	Before mounting the cover, test the correct operation of the transceiver.

3.9.2	LF - control PCB

(see diagrams 11 and 13)

For replacing the LF - control PCB, act as follows.

1.	Remove the lower cover of the transceiver, by loosening the four captive screws (1).

2.	Remove the five screws M3 (2).

3.	Now lift the side of the PCB that is near the rear side of the transceiver and disconnect flex-print connector X37 from print-connector X3.

4.	It is now possible to swing the LF - control PCB outward in the direction of the front panel of the transceiver.

5.	Disconnect the five flex-print connectors and one coax connector from the LF - control PCB and remove the PCB.

6.	Position the replacement PCB near the front of the transceiver and connect the five flex-print connectors and the coax connector.

7.	Swing the PCB back into the frame and connect the flex-print connector X37.

8.	Fasten the five screws (2). Do not forget the washers. Tighten down firmly, but not excessively.

9.	Before mounting the cover, test the correct operation of the transceiver.

3.9.3	Exciter - Power supply PCB

(see diagrams 11 and 14)

For replacing the exciter - power supply PCB, act as follows.

1.	Act as in para. 3.9.2, items 1 to 5 incl.

2.	Remove the covers from the two screened compartments of the exciter - power supply PCB (only one fixed with a screw).

3.	Remove the isolating foil mounted over the reference circuit and remove the reference PCB by loosening the three screws (3).

4.	Remove the remaining four screws (4) fixing the exciter - power supply PCB to the frame.

5.	Remove the four screws (5) fixing the four power FET's to the frame.

6.	Disconnect the flex-print connector X38 (near the battery compartment) from the PCB.

7.	The exciter - power supply PCB can now be lifted from the frame.

8.	Lower the replacement PCB carefully into the transceiver frame. Guiding pins are used to insure correct positioning. Make sure that all connector pins are properly engaged before pressing down firmly.

9.	Fasten the four screws (4) securing the PCB and the four screws (5) securing the power FET's. Do not forget the washers. Tighten down firmly, but not excessively.

10.	Mount the reference PCB on the exciter - power supply PCB. Position the coax cable under the reference PCB so that it does not interfere with the connector. Fasten the three captive screws (3) and place the isolating foil over the reference PCB.

11.	Mount the two covers of the screened compartments (only one secured with a screw).

12.	Reconnect the flex-print connector X38 (near the battery compartment).

13.	Position the LF - control PCB on the front side of the transceiver and reconnect the five flex-prints and the coax connector.

14.	Swing the LF - control PCB back into the frame and reconnect flex-print connector X37.

15.	Fasten the LF - control PCB with its five screws (2).

16.	Before mounting the cover, test the correct operation of the transceiver.

3.9.4	Reference PCB

(see diagrams 11 and 14)

For replacing the reference PCB, act as follows.

1.	Act as in para. 3.9.2, items 1 to 4 incl.

2.	Remove the covers from the two screened compartments of the exciter - power supply PCB. Also remove the isolating foil from the reference PCB.

3.	Loosen the three screws M3 (3) fixing the reference PCB to the frame.

4.	Disconnect the coax cable between the reference PCB and the LF - control PCB.

5.	Lift the reference PCB vertically from the exciter - power supply PCB.

6.	Mount the replacement reference PCB on the exciter - power supply PCB. Position the coax cable under the reference PCB so that it does not interfere with the connector. Make sure that all connector pins are properly engaged before pressing down firmly.

7.	Fasten the three screws (3) fixing the reference PCB to the frame. Tighten firmly but not excessively.

8.	Connect the coax cable from the reference PCB to the LF - control PCB.

9.	Mount the covers of the screened compartments of the exciter - power supply PCB, and the isolating foil over the reference PCB.

10.	Swing the LF - control PCB back into the frame and reconnect flex-print connector X37.

11.	Fasten the five screws fixing the LF - control PCB to the frame.

12.	Before mounting the cover, test the correct operation of the transceiver.

3.9.5	Front panel PCB

(see diagram 15)

For replacing the front panel PCB, act as follows.

1.	Remove both the upper and the lower cover of the transceiver.

2.	On the upper side, disconnect the coax cable from the transmitter - receiver PCB.

3.	On the lower side, remove the LF - control PCB as described in para. 3.9.2 item 2 to 5 incl.

4.	Loosen the two captive screws fixing the front panel to the frame. The front panel can now be removed from the frame.

As the broadest of the flex-prints cannot be 			removed without first removing the exciter - 			power supply PCB, the front panel remains 			attached to the transceiver.

5.	The front panel PCB is held in place by a frame. Remove this frame after loosening the four screws M3 (1) at the corners and the screw (2) holding down the earth wire.

6.	Remove the front panel PCB with one flex-print attached to it.

7.	Mount the replacement PCB. Re-install the frame and fasten the four screws (1). Also fasten the screw (2) for the earth wire.

8.	Thread the two flex-prints and the coax cable through the associated holes in the transceiver frame and fasten the front panel to the transceiver frame, using the two captive screws.

9.	Reconnect the five flex-prints and the coax cable to the LF - control PCB and the coax cable to the transmitter - receiver PCB.

10.	Re-install the LF - control PCB as described in para. 3.9.2 item 7 and 8.

11.	Before mounting the covers, test the correct operation of the transceiver.

3.10	Adjusting the battery holder

(see diagram 16)

The battery holder that can be used with the SPIDER manpack instead of the rechargeable battery block, holds ten type R14 dry cells. The height of these cells is not exactly the same for all manufacturers. To cope with these variations it is possible to adjust the height of the battery holder. There are two positions, with a 2 mm distance between them. The contact springs of the battery holder can cope with small variations in height of the cells.

The battery holder consist of an upper plate and a lower plate, with spacers in between. To change the height loosen the eight screws holding the upper plate to the spacers (the screws in the slotted holes). It is now possible to move the upper plate lengthwise, changing the height of the battery holder at the same time. Afterwards tighten the eight screws again.