CHAPTER 1

GENERAL

1.1	Introduction

This manual provides instructions for field maintenance of the vehicular radio set based on the SPIDER manpack radio transceiver. Only the field maintenance typical for the vehicular version is described, for the field maintenance of the transceiver itself consult the field maintenance manual of the SPIDER manpack.

1.2	Technical data

1.2.1	Power supply unit

Input voltage	:	Vehicular battery,

24V DC nominal,

limits 22...33V DC

Input current	:	200 mA or less when the set is receiving,

approx. 2A when the set is transmitting at maximum power.

Output voltage	:	14V DC nominal,

limits 13.5...14.5V DC

Output current	:	max. 2A

Cut-off current	:	3.5A or less

Heatsink temperature	:	max. 90°C

Ambient temperature	:	-30...+65°C operating

-40...+70°C storage

Dimensions	:	190 x 112 x 90 mm

1.2.2	Antenna

Frequency range	:	30...108 MHz

Impedance	:	50 ohms nominal

Standing wave ratio	:	4:1 max.

Overall length	:	2.243 m (incl. base)

Length lower rod	:	0.952 m

Length upper rod	:	1.092 m

1.2.3	Mounting

Dimensions	:	233 x 190 x 66 mm

1.2.4	Interfaces

The power supply unit has the following connectors:

a.	 3-Pole male connector

b.	10-Pole male connector

a.	3-Pole connector

A = + pole vehicular battery

B = - pole vehicular battery

C = earth connection

b.	10-Pole connector

A = common ground

B = +14V

C, D, E, F, G, H, J and K not used

1.3	Component parts

The following parts are used to convert the SPIDER manpack into a vehicular radio set.

Unit			Signaal code nr.

Vehicular mounting			9556 304 27300

Power supply unit			9556 304 27200

Broadband antenna			9556 004 14000

Consisting of: base

lower rod

upper rod

Power cable (24V)			9556 820 42100/43000

Supply cable (PSU - SPIDER)	9556 820 60100/61000

Antenna cable			9551 690 72000/72800

1.4	Differences among models

No different versions of the SPIDER vehicular radio set as described in this manual exist.