CHAPTER 3

INSTALLATION

3.1	General

This chapter contains instructions for the installation of the vehicular mounting, the power supply unit and the vehicular antenna.

3.2	Mounting the units

3.2.1	Vehicular mounting

The vehicular mounting can be installed in any convenient spot, either horizontally or vertically. The only requirement is that the controls on the front panel of the transceiver can be reached easily, and that the display can be seen. Also allow 100 mm clearance for connecting and removing the power supply cable. The drilling plan is shown in diagram 2. The mounting is fixed with four bolts M4. Use flat washers and spring washers under the nuts to prevent them from shaking loose. If the thickness of the material is sufficient, threaded holes can be used.

3.2.2	Power supply unit

The PSU can be mounted in any convenient spot, provided the on-off switch can be reached. If the situation permits, mount the unit so that the cooling fins of the heatsink are either vertical or up. This improves cooling. Also allow at least 100 mm clearance for mounting and removing the cables. The drilling plan is shown in diagram 2. The PSU is fixed with four bolts M4. Use flat washers and spring washers under the nuts to prevent them from shaking loose. If the thickness of the material is sufficient, threaded holes can be used.

3.2.3	Antenna

Install the vehicular antenna in accordance with diagram 3. A hole of 86 mm diameter must be cut. The base is fixed with the four bolts plus nuts and washers supplied with the antenna. Mount the flexible earth strip supplied with the antenna between the earthing bolt on the antenna base and a nearby metal part of the vehicle. Use toothed washers to insure a good contact.

It is possible to lock the lower antenna rod to the antenna base, using a length of thin iron or copper wire. This is also shown in diagram 3. The wire is threaded through small holes drilled in the hexagonal part of the antenna rod and through the nut on top of the antenna spring. Mount the wire in such a way that it is tightened when the antenna rod tries to work loose.

3.3	Cabling the set

The cables are connected in accordance with diagram 4. The following connections are to be made:

.	Power cable from vehicular battery terminal box to the 3-pole connector of the PSU.

.	Supply cable from the 10-pole connector of the PSU to the 10-pole connector at the side of the transceiver.

.	Antenna cable from the coaxial antenna connector on the front of the transceiver to the vehicular antenna.

.	Handset and/or headset to the 6-pole and/or the 10-pole audio connector on the front of the transceiver.

The power cable is equipped with wire pins for connection to the vehicular terminal box. These pins are marked:

POS :	connection to the positive terminal

NEG :	connection to the negative terminal

GND :	connection to earth terminal

The cable should be protected by a 10A fuse.

The Signaal code numbers of the various cables depends on the required length. Consult the table below.

| Cable | Length | Signaal code no. |
| --- | --- | --- |
| Power cable | 1 m 2 m 3 m 4 m 5 m 6 m 7 m 8 m 9 m 10 m | 9556 820 42100 9556 820 42200 9556 820 42300 9556 820 42400 9556 820 42500 9556 820 42600 9556 820 42700 9556 820 42800 9556 820 42900 9556 820 43000 |
| Supply cable | 0.5 m 1 m 2 m 3 m 4 m 5 m 6 m 7 m 8 m 9 m | 9556 820 60100 9556 820 60200 9556 820 60300 9556 820 60400 9556 820 60500 9556 820 60600 9556 820 60700 9556 820 60800 9556 820 60900 9556 820 61000 |
| Antenna cable | 1 m 1,5 m 2 m 2,5 m 3 m 3,5 m 4 m 4,5 m 5 m | 9551 690 72000 9551 690 72100 9551 690 72200 9551 690 72300 9551 690 72400 9551 690 72500 9551 690 72600 9551 690 72700 9551 690 72800 |