CHAPTER 4

FIELD MAINTENANCE

4.1	General

This chapter provides instructions for testing, trouble shooting and simple repair work on the units used to convert the SPIDER manpack into a vehicular radio set. For field maintenance of the transceiver itself consult the field maintenance manual of the SPIDER manpack.

4.2	Checking the power supply unit

Checking the power supply unit is indicated when the transceiver operates normally on batteries but not at all when using the PSU.

Act as follows:

1.	Try switching the power supply off and then on again. This will restore normal operation after a short-circuit or too high a heatsink temperature. No result: item 2.

2.	Remove the supply cable from the 10-pole connector on the PSU.

3.	Using a voltmeter, check the output voltage of the PSU. Pin B is +, pin A is -. Requirement: 13.5...14.5V DC.

4.	Instead of a voltmeter, the SPIDER testbox can be used. Connect cable 04 to the testbox, the test probe to pin B and the alligator clip to pin A. Set the range switch of the testbox to 3. One or two of the LED's 3 to 6 should light.

5.	If the output voltage of the PSU is normal, but the transceiver operates on batteries only, replace the supply cable.

6.	If there is no PSU output voltage, check the presence of vehicular battery voltage on the connector of the power cable, using a voltmeter or the testbox. Socket A is +, socket B is -. Requirement: 22...33V DC. When using the testbox with cable 04 one or two of the LED's 9...16 should light.

7.	If the vehicular battery voltage is present, replace the PSU. No battery voltage: check the relevant fuse. Fuse OK: replace the power cable.

4.3	Checking the antenna

Before checking the antenna, first check the transceiver as described in the field maintenance manual of the SPIDER manpack, but using the vehicular power supply unit instead of the internal batteries.

The test set-up for the vehicular antenna, and using the SPIDER test box, is as follows:

.	Connect the antenna connector of the transceiver to "TX POWER IN" of the testbox, using cable 01.

.	Connect the 10-pole audio connector of the transceiver to connector "SPIDER" of the testbox, using cable 03.

.	Connect the vehicular antenna to "TX POWER OUT" of the testbox.

For testing, act as follows.

1.	Switch-on the transceiver and tune to 35.000 MHz, high transmitter power (PWR 3).

2.	Turn the range switch of the testbox to "8". One or two of the LED's 5...16 lights (note which one).

3.	Turn the range switch of the testbox to "9". Note which LED lights now. It should be a LED with the same or preferable a lower number as during test 2.

4.	If a LED with a higher number lights during test 3 as compared with test 2, the antenna standing wave ratio is too high. Check if both antenna rods are present and screwed down tight. (To switch off the transmitter while doing this, turn the range switch of the testbox to "7".) Also check the antenna cable.

No result: replace the antenna.

5.	Repeat item 2, 3 and 4 for the following frequencies:

45.000 MHz

55.000 MHz

65.000 MHz

75.000 MHz

85.000 MHz

95.000 MHz

105.000 MHz

The same requirement should be met.

6.	If no deviations are found switch off the testbox and the transceiver and restore the normal cabling.

4.4	Repair

To repair a defect, the field maintenance engineer can replace the following items:

.	Power supply unit.

.	Mounting.

.	Antenna base.

.	Antenna rods.

.	Antenna cable.

.	Power cable.

.	Supply cable.

Also, the following components can be replaced:

Component		Signaal order code

Canvas strap (mounting)	3522 360 41130

Rubber block (mounting)	3522 460 71700

Dust cap     (PSU)	3522 461 36700

No special instructions are required to replace these components. It is not permitted to replace the printed circuit board of the PSU, because of the soldered connections.