CHAPTER 1

GENERAL

1.1	Introduction

This manual contains instructions for the operator of the SPIDER manpack radio transceiver.

The SPIDER manpack described in this manual is a portable VHF transmitter-receiver used to provide two-way voice communication in tactical military radio networks. When using a second transceiver retransmission is possible.

The battery compartment of the transceiver contains alternatively dry cells, or a rechargeable battery block.

A Crypto/Data unit is integrated with the set, providing facilities for high-speed data transfer, encryption of voice or low-speed data,   pre-coded messages and selective calling.

1.2	Technical data

Frequency range	:	30.000 ... 107.975 MHz.

Number of channels	:	3120 in steps of 25 kHz.

Number of preset channels:	9 user programmable channels.

(8 Crypto Channels)

Modulation system(voice)	:	FM (frequency modulation).

Modulation system

(Crypto voice and data)	:	FSK (frequency shift 				keying)

High speed data	:	16 kb/s

Low speed data	:	1200 baud (crypto)

Pre-coded messages	:	99 messages

Selective calling	:	99 addresses

Power supply	:	10 alkaline batteries type R14 (to be replaced) or battery block of NiCad cells (rechargeable).

Battery life (NiCad)	:	approx. 10 hours.

Transmitter power	:	four user selectable levels:

.	low power

.	medium power

.	high power

.	burn-through power

Ambient temperature range

Operating	:	-30 ... +65°C

Storage	:	-40 ... +70°C

Dimensions	:	240 x 175 x 83 mm

Weight	:	approx. 3 kg (transceiver without accessories)

approx. 5 kg

(complete set)

1.3	Component parts

(See diagram 1)

The SPIDER manpack transceiver is composed of the following parts.

1. 	Transceiver

2.	Carrying harness

3.	Antenna matching unit

4.	Antenna

5.	Handset and/or headset

6a.	Rechargeable battery block or

6b.	Battery holder with 10 dry cells type R14

7.	Instruction card.