CHAPTER 2

OPERATOR CONTROL FUNCTIONS

2.1	General

This chapter describes the functions for control of the SPIDER Manpack transceiver, including the keyboard and display functions.

Instructions for basic operation of the transceiver are given in chapter 4.

Procedures for programming of the preset channels are given in chapter 3.

2.2	Volume control/on-off switch

The rotary switch mounted on the front panel provides the following control functions (see diagram 2):

-	Transceiver on/off:	OFF is switched-off, any other position is switched-on.

-	Audio volume	:	W is whisper mode, positions 1...6 control the output level (6 = max. level).

-	Z (= Zeroise)	:	Zeroise position (protected against accidental use by a return spring mechanism), to enable the specific Z key on the keyboard.

2.3	Keyboard functions

(See diagram 2)

The keyboard provides a matrix of 3 x 4 keys for the input of functions, parameters and values.

The separate keys B and Z are provided for specific functions.

Matrix keys

The numeric keys (0 ... 9) have a second function implemented, indicated by a mnemonic text above each key.

For key 1 ... 6 access of the second function is provided by the F key.

For the remaining double function keys the second function will be taken for input unless a numeric input is required for additional parameters or values.

The matrix keys are given in the following table, with their functions briefly explained.

| KEY (numeric/ function) | NUMERIC VALUE | SECOND FUNCTION |
| --- | --- | --- |
| 0/LAMP | Zero | Set brightness level for the display illumination |
| 1/SEL | One | Turn selective call facility on/off for data traffic |
| 2/GEN | Two | Generate parameter/crypto data for transfer with a fill gun device |
| 3/MESS | Three | Turn message transfer facility on/off for data traffic |
| 4/FREQ | Four | Select a frequency value for the present channel |
| 5/LD | Five | Load parameter/crypto data from a fill gun device when connected or from keyboard input |
| 6/MODE | Six | Set operating mode for the selected preset channel |
| 7/CH | Seven | Select a given channel number |
| 8/PWR | Eight | Set the RF power level for the selected channel |
| 9/SQ | Nine | Turn squelch facility on/off for the receiver function |
| F | - | Keyed-in before a function input: enables the given function. Keyed-in during a function input: cancels the input procedure being started. |
| ENT | - | Enter the function input data (being given after the "F" key) into the unit: preset data is stored for the selected channel |

Separate keys

Two keys are placed apart on the front panel because of their specific function:

B-	Burn-through key, to activate an extra high RF power level (indicated by "4") for transmission  during adverse conditions. To be pressed before each transmission.

Z-	Zeroise key, only enabled when the rotary switch is set to Z simultaneously; when activated this key erases all preset channel data and crypto keys stored in the set.

2.4	Display functions

An LCD screen is used on the front panel to indicate the selected functions, parameter settings and values being entered.

| FIELD | MEANING |
| --- | --- |
| PWR 1 2 3 4 | Power level for transmission is set to 1...4 (4 = Burn-through level) as indicated by the associated fields |
| FREQ | Frequency field, to indicate that a frequency value is given in the alphanumeric display |
| MESS | Message transfer facility is turned-on for transmission or parameter entry for this facility is in progress. When flashing a message has been received |
| SEL | Selective call facility is turned-on for transmission or parameter entry for this facility is in progress. When flashing a selective call is being received. |
| LOAD | A load procedure has been initiated or is in progress for parameter/crypto data |
| GEN | A generate procedure has been initiated or is in progress for parameter/crypto data. When flashing no fill gun device is connected. |
| CH | Channel field, placed just before the leftmost character of the display to indicate the selected channel |
| SQ | Squelch facility is turned-on for the receiver function |
| F | "F" is keyed-in preceding a double function key, to enable the (second) function for control or as preset data |
| NOGO | A malfunction has been detected in the radio set (e.g. frequency synthesizer out-of-sync) |
| MODE | Mode field, parameters to be keyed-in for the mode selection are 1...4. Associated fields will indicate the selected traffic type (see next fields) |
| VCE | "Mode 1" is selected, traffic type is voice mode: normal voice with FM transfer |
| CR VCE | "Mode 2" is selected, traffic type is crypto voice mode: encoded voice with encrypted transfer |
| CR DATA | "Mode 3" is selected, traffic type is crypto data mode: low-speed data with encrypted transfer |
| DATA | "Mode 4" is selected, traffic type is data mode: high-speed data with plain text transfer. |

2.5	Connectors

The following connectors are available.

a.	Coaxial antenna connector

-------------------------

Used to connect the whip antenna for portable use, via the antenna matching unit. When used as a vehicular transceiver, the 50 Ohms vehicular antenna is connected here.

b.	6-pins audio connector

----------------------

Used to connect a handset or headset with a 5- or 6-pins plug. A handset with a channel selector switch and a volume control switch can be connected.

If such a handset is used, the preset channels 1...8 can only be selected via the handset switch, not with the keys on the frontpanel. If the handset switch is set to position 0, the keys on the frontpanel can be used to select a channel.

c.	10-pins audio connector

-----------------------

Used to connect a handset or headset with a

10-pins plug. Can also be used to connect a retransmission cable, a cable to a vehicular intercom network, a fill gun or other peripheral equipment.

d.	Supply/peripheral connector (right-hand side)

---------------------------------------------

Used to connect an external power supply for vehicular use, a booster for increased transmitter power, or other peripheral equipment.