Using the SNMP Function

The system administrator can use the SNMP Agent function to be connected with SNMP Server through the CLI command.

1. Command

[1] snmphelp

Print SNMP function list

This command prints the list of short descriptions of the CLI commands that the user uses to operate the system to display the SNMP information.

SMG3200:165.213.81.230> snmphelp snmphelp Print This List (Save in pstnmenu) Showsnmp Show SNMP Manager List Onsnmpagent Turn On the SNMP Agent Offsnmpagent Turn Off the SNMP Agent Setsnmp "ip",port Set SNMP Manager IP&Port Delsnmp ”ip”,port Delete SNMP Manager IP&Port value = 1 = 0x1 SMG3200:165.213.81.230>

[2] Showsnmp

Display current SNMP function setting

This command displays information regarding the current setting state of SNMP Agent.

SMG3200:165.213.81.230> Showsnmp >>> SNMP Agent: On >>> SNMP Manager : 1. 165.213.81.159 (162) value = 0 = 0x0 SMG3200:165.213.81.230>

[3] Onsnmpagent

Turn On the SNMP Agent function

If the system administrator sets this command, system is enabled to SNMP Agent function.

SMG3200:165.213.81.230> Onsnmpagent SNMP Agent Turn On. value = 0 = 0x0 SMG3200:165.213.81.230>

[4] Offsnmpagent

Turn Off the SNMP Agent function

If the system administrator sets this command, system is disabled to SNMP Agent function.

SMG3200:165.213.81.230> Offsnmpagent SNMP Agent Turn Off. value = 0 = 0x0 SMG3200:165.213.81.230>

[5] Setsnmp

Set SNMP Agent

The system administrator can set SNMP Agent through this command.

-> Set the IP address and port of the SNMP Server

SMG3200:165.213.81.230> Setsnmp "165.213.81.159",162 value = 0 = 0x0 SMG3200:165.213.81.230>

[6] Delsnmp

Delete SNMP Agent

The system administrator can set SNMP Agent through this command.

-> Delete the IP address and port of the SNMP Server

SMG3200:165.213.81.230> Delsnmp "165.213.81.159",162 value = 0 = 0x0 SMG3200:165.213.81.230>

2. Trap Message Format

Trap msg Format

| Generic Trap code | Specific Trap code | Detailed Information String of Trap Message |
| --- | --- | --- |

| Generic Trap code | Specific Trap code | Detailed Information String |
| --- | --- | --- |
| 0 | 0 | Coldstart of a System |
| 1 | 0 | Warmstart of a System |
| 2 | 0 | Network Link Down |
| 3 | 0 | Network Link Up |
| 4 | 0 | Failure of Authentication |
| 5 | 0 | Exterior Gateway Protocol Neighbor Loss |
| 6 | 0 | Detailed Information of System Trap |
|  | 1 | Detailed Information of Digital Trunk Trap |
|  | 2 | Detailed Information of VoIP Trap |
|  | 3 | Detailed Information of PSTN Trap |

1. Generic Trap code

: Basic SNMP trap information defined in MIB-II

-. Coldstart of a System (code : 0)

:  Trap information occurred at power on

-. Warmstart of a System (code : 1)

:  Trap information occurred when system is initialized at power on

-. Network Link Down (code : 2)

:  Trap information occurred at network link down

-. Network Link Up (code : 3)

:  Trap information occurred at network link discovery

-. Failure of Authentication (code : 4)

:  Trap information occurred at authentication fail

-. Exterior Gateway Protocol(EGP) Neighbor Loss (code : 5)

:  Trap information occurred when problem happened among the EGP neighbor

-. Enterprise Specific (code : 6)

:  Trap information related to system functional details

This is classified into following items

; System function, Digital Trunk function, VoIP function, PSTN function

2. Specific Trap code

: This trap code is to classify Generic Trap information in details.

-. System Trap message (code : 0)

-. Digital Trunk Trap message (code : 1)

-. VoIP Trap message (code : 2)

-. PSTN Trap message (code : 3)

3. Detailed Information String

: This string is for concrete SNMP trap information

3. Trap Information

 System Trap Information

| Generic Trap code | Specific Trap code | Detailed Information String |
| --- | --- | --- |
| 4 | 0 | Failure of Authentication(Console) |
| 6 | 0 | Console Login(Login ID) |
|  |  | Console Log-Out(Login ID) |
|  |  | Telnet Login (IP Address, Time, etc) |
|  |  | Telnet Log-Out (IP Address, Time, etc) |
|  |  | Change DHCP Client On/Off |
|  |  | Change Network Parameter |
|  |  | Change Login ID and Password |
|  |  | System upgrade(TFTP, FTP) |
|  |  | Reboot System by user |

 D-Trunk Trap Information

| Generic Trap code | Specific Trap code | Detailed Information String |
| --- | --- | --- |
| 6 | 1 | Digital Trunk Link Down |
|  |  | Digital Trunk Link Up |
|  |  | Digit Trunk Alarm Off |
|  |  | Digit Trunk Alarm On |

 VoIP Trap Information

| Generic Trap code | Specific Trap code | Detailed Information String |
| --- | --- | --- |
| Trap message related with call release |  |  |
| 6 | 2 | Unassigned Number |
|  |  | No Route To Dest |
|  |  | User Busy |
|  |  | Call Rejected |
|  |  | Invalid Format |
|  |  | Facility Rejected |
|  |  | No Circuit |
|  |  | Network Out Of Order |
|  |  | Equipment Congestion |
|  |  | Chan Not Available |
|  |  | Resource Unavailable |
| Trap message related with RAS event |  |  |
| 6 | 2 | GK RAS Admission Reject |
|  |  | GK RAS Registration Reject |
|  |  | GK Down |

<reference-Information string Format related with call release>

| Release Reason | Source IP(src call number) | Destination IP(dest call number) |
| --- | --- | --- |

Ex)  Invalid Format Src:165.213.81.230(2001) Dest:165.213.87.231(4237)