CDPC235. Subject 2 BD BOARDS USED, ONLY 1 PART # LISTED IN MANUAL.

SYMPTOMS

Only one (1) board listed. This board comes with 8-752-360-75 IC used with KSS-390 optical pickup. No board listed that comes with 8-752-351-94 IC used with the KSS-240A optical pickup....Please advise if we have to change the IC or if another part #  is available.

RESOLUTION

4/29/94; As per KCP the part number for the board using the KSS-240A pickup is A4649433A, this number is registered with KCP.

| A4649610A | 157 | BD BOARD |
| --- | --- | --- |

CDPC235. Subject AFTER PLAYING A FEW SELECTIONS STARTS SKIPPING - REPL OPTICS.

SYMPTOMS

After playing a few selections, the unit starts skipping.

RESOLUTION

Replaced optical device.

| 884828111 | 151 | KSS390A |
| --- | --- | --- |

CDPC235. Subject AFTER WARM UP, DISPLAY BLINKS AND STOP PLAYING.

SYMPTOMS

After unit warmed up, display blinks and stop playing during track change and look for another disc. After intensive trouble shooting, we found a defective diode in positive 9.5V supply. If 9.5V supply drops to below 6.8V, unit will reset. We think,  diode is not big enough to handle heavy loads.

RESOLUTION

We recieved Suto's information that samples are on the way. We will wait for the samples and look into this situation at that time. Update 9/21/94: The Samples have not yet arrived, since this is the only report of this type of problem regarding the  CPDC235 we will close this issue, We will however, watch for further reports.

| 871920082 | D201 | B21/DIODE 11ES2 |
| --- | --- | --- |

CDPC235. Subject BU UNIT HITS DISC TABLE WHEN CHUCKING.

SYMPTOMS

On 2 different units the laser hits the disc table when chucking. It appears disc table, ref# 59, is stopping too far clockwise. The pin on ref# 123 is against the left side of the slot in ref# 59. Is there any adjustment? What makes the table stop in  this particular place?

RESOLUTION

(Feb 5) Replacing R458 with a 3K, 1/6 W (P/N 124784200) and R457 with a 5.6k, 1/6 W (P/N 12784900) repaired a unit sent to Tech. department. R459 is a 2.1k and was already installed on the unit worked on at Tech Dept. Positive feedback from Boston and  Atlanta. -UPDATE: From Tokyo: 4/11, Replace IC401 with p/n 875284942, verify R457 - 2.4 k ohm, R458 - 750 ohm, R459 - 1.2k ohm resistor p/n's listed in S/M.

CDPC235. Subject DEAD UNIT - POWER TRANFORMER OPENED.

SYMPTOMS

Checked and found open primary (thermal fuse) in power transformer. Noticed that Q504 and Q502 had overdissipated. Checked both transistors and found shorted collector/emitters on both. Replaced Q502, Q503 and Q504. Then found that IC501 pins 13 and  19 were at -1vdc and +1vdc.

RESOLUTION

Replaced power transformer Q502, Q503, Q504 and IC501. Normal operation restored.

| 142355311 | T901 | POWER TRANSFORMER |
| --- | --- | --- |

CDPC235. Subject DISC TRAY, ROTATES CONTINUOUSLY (REPLACE D801).

SYMPTOMS

At power on disc tray would spin and not stop. Other functions were normal (open/close).

RESOLUTION

By checking D801 connector @ main board, voltages were correct but, pin 5 (T. Sens) data looked bad. Replaced D801, solved problem.

The purpose of D801 is to sense the location of the disc tray by detecting the slots in the tray and outputting pulses to the micro IC401 (pin 6).

| 874992418 | D801 | J8/IC RPI 1391 |
| --- | --- | --- |

CDPC235, CDPC335. Subject INTERMITTANT AUDIO - REPLACED BD TO MAIN BD FLEX CABLE.

SYMPTOMS

Right or left channel intermittently cutting out.

RESOLUTION

Replaced flex cable from BD board to main board.

| 164840911 | 122 | FLEX CABLE |
| --- | --- | --- |

CDPC235, CDPC335. Subject JAMS DISC TABLE, WON'T PLAY DISCS - REPLACE GEAR 61 AND 66.

SYMPTOMS

Unit will not play discs. BU assembly won't come up, due to interference with disc table. Occasionally, the CLV motor pulley is damaged.

RESOLUTION

To repair this problem as per Tokyo: Replace IC 401 with P/N 8-752-849-42, also verify values of R457 (2.4 ohm) R458 (750 ohm) and R459 (1.2k ohm). The P/N's of the resistors are listed in the S/M. A Service Bulletin is in the process of being issued to  completely list part numbers and serial number ranges.

| X49433791 | 61 | GEAR, ROTARY A, ASSY |
| --- | --- | --- |

CDPC235, CDPC335. Subject MECHANICAL ALIGNMENT PROCEDURE AVAILABLE, SEE S/M SUPP. #2.

SYMPTOMS

The unit had no mechanical movement. I found the above part out of item number 119 slot (down position on both units).

RESOLUTION

I reinstalled item number 123 into the slot on number 119 and the unit restored operation. Is there any written alignment procedure for this new mechanism?

We have asked Tokyo to provide the alignment procedure. -UPDATE: Please refer to service manual supplement #1 for the mechanical alignment procedure.

| 495728901 | 123 | BU-5BD (BASE UNIT) |
| --- | --- | --- |

CDPC235. Subject NO AUDIO - REPLACED OPTICAL DEVICE AND CABLES 122 AND 153.

SYMPTOMS

Unit had no audio traced problem to defective cables, than unit was skipping. Found defective optics.

RESOLUTION

Replaced cables 122 and 153 and optical device.

| 884828111 | 151 | KSS390A |
| --- | --- | --- |

CDPC235. Subject STATIC IN AUDIO OUT - REPLACED IC501.

SYMPTOMS

Static in audio, traced signal to IC501 sound was good going into the IC but bad going out.

RESOLUTION

Replace IC501.

| 875917588 | IC501 | D19/LINE AMP/MUTING ATT |
| --- | --- | --- |

CDPC235. Subject TABLE KEEPS ROTATING WITH LESS THAN 3 DISCS.

SYMPTOMS

With less than 3 discs in unit the table will continue to rotate unless you put some pressure on the turntable to slow it down. With 3 discs it will detect the discs and play them. With 1-2 discs it will not detect the discs and play unless you put  slight pressure on the side of the turntable.

RESOLUTION

Try replacing the resistors as outlined in FPR P0144, this mod will slightly slow the rotation of the disc table. As per our conversation, this resistor mod solved the problem.

CDPC235. Subject WEAK RIGHT CHANNEL - REPLACED LINE AMP IC501.

SYMPTOMS

Right channel is weak.

RESOLUTION

Replaced line amp IC; normal operation restored.

The line amp supplies L and R audio signals to the line out jacks and the headphone amp (IC551) the same symptom, low right audio, will be noticable through both outputs.

| 875906166 | IC501 | D19/LINE AMP |
| --- | --- | --- |

CDPC235, CDPC435, CDPC535. Subject TRAY MOTOR SPINNING BACKWARDS - REPLACE TABLE MOTOR.

SYMPTOMS

Ran into this problem 5 times in past 3 weeks. Removed tray motor. Old motor is installed with lable on motor facing up. If new motor is installed with label facing up, motor will spin backwards and tray will not function properly.

RESOLUTION

When replacing this motor, I installed it with label facing down so it spins the correct way. Replaced motor because of tray hitting the BU mech. Tried resistor mod but it did not work. If the mod doesn't work, tray motor will usually fix the problem.

| A4660322A | M801 | DISC TABLE MOTOR |
| --- | --- | --- |

CDPC235. Subject WON'T READ COMPLETE DISC - RESOLDERED IC101.

SYMPTOMS

Unit would not read a full disc.

RESOLUTION

Resoldered IC101 of CD board.

| 875235194 | IC101 | J13/CXD2515Q |
| --- | --- | --- |

CDPC235, CDPC245, CDPC335, CDPC435, CDPC445, CDPC545, CDPCA7ES. Subject TRAY MOTOR NOT LABELED OR WIRED CORRECTLY.

SYMPTOMS

After replacing the tray motor with a new one the player would not work correctly, the tray would rotate back and forth and not recognize the cd in the unit.

RESOLUTION

Checking the old motor found, it mounted in the unit with the lable facing up and the positive terminal on the left, this turns counterclockwise. I installed the new motor the same way and it did not work. The new motor installed with label down  worked correctly.

I checked the samples sent to the Tech dept. You're correct the new motor does spin in the opposite direction of the original motor. I have notified KC parts to check stock and take corrective action.

| A4660322A | M801 | TRAY TABLE MOTOR |
| --- | --- | --- |

CDPC235, CDPC265, CDPC335, CDPC365, CDPC445, CDPC545, CDPC745, CDPCA7ES, CDPCA8ES, CDPCA9ES. Subject WHISTLING NOISE - ADD RTV BTWN SPINDLE MOTOR & BD BOARD.

SYMPTOMS

A whistling noise (about 400HZ) manifests itself on certain cuts of disc (Goodbye Yellow Brick Road, Elton John, Polydor D103076). If you touch the optical block lightly, the noise disappears.

RESOLUTION

Remove the spindle motor from the BD board. Add a layer of silicone sealer (RTV) between the spindle motor and BD Board. Re-install the spindle motor, take care not to allow the base of the spindle motor to contact, or directly lay on the BD board  during reassembly.

Adding the RTV silicone compound between the motor and BD board isolates the two parts. There is no vibration transfer from the motor and spinning disc, therefore, the whistling noise stops.

| 732206519 |  | RTV SILICONE COMPOUND |
| --- | --- | --- |

CDPC435. Subject ALTERNATIVE METHOD TO REPAIRING CHASSIS VS REPLACING.

SYMPTOMS

Found plastic rod coming out of chassis, as shown on pg 35 of SM, can be repaired. Purpose of rod is to push on the rack release and release rack. If plastic rod breaks, disc exchange doesn't work properly. Method used is to drill a hole where the  rod was all the way through, then insert a screw from under unit so threads are showing in place where rod was. Screw to proper height and lock screw head. Now screw is doing job of plastic post. Part for screw 768566014.

RESOLUTION

We continue to investigate this issue

| 495728501 | 111 | LEVER, SET |
| --- | --- | --- |

CDPC435. Subject BU ASSY CONTACT TRAY SB HFP0389 UPDATE - REPL TABLE MOTOR.

SYMPTOMS

Had problem as stated in HFP0359 and put correct value resistors and replaced microprocessor with no re

RESOLUTION

of the problem. -SOLUTION: Replaced table motor, problem resolved.

| A4660322A | M801 | TABLE MOTOR |
| --- | --- | --- |

CDPC435. Subject DRAWER WILL NOT OPEN - RESOLDERED MOTOR TERMINAL.

SYMPTOMS

Drawer will not open motor does not run for open/close

RESOLUTION

Resoldered motor, one terminal was poorly soldered from the factory (intermittent contact). Resoldering restored normal operation.

| A4608834A | M802 | MOTOR ASSY LOADING |
| --- | --- | --- |

CDPC435. Subject INTERMITTENTLY READING DISC - FLEX CABLE INTERMITTANT.

SYMPTOMS

The unit would start to play fine than audio would distort and spindle goes high speed.

RESOLUTION

Replaced flex cable from main PCB to BD board unit works.

Although the flex connector may be perceived as a weak point there has been little parts movement of this piece in reference to the CDPC435.

| 175112311 | 122 | FLEX CABLE |
| --- | --- | --- |

CDPC435. Subject MECH NOISE WHEN OPEN, NO PLAY.

SYMPTOMS

Unit has mech noise when open drawer or no play. Inspection under disc table found chassis part holding lever set broken, lever set loose inside unit.

RESOLUTION

The solution is to replaced chassis, part not supplied.

We will notify KCP about stocking this chassis.

| 495728501 | 111 | LEVER SET |
| --- | --- | --- |

CDPC435. Subject SERVICE MANUAL CORRECTION - U/D GEAR P/N CORRECTION.

SYMPTOMS

The Service Manual for CDP-C435/C535 is incorrectly printed. The part number for U/D Gear (Ref# 119) shown as 4-957-286-04.

RESOLUTION

Correct Part number is 4-957-286-01.

| 495728601 | 119 | GEAR (U/D) |
| --- | --- | --- |

CDPC435. Subject UNIT HAD POWER BUT WAS INOPERATIVE - CN801 MAKNG BAD CONTACT.

SYMPTOMS

Unit had power but was inoperative.

RESOLUTION

Connector CN801 from sensor board was not making a good connection on table motor board.

CDPC435. Subject WIRING HARNESS TO ROTARY ENCODER NOT SEATED PROPERLY.

SYMPTOMS

Player exhibited symptoms similar to either flex cable or control problems. Door opening and closing by itself, B.U. assembly would go up or down or do nothing. It did not know where it was.

RESOLUTION

Disassembled unit to look at rotary encoder. Found S3 wire going to rotary encoder from main board not fully seated into socket of encoder. Pressed wire firmly into socket. Unit restored to normal operation.

CDPC535. Subject NO FUNCTION AFTER WARMUP, DISPLAY FLASHES OFF - REPL D601.

SYMPTOMS

Unit plays ok, then after warmup, display flashes off and cycles. System control is getting reset from supply as it goes down then recovers. It starts this cycling when play is pressed and disc spins up.

RESOLUTION

Traced power supply weakness to intermittent open diode of bridge. (60hz ripple on pos supply side rather than 120hz) shunted with known good diode, worked fine.

| 871920082 | D601 | C21/DIODE 1IES2 |
| --- | --- | --- |