1. Sony KVM1401U BE 2
Sony KVM1401U BE 2 DEAD VOLTS AROUND STR WRONG CHECK PS801 0 6amps IF OPEN CHECK D611 R2M Sony 8 719 303 49 STR54041 Sony 8 749 901 65 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2223.htm

2. Sony KVA213 KVA253 KVA293 KVA254 KVA294 KVS292
Sony KVA213 KVA253 KVA293 KVA254 KVA294 KVS292 TEXT PIC GEOMETRY DISTORTED CHANGE IC002 TO TMS27PC010A15FMAE217 Sony 8 759 281 63 REMOVE R2015 2016 D2002 C2009 REPLACE R2028 WITH
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2205.htm

3. Sony SLV474
Sony SLV474 When replacing heads Pan NVG33 etc cheaper than Sony ones 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr850.htm

4. Sony SLVE700UX
Sony SLVE700UX Dist sound e e L2 posn IC102 sm scart sw chip BA7632AF E2 7 from Sony Pt no 8759 44569 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr873.htm

5. Sony BE2A
Sony BE2A Dark pic with green flaring burning smell R812 1k 1w safety feed res for A1s burnt D806 leaky used BY210 800 as replacement 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1634.htm

6. Sony KV2020
Sony KV2020 Line osc drifts with time Check C507 33n C509 10 16v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1654.htm

7. Sony BE3D
Sony BE3D E W bowing excess width R870 1R 1W 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1644.htm

8. Sony AE1C
Sony AE1C Pic int col and blanks out int Dry joints on vid process vert pcb earth lands of screening can 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1604.htm

9. Sony KV2553
Sony KV2553 East West reduced width TEA2031 replacement failed Change C830 and C831 1uF caps 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1821.htm

10. Sony BE1
Sony BE1 Int goes to lines loss of sound Dry joints T01 T02 in IF can 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1624.htm

11. Sony AE2A/B
Sony AE2A B Pic off int with LED flashing 13 times Tbase fault dry joints fop chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1614.htm

12. Sony BE2A
Sony BE2A Colour intermit CT332 trimmer dodgey clean or replace pt no1 141 418 11 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1633.htm

13. Sony BE3
Sony BE3 Dead no led flashes eht briefly flashes LOPTx HR7337 fits 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1643.htm

14. Sony AE2
Sony AE2 Won t come out of st by int Field o p chip TDA8179 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1613.htm

15. Sony BE1
Sony BE1 Field collapse IC501 PC1488H faulty supply from LOPTx high dry joint on loptx esp pin 11 chassis 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1623.htm

16. Sony KV19XMTU
Sony KV19XMTU Tuning drift wont tune in new chans dry joints if unit 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1653.htm

17. Sony XE4
Sony XE4 Field non lin C527 470 low cap 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1674.htm

18. Sony KVE2922E
Sony KVE2922E Dead chopper tr s c Replace chopper 2SD1548 TEA2260 C611 47 which was cause 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1664.htm

19. Sony KV29F3U
Sony KV29F3U To enter service mode press following buttons on remote slowly in sequence osd button 5 vol TV and it will switch on in test mode 1 press menu on the remote next
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1663.htm

20. Sony XE4
Sony XE4 Dead fuse blown Mains sw internal arcing Q602 2SD1497 s c R622 R623 680R o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1673.htm

21. Sony XE4
Sony XE4 Snowy pic low tuner gain SWAF faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1676.htm

22. Sony BE2A
Sony BE2A Int flashing across pic dry joints on loptx earth pin chopper heatsink C802 heatsink for loptr 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1638.htm

23. Sony BE1
Sony BE1 Replacing STR54041 Use Sankey version has SK in small circle at bottom 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1628.htm

24. Sony KV21T1U
Sony KV21T1U No Txt excess width height poor contrast Reset eeprom press on the remote on screen display key No 5 vol up key tv set will come on with TT on screen press menu
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1658.htm

25. Sony BE4A
Sony BE4A Dead CP602 on sec psu o c Replacement cures fault only temp CP602 to N75 2 5A Add 5v6 zener between pins 1 9 IC803 add 5v6 zener between pins 9 chassis on print side of
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1648.htm

26. Sony BC4
Sony BC4 Dead st by LED flashes 2 times Excess I R624 180k hi res o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1618.htm

27. Sony KV1602
Sony KV1602 Horizontal green lines in picture when hot Remove capacitor C816 33pF 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1815.htm

28. Sony AE1
Sony AE1 Dead comes on in st by if sw on from skt Check for 0v at pins 2 3 of IC601 if pin 3 7v sus LOPTx Earth pin 3 and check for smoke 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1594.htm

29. Sony AE2
Sony AE2 No col Replaced TDA9145 dec chip IC301 TDA4660 chroma del line Xtals X301 2 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1608.htm

30. Sony KV-C25/27
Sony KV C25 27 Memory inoperative Memory chip replacement failed Check that pin 2 of memory chip is at 30 volts If not check D809 R830 L807 and the 30V smoothing cap In two cases
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1825.htm

31. Sony QE1
Sony QE1 No sound or vis snowy screen no front panel control except tv AV SW407 on front panel leaky tying up micro 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1668.htm

32. Sony BE4
Sony BE4 No txt or OSD R081 o c C810 also failed 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1646.htm

33. Sony BE2A
Sony BE2A E W bowing C806 47n 250v split and faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1636.htm

34. Sony KVX2572UB
Sony KVX2572UB LED flashes 13 times at sw on Indicates field fault s c field o p chip on 15v supply 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1666.htm

35. Sony KV2184
Sony KV2184 Chans won t lock when tuning garbled OSD LA78LR05 5 legged 5v reg with reset built in faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1656.htm

36. Sony BE1
Sony BE1 Iont sound vis goes black with lines Dry joints in VIF can 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1626.htm

37. Sony KV2720
Sony KV2720 Chopper transistor Sced after running for 10 15 mins Replace all electrolytics around uPC1394 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1823.htm

38. Sony YE2
Sony YE2 no pic sound ok HT low at 80v 135v C652 33 250v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1678.htm

39. Sony AE2B
Sony AE2B Pic blanks out int reverts to st by Dry joints Field o p chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1616.htm

40. Sony AE2
Sony AE2 Front led flashing 13 times frame problems Replaced the frame IC TDA8179 with a STV9379 O c 0 47ohm res pin 7 FBT neg 15 volt supply to the output chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1606.htm

41. Sony AE1C
Sony AE1C Int micro lockup child lock sigs Dry joints connector of EEPROM daughter board 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1603.htm

42. Sony BE1
Sony BE1 Various tuning faults dry joints in if strip C012 0 01 s c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1629.htm

43. Sony BE2A
Sony BE2A Int Red Not Q706 Q709 but vid proc chip IC302 TDA3505 V1 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1639.htm

44. Sony AE2
Sony AE2 No sound pic 1 led only lit dry joints fop chip also check other areas for drys 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1609.htm

45. Sony BC4
Sony BC4 Line foldover with LOPTr overheating R624 180k hi res o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1619.htm

46. Sony BE4A
Sony BE4A Dead st by LED flashes 4 times No lop No line drive from IC301 MC44007P chip u s 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1649.htm

47. Sony AE1
Sony AE1 Dead R1603 2R7 7w o c Chopper tr 2SD1548 s c R614 220R stand up res dry joints 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1593.htm

48. Sony [V] KVM2100D
Sony V KVM2100D EAST WEST Won t adjust properly Excessive width Change capacitor 47nF 400v parallel to E W diode 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1814.htm

49. Sony KV21V6U
Sony KV21V6U Error message asking to be reset all the time EEPROM chip ST24C16FM6TR IC002 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1659.htm

50. Sony QE3
Sony QE3 Dead IC601 1394C R632 2M2 Q202 2SD788 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1669.htm

51. Sony KV-C25/27
Sony KV C25 27 Starts on Pr0 and every time Pr is depressed it goes subsequently to Pr8 Pr16 Pr24 and to Pr0 Change D008 5Vzener located near M58655 memory chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1826.htm

52. Sony KV21V6U
Sony KV21V6U No pic screen blank OSD ok IC401 CXA1855Q vi switching chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1660.htm

53. Sony QE3
Sony QE3 Int start up C601 2 2 R608 8M2 R609 2M2 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1670.htm

54. Sony BE2A
Sony BE2A No pic field collapse sound ok R819 0R47 1 4w safety re o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1640.htm

55. Sony KV1400
Sony KV1400 Dead F601 3 15A o c R601 3R9 10w o c Q605 Chopper s c R627 0R47 1w o c C518 330pf 1kv leaky was the cause 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1650.htm

56. Sony AE1B
Sony AE1B Dead loptr s c dry joints inc at TDA2028B tbase chip IC501 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1599.htm

57. Sony AE1
Sony AE1 Dead st by light comes on briefly Running on dummy load not recommended LOPTx u s 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1595.htm

58. Sony KV2032ME
Sony KV2032ME No horizontal synchronisation when cold Replace capacitor C509 10uF near horizontal driver transformer 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1816.htm

59. Sony AE1
Sony AE1 Pic blanked out sound ok Check 18v supply read 38v Q608 2SD1761 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1597.htm

60. Sony KV2184-34
Sony KV2184 34 Won t start pin 41 stby of microcontroller is high Replace Q003 No leak or any other defect was detected with a VOM 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1818.htm

61. Sony BE1
Sony BE1 Int loss of sync No vid from txt pcb to sync sep pin 5 IC551 12v supply on txt pcb only 5v due to Q02 o p sm 2SC2873 Y faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1625.htm

62. Sony BE2A
Sony BE2A Dead no start up volts R602 270k under chopper tfmr 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1635.htm

63. Sony AE1C
Sony AE1C Sound goes tofull vol when adjusted EEPROM IC005 SDA2546 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1605.htm

64. Sony AE2B
Sony AE2B Corrupted EEPROM and no teletext set up in service menu Press and hold any two buttons at the front of the set and switch it on from the mains The letters TT appear on
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1615.htm

65. Sony BE4
Sony BE4 Dead LED blinks 6 times Field fault TDA9302H FOP chip dry jointed and u s 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1645.htm

66. Sony XE4
Sony XE4 Int dead Dry joints R622 R623 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1675.htm

67. Sony KV2184-34
Sony KV2184 34 Won t start loud audible buzz Sometimes starts and switches off after a while If stby pin 41 of Mp desoldered all ok Replace Str50115 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1820.htm

68. Sony KV2032ME
Sony KV2032ME Pic only 3 wide on lh screen increases as it warms up C514 33uF 160V C801 22uF 250V C557 2 2uF 160V 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1655.htm

69. Sony KVM531
Sony KVM531 Int col 8 8Mhz xtal 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1665.htm

70. Sony KV2704
Sony KV2704 Sometimes it won t start Check replace R606 470 kOhms resistor 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1822.htm

71. Sony BC4
Sony BC4 Dead Check st by psu o p TOP223Y BB chopper chip IC610 R602 0R1 1 2w D603 S1Wb60 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1617.htm

72. Sony BE1
Sony BE1 No sync sometimes intermittent repair print to collector of Q02 on txt pcb 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1627.htm

73. Sony KV2764
Sony KV2764 Vertical IC TDA3652 defective No longer available Foldover at the bottom reduced width Fit a TDA3654 and change R518 to1 5K 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1824.htm

74. Sony AE2
Sony AE2 Int pic blanking leaving OSD dark green raster Adj auto green cut off adj grey scale using electronic screwdriver settings 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1607.htm

75. Sony BE2A
Sony BE2A Int col CT 332 reqs adj To adj s c pin 11 IC371 to chassis then adj CT332 for stationary pic or replace CT332 with 15pf cap 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1637.htm

76. Sony QE1
Sony QE1 Burn up on crt panel R756 R758 47k 1 2w burnt Q753 J101Q blown apart used BC557B in lieu 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1667.htm

77. Sony YE2
Sony YE2 Dead no o p from psu R603 390k 1 2w o c dry joints D801 2 8 9 on line pcb 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1677.htm

78. Sony BE4
Sony BE4 Pic displaced down by 2cm Volts on Q500 501 way out Jungle chip MC44007P faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1647.htm

79. Sony KV2184
Sony KV2184 Dead STR50115 overheating Replaced chip s c pins 2 3 but still dead D608 R2M diode s c PS801 800mA cct prot o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1657.htm

80. Sony KV2032
Sony KV2032 Won t start dead Check replace R627 2 2M resistor in the power supply 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1817.htm

81. Sony SLV625U
Sony SLV625U Failure to eject Mode sw or BA6238A load mot dr chip 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr854.htm

82. Sony SLVE230
Sony SLVE230 Dead with psu pulsing after coins inserted PS600 2A fusible link 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr864.htm

83. Sony SLVE710
Sony SLVE710 Carriage out of alignment If 1st 5 digits of carriage ser no between 60203 60531 change carriage for improved typeA 6751 496 H See Nov99 TV 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr875.htm

84. Sony KV21V6U
Sony KV21V6U Recordings snow only Dry joints tuner TU101 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr844.htm

85. Sony SLVE700UX
Sony SLVE700UX Tape chewing gross tracking error TU guide buckling held in place underneath by bronze spring unit Pt no 3960 68801 TU 3960 68701 supply 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr874.htm

86. Sony SLV425
Sony SLV425 Dead 270k start up res sm o c TDA4605 u s and several electrolytics in psu 220 470 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr849.htm

87. Sony SLV825
Sony SLV825 St by only no display Back up cap 0F22 on display pcb leaked corroded pcb 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr859.htm

88. Sony SLV625
Sony SLV625 No Rec E E sound Dry joints conn CN0306 on Nicam module 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr853.htm

89. Sony SLVE225
Sony SLVE225 Squealing from drum Static disharge brush under drum lub No need to remove deck 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr863.htm

90. Sony SLVE230
Sony SLVE230 Int jams on eject Cracked sleeve on load motor worm pt no X3947 577 1 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr865.htm

91. SONY KV21X1
SONY KV21X1 DEAD REPLACE IC600 D603 R601 R605 Q601 SONY874992499 871910997 120296211 121636411 AND 872902504 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec380.htm

92. Sony SLV373
Sony SLV373 No drum rotation int at first just kicks Replace C013 14 15 0 1yellow caps on mech pcb 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr846.htm

93. SONY SLVE220
SONY SLVE220 DEAD CHECK CIRCUIT PROTECTOR J5131 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec382.htm

94. SONY KVD2512
SONY KVD2512 SOUND CUTS OUT OR IS DISTORTED REPLACE C615 1000MFD25V 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec381.htm

95. Sony SLV715UB
Sony SLV715UB Loads tape threads head spins then stops unthreads Dry loints on lower drum pcb 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr856.htm

96. Sony SLV270
Sony SLV270 Dead C1325 1 100v C1326 47 25v in psu pri 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr845.htm

97. Sony SLV715U
Sony SLV715U dead no o ps from psu replace all electros in psu secondary 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr855.htm

98. Sony SLVE25V
Sony SLVE25V Cuts out aft few secs in play Drum too slow motor faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr866.htm

99. Sony SLVE720
Sony SLVE720 Cass Mechs jammed Driving gear from main deck item 262 damaged teeth Fit replacement gear set which includes gear 262 mating master cam 261 which MUST also be
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr876.htm

100. Sony SLVE220
Sony SLVE220 Cuts out in rec pback ok PR512 1A fuse increased resistance on psu towards front can be reached from underneath 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr860.htm

101. Sony SLVE520
Sony SLVE520 No rec m c sw off PR511 fusible unable to cope with current 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr871.htm

102. Sony SLVE8
Sony SLVE8 Capst motor graunching at end of tape Capst belt tension wrong should use torque screwdriver to set up 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr880.htm

103. Sony SLVE520
Sony SLVE520 Droning noise on playback audio Fusible link PR512 in psu 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr870.htm

104. Sony SLVE220
Sony SLVE220 Cuts out in record but plays Ok PR512 1000ma fuse had increased in value On PSU towards front 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr861.htm

105. Sony SLVE220
Sony SLVE220 Int stops switches off in Rec mode only PR512 1R res in psu risen in value 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr862.htm

106. Sony SLV625
Sony SLV625 Knocking noise in playback Timing belt stretched 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr852.htm

107. Sony SLV615
Sony SLV615 Damages tapes on eject Half load arm sticking clean lub 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr851.htm

108. Sony SLVE520UK
Sony SLVE520UK No record sw off with no erase PR512 CP in 5v rail increases from 0R3 to 0R6 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr872.htm

109. Sony SLVE720
Sony SLVE720 No PLAY or STOP with bottom cover removed This is normal if earthing strap not fitted FFwd Rew OK 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr877.htm

110. Sony SLV415
Sony SLV415 Poor sync replace modulator 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr848.htm

111. Sony SLV815B
Sony SLV815B Interference on pback pic Elna caps in psu C207 820 10v leaked 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr858.htm

112. Sony SLVE40
Sony SLVE40 Won t open cassette front flap when tape inserted Post broken same as used in Aiwa HVF11 series see explopded view in CHS cat 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr869.htm

113. Sony SLVE720UX
Sony SLVE720UX Int loss e e sound pic not always together Dry joints earths on edges of IF module 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr879.htm

114. Sony SLVE280 etc
Sony SLVE280 etc Int reverts to st by in Rec PR512 cct prot gone hi res can also cause excess audio buzz 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr868.htm

115. Sony SLV777B
Sony SLV777B No drum or capst 1st time tape loaded Check for electrolyte gunge around the 5 pins of 9v reg 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr857.htm

116. Sony SLVE280
Sony SLVE280 PSU display pulses every sec JS130 cct prot had been replaced with Wickman fuse use correct cct prot 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr867.htm

117. Sony SLVE720
Sony SLVE720 Tuner low gain ok on loop thro pback Dry Joints tuner receiver module 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr878.htm

118. Sony SLV415
Sony SLV415 Head twitches Hall effect sensor on drum can be cause good reading 200 300R between any 2 pins faulty 1500R 
URL: http://homepages.nildram.co.uk/~mkstevo/vcrpages/vcr847.htm

119. SONY SLVE220
SONY SLVE220 SWITCHES OFF ON RECORD REPLACE PR512 SONY180156700 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec391.htm

120. SONY KV32WF1
SONY KV32WF1 NO SOUND REPLACE IC1200 PS602 Q1200 AND Q1201 SONY875925068 153268621 MITS240P419030 AND ALCO00800143002 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec392.htm

121. Sony AE1B
Sony AE1B Pic int blanks out Comes on with tapping on rear panel Q1404 faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1601.htm

122. SONY KVD2912
SONY KVD2912 FRAME FOLDOVER REPLACE C531 680MFD25V 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec390.htm

123. SONY KV21X4U
SONY KV21X4U NO PICTURE LED BLINKS TWICE RESOLDER IC604 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec393.htm

124. SONY KV29F2
SONY KV29F2 NO SOUND FROM COLD REPLACE IC3 SONY875947773 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec396.htm

125. SONY KVX2942
SONY KVX2942 INT NO SOUND FROM SURROUND SPEAKERS REPLACE R627 IN POWER SUPPLY 270 OHM 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec397.htm

126. SONY KVX2582U
SONY KVX2582U INT CRACKLING SYMBOLS ON SCREEN JUMPING BETWEEN NICAM AND MONO REPLACE X1101 ON NICAM PCB 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec394.htm

127. SONY SLVE25UY
SONY SLVE25UY LEAVES LOOP OF TAPE ON EJECT CHECK CONDITION OF FELT ON BACK TENSION ARM 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec395.htm

128. Sony AE2
Sony AE2 Trips 13 times indicates field fault STB9379 field chip can use TDA8179S 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1611.htm

129. Sony AE1A
Sony AE1A Field partial collapse C531 680 25v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1598.htm

130. Sony XE4
Sony XE4 Dead Dry joints on 2 large upright res in psu 4A fuse o c 2SD1497 s c replaced with BU208A 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1671.htm

131. Sony AE1
Sony AE1 Field non lin and short pic C531 680 25v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1596.htm

132. Sony KV2184-34
Sony KV2184 34 Won t start Nothing heard upon switch on Diode D604 short circuited 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1819.htm

133. Sony KV2206
Sony KV2206 Dead Dry joints chopper tfmr pins 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1661.htm

134. Sony BE2
Sony BE2 St by only PS801 cct prot 600mA in HT line o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1631.htm

135. Sony BE1
Sony BE1 Dead HT 110v present at test point No B volts at loptr PS01 N15 600mA cct prot o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1621.htm

136. Sony KV1422
Sony KV1422 Won t power up Q503 leaky tested ok out of cct also C801 22 250v s c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1651.htm

137. Sony BE2A
Sony BE2A No start up R602 270k 1 2w under chopper tfmr o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1641.htm

138. SONY KVX2932U
SONY KVX2932U FRAME FOLDOVER REPLACE C531 1000MFD50V CPC CANHE31 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec389.htm

139. SONY KV21T3
SONY KV21T3 VOLUME GOES TO MAX NO REMOTE OR FRONT CONTROL REPLACE MICRO SONY875947506 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec388.htm

140. SONY KV28WS2U
SONY KV28WS2U BUZZING NOISE THROUGH SPEAKERS CHECK EARTH CONNECTION ON CN303 PLUG FOR O C OR DRY JOINT 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec371.htm

141. SONY KV29F2
SONY KV29F2 SET COMES ON WITH FLYBACK LINES THEN GOES DEAD REPLACE Q707 GREEN Q710 BLUE OR Q704 RED TRANBF423 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec370.htm

142. SONY KVM2101U
SONY KVM2101U WIDTH AND E W PROBLEMS REPLACE C806 47NFD250V SONY113618711 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec387.htm

143. SONY KVX2512U
SONY KVX2512U FRAME FOLDOVER REPLACE C531 532 NEAR FRAME O P IC SONY112412211 AND 1124190000 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec384.htm

144. SONY KV25KSU
SONY KV25KSU FRAME IC HOT LINES ON SCREEN REPLACE C506 0 01MFD SONY116423211 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec383.htm

145. SONY SLVE920
SONY SLVE920 NO CLOCK DISPLAY Q607 2SD774 IN POWER SUPPLY 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec386.htm

146. SONY KVA2542
SONY KVA2542 DEAD CHECK CIRCUIT PROTECTORS PS682 AND PS681 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec385.htm

147. SONY KVDX271TU
SONY KVDX271TU SET TAKES 10 20 MINS TO COME ON REPLACE C615 IN POWER SUPPLY 1000MFD35V CAP CA00180 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec372.htm

148. SONY KVX2902
SONY KVX2902 NO REMOTE FUNCTIONS RESOLDER RA1 RA2 AND RA3 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec368.htm

149. SONY KV29X1
SONY KV29X1 INT BLANK SCREEN REPLACE A BOARD SONYA1632400A 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec369.htm

150. SONY KVX2942U
SONY KVX2942U GRAPHICS WOBBLE FROM SISE TO SIDE RESOLDER 12V REGULATOR UNDER PLASTIC CRADLE 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec378.htm

151. SONY KVX2952Y
SONY KVX2952Y FRAME CRAMPED WITH FLYBACK LINES REPLACE C532 100MFD50V CAP CE100MF50V 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec379.htm

152. SONY SLVE7UB
SONY SLVE7UB DIM DISPLAY REPLACE C207 1000MFD63V IN POWER SUPPLY SONY112691651 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec377.htm

153. SONY KV32WF1U
SONY KV32WF1U DEAD WITH POWER SUPPLY SCREAMING REPLACE LINE O P TRANSISTOR Q802 SONY872904286 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec374.htm

154. SONY KVA2942S
SONY KVA2942S FOR VERTICAL SHIFT AND HEIGHT PROBLEMS ENTER SERVICE MODE AND MAKE SURE GEOMETRY SET IS OFF 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec373.htm

155. SONY SLVE220
SONY SLVE220 DRONING NOISE ON PLAYBACK REPLACE IC101 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec376.htm

156. SONY KV28WS2
SONY KV28WS2 VOLUME VARIES CHANNELS WIPE FROM MEMORY UNUSUAL CHARACTERS DOWN SIDE OF SCREEN REPLACE IC3 WITH PREPROGRAMMED NEW TYPE SONY875945310 
URL: http://homepages.nildram.co.uk/~mkstevo/tecpage/tec375.htm

157. Sony BE1
Sony BE1 Won t stop lock when search tuning Sync gen Tr Q071 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1630.htm

158. Sony 27XRTU
Sony 27XRTU DEAD CHECK R 822 1K2 LINE DRIVE 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2143.htm

159. Sony 27XRTU
Sony 27XRTU NO SYNC SAA5230 AND Q002 12v REG 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2144.htm

160. Sony 1412UB
Sony 1412UB DEAD ic OSCILLATING MAY START IF ic HEATED R 602 2 2M 1 2 W 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2141.htm

161. Sony 1420
Sony 1420 LINE TEARING E W VARIES CHECK Q803 PIN AMP TIP41 WORKS 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2142.htm

162. Sony 27XRTU
Sony 27XRTU NO SYNC NO INPUT AT PIN 5 IC501 NO VIDEO OUTPUT AT PIN 1 SAA5230 ON TEXT PCB SAA5230 US CHECK Q002 12v REG 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2145.htm

163. Sony AE 2
Sony AE 2 RANDOM SPIKES AND DIST HIGH SOUND LEFT SPEAKER ADD 3K3 RES AND 0 015mfd CAP PARALLEL TOC201 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2148.htm

164. SONY AE1
SONY AE1 Line jitter and rolling Not if coils Check Q598 Video amp beneath tuner 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2149.htm

165. Sony AE 2 AE 2A AE 2B AE 2F
Sony AE 2 AE 2A AE 2B AE 2F NO COLOUR WITH VTR PLAYBACK ESP 2nd GEN COPIES REPLACE IC301 FROM TDA9145 N1 OR TDA9145N2B TO TDA9145N3C Pt No 8 759 343 40 REPLACE L307 4 7uH BY LINK
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2146.htm

166. Sony AE 2A
Sony AE 2A FAILURE OF VERT OUT IC 1501 REPLACE TDA8179S WITH STV9379 8 759 192 71 AND CHANGE C1506 1 130 777 00 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2147.htm

167. Sony KVA2912 AE 1C
Sony KVA2912 AE 1C Pic flicks from colour to B W 5V supply on B board missing Circuit protector o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2209.htm

168. Sony KV2762 PE 3
Sony KV2762 PE 3 Field output IC replaced with version TDA3654 change R518 from 1K5 to 6K8 on D board 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2201.htm

169. Sony KV27XRTU
Sony KV27XRTU DEAD ENERGISES BUT SHUTS DOWN LOPT FAT 3925 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2202.htm

170. Sony KVA2942U
Sony KVA2942U 14 flashes then dead Replace Sound o p IC on sub panel 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2210.htm

171. Sony KV2756UB
Sony KV2756UB NO RASTER BLANK SCREEN FIELD COLLAPSE CHECK D804 1R2 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2200.htm

172. Sony KV2972 AE 2B
Sony KV2972 AE 2B Random OSD characxters appear IC2002 u s SDA5237P 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2203.htm

173. Sony KVA2512U AE1C
Sony KVA2512U AE1C Int colour Tc1 on video pcb dry Check screening cans etc for drys also 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2207.htm

174. Sony KVA2912 AE 1C
Sony KVA2912 AE 1C FLICKS FROM COLOUR TO B W NO 5v ON BOARD B CHECK 5v REG 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2208.htm

175. Sony KV2972U AE 2B
Sony KV2972U AE 2B OSD APPEARS AT RANDOM IC002 IC2002 SDA5273P 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2204.htm

176. Sony KVA2512
Sony KVA2512 INT ROLLS GOES B W LINE TWITCHES DRY JOINTS INSIDE CAN ON DIGITAL VIDEO PANEL 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2206.htm

177. Sony KV211MTU
Sony KV211MTU EW FAULT RH SCREEN BLANK DRY JOINTS ON AV BOARD 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2182.htm

178. Sony KV211XMTU AE1
Sony KV211XMTU AE1 PICTURE COVERED BY FINE SHIMMERING HORIZONTAL BLACK LINES R544 330R O C IN FIELD CCT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2183.htm

179. Sony KV211 AE 1
Sony KV211 AE 1 Lack of height field foldover cramping in centre R801 0 47R 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2180.htm

180. Sony KV211 AE 1
Sony KV211 AE 1 No sound or pic 12V at L606 low R822 1K o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2181.htm

181. Sony KV21MU ( I think ) 21" TEXT
Sony KV21MU I think 21 TEXT Int picture touching adjusting A1 brings picture back Check for drys on text pcb particularly on top mounted plugs sockets 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2184.htm

182. Sony KV21XRTU
Sony KV21XRTU WHEN TEXT ON ONLY TV CHROMA IN MIX ONLY TV DJ PIN 5 OF CONNECTOR D3 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2187.htm

183. Sony KV2201
Sony KV2201 LOSES LINE SYNC ON CHANNEL CHANGE D811 R825 D810 R838 ALL DRY JOINTED 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2188.htm

184. Sony KV21XMTU
Sony KV21XMTU LOSES PICTURE ON CHANNEL CHANGE INT IC1003 PIN 2 NOT 30v L8O7 INT OC 30v FEED 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2185.htm

185. Sony KV21XMTU
Sony KV21XMTU Picture dissapears and goes to snow Check memory IC for 30v if low check L807 10uH 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2186.htm

186. Sony KVX2172 AE 2B
Sony KVX2172 AE 2B Set will only display one station if another is selected it shows for 2 to 3 secs then revert to original station EAROM ST24C16 Data corrupted replace set up
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2249.htm

187. Sony KVS3412U AE 2
Sony KVS3412U AE 2 Set comes on then collapses into lines in the centre of screen with high pitched whistle from line stage Check H T for 119V if low at 70V Q613 IN AUDIO
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2242.htm

188. Sony Kvx 2952 AE1C
Sony Kvx 2952 AE1C Blows Chopper Transistor After Few Mins Replace C611 47uf In Base Circuit Also Check R651 100 OHM SMD FOR O C 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2243.htm

189. Sony KVM2141
Sony KVM2141 NO PIC LOSS OF SYNC C14 4 7uF 50V P501 N15CCT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2240.htm

190. Sony KVM2141U
Sony KVM2141U LOW CONTRAST ONLY GOES UP SO FAR THEN LEVELS R816 PIN 4 LOPT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2241.htm

191. Sony KVX2121
Sony KVX2121 Crackle on sound D J around earth points check mains bridge 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2244.htm

192. Sony KVX2162 AE 2
Sony KVX2162 AE 2 LED display indicates set in protection mode Disconnect protection line from micro if set comes on with small field EEPROM u s 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2247.htm

193. Sony KVX2162 AE 2
Sony KVX2162 AE 2 Sound cuts out when vol turned up D J on X1101 crystal on Nicam PCB 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2248.htm

194. Sony KVX2132 AE 1B
Sony KVX2132 AE 1B No field scan Check R802 0 47R R530 1R2 feeds to IC502 TDA8170 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2245.htm

195. Sony KVX2162 AE 2
Sony KVX2162 AE 2 Cuts out goes into protection mode RGB lights edges of screen D J on IC1501 Field output IC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2246.htm

196. Sony KVDX21TU AE 1
Sony KVDX21TU AE 1 CHOPPER TRANSISTOR SC R614 OC MAYBE IC601 ALSO TEA2164 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2211.htm

197. Sony KVX2532 KVX2952 KVX2152 AE 1B
Sony KVX2532 KVX2952 KVX2152 AE 1B POOR FIELD LIN C531 680mfd 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2257.htm

198. Sony KVX2521U KVM2511 AE 1
Sony KVX2521U KVM2511 AE 1 CAN T STORE STATIONS PIN 2 ICOO3 NOT 33v L807 OC PtNo 140750400 also check R008 C017 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2256.htm

199. Sony KVX2552 AE 1C
Sony KVX2552 AE 1C BLANK RASTER Q305 DTA144EK ON DECODER BOARD B 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2259.htm

200. Sony KVX2542A
Sony KVX2542A INT RIGHT CHANNEL IC261 PIN4 DRY 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2258.htm

201. Sony KVX2521 AE 1
Sony KVX2521 AE 1 STARTS THEN SHUTS DOWN REMOVE T601 RESOLDER JOINTS ON ALL PINS THEN REFIT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2255.htm

202. Sony KVX2521 AE 1
Sony KVX2521 AE 1 INT LOSS PICTURE SOUND Q808 12v REG DRY 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2252.htm

203. Sony KVX2182U KVX2582U
Sony KVX2182U KVX2582U LINE PULLING ON MACRO TAPES REPLACE JUMPER WIRE JW802 MOUNTED IN LOC L800 with 330R 3W res PtNo 121590821 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2251.htm

204. Sony KVX2521 AE 1
Sony KVX2521 AE 1 Motor boating noise in StBy D J on Heat sink audio output ic earth also check C612 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2254.htm

205. Sony KVX2521 AE 1
Sony KVX2521 AE 1 Int loss sound pic ok D J on IC251 or IC252 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2253.htm

206. Sony KV2217 KV2202 YE2
Sony KV2217 KV2202 YE2 BLANK SCREEN IN TEXT RV3 ON TEXT PCB 100R 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2190.htm

207. Sony KV2705 QE1
Sony KV2705 QE1 No sound or picture no response from front panel buttons SW407 leaky 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2197.htm

208. Sony KV2704
Sony KV2704 POOR STARTING R605 R606 OR C623 33uF 250v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2196.htm

209. Sony KV2715UB
Sony KV2715UB NO COLOUR T302 OC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2199.htm

210. Sony KV2712
Sony KV2712 SCREEN ONE HALF GREEN IC 302 TDA4661 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2198.htm

211. Sony KV2576UB
Sony KV2576UB NO RASTER FIELD COLLAPSE BUT BLANKED OUT CHECK RES IN SERIES WITH D804 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2195.htm

212. Sony KV2256 RX
Sony KV2256 RX 6 wide bands across screen Black Grey White Replace C655 220uF 5V supply to micro 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2192.htm

213. Sony KV2217
Sony KV2217 STUCK ON CHANNEL 6 NO SOUND OR PIC VOLUME DOWN BUTTON SC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2191.htm

214. Sony KV2562
Sony KV2562 COLOUR FLICKERS OFF AFTER 30mins IC301 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2194.htm

215. Sony KV2512
Sony KV2512 LOOKS LIKE FINE TUNE OFF WILL NOT LOCK NICAM IC601 DRY 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2193.htm

216. Sony KVG2515U AE 2B
Sony KVG2515U AE 2B NO SATELITE CHECK CP605 CIRCUIT PROTECTOR 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2218.htm

217. Sony KVFX29
Sony KVFX29 Frame Foldover At The Top Of Pic Replace C524 3 3uf 160v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2217.htm

218. Sony AE2 Chassis
Sony AE2 Chassis Blank Raster With Purity Error This Fault Is Actually Frame Collapse Which Is Shifted Of Screen With Light Reflected From Bowl Of Crt Check O P ic Also L1501
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2150.htm

219. Sony KVG2515U AE 2B
Sony KVG2515U AE 2B SATELLITE SECTION DEAD CHECK PS605 ON PCB D N15 600ma CORRECT TYPE 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2219.htm

220. Sony KVFX29 FX
Sony KVFX29 FX Field foldover D506 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2216.htm

221. Sony KVE2531 KVE2532 KVS2913 AE 2
Sony KVE2531 KVE2532 KVS2913 AE 2 PERIODICAL SWITCHING OFF AND ON 10 secs REPLACE IC003 WITH AE20 TYPE VERSION V2 82 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2213.htm

222. Sony KVDX271 AE 1
Sony KVDX271 AE 1 NO NICAM BPF 1101 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2212.htm

223. Sony KVE2922U AE 1C
Sony KVE2922U AE 1C INT COL FLASHING PIC CHECK FOR DRY JOINTS ON TAGS OF SCREENING CAN 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2215.htm

224. Sony KVE2912U
Sony KVE2912U No Sound for the first 5 minutes Replace C615 1000mfd 25V 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2214.htm

225. Sony AEl Chassis
Sony AEl Chassis Intermittent loss of the luminance signal Delay line DL332 on PCB B Its part no is 1 236 062 11 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2151.htm

226. Sony KV1412
Sony KV1412 Tuner out of action If 33V ok check Q10 for control voltage VC point on tuner if low Replace C134 inside tuner 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2158.htm

227. Sony KV1412
Sony KV1412 DEAD PIN 4 CHOPPER IC NOT 2 9v R602 2M2 OC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2157.htm

228. Sony KVX2172 AE 2B
Sony KVX2172 AE 2B VOL WONT ALTER REPLACE IC072 EAROM 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2250.htm

229. Sony KV1460 GP1
Sony KV1460 GP1 DEAD HT FUSE BLOWN SR2M OVERVOLTS DIODE S C STR50115 THEN NO LINE DRIVE IC501 UPC1377C 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2159.htm

230. Sony CRTS
Sony CRTS NO FOCUS CHECK BEHIND CRT BASE 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2156.htm

231. Sony BE 2A
Sony BE 2A IC1001 REPLACEMENT FROM PCA84C840P 016 WHEN FITTING NEW TYPE WITH A SUFFIX CHANGE R084 FROM 2 2K TO 5 1K 1 216 066 00 R025 FROM 100K TO 82K 1 216 093 00 C006 FROM
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2153.htm

232. Sony AEl Chassis
Sony AEl Chassis intermittent teletext contrast variations Dry joint at the emitter of Q02 on board V 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2152.htm

233. Sony BE 3B
Sony BE 3B GOES TO STBY AFTER REPLACING IC605 7812 REPLACE WITH ORIGINAL TYPE ICMCT7812CT Pt No 8 759 279 76 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2155.htm

234. Sony BE 3B
Sony BE 3B Clicking noise during Nicam reception TV02794R 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2154.htm

235. Sony KVX25TU AE 1
Sony KVX25TU AE 1 LINE OP TRANSISTOR KEEPS BLOWING L806 SC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2266.htm

236. Sony KVX25TU AE 1
Sony KVX25TU AE 1 POWERS UP THEN TRIPS OFF IC251 LA4280 SHORT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2267.htm

237. Sony KVX2572 AE 2B
Sony KVX2572 AE 2B HISSING ON RH CHANNEL DELETE SPATIAL SOUND ONLY USED ON MONO TRANSMISSION 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2264.htm

238. Sony KVX25TU AE 1
Sony KVX25TU AE 1 INT LOSS SOUND AND PICTURE Q608 12v REG DRY RESOLDER CHOPPER TX AND METAL PLATES EARTH TABS 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2265.htm

239. Sony KVX2902 BE 3B
Sony KVX2902 BE 3B All supply lines come up then go low StBy Check pin 9 CN001 if high set in protection mode disconnect pin if set still fails to come on Replace IC002 Memory
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2268.htm

240. Sony SLV615
Sony SLV615 NO PICTURE BUT HI FI SOUND OK HEAD AMP HA118019NT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst268.htm

241. Sony SLV280
Sony SLV280 Chassis uses a Sanyo mech Chews tape on eject The capstan brake pad becomes sticky 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst264.htm

242. Sony KVX2932 AE 1B
Sony KVX2932 AE 1B NO EHT LINE SCAN DRIVE APPROX 10v D1504 SC ON SCART PCB 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2269.htm

243. Sony SLV-270
Sony SLV 270 DEAD SOLENOID MAY RATTLE ALL SUPPLY RAILS LOW REPLACE C1326 47uF 25v C1325 1uF 100v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst263.htm

244. Sony KVX2562 AE 2
Sony KVX2562 AE 2 ODD LINEARITY FAULTS LOOKS LIKE RINGING CXD2018Q IC561 48 PIN SURFACE MOUNT DEVICE GOOD LUCK Pt No 8 752 347 92 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2263.htm

245. Sony KV2075
Sony KV2075 POOR CONVERGENCE L551 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2166.htm

246. Sony KV2090 XE4
Sony KV2090 XE4 BLOWS CHOPPER AND OR LINE TRANSISTORS REPLACE D605 614 RGP15J AND C513 2 700pF IN LINE OSC CCT RESOLDER STAND OFF RESISTORS 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2167.htm

247. Sony KV2060UB XE3 Chassis
Sony KV2060UB XE3 Chassis Dead with no drive at pin 7 of the chopper control chip IC601 despite the fact that its supply was present and the internal oscillator was working R632
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2164.htm

248. Sony KV2062
Sony KV2062 BLANK SCREEN SOUND OK uPC1377 LINE OSC IC FAULTY 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2165.htm

249. Sony KV2090 XE4
Sony KV2090 XE4 DEAD LOP TR SC PREVIOUSLY SET SHUT DOWN INT C513 2700pF IN LINE OSC CCT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2168.htm

250. Sony KVX2552 AE 1C
Sony KVX2552 AE 1C WONT TUNE CHECK RR05 10K 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2261.htm

251. Sony KVX2562 AE 2
Sony KVX2562 AE 2 Chopper TR Q601 s c BUZ91A f e t Check R604 150K R610 180K 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2262.htm

252. Sony KV2092
Sony KV2092 INT FAILURE OF LOP TRANSISTOR 2SD1398 CHECK BASE DRIVE COUPLING COIL L801 FOR INT OC OR SC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2169.htm

253. Sony KVX2552 AE 1C
Sony KVX2552 AE 1C SOUND ON VISION AT HIGH VOLUME LEVELS NORMAL 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2260.htm

254. Sony BE2A
Sony BE2A Pic goes to bright white raster int S castle pulse pin 10 IC302 dist D504 by lopt leaky 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1642.htm

255. Sony KV1462
Sony KV1462 Negative pic Dry joints IF201 module 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1652.htm

256. Sony BE1
Sony BE1 Field collapse Replace fop chip with a PC1488H orig LA7830 check cct prot 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1622.htm

257. Sony BE2A
Sony BE2A Blank raster sound ok Scastle pulse dist DAN202 sm diode D504 leaky 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1632.htm

258. Sony KV2762
Sony KV2762 Field reduction non lin TDA3652 field o p chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1662.htm

259. Sony AE2
Sony AE2 Pic int blanks out then shuts down restarts Loss of field o p dry joints fop chip next to lopt by back panel 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1610.htm

260. Sony BE1
Sony BE1 6 flyback lines 2 of each col top of pic IC501 PC1488H chip dry joints 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1620.htm

261. Sony XE4
Sony XE4 Dead fuse blown C815 faulty tho check ok Q602 2SD1497 s c D605 D614 RGP10G 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1672.htm

262. Sony AE1B
Sony AE1B Field cramped at bottom stretched at top C531 680 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1600.htm

263. Sony AE2
Sony AE2 vertical ripple staining effect D613 double diode in psu leaky 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1612.htm

264. Sony SLVE8
Sony SLVE8 INT EJECT IC201 DRY 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst272.htm

265. Sony Panasonic SLV425 Panasonic G MECH
Sony Panasonic SLV425 Panasonic G MECH STARTS TO LACE THEN UNLACES AND EJECTS OC SOLENOID 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst262.htm

266. Sony SLVE40
Sony SLVE40 Won t play loops tape shows L in display very int Replace mode switch 3 946 958 01 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst270.htm

267. Sony SLV615
Sony SLV615 Loss of playback picture screen going to blue mute But Hi Fi sound ok Check HA118019NT head amp chip 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst267.htm

268. Sony SLV415
Sony SLV415 Head drum twitches Hall sensor on drum motor US Poor sync Modulator faulty 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst266.htm

269. Sony SLV625
Sony SLV625 CHEWS TAPES ON EJECT NO 9v Q203 OC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst269.htm

270. Sony AE1B
Sony AE1B St by only Dry joints audio o p chip IC261 blew chip TDA2050 then destroyed D613 614 19v rects 
URL: http://homepages.nildram.co.uk/~mkstevo/faults2/flt1602.htm

271. Sony SLVE8
Sony SLVE8 Fails to eject Dry joints on IC201 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst271.htm

272. Sony SLV315 and others
Sony SLV315 and others Int start No sound Tape damage Half load arm becomes very tight on shaft removed clean and oil 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst265.htm

273. Sony KV2096 XE4
Sony KV2096 XE4 INT PICTURE SOLDER IC003 DIRECT TO PCB ON TEXT PCB 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2171.htm

274. Sony KV2096 XE4
Sony KV2096 XE4 INT STUCK ON AV AT SWITCH ON OR IR DIODE FLASHES AS IF REM BEING USED DRIES ON SCREENING CAN AROUND MICRO ALSO CHECK MAIN PCB WHERE SUBPANEL IS PLUGGED IN
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2172.htm

275. Sony KVM2140
Sony KVM2140 LOW CONTRAST IC302 PIN 19 LOW SHOULD BE BETWEEN 1 4v CHECK R817 R818 R816 150K 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2239.htm

276. Sony KV2092
Sony KV2092 SOUND VARIES HARD TO ADJUST D016 LEAKY 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2170.htm

277. Sony KV2096 XE4
Sony KV2096 XE4 INT AV CHECK FOR DRY JOINTS ON SCREENING CAN AROUND MAIN MICRO 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2173.htm

278. Sony KV2096 XE4
Sony KV2096 XE4 SHIMMER ON VERTICALS WORSE WHEN WARM DRYS ON FIELD HEATSINK 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2176.htm

279. Sony KV2096 XE4
Sony KV2096 XE4 SOMETIMES COMES ON IN AV MODE REMOTE LED FLASHES AT RANDOM POOR SOLDERING AROUND IR SCREENING CAN 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2177.htm

280. Sony KV2096 XE4
Sony KV2096 XE4 LINE TRANSISTOR SC FUSE OC TRANSISTOR BLOWS AT SWITCH ON LOPT SC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2174.htm

281. Sony KV2096 XE4
Sony KV2096 XE4 POOR FIELD LIN C527 470uF 25v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2175.htm

282. Sony KVM2140
Sony KVM2140 DEAD D611 SHORT DUE TO HIGH HT C618 OPEN 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2238.htm

283. Sony KVM2121U
Sony KVM2121U LINES OF RED BLUE GREEN AT TOP OF SCREEN IC501 FIELD OUTPUT DRY u s R801 1K 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2231.htm

284. Sony KVM2131 BE 1
Sony KVM2131 BE 1 DEAD NO HT BU506 SC CHECK R606 68R OR REPLACEMENTS WILL BLOW AS HT MAY BE TOO HIGH 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2232.htm

285. Sony KV2202 KV2212
Sony KV2202 KV2212 From Cold Top 2 3 of pic blanked out improves as set warms C818 4 7uF 250V 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2189.htm

286. Sony KVM2120U BE1
Sony KVM2120U BE1 DEAD ONLY 25v ON D604 119v IS CORRECT C608 680pF LEAKY CONNECTS ACROSS D604 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2230.htm

287. Sony KVM2131 BE 1
Sony KVM2131 BE 1 Picture blanks out leaving OSD or pic breaks up DJ on T01 in if can 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2233.htm

288. Sony KVM2131 BE 1
Sony KVM2131 BE 1 SOUND WONT ADJUST OSD OPERATING NORMALLY D201 1SS133 ON AF MUTE LINE 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2236.htm

289. Sony KVM2131 BE1
Sony KVM2131 BE1 WHITE FLASHES ON PICTURE Q301 NOISY JC501Q 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2237.htm

290. Sony KVM2131 BE 1
Sony KVM2131 BE 1 SET WON T TUNE IN BUT OSD MOVES CORRECTLY IF 33v ON IC004 OK Check Q004 D156 SC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2234.htm

291. Sony KVM2131 BE 1
Sony KVM2131 BE 1 SOUND STUCK AT ZERO OSD MOVES PIN 7 IC201 TDA1013 AT 6v D201 SC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2235.htm

292. Sony KVM1420U BE 2A
Sony KVM1420U BE 2A FAINT VERTICAL LINES LEFT SIDE OF SCREEN FIT 150nf 250v CAP BETWEEN PINS 2 AND 3 OF CNC74 IS NEAR FOCUS POT 3 PIN PLUG CAP PtNo 1 137 146 11 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2224.htm

293. Sony KVM1420U BE 2A
Sony KVM1420U BE 2A NO BLUE IC302 TDA3505 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2225.htm

294. Sony KVM1400
Sony KVM1400 There was no power The red standby light was on however it flashed twice A quick check in the line output stage showed that there was no HT supply This was simply
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2222.htm

295. Sony KV1612
Sony KV1612 NO S V MAIN SUPPLIES ALL CORRECT CHECK 12v SUPPLY TO TUNER FOLLOWS COMPLEX PATH VIA R811 D811 CONNECTOR D5 AND A12 PIN 1 Q207 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2161.htm

296. Sony KVM1420U BE 2A
Sony KVM1420U BE 2A NO PIC VIDEO MUTED C329 22uF TAN SHORT 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2226.htm

297. Sony KVM1421 KVM2121 BE 1
Sony KVM1421 KVM2121 BE 1 Field collapse Check R801 0 47R PS501 if both o c IC501 uPC1488 u s if ok check for D J around ic 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2228.htm

298. Sony KVM19TU
Sony KVM19TU LOW WIDTH L804 O C 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2229.htm

299. Sony KVM1421 BE 2A
Sony KVM1421 BE 2A BLUE SCREEN FLYBACK LINES TDA3505G VIDEO PROC 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2227.htm

300. Sony KV1460 GP1
Sony KV1460 GP1 NO SOUND OR VIS JUST BLANK RASTER AND FLYBACK LINES WHEN A1 TURNED UP NO KEY SCAN PULSES AT PINS 5 6 32 AND 33 OF M50431 M50431 511SP 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2160.htm

301. Sony KV2000 Mk2
Sony KV2000 Mk2 TOP OF PICTURE DARK IMPROVING WHEN WARM C827 4u7 250v 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2162.htm

302. Sony KV2052
Sony KV2052 INT DEAD WHEN ON PIC TEARS Q001 BC237 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2163.htm

303. Sony KX27PSI
Sony KX27PSI REPEAT FAILURE OF Q651 CHANGE DAMPING DIODE D654 ON FA BOARD FIT RGP02 17 Pt No 8 719 976 64 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2270.htm

304. Sony KV2096 XE4
Sony KV2096 XE4 VERTICAL DISTORTION FROM COLD RESOLDER FIELD IC AND HEATSINK 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2178.htm

305. Sony KV211 AE 1
Sony KV211 AE 1 Blank screen with flyback lines R805 1R2 o c 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2179.htm

306. Sony
Sony NO E W CORRECTION R1584 150 ohm L 1553 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2271.htm

307. Sony KVM 2171U BE4/ BE4A Chassis
Sony KVM 2171U BE4 BE4A Chassis No EW Correction Replace IC301 MC 44007P 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2220.htm

308. Sony KVM1120
Sony KVM1120 USER DEFINED SECURITY CODE IS UNKNOWN WHEN CODE IS ASKED FOR PRESS TEST ON A SERVICE 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2221.htm

309. Sony
Sony POOR PURITY SLIDE SCAN COILS FORWARD 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2272.htm

310. SONY SLVE220
SONY SLVE220 Cuts out on record patterning on picture Replace J5130 1 amp CP res gone high 
URL: http://homepages.nildram.co.uk/~mkstevo/faults/tst2273.htm