DCRTRV103. Subject BAD DIGITAL PB ON STANDARD TAPES - REPLACED DRUM ASSY.

SYMPTOMS

Digital looks good but has intermittent dropouts, resembles bad heads.

RESOLUTION

Replaced drum assy resolved symptom. Source:

| A7048904A | M901 |
| --- | --- |

DCRTRV103. Subject BOARD NOT INITIALIZED - INITIALIZE EVR DATA PAGES.

SYMPTOMS

After replacing vc213 board radar would not communicate with camera. Found that if you initialized page D of the camera , page 5-12 in manual, unit would then do a complete download and the camera would work correctly.

RESOLUTION

We're adding this feature to radar. A beta version of the program was available as of Nov 99. Please check Radar web page for the final version.

| A7094282A | VC213 |
| --- | --- |

DCRTRV103, DCRTRV110, DCRTRV310, DCRTRV315, DCRTRV510. Subject CAUTION WHEN REPLACING VC213 BOARD..

SYMPTOMS

There are two types of DV Interface IC303 on the VC-213 boards. One is a Sony and the other is TI. Both manufacturers are labled on the top of the IC. Whenever the board is replaced, the proper page C, address AD data must be installed. Incorrect  data will result in NO DV IN/OUT. As most facilities do not have the VC-213 board layout diagram, I will discribe the IC303's location. IC303 is the TGA IC located next to the white 26 pin connector. This connector attaches to flex FP-78 which feeds  analog and digital A/V to the PJ and SE boards.

RESOLUTION

When IC303 is Sony install data 00 in page C, address AD. When IC303 is TI install data 02.

DCRTRV103. Subject EATS TAPES - REPLACED TU REEL & TG4.

SYMPTOMS

UNIT ATE TAPES.

RESOLUTION

FOUND TU-REEL TENSION WAS TOO LOW AND TG4 WAS BENT. REPLACE BOTH FIX THE PROBLEM.

DCRTRV103. Subject EJECT SW JAMS.

SYMPTOMS

Eject SW is a little short and jams up.

RESOLUTION

2 cases have been reported and one case found in Japan. Samples were sent to Tokyo for analysis - See comments for details. Japan told us that the failures were due to a few isolated bad parts. we will close the issue for now. Please continue to  report those failures.

DCRTRV103. Subject SHUTS OFF WHEN EJECT PRESSED- REPLACED CAPSTAN MOTOR.

SYMPTOMS

UNIT WILL SHUT DOWN AS SOON AS THE EJECT SWITCH HAS BEEN PRES SED.

RESOLUTION

FOUND CAPSTAN MOTOR IS SHO RTED.REPLACED CAPSTAN MOTOR SO LVE PROBLEM.

DCRTRV103. Subject TAPE INT EJECTING - REPLACED MODE SW.

SYMPTOMS

TAPE WAS INTERMITTENTLY EJECTING WHEN CHANGING INPUT COMMENS.

RESOLUTION

REPLAC ED ENCODER SWITCH S901 ,PART# 176243615, UNIT RESTORED TO NORMAL OPERATION.

DCRTRV103. Subject WON'T LOAD - REPLACE BROKEN LOCK GUIDE #768.

SYMPTOMS

UNIT WOULD NOT LOAD.

RESOLUTION

FOUND LOCK GUIDE BROKEN. REF#768 PT# 396556811.

CCDTR517, CCDTRV37, CCDTRV57, CCDTRV67, CCDTRV87, DCRTR7000, DCRTRV103, DCRTRV110, DCRTRV310, DCRTRV315. Subject P/N for Compression Springs are Swapped in SM..

SYMPTOMS

P/N for Compression Springs are Swapped in SM.

CAUSE

In SM Page 6-5, the P/N for Compression Springs (Ref # 207 & 212) are reversed.

RESOLUTION

The part numbers should be the following.
Ref # 207/157: 3-302-492-00 (Eject lever SW)
Ref # 212/161: 3-979-642-01 (Rec/Stop SW)

Note1: In the CCD models the ref numbers for Eject Lever SW and Rec/Stop SW are 212 & 207. In  DCR models the ref numbers are 161 & 157 respectively.

DCRTRV103. Subject Reference number 455 has incorrect part number - Correct P/N is 3-053-827-01..

SYMPTOMS

Reference number 455 has incorrect part number - Correct P/N is 3-053-827-01.

CAUSE

Part number for IR lever listed in service manual on page 6-10 reference number 455 is incorrect.

RESOLUTION

Correct part number should read 3-053-827-01.

| Model DCRTRV103: Features |  |
| --- | --- |
|  | ZOOM LENS 20X OPTICAL, 360X DIGITAL |
|  | PCM DIGITAL STEREO AUDIO |
|  | STEADYSHOT YES |
|  | SPEAKER BUILT IN SPEAKER |
|  | AE MODES 7 MODES |
|  | LASERLINK YES |
|  | DIGITAL FADER 5 MODES |
|  | PICTURE EFFECTS8 EFFECTS |
|  | 16:9 RECORDING YES |
|  | AUTO FOCUS YES |
|  | DNR YES |
|  | LIGHT NO |
|  | ACCESORY SHOE INTELLIGENT |
|  | DIGITAL EFFECTS6 EFFECTS |
|  | EDIT/SEARCH YES |
|  | SLOW SHUTTER YES |
|  | BACK LIGHT YES |
|  | I LINK YES |
|  | S VIDEO YES |

| Model DCRTRV103: Specifications |  |
| --- | --- |
| VIDEO SYSTEM | 2 ROTARY HEADS |
| AUDIO SYSTEM | ROTARY HEADS, PCM SYSTEM |
| VIDEO SIGNAL | NTSC COLOR, EIA STANDARDS |
| USABLE CASSETTE | HI 8 VIDEO CASSETTE |
| REC/PLAYBACK | 120 MIN. TAPE / 1 HR |
| FF/REWIND | 120 MIN. TAPE / APPROX. 8 MIN |
| IMAGE DEVICE | 1/4 INCH CCD |
| LENS | 37 MM, 20X OPTICAL, 360X DIGITAL |
| FOCAL DISTANCE | 5/32- 2 7/8 INCH, IN 35MM STILL |
| COLOUR TEMP. | AUTO |
| LUX RATING | 1 LUX, 0 LUX IN NIGHTSHOT MODE |
| LCD SIZE | 2.5 INCHES |
| TOTAL DOT NUM. | 61,380 |
| S VIDEO OUT | S VIDEO IN/OUT |
| AUDIO OUT | AUDIO IN/OUT |
| VIDEO OUT | VIDEO IN/OUT |
| DV OUTPUT | DV IN/OUT, 4 PIN CONNECTOR |
| HEADPHONES JACK | STEREO MINIJACK |
| MIC INPUT | MINIJACK |
| LANC JACK | CONTROL L |
| POWER | 7.2V BATTERY, 8.4V AC POWER ADAPTOR |
| DIMENSIONS | 4 X 41/4 X 8 5/8 INCH |
| MASS | 1 LB, 15 OZ |

DCRTRV110. Subject Colored blocks and beeping noise on D8 playback - Replace IC-302 P/N 875239736..

SYMPTOMS

Colored blocks and beeping noise on D8 playback.

RESOLUTION

Replace IC-302 P/N 875239736.

| Model DCRTRV110: Features |  |
| --- | --- |
|  | ZOOM LENS 20X VARIABLE SPEED, 360 DIGITAL |
|  | PCM DIGITAL STEREO |
|  | STEADYSHOT YES |
|  | SPEAKER BUILT IN |
|  | REC. MODE ANALOG RECORDING CAPABILITY |
|  | MIN.ILUMINATION 4 LUX |
|  | AE MODES 7 MODES |
|  | LCD SCREEN 2.5 INCH. |
|  | ACCUPOWER SYST. YES |
|  | LASERLINK YES |
|  | PHOTE MODE YES |
|  | INDEX TITLER 5 LANGUAGE TITLER |
|  | VIDEO INTERFACE NO |
|  | TIME CODE TIME BASED CORRECTION |
|  | PICTURE EFFECTS 8 EFFECTS |
|  | 16:9 RECORDING YES |
|  | CCD IMAGE 460,000 PIXELS |
|  | AUTO FOCUS YES |
|  | DNR DIGITAL NOISE REDUCTION |
|  | VIEWFINDER B/W |
|  | LIGHT BUILT IN |
|  | WORLD CLOCK YES |
|  | REMOTE |
|  | ACCESORY SHOE INTELLIGENT |
|  | DIGITAL EFFECTS 6 EFFECTS |
|  | EDIT/SEARCH YES |
|  | I LINK YES |

| Model DCRTRV110: Specifications |  |
| --- | --- |
| VIDEO SYSTEM | 2 ROTARY HEADS |
| AUDIO SYSTEM | ROTARY HEADS, PCM SYSTEM |
| VIDEO SIGNAL | NTSC COLOR, EIA STANDARDS |
| USABLE CASSETTE | HI 8 VIDEO CASSETTE |
| REC/PLAYBACK | 120 MIN. TAPE / 1 HR |
| FF/REWIND | 120 MIN. TAPE / APPROX. 8 MIN |
| IMAGE DEVICE | 1/4 INCH CCD |
| LENS | 37 MM, 20X OPTICAL, 360X DIGITAL |
| FOCAL DISTANCE | 5/32- 2 7/8 INCH, IN 35MM STILL |
| COLOUR TEMP. | AUTO |
| LUX RATING | 4 LUX, 0 LUX IN NIGHTSHOT MODE |
| LCD SIZE | 2.5 INCHES |
| TOTAL DOT NUM. | 61,380 |
| S VIDEO OUT | S VIDEO IN/OUT |
| AUDIO OUT | AUDIO IN/OUT |
| VIDEO OUT | VIDEO IN/OUT |
| DV OUTPUT | DV IN/OUT, 4 PIN CONNECTOR |
| HEADPHONES JACK | STEREO MINIJACK |
| MIC INPUT | MINIJACK |
| LANC JACK | CONTROL L |
| POWER | 7.2V BATTERY, 8.4V AC POWER ADAPTOR |
| DIMENSIONS | 4 X 41/4 X 8 5/8 INCH |
| MASS | 1 LB, 15 OZ |

DCRTRV310. Subject END SEARCH NOT WORKING - REPLACED CF-63 BOARD.

SYMPTOMS

UNIT HAD INT. END SEARCH FUNCT ION. NO VISIBLE BOARD TROUBLE.

RESOLUTION

REPLACED CF BOARD REF#154 PT#A7073955A. (CONFIRMED PROBLEM ON THIS BOA RD BY SUBBING BOARD WITH TEST JIG CAMERA)

DCRTRV310. Subject EVF WON'T TURN ON WHEN LCD FLIPPED.

SYMPTOMS

EVF WOULD NOT COME ON WHEN LCD WAS CLOSED UNLESS LCD WAS TURNED OUTWARD.

RESOLUTION

FOUND SW ON FP642 PCB(#306) WAS NOT MAKING COMPLETE CONTACT. REPLACED FP642 AND UNIT THEN WORKED PROPERLY.

DCRTRV310. Subject IMAGE UNSTABLE.

SYMPTOMS

FOUND IMAGE MOVING FROM SIDE TO SIDE IN STEADYSHOT.

RESOLUTION

REPLACED SE BOARD UNIT WORKED OK.

DCRTRV310. Subject INT. EJECT.

SYMPTOMS

UNIT INTERMITTENT HAD EJECT SYMBOL.

RESOLUTION

FOUND TAKE UP REEL WAS CLOGGED (DIRTY AT THE BOTTOM REEL). REPLACE NEW REEL, TEST OK SEND UNIT BACK TO CUSTOMER WITH A NEW HI8 TAPE TO TEST.

DCRTRV310. Subject INT. NIGHTSHOT.

SYMPTOMS

UNIT CAME IN WITH INTERMITTENT NIGHT VISION OPERATION.

RESOLUTION

CHECKED AND RESOLDERED THE LASER LIN K DIODES. UNIT WORKED FINE.

DCRTRV310. Subject LOOSE BALANCING WEIGHTS ON DRUM, NOISE.

SYMPTOMS

The plastic piece that holds the balancing weights on the bottom of the drum came loose from the double-stick tape and was interfering with the anti-static brush mounting screw.

RESOLUTION

Applied tiny amount of lock tight to hold in place.

It is important that the glue does not put the drum out of balance. Next time, you may want to use double sided tape as in production

DCRTRV310. Subject LOUD DRUM NOISE IN D8 MODE.

SYMPTOMS

Excessive drum noise in D8 mode.

RESOLUTION

Japan changed drum grease and improved box cushion. S/N range DCR-TR7000: 34101, DCR-TRV110: 73001- CR-TRV103: 97801 , DCR-TRV315: 1030351 , DCR-TRV310: 1238370

DCRTRV310. Subject MENU MALFUNCTION.

SYMPTOMS

UNABLE TO SCROLL TO NEXT SCREEN ON MENU WITH SEL/EXECUTE BUTTON.

RESOLUTION

FOUND POOR SOLDER ON S015 ON CF BOARD. RESOLDERED ALL OK.

DCRTRV310. Subject NO AUTO FOCUS.

SYMPTOMS

THE UNIT WOULD NOT SWITCH TO A UTO MODE FOR FOCUS WAS STUCK I N MANUAL MODE.

RESOLUTION

TRACED PROBLEM TO CF63 PCBD THERE WERE BAD CONNECTIONS REPLACED CF63 PCBD TO SOLVE PR OBLEM.

DCRTRV310. Subject NO EVF PICTURE.

SYMPTOMS

NO PICTURE ON VF.

RESOLUTION

CONNETOR FROM VF TO CF-65 BOARD LOOSE. REINSTAL CONNECTOR.

DCRTRV310. Subject NO EVF VIDEO.

SYMPTOMS

CONFIRMED THE CUSTOMER COMPLAINT THAT THE EVF WAS NOT TURNING ON WHEN THE LCD PANEL IS CLOSED. FOUND THAT THE SW S905 WAS LOOSE.

RESOLUTION

RESOLDERED THE SW TO RESTORE NORMAL OPERATION.

DCRTRV310. Subject NO FOCUS.

SYMPTOMS

B/W VIEWFINDER WILL GO OUT OF FOCUS.

RESOLUTION

FOUND THAT C916 ON THE VF129 BOARD WAS CRACKED AT BOT H ENDS. REPLACED BOARD AND RES TORED NORMAL OPERATION.

DCRTRV310. Subject NO LCD VIDEO.

SYMPTOMS

NO DISPLAY IN FLIP OUT LCD. -SOLUITION: FOUND Q837 ON DD BOARD BLOWN APART CAUSING NO 2.3VDC TO RGB DECODER.

RESOLUTION

We continue to investigate this issue

DCRTRV310. Subject NO MENU SCROLL FUNCTION.

SYMPTOMS

UNABLE TO SCROLL TO NEXT SCREE N ON MENU WITH SEL/EXECUTE BUTTON.

RESOLUTION

FOUND POOR SOLDER ON S015 ON CF BOARD. RESOLDERED ALL OK.

DCRTRV310. Subject NO S-VIDEO.

SYMPTOMS

S VIDEO JACK NOT WORKING.

RESOLUTION

FOUND FLEX CABLE REF # 231 NOT PROPERLY SEATED IN CONNECT OR REF#CN101 ON PJ96 BOARD.

DCRTRV310. Subject NO S-VIDEO -COLD SOLDER ON J101 ON PJ-95 BD.

SYMPTOMS

FOUND THAT FLEXING VIDEO CABLE WOULD CAUSE SVIDEO TO CUT OUT.

RESOLUTION

PROBLEM WAS J101 ON PJ95 BOARD, HAVING COLD SOLDER CONNECTION. RESOLDERING J101 RESTORED NORMAL OPERATION.

DCRTRV310. Subject REEL ERRORS.

SYMPTOMS

UNIT HAD REEL ERRORS AND WOULD STOP IN PLAY FF AND REWIND GIVING EJECT SYMBOL.

RESOLUTION

FOUND FLEX REF#762, PT#165778613 TORN AT SOLDER JOINTS ON LS ASSY CAUED BY REF#225 BEING CRINKLED.

DCRTRV310. Subject UNIT STUCK IN STD MODE / WON'T RECORD.

SYMPTOMS

Camera stuck in STD mode. Won't record. PB is OK

RESOLUTION

Japan told us the problem may be caused by a jittery REC/STBY SW.

DCRTRV310. Subject VF STAYS ON.

SYMPTOMS

WILL NOT TURN OFF VIEWFINDER.

RESOLUTION

REPAIRED BAD CONTACT CN911.

DCRTRV310. Subject WON'T LOAD DATA ON NEW BOARD - CHANGE DATA PAGE D FF TO 00.

SYMPTOMS

If data on page D address 10 is ff on the new board you cannot download data from radar.

RESOLUTION

Change the data to 00 and it works. You can also try initializing all pages using the RM-95.

DCRTRV310. Subject WON'T PLAY D8 TAPES - REINSTALLED PAGE C DATA.

SYMPTOMS

UNIT WOULD NOT PLAYBACK A D8 RECORDING. IT WOULD NOT RECOGNIZE D8 FORMAT AND WOULD SWITCH BACK AND FORTH FROM HI8 TO D8 MODE.

RESOLUTION

FOUND PROBLEM WITH PAGE C DATA AND REPLACED DATA TO REPAIR UNIT TO NORMAL OPERATION.

DCRTRV310. Subject WILL NOT CHARGE BATTERY - REPL BAT TERMINAL BOARD.

SYMPTOMS

Unit would operate normally with the battery and the ac adapter, battery would not charge.

RESOLUTION

Open the lcd assy, turn the camers on with a known good battery attached. After 30 seconds, battery time remainder symbol will appear on screen.. Plug ac adapter in and battery remain symbol should turn off. If battery charge remain symbol continues  to stay on after the ac adapter has been connected this is an indication that the battery terminal board has failed. Replace with 169438411, ref #9.

DCRTRV310, DCRTRV510. Subject Keeps searching back and forth from reg8 to D8 when D8 Tape is used.

SYMPTOMS

Keeps searching back and forth from reg8 to D8 when D8 Tape is used

CAUSE

Tried loading page C from good working camera, also attempted the PLL F0 adjustment , these two usually fix most cameras with this problem.

RESOLUTION

Replaced IC103 on VC-213 board.

| 875208652 | IC103 | IC CXA2071R-T4 |
| --- | --- | --- |

| Model DCRTRV310: Features |  |
| --- | --- |
|  | MENU SYSTEM YES |
|  | IMAGE STABILIZ STEADY SHOT |
|  | AUTO FOCUS YES |
|  | MANUAL FOCUS MANUAL FOCUS RING |
|  | QUICK RECORD YES |
|  | AUDIO STEREO |
|  | MICROPHONE BUILT-IN |
|  | BUILT-IN SPEAKR YES |
|  | LUX 1.0 LUX, 0 LUX IN NIGHTSHOT |
|  | AUTO EXPOSURE YES |
|  | PROGRAM AE SPOTLIGHT, SOFT PORTRAIT, SPORTS, LOW LUX |
|  | PROGRAM AE BEACH & SKI, SUNSET & MOON, LANDSCAPE, |
|  | MANUAL BACKLIGT YES |
|  | VIEWFINDER BLACK & WHITE |
|  | VIEWFNDR METERS YES |
|  | LENS POWER ZOOM, 20 OPTICAL X 360 DIGITAL |
|  | LENS COVER LENS CAP |
|  | FILTER 37 MM DIA. |
|  | BEEP FUNCTION YES, ALSO MELODY |
|  | EDIT SEARCH YES |
|  | COUNTER ON PLAYBACK ON HI8/STANDARD 8 FORMAT |
|  | DATE/TIME REC YES |
|  | AUTO DATE YES |
|  | DATA CODE ONLY ON RECORDING MADE IN DIGITAL8 SYSTEM |
|  | FADER BLACK, MOSIAC |
|  | FADER FADE IN ONLY: OVERLAP, BOUNCE |
|  | DIGITAL NR YES |
|  | DIGITAL EFFECTS STILL, TRAIL, OLD MOVIE, SLOW SHUTTER |
|  | DIGITAL EFFECT FLASH, LUMI (LUMINANCKEY) |
|  | CRYSTAL CLEAR PLAYBACK ON LCD SCREEN |
|  | FORMAT DIGITAL8 SYSTEM |
|  | ZOOM 20X OPTITAL 360X DIGITAL |
|  | LCD SCREEN 3.5 INCH CLOLR |
|  | EARPHONE JACK YES |
|  | 5 SEC SNAP REC YES |
|  | ACCUPOWER SYS INFOLITHIUM BATTERY ACCUPOWER SYSTEM |
|  | 16:9 CINEMA MOD NO |
|  | INTEL ACCY SHOE YES |
|  | FOCUS RING MANUAL FOCUS RING |
|  | NIGHTSHOT FOR TOTAL DARKNESS |
|  | REMOTE CONTROL YES |
|  | PICTURE EFFECTS SEPIA, SLIM, STRETCH, PASTEL, MOSIAC |
|  | PICTURE EFFECTS NEGATIVE ART, SOLARIZE, BLACK & WHITE |
|  | TIME CODE TAPES RECORDED IN THE DIGITAL8 SYSTEM ONLY |

| Model DCRTRV310: Specifications |  |
| --- | --- |
| VID REC SYS | TWO ROTARY HEADS, HELICAL SCANNING |
| AUDIO REC SYSTM | ROTARY HEADS PCM SYSTEM 12/16 BIT |
| VIDEO SIGNAL | NTSC COLOR, EIA STANDARD |
| REC/PB TIME | 1 HOUR USING 120 MIN CASSETTE |
| FF/REW TIME | APPROXIMATELY 8 MIN USING 120 MIN CASSETTE |
| IMAGE DEVICE | 1/4 INCH CCD 460,000 PIXELS |
| VIEWFINDER | BLACK & WHITE |
| LENS | COMBINED POWE ZOOM LENS, FILTER DIA 37MM |
| COLOR TEMP. | AUTO |
| MINIM. ILLUMIN. | 1.0 LUX, 0 LUX IN NIGHTSHOT MODE |
| INPUT/OUT CONNE | IN/OUTPUT: S-VIDEO AUDIO/VIDEO, DV |
| EARPHONE JACK | STEREO MINIJACK |
| REMOTE JACK | LANC CONTROL STEREO MINI-MINIJACK |
| MIC JACK | MINIJACK .388MV LOW IMPEDANCE |
| POWER REQUIRE | 7.2 V BATTERY, 8.4V AC POWER ADAPTER |
| POWER CONSUMP | 4.2W SCREEN, 3.2W VIEWFINDER |
| OPERATING TEMP | 32F - 104F, 0C - 40C |
| STORAGE TEMP | -4F - +140F, -20C - +60C |
| DIMENSIONS | 4 3/8X4 1/4X8 5/8 (108X107X195MM) WXHXD |
| WEIGHT | 2 LB 6 OZ INC. BATTERY, CASSETTE, STRAP |
| RF OUTPUT | RF DC OUTPUT SPECIAL MINIJACK, DC 5V |
| VIDEO IN | PHONO JACK |
| VIDEO OUT | PHONO JACK |
| AUDIO IN | STEREO PHONO JACK |
| AUDIO OUT | STEREO PHONO JACK |
| CAMERA CONNECT | S-VIDEO, A/V JACKS, RF OUT |
| FOCAL DISTANCE | 5/32-2 7/8 IN WHEN CONVERTED TO 35MM STILL |
| S-VIDEO | INPUT OUTPUT |
| LANC CONTROL | STEREO MINI-MINIJACK |
| VIDEO IN/OUT | PHONO JACK |
| AUDIO IN/OUT | STEREO PHONO JACK |
| LCD PICTURE | COLOR |
| LCD DISPLAY | 3.5 INCHES |
| MIC JACK | STEREO MINI-MINIJACK |
| SPEAKER | BUILT IN |
| LASER LINK | YES |

DCRTRV315. Subject CORRECT PART FOR 3 LCD PANEL - CORRECT PART IS 180335921.

SYMPTOMS

Service manual incorrectly lists part for 2.5 panel for lcd901 3 version. This applies to second listing for lcd901, all models except TRV310.

RESOLUTION

Correct part is 180335921. Model ref. also CCDTRV210/203

DCRTRV315. Subject INCORRECT SPECS IN ACMO.

SYMPTOMS

Acmo screen lists DCRTRV315 with a b & w viewfinder, this is not correct it has a color viewfinder.

RESOLUTION

We continue to investigate this issue

DCRTRV315. Subject CAN'T ID TAPE 8MM /D8 - REPLACED POWER SUPPLY.

SYMPTOMS

When loading a tape the unit could not determine what type of tape you were trying to play. It cycles between 8mm and d8 and will not play.

RESOLUTION

Found the power supply caused this problem replaced supply returned unit to proper operations.

| A7073922A | NA | POWER BOARD |
| --- | --- | --- |

DCRTRV315. Subject Distorted low audio on analog 8 playback and in EE mode. D8 playback was OK - Replace C-728 P/N113525911..

SYMPTOMS

Distorted low audio on analog 8 playback and in EE mode. D8 playback was OK.

RESOLUTION

Replace C-728 P/N 113525911.

DCRTRV315. Subject No drum start, will not load tape - Replace C-451 P/N 112577711..

SYMPTOMS

No drum start, will not load tape - Replace C-451 P/N 112577711.

CAUSE

No drum start, will not load tape.

RESOLUTION

Replace C-451 P/N 112577711.

DCRTRV315. Subject Unit cycles from D8 to analog 8 when a D8 recording is played back - Replace IC103 P/N 875208652..

SYMPTOMS

Unit cycles from D8 to analog 8 when a D8 recording is played back.

RESOLUTION

Replace IC103 P/N 875208652.

DCRTRV315. Subject Dead, no power in any mode - Replaced IC-801 P/N 875958954..

SYMPTOMS

Dead, no power in any mode.

RESOLUTION

Replaced IC-801 P/N 875958954.

DCRTRV315. Subject No analog 8 playback video - Replaced IC-601 P/N 875957981..

SYMPTOMS

No analog 8 playback video.

RESOLUTION

Replaced IC-601 P/N 875957981.

DCRTRV315. Subject No analog 8 playback video - Replace X-601 P/N 178130921..

SYMPTOMS

No analog 8 playback video.

RESOLUTION

Replace X-601 P/N 178130921.

DCRTRV315. Subject Unit had no functions and could not access data on page C or D.

SYMPTOMS

Unit had no functions and could not access data on page C or D.

RESOLUTION

Replace IC-401 P/N 875957629.

DCRTRV315. Subject No power . With RM-95 write protect can be released but cannot access or change data on any page and cannot initialize data..

SYMPTOMS

No power . With RM-95 write protect can be released but cannot access or change data on any page and cannot initialize data - Replace R-407 P/N 121895311.

CAUSE

No power . With RM-95 write protect can be released but cannot access or change data on any page and cannot initialize data.

RESOLUTION

Replace R-407 P/N 121895311.

DCRTRV315. Subject No EE video just green.

SYMPTOMS

No EE video just green screen - Replace C-520 P/N 112777711.

CAUSE

No EE video just green screen.

RESOLUTION

Replace C-520 P/N 112777711.

DCRTRV315. Subject No zoom or focus functions.

SYMPTOMS

No zoom or focus functions.

RESOLUTION

Replace IC-251 P/N 875957834.

DCRTRV315. Subject No D8 playback video and no DV.

SYMPTOMS

No D8 playback video and no DV.

RESOLUTION

Replace IC-301 P/N 875958436.

DCRTRV315. Subject Continues to switch from D8 to analog 8 when playing a D8 recording..

SYMPTOMS

Continues to switch from D8 to analog 8 when playing a D8 recording.

RESOLUTION

Replaced C-103 P/N 875208652.

DCRTRV315. Subject No color on analog 8 playback..

SYMPTOMS

No color on analog 8 playback.

RESOLUTION

Replace Damaged R-274 P/N 121896711.

DCRTRV315. Subject Dot noise in E-E video.

SYMPTOMS

Dot noise in E-E video.

RESOLUTION

Replaced IC-251 P/N 875957834.

DCRTRV315. Subject No focus functions. E61:00 error.

SYMPTOMS

No focus functions. E61:00 error.

RESOLUTION

Replace IC-551 P/N 87599231.

DCRTRV315. Subject No audio.

SYMPTOMS

No audio.

RESOLUTION

Replace C-711 P/N 111975011.

DCRTRV315. Subject Unit would power off immediately after power-up.

SYMPTOMS

Unit would power off immediately after power-up.

RESOLUTION

Replaced Camera control IC 902 P/N 875957924.

| DCRTRV315 |  |
| --- | --- |
| Subject No analog 8 playback - Replace IC-201 P/N 875207531. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Powers up and then shuts down after a minute of operation - Replace IC-401 P/N 875957629. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Vertical line on left side of picture in camera ee mode - Replace IC-251 P/N 875957834. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Intermittent drum start - Replaced C-479 P/N 110781911. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Record start/stop button does not function - Replace R-815 P/N 121898511. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No power and cannot access data - Replace IC-801 875958954. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No drum rotation - Resolder lifted R-425 on VC board. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No playback of analog or D8 tapes - REPLACE IC-401 P/N 875957629. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Intermittent shut down with T- reel errors stored in playback or record - Replace IC-452 P/N 875932761. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No DV out. (Sony IC-303) - Replace IC-303 P/N 875239806. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No D8 playback audio - Replace C-302 P/N 116517611. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Intermittent dead when powered up with battery - Replaced D-802 P/N 871907302. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Low battery indicator flashing with fully charged battery - Replace IC-402(EEPROM) P/N 875944594. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Bright light would cause camera E-E video to become grainy - Downloaded data. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No composite video from RCA jacks - Replace R-679 P/N 121893511. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject E-E picture degrades and iris starts hunting - Replace IC-902 P/N 875957924. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Dot noise in E-E video - Replace C-519 P/N 116494311. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No i-Link video - Change data on page C address AD from 00 to 02 For TI IC-303. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No analog 8 playback Y only chroma - Resoldered IC-641. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No playback video D8 or analog - Replace IC-104 P/N 875958435. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Red blocks on playback of D8 recordings - Replace IC-302 P/N 875239736. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No playback analog or D-8. Erases tape during playback - Replaced C-144 P/N 116487411 |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Color dot noise on playback - Replaced IC-301 P/N 875958436. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Bars and digital noise on playback - Replaced IC-351 P/N 875958466. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No power - Replaced C-821 P/N 110781911 |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Low analog 8 playback audio - Replaced C-785 P/N 110998211. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Red screen - Replaced IC-352 P/N 875958386. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Vertical roll on playback of anolog 8 recordings - Replaced IC-601 P/N 875957981. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Static in audio from DV out in camera E-E mode and from analog out on playback of D8 recordings - Replaced IC-301 P/N 875958436. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Buzzing in E-E audio on left channel with tape in and drum motor running - Replace IC-301 P/N 875958436. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No remote control operation - Replace X802 P/N 176045821. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Grainy camera E-E picture - Performed AGC gain adj. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Blue screen in pause and cue and review - Performed PLL FO ADJ. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No playback RF - Replace IC-102 P/N 875208653. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No focus or zoom - Replace IC-902 P/N 875957924. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Unit had a horizontal line from composite out on playback and from DV out in EE mode - Replace IC-351 P/N 875958466. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No power. Unit will power up with RM-95 but cannot access or initialize data - Replace IC-401 P/N 875957629. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No E-E video - Replace IC-401 P/N 875957629. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No DV or composite video on playback of D8 and anolog 8 tapes or in camera EE mode - Replace open FB501 P/N 150023822. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Blocking on playback in composite video and blocking in DV in camera E-E mode - Replace IC-301 P/N 875958436. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No playback of self recordings. Prerecorded tapes playback OK - Replace IC-102 P/N 875208653. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No camera E-E video and distorted chroma on analog 8 playback - Replaced IC-502 P/N 875957081. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Solarized camera E-E video - Replace IC-502 P/N 875957081. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject No capstan phase lock, capstan runs fast - Replace Q-453 P/N 872903753. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Unit switches from D8 to analog 8 when a D8 recording is played back. Poor solder connection on FB101- Resolder FB101 on VC-213 board. |  |

| DCRTRV315 |  |
| --- | --- |
| Subject Solarized E-E video - Replace C-529 P/N 112577711. |  |

| Model DCRTRV315: Features |  |
| --- | --- |
|  | MENU SYSTEM YES |
|  | IMAGE STABILIZ STEADY SHOT |
|  | AUTO FOCUS YES |
|  | MANUAL FOCUS MANUAL FOCUS RING |
|  | QUICK RECORD YES |
|  | AUDIO STEREO |
|  | MICROPHONE BUILT-IN |
|  | BUILT-IN SPEAKR YES |
|  | LUX 1.0 LUX, 0 LUX IN NIGHTSHOT |
|  | AUTO EXPOSURE YES |
|  | PROGRAM AE SPOTLIGHT, SOFT PORTRAIT, SPORTS, LOW LUX |
|  | PROGRAM AE BEACH & SKI, SUNSET & MOON, LANDSCAPE, |
|  | MANUAL BACKLIGT YES |
|  | VIEWFINDER COLOR |
|  | VIEWFNDR METERS YES |
|  | LENS POWER ZOOM, 20 OPTICAL X 360 DIGITAL |
|  | LENS COVER LENS CAP |
|  | FILTER 37 MM DIA. |
|  | BEEP FUNCTION YES, ALSO MELODY |
|  | EDIT SEARCH YES |
|  | COUNTER ON PLAYBACK ON HI8/STANDARD 8 FORMAT |
|  | DATE/TIME REC YES |
|  | AUTO DATE YES |
|  | DATA CODE ONLY ON RECORDING MADE IN DIGITAL 8 SYSTEM |
|  | FADER BLACK, MOSAIC |
|  | FADER FADE IN ONLY: OVERLAP, BOUNCE |
|  | DIGITAL NR YES |
|  | DIGITAL EFFECTS STILL, TRAIL, OLD MOVIE, SLOW SHUTTER |
|  | DIGITAL EFFECTS FLASH, LUMI (LUMINANCKEY) |
|  | CRYSTAL CLEAR PLAYBACK ON LCD SCREEN |
|  | FORMAT DIGITAL8 SYSTEM |
|  | ZOOM 20X OPTICAL 360X DIGITAL |
|  | LCD SCREEN 3.0 INCH COLOR |
|  | EARPHONE JACK YES |
|  | 5 SEC SNAP REC YES |
|  | ACCUPOWER SYS INFOLITHIUM BATTERY ACCUPOWER SYSTEM |
|  | 16:9 CINEMA MOD YES |
|  | INTEL ACCY SHOE YES |
|  | FOCUS RING ANUAL FOCUS RING |
|  | NIGHTSHOT YES |
|  | REMOTE CONTROL YESS |
|  | PICTURE EFFECTS SEPIA, SLIM, STRETCH, PASTEL, MOSIAC |
|  | PICTURE EFFECTS NEGATIVE ART, SOLARIZE, BLACK & WHITE |
|  | TIME CODE TAPES RECORDED IN THE DIGITAL8 SYSTEM ONLY |

| Model DCRTRV315: Specifications |  |
| --- | --- |
| VID REC SYS | TWO ROTARY HEADS, HELICAL SCANNING |
| AUDIO REC SYSTM | ROTARY HEADS PCM SYSTEM 12/16 BIT |
| VIDEO SIGNAL | NTSC COLOR, EIA STANDARD |
| REC/PB TIME | 1 HOUR USING 120 MIN CASSETTE |
| FF/REW TIME | APPROXIMATELY 8 MIN USING 120 MIN CASSETTE |
| IMAGE DEVICE | 1/4 INCH CCD 460,000 PIXELS |
| VIEWFINDER | COLOR |
| LENS | COMBINED POWER ZOOM LENS, FILTER DIA 37MM |
| COLOR TEMP. | AUTO |
| MINIM. ILLUMIN. | 1.0 LUX, 0 LUX IN NIGHTSHOT MODE |
| INPUT/OUT CONNE | IN/OUTPUT: S-VIDEO AUDIO/VIDEO, DV |
| EARPHONE JACK | STEREO MINIJACK |
| REMOTE JACK | LANC CONTROL STEREO MINI-MINIJACK |
| MIC JACK | MINIJACK .388MV LOW IMPEDANCE |
| POWER REQUIRE | 7.2 V BATTERY, 8.4V AC POWER ADAPTER |
| POWER CONSUMP | 4.2W SCREEN, 3.2W VIEWFINDER |
| OPERATING TEMP | 32F - 104F, 0C - 40C |
| STORAGE TEMP | -4F - +140F, -20C - +60C |
| DIMENSIONS | 4 3/8X4 1/4X7 3/4 (108X107X217MM) W/H/D |
| WEIGHT | 2 LB 6 OZ INC. BATTERY, CASSETTE, STRAP |
| RF OUTPUT | RF DC OUTPUT SPECIAL MINIJACK, DC 5V |
| VIDEO IN | PHONO JACK |
| VIDEO OUT | PHONO JACK |
| AUDIO IN | STEREO PHONO JACK |
| AUDIO OUT | STEREO PHONO JACK |
| CAMERA CONNECT | S-VIDEO, A/V JACKS, RF OUT |
| FOCAL DISTANCE | 5/32-2 7/8 IN WHEN CONVERTED TO 35MM STILL |
| S-VIDEO | INPUT OUTPUT |
| LANC CONTROL | STEREO MINI-MINIJACK |
| VIDEO IN/OUT | PHONO JACK |
| AUDIO IN/OUT | STEREO PHONO JACK |
| LCD PICTURE | COLOR |
| LCD DISPLAY | 3.0 INCHES |
| MIC JACK | STEREO MINI-MINIJACK |
| SPEAKER | BUILT IN |
| LASER LINK | YES |

DCRTR7000. Subject CAUTION WHEN REPLACING VC213 BOARD; (DV IC HAS 2 TYPES).

SYMPTOMS

There are two types of DV Interface IC303 on the VC-213 boards. One is a Sony and the other is TI. Both manufacturers are labled on the top of the IC. Whenever the board is replaced, the proper page C, address AD data must be installed. Incorrect  data will result in NO DV IN/OUT. As most facilities do not have the VC-213 board layout diagram, I will discribe the IC303's location. IC303 is the TGA IC located next to the white 26 pin connector. This connector attaches to flex FP-78 which feeds  analog and digital A/V to the PJ and SE boards.

RESOLUTION

When IC303 is Sony install data 00 in page C, address AD. When IC303 is TI install data 02.

DCRTR7000. Subject DD TO VC EXTENDER INCORRECT; ORDER J6082456A.

SYMPTOMS

Page 5-8 of s/m shows to use part J6082439a as extension between cn914 of vc213 board and cn931 of dd117 board. the connectors on the circuit board are narrower than on the cable. This model does not have the same size connectors as similar boards for  other models covered in same manual. Could you please identify and supply correct part # for extension cables used on this modes.

RESOLUTION

Oder J6082456A

Thanks Carl. I have been working on this for a few months. The part number is now registered and availability should be November '99

DCRTR7000. Subject FLASHING BLUE PIX IN CAM MODE, PB GOOD; RESEAT CCD CN501.

SYMPTOMS

UNIT CAME IN WITH A FLASHING LIGHT BLUE SCREEN IN CAMERA MODE, PLAYBACK OK. FOUND CCD CONNECTOR CN501 WAS PUSHED SIDWAYS (TOWARD THE FRONT OF THE UNIT) THE SMALL CLEARANCE BETWEEN THE EVF CABINET ASSY (#352) ALLOWED THE CABNET TO PUSH ON THE CONNECTOR.

RESOLUTION

RESEAT CCD IMAGE FLEX IN CN501

DCRTR7000. Subject INT C:31 IN REWIND MODE; REPLACE CAPSTAN.

SYMPTOMS

IN REWIND MODE DESPLAYS ERROR CODE (C:31) PROBLEMS OCCURS MOST OF THE TIME. THE MECH WONT LOAD OR EJECT TAPE PROPERLY.

RESOLUTION

REPLACED CAPTAN MOTOR.

| 883563501 |  | CAPSTAN |
| --- | --- | --- |

DCRTR7000. Subject NO EE AUDIO; RESEAT FP76 INTO CN300 ON THE MA BD..

SYMPTOMS

NO AUDIO IN E/E. FOUND FP76 BD NOT COMPLETELY SEATED IN CN300 OF THE MA BD.

RESOLUTION

RESEAT FP76 INTO CN300 ON THE MA BD.

DCRTR7000. Subject NO VIDEO COMP OUT;EF, S-VID, LCD GOOD; REPL R101 ON PJ BD..

SYMPTOMS

There will be no/intermittent video on the composite output. The EVF, S-Video & Side Panel LCD all have good video. This is a series inductor directly before the composite output jack.

RESOLUTION

Replace R101 on the PJ-98 Board; Japan will replace inductor with 0 ohm resistor.

| 150028311 | R101 | INDUCTOR, FERRITE BEAD |
| --- | --- | --- |

| DCRTR7000 |  |
| --- | --- |
| Subject NO/DARK PICTURE IN EVF; REPLACE VF-126 BD. |  |

DCRTR7000. Subject SHUTS DOWN / DRUM FG ERROR W/ ANALOG 8 OR D8 TAPE.

SYMPTOMS

Unit shuts down and giveS Drum FG Error with Reg or D8 Tape FG were at the IC's on the main Cba

RESOLUTION

Replaced VC-213 Cba A7097281A.

DCRTR7000. Subject WILL NOT EJECT - ERROR C:31:00; REPL LOADING MOTOR.

SYMPTOMS

This is the second dcrtr7000 with this same problem. The camcorder will power up in cam/vtr modes and after a few seconds beeps and shows error c31:00. During this interval checking the load motor terminals we found bias being applied with no  rotatin from the motor. Inspection of the chassis showed it was free to move up and down when driven by hand.

RESOLUTION

Replacement of loading motor resolved this problem in both cases.

| DCRTR7000 |  |
| --- | --- |
| Subject E-E video top half picture solarized bottom half blue screen - Replaced C-514 P/N 112577711. |  |

| DCRTR7000 |  |
| --- | --- |
| Subject No composite out video - Replace C-650 P/N 110998211. |  |

| Model DCRTR7000: Features |  |
| --- | --- |
|  | ZOOM LENS 20X OPTICAL, 360 DIGITAL |
|  | STEADYSHOT YES |
|  | AE MODES SPOTLIGHT,PORTRAIT,SUNSET,LANDSCAPE |
|  | ACCUPOWER SYST. YES |
|  | LASERLINK YES |
|  | PHOTE MODE YES |
|  | DIGITAL FADER MOSAIC,BOUNCE,OVERLAP |
|  | DATA CODE YES |
|  | PICTURE EFFECTS SEPIA,NEG.ART,B&W,SOLARIZE,SLIM,PASTEL |
|  | CONTROL L YES |
|  | 16:9 RECORDING YES, VIEWABLE IN 16:9 ONLY ON CAMERA |
|  | ACCESORY SHOE INTELLIGENT ACCESSORY SHOE |
|  | DIGITAL EFFECTS STILL,FLASH,LUMINANCE,TRAIL,SLOW,OLD |
|  | EDIT/SEARCH YES |
|  | SLOW SHUTTER YES |
|  | TRIPOD RECEPTACLE ONLY |
|  | BACK LIGHT YES |
|  | I LINK YES |
|  | S VIDEO YES |

| Model DCRTR7000: Specifications |  |
| --- | --- |
| VIDEO SYSTEM | 2 ROTARY HEADS |
| AUDIO SYSTEM | ROTARY HEADS, PCM |
| VIDEO SIGNAL | NTSC COLOR |
| REC/PLAYBACK | 1 HR |
| FF/REWIND | APPROX. 8 MIN |
| IMAGE DEVICE | 1/4 INCH CCD |
| LENS | 20 OPTICAL, 360 DIGITAL |
| FOCAL DISTANCE | 5/32 - 2 7/8 INCH |
| COLOUR TEMP. | AUTO |
| LUX RATING | 1 LUX, 0 LUX WITH NIGHT SHOT |
| S VIDEO OUT | S VIDEO OUT/IN |
| AUDIO OUT | IN/OUT |
| VIDEO OUT | IN/OUT |
| DV OUTPUT | DV INPUT/OUTPUT 4 PIN CONNECTOR |
| HEADPHONES JACK | STEREO MINIJACK |
| MIC INPUT | MINIJACK |
| LANC JACK | STEREO MINI-MINIJACK |
| POWER | 7.2V BATTERY, 8.4V AC ADAPTOR |
| DIMENSIONS | 4 X 4 1/4 X 7 3/4 W,H,D |
| MASS | 1 LB 11 OZ |

Model DCR300, DCRIP220, DCRIP45, DCRIP5, DCRIP55, DCRIP7, DCRPC1, DCRPC10, DCRPC100, DCRPC101, DCRPC110, DCRPC115, DCRPC120, DCRPC120, DCRPC3, DCRPC5, DCRPC7, DCRPC9, DCRSC100, DCRTR7000, DCRTR720, DCRTRV10, DCRTRV103, DCRTRV11, DCRTRV110, DCRTRV120, DCRTRV130, DCRTRV140, DCRTRV15, DCRTRV16, DCRTRV17, DCRTRV18, DCRTRV20, DCRTRV203, DCRTRV210, DCRTRV220, DCRTRV230, DCRTRV240, DCRTRV25, DCRTRV27, DCRTRV27, DCRTRV30, DCRTRV310, DCRTRV315, DCRTRV320, DCRTRV330, DCRTRV340, DCRTRV350, DCRTRV40, DCRTRV410, DCRTRV5, DCRTRV50, DCRTRV510, DCRTRV520, DCRTRV525, DCRTRV530, DCRTRV6, DCRTRV7, DCRTRV720, DCRTRV730, DCRTRV740, DCRTRV8, DCRTRV820, DCRTRV830, DCRTRV840, DCRTRV9, DCRTRV900, DCRTRV950, DCRVX1000, DCRVX2000, DCRVX700, DCRVX9000, DHR1000, DVMCDA1, DVMCDA2, DVMCMS1, GVD1000, GVD200, GVD300, GVD800, GVD900, GVD900E. Subject No iLink communication, iLink IC damaged by excessive external voltage..

SYMPTOMS

There is no communication between the camera and the PC via the iLink connection. The iLink IC is damaged internally, and sometimes there is damage to the IC that can be seen on the top surface of the IC.

CAUSE

This symptom and damage are caused by excessive voltage being applied to the iLink IC from an outside source (the outside source would typically be the computer that the camera is connected to). The damage would occur when the iLink cable is connected  to a 6-pin iLink connector on the computer. These 6-pin connectors on the computer have voltages of approx. 18 Vdc that do not normally appear at the 4-pin end of the cable that plugs into the camera. Following are several examples of how the excessive  voltage might appear at the 4-pin end of the cable, resulting in damage to the iLink IC:
1- The 6-pin end of the cable is plugged in backwards into the computer. Once a cable is forced into the connector backwards just one time, the keying  characteristic of the connector may become compromised which allows the cable to be plugged in backwards easily the next time.

2- 6-pin connector on computer may be loose internally, allowing improper contact to be made with voltage sources when the  cable is plugged-in, moved, or wiggled (may be intermittent).

3- Cable has short circuit at 6-pin connector end (may be intermittent).

4- The cable may have had extra length spliced into it and the workmanship may be poor.

RESOLUTION

Estimate the repair cost and advise the customer that the camera was damaged by excessive voltage from outside the camera, and that the repair will not be covered under warranty. If the customer accepts the estimate, explain to him that the conditions  that caused the damage probably still exist in the customer's environment, and that the repaired camera or deck will likely be damaged again, perhaps immediately, if the situation that caused the damage is not found and corrected before connecting the  camera to the computer again.