This disk contains:

- AWC.EXE Arbitrary Waveform Composer and associated files

- LABVWZIP.EXE self-extracting archive of LabVIEW drivers

The following are notes about using AWC and extracting the LabView drivers:

Arbitrary Waveform Composer (AWC) Notes Version 1.10

- In the following notes the term DS34X refers to both the SRS model DS340

and the SRS model DS345.

- Please Read the "AWC Software" section in the DS34X manual before

attempting to use AWC.

- Sample files are included on the AWC program disk.  The files with

.wav extension are waveform files that can be loaded directly

from the AWC "Send Data -> Waveform" submenu.  The files with the .asc

extension are command files that contain GPIB commands from the

abridged command list.  These files can be loaded with the

"Send Data -> ASCII" submenu to preset the DS34X to a certain state.

Both types of files are ASCII files and editable with a text editor.

- When reading or storing files in AWC, make sure that those files are

in the same directory as AWC.EXE.

- AWC uses its own directory (where awc.exe resides) to save

temporary images of screens.  If you run AWC from the disk

it is shipped with, you will run out of disk space and

get an error ( Err# 105 ) when exiting the program.  One way

to prevent this is to run AWC from a hard disk.

- If you are creating your own waveform file, you need to specify

the DS34X trigger source code on the third line of the file. The

trigger source can be any integer number from 0 to 6. Please refer to

the listing below for the trigger source you might need.

0 = Line Trigger

1 = P.C. mouse

2 = Internal Rate

3 = - Slope External

4 = - Slope External

5 = DS34X Keypad

6 = None (Continuous output)

- If you are using a Capital Equipment Corporation (CEC) GPIB

controller card, make sure the card's firmware is rev Q or later.

- If you are using a National GPIB controller card, make sure the

ibconf timeout is set for 100 seconds when loading large waveforms

(2500 points or more).

- If you are running AWC on a PC AT, or 386, the addition of a

math coprocessor is highly recommended.  A coprocessor could increase

the speed up to 10 fold!

- Even though AWC can be used with the keyboard, it is highly recommended

that a mouse be used instead.

- When operating on large waveforms (5000 point or higher),  AWC will

run slow especially if no coprocessor is available.  Floating point

math on a 16300 point waveform can take a few minutes on a PC/AT.

- A 2400 baud rate selection has been added to AWC.

*****************************************************************************

National Instruments LabVIEW Driver Notes

Included on this disk are LabVIEW Drivers for the DS335, DS340, and DS345.

These drivers can be used with National Instruments' LabVIEW program on

the Apple Macintosh, IBM PC and clones, and Sun workstations. The files

are in a self-extracting ZIP file called "LABVWZIP.EXE". To unzip this

file, put the floppy disk in your floppy disk drive (e.g. a: or b:), then go

to the directory on your hard disk where you want the files to be.

Type "a:labvwzip" (or "b:labvwzip") and the drivers will be unzipped and

copied into your current directory.

NOTE:   If you try to unzip the files while the current drive is the floppy

with the ZIP file on it, you will get a disk full error. You must

first change drives to your hard disk or to a floppy drive with a

blank disk.

*****************************************************************************