SUBCOURSE	EDITION

AV0991	A

US ARMY AVIATION CENTER

NAVIGATIONAL AIDS

THIS SUBCOURSE HAS BEEN REVIEWED FOR

OPERATIONS SECURITY CONSIDERATIONS.

NAVIGATIONAL AIDS

Subcourse Number AV0991*

EDITION A

United States Army Aviation Center

Fort Rucker, Alabama 36362-5255

1 Credit Hour

Edition Date: June 1993

SUBCOURSE OVERVIEW

This correspondence course provides a basic knowledge of navigation aids (NAVAIDs).

There are five prerequisites that must be met before you begin studying the material in this subcourse:

AV0986	Airport Traffic Control Procedures

AV0987	Flight Rules

AV0988	Communications Operating Procedures

AV0989	Flight Assistance

AV0990	Aviation Weather

This subcourse reflects the current doctrine when it was prepared.  In your own work, always refer to the latest publications.

Unless otherwise stated, the masculine gender of singular pronouns refers to men and women.

TERMINAL LEARNING OBJECTIVE

ACTION: 	Describe the relationship between air traffic control (ATC) and NAVAIDs.

CONDITION: 	You will use the material in this correspondence course.

STANDARD: 	To prove competency of this task, you must achieve a minimum of 70 percent on the examination.

*Supersedes Aviation Subcourse 0991, Edition 8.

AV0991

TABLE OF CONTENTS

Section 	Page

Administrative Instructions 		iv

Grading and Certification Instructions		iv

Lesson:	NAVIGATIONAL AIDS		1

OVERVIEW 		1

INTRODUCTION		1

EXTRACT REVIEW		3

Practice Exercise		5

Answer Key and Feedback		6

Appendix:  Publication Extracts		7

Airman's Information Manual	October 1987

FAA Handbook 7110.65	April 1987,

with Change 3

Use the above publication extracts to take this correspondence course.  When we wrote this correspondence course, these were the current publications.  In your own work, always refer to the latest publications.

i	AV0991

LESSON

NAVIGATIONAL AIDS

Task:  NA

OVERVIEW

TASK DESCRIPTION

In this lesson you will learn the relationship between air traffic control (ATC) and navigational aids (NAVAIDs).

LEARNING OBJECTIVE

ACTIONS: 	Describe the relationship between ATC and NAVAIDs.

CONDITIONS: 	You will be given extracts from the Federal Aviation Administration Handbook (FAAH) 7110.65 and Airman's Information Manual (AIM).

STANDARDS:	You must describe the relationship between ATC and NAVAIDs according to the extracts provided.

REFERENCES:	The material in this lesson was derived from the following publications:

AIM, October 1987

FAAH 7110.65, April 1987

INTRODUCTION

Having a basic knowledge of navigational principles and electronic aids to navigation will enable the controller to relate more closely to the pilot concerning aircraft navigation and its relationship to ATC.  Comprehension of how pilots navigate, as well as a good basic knowledge of the instruments and methods used in air navigation, are requirements to function as a full-performance level controller.

1	AV0991

TURN TO THE APPENDIX AND STUDY THE EXTRACTS.  WHEN YOU HAVE COMPLETED STUDYING THE EXTRACTS, BEGIN THE EXTRACT REVIEW ON THE NEXT PAGE.

WRITE YOUR RESPONSES TO THE QUESTIONS OR STATEMENTS ON EACH RIGHT-HAND (ODD-NUMBERED) PAGE BEFORE CHECKING YOUR RESPONSES WITH THE ANSWERS ON THE FOLLOWING LEFT-HAND (EVEN-NUMBERED) PAGE.  THEN CONTINUE WITH THE NEXT PAGE OF QUESTIONS.

2	AV0991

EXTRACT REVIEW

1.  A nondirectional radio beacon (NDB) transmits nondirectional signals whereby the pilot of a properly equipped aircraft can determine the aircraft's ____________ and ___________ on the station.

2.  NDBs are subject to disturbances that may result in erroneous bearing information.  This is usually caused by ______________ or ________________.

3.  A VHF omnidirectional range (VOR) is a ground based electronic NAVAID transmitting VHF navigational signals 360 degrees in azimuth, orientated from ____________________.

4.  The basic restriction to the use of VOR NAVAIDs is _______________________.

5.  Tactical Air Navigation (TACAN) is a UHF electronic air NAVAID that provides suitably equipped aircraft with a continuous indication of ____________ and ___________ to the TACAN station.

6.  TACAN is a pulse system and operate in the __________ band of frequencies.

7.  TACANs are specifically designed for the use of ___________ and ___________________ aircraft.

8.  The letters DME mean ____________________________________.

9.  A VOR TACAN is a NAVAID that provides VOR azimuth, TACAN azimuth, and TACAN ________ at one site.

10.  An instrument landing system (ILS) approach is classified as a ____________________ approach.

11.  The purpose of marker beacons in conjunction with the ILS is to determine __________________.

3	AV0991

1.	bearing, home  (AIM, paragraph 2a)

2.	lightning, precipitation static (AIM, paragraph 2d)

3.	magnetic north  (FAAH 7110.65, Appendix A, VOR/VERY HIGH FREQUENCY OMNIDIRECTIONAL RANGE STATION)

4.	line-of-sight (AIM, paragraph 3a)

5.	bearing, distance  (FAAH 7110.65, Appendix A, TACTICAL AIR NAVIGATION/TACAN )

6.	UHF (AIM, paragraph 5b)

7.	military, naval forces (AIM, paragraph Sa)

8.	distance measuring equipment (AIM, paragraph 7)

9.	DME  (FAAH 7110.65, Appendix A, VORTAC/VHF OMNIDIRECTIONAL RANGE/TACTICAL AIR NAVIGATION)

10.	precision instrument  (FAAH 7110.65, Appendix A, INSTRUMENT LANDING SYSTEM/ILS)

11.	range information  (AIM, paragraph 12a(3)(b)

4	AV0991

LESSON

PRACTICE EXERCISE

The following items will test your grasp of the material covered in this lesson.  There is only one correct answer for each item.  When you have completed the exercise, check your answers with the answer key that follows.  If you answer any item incorrectly, study again that part of the lesson which contains the portion involved.

1.	What is the basis for navigation within the National Airspace System?

A.	NDB

B.	VOR

C.	TACAN

D.	radar

2.	What are sectional charts designed for?

A.	clearance delivery

B.	visual navigation

C.	terminal control areas

D.	en route instrument navigation

3.	How many usable magnetic radials are available from a VOR?

A.	90

B.	180

C.	270

D.	360

4.	How many components does an ILS consist of?

A.	one

B.	three

C.	five

D.	six

5.	How many components does a microwave landing system (MLS) consist of?

A.	three

B.	five

C.	six

D.	seven

5	AV0991

LESSON

PRACTICE EXERCISE

ANSWER KEY AND FEEDBACK

Item	Correct Answer and Feedback

1.	B.  VOR

This NAVAID was selected because of its omnidirectional capability.  (FAAH 7110.65, Appendix A, VOR/VERY HIGH FREQUENCY OMNIDIRECTIONAL RANGE STATION)

2.	B.  visual navigation

The name of the charts is aeronautical which means distance is measured in nautical miles (FAAH 7110.65, Appendix A, AERONAUTICAL CHART 1)

3.	D.  360

A VOR is nondirectional; this means the signal is put out 360 degrees.  (FAAH 7110.65, Appendix A, VOR/VERY HIGH FREQUENCY OMNIDIRECTIONAL RANGE STATION)

4.	C.  five

This is the number of components required to make up an ILS system.  (AIM, paragraph 12a (2))

5.	A.  three

This is the number of components required to make up an MLS system.  (AIM, paragraph 14a (5))

6	AV0991