| #### CHAPTER 5 |  |
| --- | --- |
| ADDITIONAL SUBJECTS - LESSON OUTLINES |  |
| LESSON TITLE: | DRIVE A VEHICLE WITH BALANCED (PINTLE-CONNECTED) TRAILER |
| TASK NUMBER: | 551-721-1380 (Transport General Cargo in Trailer) and 551-721-1385 (Couple/Uncouple Pintle-Connected Trailer) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Drive a vehicle with balanced (pintle-connected) trailer. |
| --- | --- | --- |
|  | CONDITIONS | Given instruction, DD Form 1970, DA Form 2404, pencil, vehicle operator's manual, trailer operator's manual, equipment records folder, rags, lubricants, coolant, a suitable training area, a trailer, and a wheeled vehicle with BII. |
|  | STANDARD: | Without accident or injury, drive the designated route, connect and disconnect the trailer, back the trailer (in a straight line), and perform basic driving maneuvers. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Motor pool, training area, and driver training route.

3.  Training type:  Conference, demonstration, and practical exercise.

4.  Students:  Scheduled personnel.

5.  Principal and assistant instructors required:  One primary instructor for the conference and one assistant instructor for each three students for the demonstration and practical exercise.

6.  Training aids and equipment:  Rags, lubricants, coolant, DA Form 2404, DD Form 1970, pencil, 40 traffic cones or empty POL drums, vehicle operator's manual, trailer operator's manual, equipment records folder, and a trailer and a wheeled vehicle with BII for each three students.

7.  References:  FM 21-305, trailer operator's manual, and vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Practical exercise.

(3)  Summary.

2.  Explanation and demonstration:

NOTE:  The instructions contained in this lesson outline for a balanced (pintle-connected) trailer are in general terms because this lesson covers a variety of trailers.  Although procedures for these type trailers are similar, the instructor should consult the specific trailer operator's manual for detailed instructions.

a.  Connect the trailer to the vehicle.

(1)  Sound the horn before backing, and using ground guide(s), back the vehicle to the trailer ensuring no one stands between the vehicle and the trailer.

(2)  Stop the vehicle, engage the parking brake, shut off the engine, and get out of the vehicle to connect the trailer.

(3)  Lift the trailer.  (Some trailers are light and can be lifted by one person, while others will require several people, and still others have a mechanical lift.)  Secure the trailer lunette in the pintle of the towing vehicle.  Secure the pintle, and install the pintle safety cotter pin in the pintle.

(4)  Hook the two safety chains from the trailer to the rear lifting shackles of the towing vehicle.

(5)  Release the trailer hand brake lever.

(6)  Raise the retractable support (landing leg).

(7)  Connect the intervehicular brake hoses to the towing vehicle, and turn on the air supply.

(8)  Connect the intervehicular electrical cable to the towing vehicle.

(9)  Check the brake action and operation of all lights by operating the controls in the towing vehicle.

b.  Disconnect the trailer from the vehicle.

(1)  Position the trailer on level ground, if possible.

(2)  Apply the trailer hand brake or chock wheels to prevent movement of the trailer.

(3)  Disconnect the intervehicular brake hoses, and stow on the bracket provided on the trailer.

(4)  Lower the retractable support (landing leg).

(5)  Unhook the safety chains and lunette from the towing vehicle.

(6)  Secure the pintle, and reinstall the pintle safety cotter pin in the pintle.

(7)  Level trailer, if required.

c.  Drive the towing vehicle and trailer.  Keep in mind the overall length of the towing vehicle and trailer when passing other vehicles and turning.  Backing is also affected because the unit is hinged in the middle.

(1)  Safely towing a trailer requires increased concentration, alertness, and strict compliance with towing speeds.

(2)  Trailers have a tendency to fishtail or swerve very easily when the driver makes an erratic movement of any magnitude.  This can be caused by--

(a)  Speed.

(b)  A slight swerve to miss a pothole.

(c)  A quick lane change.

(d)  A slip of the hand on the steering wheel.

(e)  Rough or uneven road.

(3)  To prevent trailer fishtail or swerve--

(a)  Never exceed the recommended speed for the trailer that you are towing.

(b)  Never exceed the trailer's maximum recommended weight capacity.

(c)  Always slow down for curves, wet or rough road, or downgrades.

(d)  Anticipate that all stops will take longer as a result of the added weight and brake early and gently.  Do not slam on the brakes.

(e)  On downgrades, slow down before starting downhill.  Use a lower gear, and let the engine compression help slow the vehicle.

(4)  To correct trailer fishtail or swerve--

(a)  Steer straight ahead.

(b)  Gradually decelerate.

(c)  Do not brake until both the towing vehicle and trailer have stabilized.

(5)  When turning corners, allow for the fact that the trailer wheels turn inside the turning radius of the towing vehicle.

(a)  To make a right turn--

- Check traffic ahead, to the rear, and to the right side.

- If not in the right-hand lane, make sure that lane is clear, signal, and move into the lane well in advance of the turn.

- Signal for the turn at least 150 feet in advance, and reduce speed.

- Check other traffic that is in, at, or approaching the intersection.  (Make sure there is no bicycle or motorcycle to the right, and be especially alert for pedestrians.)

- Take a position farther from the curb or edge of the pavement than the driver would if driving a straight truck.

- Drive the towing vehicle about halfway into the intersection, and then cut sharply to the right.  This will keep the trailer wheels off the curb.

- Keep the vehicle close enough to the edge of the road to prevent following vehicles from attempting to pass on the right.

- If swinging wide to the left or oncoming lane, watch oncoming vehicles.

- During the turn, monitor the mirrors for off-tracking.

- Do not shift gears during the turn.  Keep both hands on the steering wheel.

- If unable to complete the turn, do not back up.

- After completing the turn, cancel the signal.

- When it is safe to do so, steer the vehicle into the desired lane (on four-lane roads).

(b)  To make a left turn--

- Check traffic ahead, to the rear, and to both sides.

- Signal for the turn at least 150 feet in advance and reduce speed.

- Check other traffic that is in, at, or approaching the intersection.

- Ensure there is an adequate gap to make a turn in front of traffic.

- Before turning, drive the towing vehicle into the center of the intersection to allow for the trailer.

- Turn hard to the left and watch for oncoming traffic, if applicable.

- During the turn, monitor the mirrors for off-tracking.

- Do not shift gears during the turn.  Keep both hands on the steering wheel.

- After completing the turn, cancel the signal.

- When it is safe to do so, steer the vehicle into the desired lane (on four-lane roads).

(6)  Stopping requires more distance when pulling a trailer.  Apply brakes gradually and smoothly.  Stepping on the brake pedal will stop both the towing vehicle and the trailer.

d.  Back the trailer (in a straight line).  Since the driver cannot see directly behind the vehicle, backing is always a dangerous maneuver.  Avoid backing whenever possible, even if it involves going around the block.  When backing, follow these rules:

(1)  Get out of the vehicle, and check the area to the sides, rear, and overhead for obstructions.

(2)  Use ground guide(s) to direct while backing, when possible.

(3)  Adjust the rear view mirrors before backing.

(4)  Sound the horn before backing where it is legal to do so.

(5)  Remember that when backing the rear of the trailer will move in the opposite direction from which the front towing vehicle wheels are turned.

(a)  If the wheels are turned to the right, the trailer will go left.

(b)  If the wheels are turned to the left, the trailer will go right.

(6)  Make gradual steering corrections in relation to trailer alignment.

(7)  Pull up, if necessary, to gain improved alignment of trailer.

(8)  Stop the vehicle when the desired position is achieved.

e.  Explain to the students that they will be required to perform before-during-, and after-operation PMCS on their assigned vehicle.

f.  Demonstrate hand and arm signals required for this exercise.

g.  Demonstrate driving through each maneuver.

3.  Practical exercise:

a.  Assign students to vehicles/trailers, and issue vehicle operator’s manuals, trailer operator's manuals, pencils, DA Form 2404, DD Form 1970, and the equipment records folder.   Instruct students on the location of rags, lubricants, and coolant.

b.  Students perform before-operation PMCS.

c.  Students practice maneuvering the vehicle through the courses laid out in the training area(s).  (Sample training areas are at Chapter 6, Figures 6-2 through 6-5 and 6-8).  They also conduct during-operation PMCS at this time.

d.  After the students have mastered driving the vehicle in the training area, they will practice driving on the road.

NOTE:  As each student practices driving, an assistant instructor rides in the right front seat.  The other two students will ride in the rear seats or troop seats and rotate driving duties.  The assistant instructor explains driving techniques, ensures the driver is aware of driving situations, and conducts after-action reviews with each driver.  Now is the time to pass on valuable experience and correct any bad driving habits.

e.  Students perform after-operation PMCS and ensure all operator entries required on DA Form 2404 and DD Form 1970 are accurate, complete, and legible.

4.  Evaluation:  Check each student's performance of PMCS and driving.

5.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

6.  Retraining:  Retrain No-Gos and slow learners.

E.  SAFETY RESTRICTIONS.

1.  Ensure all chock blocks (if required) are in place when vehicles are parked.

2.  Ensure the transmission is always placed in neutral (some automatics are placed in park), the parking brake is set, the engine is shut off, and the ignition key is removed (if equipped) before leaving the vehicle.

3.  Ensure students remove all jewelry and identification tags before performing PMCS.

4.  Ensure students pay particular attention to the cautions and warnings listed in the operator's manual.

5.  Ensure ground guide(s) is used when backing the vehicle or trailer during training.

6.  Ensure a safe following distance and speed are maintained when driving in the training area and the driver training route (as determined by the local command).

7.  Ensure all occupants wear seat belts (if equipped) while vehicle is in operation.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended instructional time is 8 hours (.5 conference, .5 demonstration, and 7.0 practical exercise, including 1.0 PMCS).

| LESSON TITLE: | DRIVE A WHEELED VEHICLE IN A CONVOY |
| --- | --- |
| TASK NUMBER: | 551-721-1359 (Drive Vehicle in a Convoy) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Drive a wheeled vehicle in a convoy. |
| --- | --- | --- |
|  | CONDITIONS | Given instruction, a DD Form 1970, a DA Form 2404, a pencil, vehicle operator's manual, equipment records folder, rags, lubricants, coolant, a suitable training area, and a wheeled vehicle with BII. |
|  | STANDARD: | Operate the vehicle in accordance with specific instructions of the march unit commander. Using defensive driving (accident avoidance) methods; maintain vehicle interval, obey highway warning and regulatory signs, interpret and relay all mechanical/ hand signals correctly, and use correct braking procedures without accident or injury. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Motor pool and convoy route.

3.  Training type:  Conference and practical exercise.

4.  Students:  Scheduled personnel.

5.  Principal and assistant instructors required:  One primary instructor for the conference and one assistant instructor for each three students for the practical exercise.

6.  Training aids and equipment:  Rags, lubricants, and coolant, DA Form 2404, DD Form 1970, pencil, vehicle operator's manual, equipment records folder, convoy strip map, a wheeled vehicle with BII for each three students, convoy signs ("CONVOY AHEAD" and "CONVOY FOLLOWS"), convoy flags (blue, green, and black/white), and convoy control vehicles (minimum of two vehicles required).  A communication system for the control vehicles is recommended.

7.  References:  FM 55-312, FM 21-305, and appropriate vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Practical exercise.

(3)  Summary.

2.  Explanation and demonstration:

a.  Give convoy safety briefing (see attached sample convoy commander's briefing).

b.  The students will be required to perform before-, during-, and after-operation PMCS on their assigned vehicle.

c.  Demonstrate hand and arm signals required for this exercise.

3.  Practical exercise:

a.  Assign students to vehicles and issue vehicle operator's manuals, pencils, DA Form 2404, DD Form 1970, convoy strip map, and equipment records folder.  Instruct students on the location of rags, lubricants, and coolant.

b.  Students perform before-operation PMCS.

c.  Students practice driving the vehicle on assigned convoy route.  During-operation PMCS is conducted at this time.

NOTE:  As each student practices driving, an assistant instructor rides in the right front seat.  The other two students will ride in the rear seats or troop seats and rotate driving duties.  The assistant instructor explains driving techniques, ensures the driver is aware of driving situations, and conducts after-action reviews with each driver.  Now is the time to pass on valuable experience and correct any bad driving habits.

d.  Students perform after-operation PMCS and ensure all operator entries required on DA Form 2404 and DD Form 1970 are accurate, complete, and legible.

4.  Evaluation:  Check each student's performance of PMCS and driving the vehicle in convoy.

5.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

6.  Retraining:  Retrain No-Gos and slow learners.

E.  SAFETY RESTRICTIONS.

1.  Ensure all chock blocks (if required) are in place when vehicles are parked.

2.  Ensure the transmission is always placed in neutral (some automatics are placed in park), the parking brake is set, the engine is shut off, and the ignition key is removed (if equipped) before leaving the vehicle.

3.  Ensure students remove all jewelry and identification tags before performing PMCS.

4.  Ensure students pay particular attention to the cautions and warnings listed in the operators manual.

5.  Ensure ground guide(s) is used when backing the vehicle during training.

6.  Ensure a safe following distance and speed are maintained when driving on the convoy route (as determined by the local command).

7.  Ensure all occupants wear seat belts (if equipped) while the vehicle is in operation.

8.  Ensure no one walks between vehicles parked in a column.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended instructional time is 5 hours (.5 conference and 4.5 practical exercise, including 1.0 PMCS).

SAMPLE CONVOY COMMANDER'S BRIEFING

1.  Always follow civilian and military police instructions when given.

2.  Use only truck parking areas on controlled access highways.

3.  Make only emergency halts on the roadside of controlled access highways.

4.  Do not stand on the traffic side of a convoy during halts on conventional highways.

5.  Perform vehicle operation maintenance and check cargo security at every halt.

6.  Move vehicles off of the highway before beginning maintenance.

7.  Have reflectors and warning devices in place before beginning maintenance.

8.  Use warning lights during periods of darkness or reduced visibility.

9.  Begin convoy movement only at the convoy commander's signal.

10.  Vehicle speed restrictions:  as determined by the local commander.

11.  Vehicle intervals (minimums):

a.  Controlled access highway - 200 yards.

b.  Rural conventional highway - 150 yards.

c.  Urban conventional highway - 50 yards.

12.  Maintain a close interval until reaching main convoy route.

13.  Use the acceleration lane, when available, to reach convoy speed.

14.  Gradually attain the proper vehicle interval once on the main convoy route.

15.  In the case of an accident, the main column does not stop to provide assistance.  The next following vehicle provides immediate assistance to the accident vehicle.

16.  If an accident occurs to a vehicle ahead, make the maximum effort to clear traffic lanes.

17.  Operate all vehicles with headlights on at all times.

18.  Use warning devices correctly.

19.  Add any additional comments as local conditions warrant.

| LESSON TITLE: | DRIVE A WHEELED VEHICLE IN A CONVOY UNDER BLACKOUT CONDITIONS |
| --- | --- |
| TASK NUMBER: | 551-721-1363 (Drive Vehicle in Blackout Conditions) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Drive a wheeled vehicle in a convoy under blackout conditions. |
| --- | --- | --- |
|  | CONDITIONS | Given instruction, DD Form 1970, DA Form 2404, pencil, vehicle operator's manual, equipment records folder, rags, lubricants, coolant, a suitable training area, and a wheeled vehicle with BII. |
|  | STANDARD: | Operate the vehicle in accordance with specific instructions of the march unit commander. Drive the designated convoy route at night using defensive driving (accident avoidance) methods, maintain vehicle interval, and operate the tactical light switch, including headlights and blackout drive without accident or injury. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Motor pool and convoy route.

3.  Training type:  Conference and practical exercise.

4.  Students:  Scheduled personnel.

5.  Principal and assistant instructors required:  One primary instructor for the conference and one assistant instructor for each three students for the practical exercise.

6.  Training aids and equipment:  Rags, lubricants, and coolant, DA Form 2404, DD Form 1970, pencil, vehicle operator's manual, equipment records folder, convoy strip map, a wheeled vehicle with BII for each three students, convoy signs ("CONVOY AHEAD" and "CONVOY FOLLOWS"), convoy flags (blue, green, and black/white), and convoy control vehicles (minimum of two vehicles required).  A communication system for the control vehicles is recommended.

7.  References:  FM 55-312, FM 21-305, and appropriate vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Practical exercise.

(3)  Summary.

2.  Explanation and demonstration:

a.  Demonstrate night convoy control signals, as listed in FM 21-305.

b.  Demonstrate the operation of the tactical light switch for turning on/off headlights, blackout drive lights, and blackout marker lights.

c.  Have three vehicles positioned with blackout drive lights on so the students can see the rear blackout marker and stop light at different distances:

(1)  Too far - more than 180 feet.

(2)  Proper distance - between 60 and 180 feet.

(3)  Too close - less than 60 feet.

d.  Give convoy safety briefing (see attached sample convoy commander's briefing).

3.  Practical exercise:

a.  Assign students to vehicles and issue vehicle operator's manual, pencils, DA Form 2404, DD Form 1970, convoy strip map, and equipment records folder.  Instruct students on the location of rags, lubricants, and coolant.

b.  Students perform before-operation PMCS to include the operation and cleaning of all lights.

c.  Students practice driving the vehicle first with headlights on and then under blackout conditions on the assigned convoy route.  During-operation PMCS is conducted at this time.

NOTE:  As each student practices driving, an assistant instructor rides in the right front seat.  The other two students will ride in the rear seats or troop seats and rotate driving duties.  The assistant instructor explains driving techniques, ensures the driver is aware of driving situations, and conducts after-action reviews with each driver.  Now is the time to pass on valuable experience and correct any bad driving habits.

d.  Students perform after-operation PMCS and ensure all operator entries required on DA Form 2404 and DD Form 1970 are accurate, complete, and legible.

4.  Evaluation:  Check each student's performance of night driving both with headlights and blackout drive.

5.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

6.  Retraining:  Retrain No-Gos and slow learners.

E.  SAFETY RESTRICTIONS.

1.  Ensure all chock blocks (if required) are in place when vehicles are parked.

2.  Ensure the transmission is always placed in neutral (some automatics are placed in park), the parking brake is set, the engine is shut off, and the ignition key is removed (if equipped) before leaving the vehicle.

3.  Ensure students remove all jewelry and identification tags before performing PMCS.

4.  Ensure students pay particular attention to the cautions and warnings listed in the operator's manual.

5.  Ensure ground guide(s) is used when backing the vehicle during training.

6.  Ensure a safe following distance and speed are maintained when driving on the convoy route (as determined by the local command).

7.  Ensure all occupants wear seat belts (if equipped) while vehicle is in operation.

8.  Ensure no one walks between vehicles parked in a column.

9.  Ensure ground guides and road guides wear reflective vests and carry filtered flashlights at night.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended instructional time is 5 hours (.5 conference and 4.5 practical exercise, including 1.0 PMCS).

SAMPLE CONVOY COMMANDER'S BRIEFING

1.  Always follow civilian and military police instructions when given.

2.  Use only truck parking areas on controlled access highways.

3.  Make only emergency halts on the roadside of controlled access highways.

4.  Do not stand on the traffic side of a convoy during halts on conventional highways.

5.  Perform vehicle operation maintenance and check cargo security at every halt.

6.  Move vehicles off of the highway before beginning maintenance.

7.  Have reflectors and warning devices in place before beginning maintenance.

8.  Use warning lights during periods of darkness or reduced visibility.

9.  Begin convoy movement only at the convoy commander's signal.

10.  Vehicle speed restrictions:  as determined by the local commander.

11.  Vehicle intervals (minimums):

a.  Controlled access highway - 200 yards.

b.  Rural conventional highway - 150 yards.

c.  Urban conventional highway - 50 yards.

d.  Blackout conditions - 60 to 180 feet.

12.  Maintain close interval until reaching the main convoy route.

13.  Use the acceleration lane, when available to reach convoy speed.

14.  Gradually attain proper vehicle interval once on the main convoy route.

15.  In case of an accident, the main column does not stop to provide assistance.  The next following vehicle provides immediate assistance to the accident vehicle.

16.  If an accident occurs to a vehicle ahead, make the maximum effort to clear traffic lanes.

17.  Operate all vehicles with headlights on at all times (except during blackout conditions).

18.  Use warning devices correctly.

19.  Add any additional comments as local conditions warrant.