SUBCOURSE	EDITION

IT 0306 	A

US ARMY INTELLIGENCE CENTER

BORDER COORDINATES

BORDER COORDINATES

Subcourse Number IT 0306

EDITION A

US ARMY INTELLIGENCE CENTER

FORT HUACHUCA, AZ 85613-6000

4 Credit Hours

Edition Date: MAY 1995

SUBCOURSE OVERVIEW

This subcourse is designed to help you in the construction of a DF plotting facility by explaining how to use border coordinate tables.  When you have finished this subcourse you will know how to get and use border coordinate tables for your own plotting facility.  Map GT5a and border coordinate tables are supplied with this course.  A working surface, masking tape, a very long straight edge, three different colored pens and paper will be needed for the successful completion of this subcourse.  All materials should be retained until you are notified of the successful completion of the subcourse.

There are no prerequisites for this subcourse, however it is assumed you have already finished a comprehensive map reading course.

This subcourse reflects the doctrine which was current at the time it was prepared.  In your own work situation, always refer to the latest official publications.

Unless otherwise stated, the masculine gender of singular pronouns is used to refer to both men and women.

TERMINAL LEARNING OBJECTIVE

ACTION:	You will request and read border coordinate tables for the map you wish to use in constructing your DF plotting facility, and identify the materials required for the DF plotting facility.

CONDITION:	You will use the information provided in this subcourse.

STANDARD:	To demonstrate competency of this task, you must achieve a minimum of 70% on the subcourse examination.

i	IT 0306

TABLE OF CONTENTS

Section	Page

Subcourse Overview

Lesson 1: Explanation	1-1

Introduction	1-1

Lesson 2: Practice Exercise	2-1

IT 0306	ii

LESSON 1

EXPLANATION

Critical Task: 301-794-1307

OVERVIEW

LESSON DESCRIPTION:

Upon completion of this lesson you will be able to show your ability to request and read border coordinate tables for the map you wish to use in constructing your DF plotting facility.

TERMINAL LEARNING OBJECTIVE:

ACTION:	Request and read border coordinate tables for the map you wish to use in constructing your DF facility.

CONDITION:	Given the information and materials provided in this subcourse.  A working surface, masking tape, a very long straight edge, three different colored pens and paper will be needed as additional materials.

STANDARD:	To demonstrate competency of this task, you must achieve a minimum of 70% on the subcourse examination.

INTRODUCTION

The use of border coordinate tables is considered the most accurate method of plotting DF bearings on a gnomonic projection.  The ability of the plotter to use border coordinate tables directly affects the value of the strategic direction finding net.

The system of border coordinate is based on the fact that on a gnomonic projection all great circles are represented by straight lines.  Therefore, a line bearing, which of course is a great circle, may be represented by a straight line drawn between an two border points on the edge of the map (See figure 1-1).

1-1	IT 0306

Figure 1-1.  Points on edge of the map.

IT 0306	1-2

Naturally, just picking two arbitrary points doesn't do us much good, so we have to have two important items.

First, a method of identifying the particular spots on the edge of the map that are the points where the great circle path enters or leaves the map.  Second, and equally important, we need to know where the DF site is which is going to report the bearing we want to identify.  Let's start with identifying the border coordinates first.

Border coordinates are constructed by equally dividing the edges of a gnomonic projection into 100 major increments.  These divisions are numbered clockwise from the upper left hand corner.  The maps are constructed so that the divisions are numbered from 00 to 30 across the top, 30 to 50 down the right border, 50 to 80 across the bottom, and 80 to 00 (100) up the left border (See figure 1-2).

Figure 1-2.  Division of numbers.

1-3	IT 0306

Each border coordinate's value in the table furnished consists of four numbers.  These numbers may be located on the map edge as follows: (Refer to gnomonic projection furnished).

The first two numbers indicate the major increments, all of which are numbered form 00 to 99 clockwise around the map.  Find 73 on your map.

Each major increment is divided into 10 minor increments on the map in much the same way as a ruler is divided into fractions of an inch.  These minor increments are not numbered on the map, but they are indicated on the map by tick marks.  The third number in the border coordinate refers to one of these tick marks.  Find 733 on the map.

Each of the minor increments can be visually interpolated into tenths.  If the map projection is large enough, you may even be able to find some physical means of measuring these sub-increments, such as an engineers scale or map protractor.  This interpolation equates to the fourth number in the border coordinate value.  Find 7332 on your map.  Since there are 1000 increments (major and minor) actually indicated on the projection, this interpolation divides the edge of the map into 10,000 increments.  With these 10,000 increments we can pretty well pick out any point on the edge of the map and identify it.  So now we have a map of an area in which we wish to make a DF survey.  First, we make sure that we are actually using a gnomonic projection, and second, we find out the exact geographic coordinates of the DF sites we wish to use.  Aha! You think we've goofed! You are going to use a DF site which doesn't appear on the map! Surprise! That doesn't matter! Simply send the number of the map projection and the exact geographic coordinates of the DF site to the National Security Agency (NSA).  Sit back and wait.  NSA will return to you a border coordinate table for each and every DF site you listed.  If the DF site does not appear on the map you will receive border coordinates for only that portion of the circle which actually falls within the map borders.

Each border coordinate table is made up of four column of figures.  The first and second columns list all bearings that can be reported (0-359 for any site on the map).  The third and fourth columns list the points on the edge of the map where this great circle path crosses the edge of the map.  This line will always cross the DF site if the site is on the map.  A straight line connecting these two values will represent the great circle path for the bearing and its reciprocal.  The bearing and its reciprocal can both be found by laying the straight edge across the two points, so it should not really bother you to discover that somewhere in the course of developing the tables the column relationship reverses, so that column one and three start out equating as do two and four, while by the time the entire table is complete, columns one and four equate, as do two and three.  What this means in terms of this lesson is that for site A the columns for 124.5 304.5 read 6105 9999 and immediately under that is 125.0 305.0 reading 0015 6122.  This should not confuse

IT 0306	1-4

you.  The computer lists the lower number first despite the fact that you might not be using both coordinates for some reason.  Also, the print-out does not show the initial zero in any number, so that the 00 coordinate might be one or two digits instead of four, e.g., 15 instead of 0015.  As long as you use both coordinates to establish your map markings, you should have no trouble.

The advantages of the border coordinate tables should be obvious.  NSA uses machine computation to develop the table and human error is almost eliminated.  The disadvantages are slowness and laboriousness in converting to usable bearings.  But constructing the DF plotting facility can eliminate these drawbacks! We simply use the tables once, to construct extended edge coordinates, and then forget about them! At least we forget about them until we either change maps or move a DF site.

1-5	IT 0306

LESSON 2

PRACTICE EXERCISE

Critical Tasks: 301-794-1307

OVERVIEW

LESSON DESCRIPTION:

In this lesson you will accurately place two DF sites and mark the extended edge coordinates for each site using the gnomonic projection and border coordinate tables furnished.

TERMINAL LEARNING OBJECTIVE:

ACTION:	Accurately place two DF sites and mark the extended edge coordinates for each site using 'the gnomonic projection and border coordinate tables furnished.

CONDITION:	You will use the information and materials provided in this subcourse, including Map GT5a and the border coordinates tables.  Additionally materials needed are a working surface, masking tape, a very long straight edge, three different colored pens and paper.

STANDARD:	To demonstrate competency of this task, you must achieve a minimum of 70 % on the subcourse examination.

EXERCISE

Step 1: Provide yourself with a flat surface large enough to lay the map out completely and allow at least three inches of working space on all sides.  This means a space slightly more than 28 x 20 inches.

Step 2: Cover the space with white paper, completely smooth and fasten it securely with masking tape.  If you do not have a single sheet large enough to do the job, cover only the outside edges at the rectangle, do not overlap sheets of paper in the area where the map will be placed.

Step 3: Place the map in the center of the prepared area, flatten it out - if necessary, for a "really smooth" map iron it.  Being extremely CAREFUL NOT TO COVER ANY NUMBER ON THE MAP, fasten the map securely with tape.

2-1	IT 0306

Step 4: Now look at your coordinate tables.  The first thing you will notice is that after carefully explaining to you that the tables are arranged in four columns, we supplied you with a table bearing 12 columns.  But do not be fooled.  It is simply four columns arranged in sets of three.  This particular set of coordinates is in half degrees.  You may not need half degrees, or conversely you may want tenths.  The good old computer does not care.  Because of the size of map, you will be using in this lesson, for most of the lesson you will use only full degree markings.

Step 5: Gather your additional materials.  You will need a straight edge at least 36 inches long, two different colored pencils - (we will assume red and blue although they do not have to be these colors), - a few thumb tacks or map tacks, a pencil sharpener, and your coordinate tables.

Step 6: At least 1 inch outside the edge of your map, draw a straight line on all four sides of the map forming a rectangle.  Use the color you have chosen for your first site.  We will assume it is red, but you could have used any color, as long as everything associated with the first site is the same color.

Step 7: One inch around the first rectangle, draw another, use blue (or whatever color will be associated with the second site).

Step 8: Now we have to physically locate our sites.  Going back to the tables, we find site "A" has bearing coordinates of 0 and 180 degrees at 1118 and 7332.  We have already located 7332, but just as a refresher, find 73, count off 3 tick marks clockwise, mentally divide the space between the third and fourth tick mark into tenths and mark off two, 7332.  Simple? Do it with 1118, and check your results with figure 2-1.

Figure 2-1.  1118 and 7332.

IT 0306	2-2

We carefully place our straight edge so that it touches both our reference points.  We double check to make sure we are using the same edge of the ruler or straight edge at both ends many maps have been ruined using the wrong edge at one end.  When the straight edge is aligned with both tick marks, the azimuth is laid out.  The simplest, safest, and most accurate method is to hold a pencil on each edge coordinate and gently place the straight edge against both pencils.  It takes four hands.  Without moving the straight edge, mark a one inch line, along the edge outside the reference line for site "A" at each end.  Also lightly mark the approximate locations of the site on the map (Somewhere in England).  Having done this for 0-180 we now do the same thing for 90-270 using 4403 and 9240 as our reference points.  Where the two lines intersect on the map is our site location.  If we were marking a permanent plotting facility, we might drill a hole through the map (table and, all) at this point, so we could pass a string through it.

You should now have a map with the exact site location marked with a cross (+), and two rectangles around the map perimeter (one red, one blue ) - each one inch apart.

When producing the border coordinate scales, we will need a method of identifying the degrees.  Numbering each degree line would be very difficult because of the small distance between degree lines.  Therefore, we will mark each 10 degrees with a line one inch in length, each 5 degrees with a line one-half inch in length, and each degree in-between with a line one-quarter inch in length.  (NOTE: If we needed to use half degree marks, we would use a line one-eighth inch in length.)  Then we number only the 10 degree lines (the lines one inch in length).  Put all the degree marks for each site inside the color coded rectangle that corresponds to that particular site.

If we need half-degrees, we use eighth inch lines or increase the relative sizes of the other lines if space permits.  For speed in reading, we number the 10 degree markings.  After we have marked the lines as far as we need, we use a colored pencil to mark each line so it is readily identifiable as belonging to that particular site.

Step 9: Mark site "A" bearings from 0 to (360) WEST to 3450 in degrees, and from 180 EAST to 1650 in half degrees.  It is probably obvious to you that either type of line could be entered all the way around the map, and equally obvious that the half degree markings become difficult to read at this small a scale.

Step 10: Locate site "B" on the map (near the Adriatic Sea) and mark the reference line in degrees from 250 to 260 and in half degrees from 260 to 270 (Site "B" 0-180 1568 6349).

2-3	IT 0306

Save the map, you will need it for the examination.  The remainder of the lesson concerns nice to know information.  At this point, you are ready to use border coordinates in your own work.

After your map has been prepared for all the sites in your net, you may want to bring the borders into the edge of a map, hiding the edge coordinates from view.  To do this, it is only necessary to put the original lines far enough from the edge of the map to enable you to put a collar around the map inside the original markings.  Then replace the straight edge so that is corresponds to the marked edge coordinates and mark the collar in the same fashion as the original rectangle was marked.  If this is done carefully, the new edge marking will be as accurate as the old, but the edge coordinate markings and any marginal information not needed will be out of view.

Many different methods have been devised to keep the string taunt on the plotting board, all of which can be used.  Perhaps the simplest is to fasten a weight to the end of the string which passes through a hole in the map and allows the weight to dangle below the map.  The other end of the string is attached to a magnet which is placed on a metal strip on the edge of the plotting board.  Such things as self winding reels, pulleys, weights, cork board and tacks have been successfully used.  The best string is probably a black nylon fishing line, which is small (diameter) and lightweight, also strong and relatively inexpensive.  Black elastic thread may also be used, which might eliminate the need to drill holes in the board.

IT 0306	2-4

SITE A

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

0	180	1118	7332	30	210	1785	8070	60	240	3291	8761

0.5	0.5	1127	7342	0.5	0.5	1801	8087	0.5	0.5	3311	8770

1	181	1135	7352	31	211	1817	8104	61	241	3331	8778

0.5	0.5	1144	7361	0.5	0.5	1833	8120	0.5	0.5	3350	8786

2	182	1153	7371	32	212	1849	8136	62	242	3370	8795

0.5	0.5	1162	7381	0.5	0.5	1865	8151	0.5	0.5	3389	8803

3	183	1171	7391	33	213	1882	8167	63	243	3409	8812

0.5	0.5	1180	7401	0.5	0.5	1899	8182	0.5	0.5	3428	8820

4	184	1190	7411	34	214	1917	8197	64	244	3447	8828

0.5	0.5	1199	7421	0.5	0.5	1935	8211	0.5	0.5	3466	8836

5	185	1208	7431	35	215	1953	8226	65	245	3485	8844

0.5	0.5	1217	7441	0.5	0.5	1971	8240	0.5	0.5	3504	8853

6	186	1227	7452	36	316	1990	8254	66	246	8523	8861

0.5	0.5	1236	7462	0.5	0.5	2009	8268	0.5	0.5	3541	8869

7	187	1245	7472	37	217	2029	8281	67	247	2560	8877

0.5	0.5	1255	7483	0.5	0.5	2049	8295	0.5	0.5	3579	8885

8	188	1264	7493	38	218	2069	8308	68	248	3597	8893

0.5	0.5	1274	7504	0.5	0.5	2090	8321	0.5	0.5	3616	8901

9	189	1284	7514	39	219	2111	8333	69	249	3634	8909

0.5	0.5	1293	7525	0.5	0.5	2133	8346	0.5	0.5	3653	8917

10	190	1303	7536	40	220	2155	8358	70	250	3671	8925

0.5	0.5	1313	7547	0.5	0.5	2178	8371	0.5	0.5	3690	8933

11	191	1323	7557	41	221	2201	8383	71	251	3708	8940

0.5	0.5	1333	7568	0.5	0.5	2225	8395	0.5	0.5	3726	8948

12	192	1343	7580	42	222	2250	8407	72	252	3744	8956

0.5	0.5	1353	7591	0.5	0.5	2275	8418	0.5	0.5	3763	8964

13	193	1364	7602	43	223	2300	8430	73	253	8781	8972

0.5	0.5	1374	7613	0.5	0.5	2326	8441	0.5	0.5	3799	8980

14	194	1384	7625	44	224	2353	8452	74	254	3817	8988

0.5	0.5	1395	7636	0.5	0.5	2381	8463	0.5	0.5	3835	8995

15	195	1405	7648	45	225	2409	8474	75	255	3853	9003

0.5	0.5	1416	7660	0.5	0.5	2439	8485	0.5	0.5	3871	9011

16	196	1427	7672	46	226	2469	8496	76	256	3890	9019

0.5	0.5	1438	7684	0.5	0.5	2499	8507	0.5	0.5	3908	9027

17	197	1449	7696	47	227	2531	8517	77	257	3926	9034

0.5	0.5	1460	7708	0.5	0.5	2564	8528	0.5	0.5	3944	9042

18	198	1471	7720	48	228	2598	8538	78	258	3962	9050

0.5	0.5	1483	7733	0.5	0.5	2632	8548	0.5	0.5	3980	9058

19	199	1494	7745	49	229	2668	8558	79	259	3998	9065

0.5	0.5	1506	7758	0.5	0.5	2705	8568	0.5	0.5	4016	9073

2-5	IT 0306

SITE A (continued)

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

20	200	1517	7771	50	230	2743	8578	80	260	4034	9081

0.5	0.5	1529	7784	0.5	0.5	2783	8588	0.5	0.5	4052	9089

21	201	1541	7797	51	231	2824	8598	81	261	4070	9097

0.5	0.5	1553	7810	0.5	0.5	2866	8607	0.5	0.5	4088	9104

22	202	1566	7824	52	232	2910	8617	82	262	4107	9112

0.5	0.5	1578	7J37	0.5	0.5	2956	8626	0.5	0.5	4125	9120

23	203	1591	7851	53	233	3001	8636	83	263	4143	9128

0.5	0.5	1603	7865	0.5	0.5	3023	8645	0.5	0.5	4161	9136

24	204	1616	7879	54	234	3044	8655	84	264	4180	9144

0.5	0.5	1629	7894	0.5	0.5	3066	8664	0.5	0.5	4198	9152

25	205	1642	7908	55	235	3087	8673	85	265	4216	9159

0.5	0.5	1656	7923	0.5	0.5	3108	8682	0.5	0.5	4235	9167

26	206	1669	7938	56	236	3129	8691	86	266	4253	9157

0.5	0.5	1683	7953	0.5	0.5	3150	8700	0.5	0.5	4272	9183

27	207	1697	7968	57	237	3170	8709	87	267	4290	9191

0.5	0.5	1711	7984	0.5	0.5	3191	8718	0.5	0.5	4309	9199

28	208	1726	7999	58	238	3211	8726	88	268	4327	9207

0.5	0.5	1740	8018	0.5	0.5	3231	8735	0.5	0.5	4346	9215

29	209	1755	8035	59	239	3251	8744	89	269	4365	9224

0.5	0.5	1770	8053	0.5	0.5	3271	8752	0.5	0.5	4384	9232

90	270	4403	9240	120	300	5935	9860	150	330	599	6763

0.5	0.5	4422	9248	0.5	0.5	5955	9874	0.5	0.5	608	6773

91	271	4441	9256	121	301	5975	9889	151	331	617	6783

0.5	0.5	4460	9264	0.5	0.5	5994	9904	0.5	0.5	627	6793

92	272	4479	9273	122	302	6014	9919	152	332	636	6803

0.5	0.5	4498	9281	0.5	0.5	6032	9935	0.5	0.5	645	6813

93	273	4518	9289	123	303	6051	9950	153	333	654	6823

0.5	0.5	4537	9298	0.5	0.5	6069	9966	0.5	0.5	663	6833

94	274	4557	9306	124	304	6087	9983	154	334	672	6843

0.5	0.5	4577	9315	0.5	0.5	6105	9999	0.5	0.5	681	6852

95	275	4597	9323	125	305	15	6122	155	335	689	6862

0.5	0.5	4616	9332	0.5	0.5	31	6139	0.5	0.5	698	6872

96	276	4636	9341	126	306	46	6156	156	336	707	6881

0.5	0.5	4657	9349	0.5	0.5	61	6172	0.5	0.5	716	6891

97	277	4677	9358	127	307	76	6188	157	337	725	6901

0.5	0.5	4697	9367	0.5	0.5	90	6204	0.5	0.5	733	6910

98	278	4718	9376	128	308	105	6220	158	338	742	6920

0.5	0.5	4739	9384	0.5	0.5	119	6236	0.5	0.5	751	6929

99	279	4759	9393	129	309	133	6251	159	339	759	6939

0.5	0.5	4780	9402	0.5	0.5	147	6266	0.5	0.5	768	6948

IT 0306	2-6

SITE A - Continued

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

100	280	4801	9412	130	310	160	6281	160	340	777	6958

0.5	0.5	4823	9421	0.5	0.5	174	6296	0.5	0.5	785	6967

101	281	4844	9430	131	311	187	6310	161	341	794	6977

0.5	0.5	4866	9439	0.5	0.5	200	6325	0.5	0.5	802	6986

102	282	4887	9449	132	312	213	6339	162	342	811	6995

0.5	0.5	4909	9458	0.5	0.5	225	6353	0.5	0.5	819	7005

103	283	4931	9468	133	313	238	6366	163	343	828	7014

0.5	0.5	4954	9477	0.5	0.5	250	6380	0.5	0.5	836	7023

104	284	4976	9487	134	314	263	6394	164	344	845	7033

0.5	0.5	4999	9497	0.5	0.5	275	6407	0.5	0.5	853	7042

105	285	5043	9506	135	315	287	6420	165	345	862	7051

0.5	0.5	5087	9516	0.5	0.5	299	6433	0.5	0.5	870	7060

106	286	5129	9526	136	316	310	6446	166	346	879	7070

0.5	0.5	5169	9537	0.5	0.5	322	6459	0.5	0.5	887	7079

107	287	5209	9547	137	317	333	6471	167	347	895	7088

0.5	0.5	5247	9557	0.5	0.5	345	6484	0.5	0.5	904	7097

108	288	5284	9568	138	318	356	6496	168	348	912	7107

0.5	0.5	5320	9578	0.5	0.5	367	6508	0.5	0.5	921	7116

109	289	5355	9589	139	319	378	6520	169	349	929	7125

0.5	0.5	5389	9600	0.5	0.5	389	6532	0.5	0.5	938	7134

110	290	5421	9611	140	320	400	6544	170	350	946	7144

0.5	0.5	5454	9622	0.5	0.5	411	6556	0.5	0.5	954	7153

111	291	5485	9633	141	321	421	6568	171	351	963	7162

0.5	0.5	5515	9644	0.5	0.5	432	6579	0.5	0.5	971	7171

112	292	5545	9655	142	322	442	6591	172	352	980	7181

0.5	0.5	5574	9667	0.5	0.5	452	6602	0.5	0.5	988	7190

113	293	5602	9679	143	323	463	6612	173	353	997	7199

0.5	0.5	5629	9690	0.5	0.5	473	6624	0.5	0.5	1005	7209

114	294	5656	9702	144	324	483	6635	174	354	1014	7218

0.5	0.5	5682	9715	0.5	0.5	493	6646	0.5	0.5	1022	7227

115	295	5708	9727	145	325	503	6657	175	355	1031	7237

0.5	0.5	5733	9739	0.5	0.5	513	6668	0.5	0.5	1039	7246

116	296	5757	9752	146	326	523	6679	176	356	1048	7256

0.5	0.5	5781	9765	0.5	0.5	532	6690	0.5	0.5	1057	7265

117	297	5804	9778	147	327	542	6700	177	357	1065	7275

0.5	0.5	5827	9791	0.5	0.5	552	6711	0.5	0.5	1074	7284

118	298	5850	9804	148	328	561	6721	178	358	1083	7294

0.5	0.5	5871	9818	0.5	0.5	571	6732	0.5	0.5	1091	7303

119	299	5893	9832	149	329	580	6742	179	359	1100	7313

0.5	0.5	5914	9846	0.5	0.5	590	6752	0.5	0.5	1109	7323

2-7	IT 0306

SITE B

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

0	180	1568	6349	30	210	2532	6486	60	240	3874	6746

0.5	0.5	1583	6352	0.5	0.5	2551	6489	0.5	0.5	3891	6753

1	181	1598	6354	31	211	2570	6492	61	241	3908	6761

0.5	0.5	1613	6356	0.5	0.5	2590	6495	0.5	0.5	3925	6769

2	182	1628	6358	32	212	2609	6497	62	242	3941	6778

0.5	0.5	1642	6360	0.5	0.5	2629	6500	0.5	0.5	3958	6786

3	183	1657	6362	33	213	2649	6503	63	243	3974	6795

0.5	0.5	1672	6364	0.5	0.5	2670	6506	0.5	0.5	3990	6804

4	184	1678	6366	34	214	2690	6509	64	244	4006	6814

0.5	0.5	1702	6368	0.5	0.5	2711	6512	0.5	0.5	4021	6824

5	185	1717	6370	35	215	2732	6515	65	245	4037	6834

0.5	0.5	1731	6373	0.5	0.5	2753	6518	0.5	0.5	4052	6844

6	186	1746	6375	36	216	2775	6521	66	246	4068	6855

0.5	0.5	1761	6377	0.5	0.5	2797	6524	0.5	0.5	4083	6867

7	187	1776	6379	37	217	2819	6527	67	247	4098	6878

0.5	0.5	1791	6381	0.5	0.5	2841	6530	0.5	0.5	4113	6890

8	188	1806	6383	38	218	2864	6534	68	248	4128	6903

0.5	0.5	1821	6385	0.5	0.5	2886	6537	0.5	0.5	4143	6916

9	189	1836	6387	39	219	2910	6540	69	249	4157	6930

0.5	0.5	1851	6390	0.5	0.5	2933	6543	0.5	0.5	4172	6944

10	190	1866	6392	40	220	2957	6547	70	250	4168	6959

0.5	0.5	1881	6394	0.5	0.5	2981	6550	0.5	0.5	4200	6975

11	191	1897	6396	41	221	3007	6554	71	251	4214	6991

0.5	0.5	1912	6398	0.5	0.5	3038	6557	0.5	0.5	4228	7008

12	192	1927	6400	42	222	3069	6561	72	252	4242	7326

0.5	0.5	1942	6403	0.5	0.5	3098	6564	0.5	0.5	4256	7044

13	193	1958	6405	43	223	3128	6568	73	253	4270	7064

0.5	0.5	1973	6407	0.5	0.5	3156	6572	0.5	0.5	4284	7085

14	194	1989	6409	44	224	3184	6576	74	254	4297	7107

0.5	0.5	2004	6411	0.5	0.5	3212	6580	0.5	0.5	4311	7130

15	195	2020	6414	45	225	3239	6583	75	255	4325	7154

0.5	0.5	2035	6416	0.5	0.5	3265	6587	0.5	0.5	4338	7180

16	196	2051	6418	46	226	3291	6591	76	256	4351	7207

0.5	0.5	2067	6420	0.5	0.5	3317	6596	0.5	0.5	4365	7236

17	197	2083	6423	47	227	3342	6600	77	257	4378	7267

0.5	0.5	2099	6425	0.5	0.5	3366	6604	0.5	0.5	4391	7301

18	198	2115	6427	48	228	3391	6608	78	258	4404	7336

0.5	0.5	2131	6429	0.5	0.5	3414	6613	0.5	0.5	4417	7374

19	199	2147	6432	49	229	3438	6-17	79	259	4430	7415

0.5	0.5	2163	6434	0.5	0.5	3461	6622	0.5	0.5	4443	7459

IT 0306	2-8

SITE B - Continued

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

20	000	2180	6436	50	230	3483	6627	80	260	4456	7507

0.5	0.5	2196	6439	0.5	0.5	3506	6631	0.5	0.5	4468	7559

21	201	2213	6441	51	231	3528	6636	81	261	4481	7616

0.5	3.5	2229	6443	0.5	0.5	3549	6641	0.5	0.5	4494	7679

22	202	2246	6446	52	232	3570	6646	82	262	4507	7747

0.5	0.5	2263	6448	0.5	0.5	3591	6652	0.5	0.5	4519	7823

23	203	2280	6451	53	233	3612	6657	83	263	4532	7907

0.5	0.5	2297	6453	0.5	0.5	3632	6662	0.5	0.5	4544	8000

24	204	2314	6455	54	234	3652	6668	84	264	4557	8015

0.5	0.5	2332	6458	0.5	0.5	3672	6673	0.5	0.5	4569	8030

25	205	2349	6460	55	235	3692	6679	85	265	4582	8045

0.5	0.5	2367	6463	0.5	0.5	3711	6685	0.5	0.5	4594	8060

26	206	2384	6465	56	236	3730	6691	86	266	4607	8075

0.5	0.5	2402	6468	0.5	0.5	3749	6697	0.5	0.5	4619	8090

27	207	2420	6471	57	237	3767	6704	87	267	4631	8105

0.5	0.5	2438	6473	0.5	0.5	3786	6710	0.5	0.5	4644	8120

28	208	2457	6476	58	238	3804	6717	88	268	4656	8134

0.5	0.5	2475	6478	0.5	0.5	3822	6724	0.5	0.5	4668	8149

29	209	2494	6481	59	239	3839	6731	89	269	4681	8146

0.5	0.5	2513	6484	0.5	0.5	3857	6733	0.5	0.5	4693	8179

90	270	4705	8194	120	300	5906	9149	150	330	568	6207

0.5	0.5	4718	8209	0.5	0.5	5915	9168	0.5	0.5	589	6210

91	271	4730	8223	121	301	5924	9187	151	331	609	6213

0.5	0.5	4742	8238	0.5	0.5	5933	9206	0.5	0.5	629	6216

92	272	4754	8253	122	302	5942	9225	152	332	649	6219

0.5	0.5	4766	8268	0.5	0.5	5950	9245	0.5	0.5	668	6221

93	273	4779	3282	123	303	5958	9265	153	333	688	6224

0.5	0.5	4791	8297	0.5	0.5	5966	9285	0.5	0.5	707	6230

94	274	4803	8312	124	304	5973	9305	154	334	726	6232

0.5	0.5	4815	8327	0.5	0.5	5981	9325	0.5	0.5	745	6232

95	275	4828	8341	125	305	5988	9346	155	335	764	6235

0.5	0.5	4840	8356	0.5	3.5	5995	9367	0.5	0.5	782	6238

96	276	4852	8371	126	306	6001	9388	156	336	801	6240

0.5	0.5	4865	8386	0.5	0.5	6008	9410	0.5	0.5	819	6243

97	277	4877	8401	127	307	6014	9431	157	337	837	6245

0.5	0.5	4889	3416	0.5	0.5	6021	9453	0.5	0.5	855	6248

98	278	4902	8430	128	308	6027	9475	158	338	873	6250

0.5	0.5	4914	8445	0.5	0.5	6033	9498	0.5	0.5	890	6253

99	279	4926	8460	129	309	6039	9521	159	339	908	6255

0.5	0.5	4939	8475	0.5	0.5	6044	9544	0.5	0.5	925	6258

2-9	IT 0306

SITE B - Continued

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

100	280	4951	8490	130	310	6050	9567	160	340	942	6260

0.5	0.5	4964	8505	0.5	0.5	6055	9591	0.5	0.5	959	6263

101	281	4976	8520	131	311	6061	9615	161	341	976	6265

0.5	0.5	4989	8536	0.5	0.5	6066	9639	0.5	0.5	993	6268

102	282	5009	8551	132	312	6071	9664	162	342	1010	6270

0,5	0.5	5074	8566	0.5	0.5	6076	9689	0.5	0.5	1027	6272

103	283	5133	8581	133	313	6081	9714	163	343	1043	6275

0.5	0.5	5187	8596	0.5	0.5	6086	9740	0.5	0.5	1060	6277

104	284	5236	8612	134	314	6091	9766	164	344	1076	6279

0.5	0.5	5282	8672	0.5	0.5	6095	9793	0.5	0.5	1093	6282

105	285	5324	8643	135	315	6100	9820	165	345	1109	6284

0.5	0.5	5363	8658	0.5	0.5	6104	9847	0.5	0.5	1125	6286

106	286	5400	8674	136	316	6109	9875	166	346	1141	6289

0.5	0.5	5434	8689	0.5	0.5	6113	9903	0.5	0.5	1157	6291

107	287	5466	8705	137	317	6117	9932	167	347	1173	6293

0.5	0.5	5496	8721	0.5	0.5	6121	9962	0.5	0.5	1189	6295

108	288	5524	8737	138	318	6125	9992	168	348	1205	6298

0.5	0.5	5550	8753	0.5	0.5	20	6129	0.5	0.5	1220	6300

109	289	5575	8769	139	319	48	6133	169	349	1236	6302

0.5	0.5	5599	8785	0.5	0.5	75	6137	0.5	0.5	1251	6304

110	290	5621	8801	140	320	102	6141	170	350	1267	6307

0.5	0.5	5642	8817	0.5	0.5	129	6145	0.5	0.5	1282	6309

111	291	5662	8834	141	321	155	6149	171	351	1298	6311

0.5	0.5	5681	8850	0.5	0.5	181	6152	0.5	0.5	1313	6313

112	292	5699	8867	142	322	206	6156	172	352	1328	6315

0.5	0.5	5717	8884	0.5	0.5	231	6159	0.5	0.5	1344	6317

113	293	5733	8900	143	323	256	6163	173	353	1359	6320

0.5	0.5	5749	8917	0.5	0.5	280	6166	0.5	0.5	1374	6322

114	294	5764	8934	144	324	304	6170	174	354	1389	6324

0.5	0.5	5779	8951	0.5	0.5	327	6173	0.5	0.5	1404	6326

115	295	5793	8969	145	325	351	6176	175	344	1419	6328

0.5	0.5	5806	8986	0.5	0.5	374	6180	0.5	0.5	1434	6330

116	296	5819	9003	146	326	396	6183	176	356	1449	6332

0.5	0.5	5831	9021	0.5	0.5	419	6186	0.5	0.5	1464	6335

117	297	5843	9038	147	327	441	6189	177	357	1479	6337

0.5	0.5	5885	9057	0.5	0.5	463	6192	0.5	0.5	1494	6339

118	298	5866	9057	148	328	484	6195	178	358	1509	6341

0.5	0.5	5876	9093	0.5	0.5	506	6198	0.5	0.5	1524	6343

119	299	5887	9112	149	329	527	6201	179	359	1539	6345

0.5	0.5	5897	9130	0.5	0.5	548	6204	0.5	0.5	1553	6347

IT 0306	2-10

SITE C

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

0	180	1859	5716	30	210	2764	5883	60	240	4009	6142

0.5	0.5	1874	5718	0.5	0.5	2780	3886	0.5	0.5	4021	6149

1	181	1889	5721	31	211	2797	5889	61	241	4034	6156

0.5	0.5	1904	5724	0.5	0.5	2814	5892	0.5	0.5	4046	6163

2	182	1919	1727	32	212	2831	5895	62	242	4057	6170

0.5	0.5	1933	5729	0.5	0.5	2848	5899	0.5	0.5	4069	6177

3	183	1948	5732	33	213	2866	5902	63	243	4081	6185

0.5	0.5	1963	5735	0.5	0.5	2883	5905	0.5	0.5	4092	6192

4	184	1978	5738	34	214	2901	5908	64	244	4103	6200

0.5	0.5	1993	5740	0.5	0.5	2918	5912	0.3	0.5	4114	6208

5	185	2007	5743	35	215	2936	5915	65	245	4125	6217

0.5	0.5	2022	5746	0.5	0.5	2954	5918	0.5	0.5	4136	6225

6	186	2037	5748	36	216	2972	5922	66	246	4147	6234

0.5	0.5	2051	5751	0.5	0.5	2990	5925	0.5	0.5	4157	6243

7	187	2066	5754	37	217	3019	5928	67	247	4167	6252

0.5	0.5	2080	5757	0.5	0.5	3058	5932	0.5	0.5	4178	6261

8	188	2095	5759	38	218	3094	5935	68	248	4188	6271

0.5	0.5	2110	5762	0.5	0.5	3130	5939	0.5	0.5	4198	6231

9	198	2124	5765	39	219	3164	5942	69	249	4208	6291

0.5	0.5	2139	5767	0.5	0.5	3198	5946	0.5	0.5	4217	6302

10	190	2153	5770	40	220	3230	5949	70	250	4227	6313

0.5	0.5	2168	5773	0.5	0.5	3261	5953	0.5	0.5	4237	6324

11	191	2183	5775	41	221	3291	5957	71	251	4246	6336

0.5	0.5	2197	5778	0.5	0.5	3220	5961	0.5	0.5	4255	6348

12	192	2212	5781	42	222	3348	5964	72	252	4265	6360

0.5	0.5	2227	5784	0.5	0.5	2276	5968	0.5	0.5	4274	6373

13	193	2241	5786	43	223	3402	5972	73	253	4283	6386

0.5	0.5	2256	5789	0.5	0.5	3428	5976	0.5	0.5	4292	6400

14	194	2270	5792	44	224	3453	5980	74	254	4301	6414

0.5	0.5	2285	5794	0.5	0.5	3477	5934	0.5	0.5	4310	6429

15	195	2300	5797	45	225	3501	5988	75	255	4318	6444

0.5	0.5	2315	5800	0.5	0.5	3524	5992	0.5	0.5	4327	6460

16	196	2329	5803	46	226	3547	5996	76	256	4335	6476

0.5	0.5	2344	5805	0.5	0.5	3569	6000	0.5	0.5	4344	6494

17	197	2359	5808	47	227	3590	6005	77	257	4352	6512

0.5	0.5	2374	5811	0.5	0.5	3611	6009	0.5	0.5	4361	6530

18	198	2389	5814	48	228	3631	6013	78	258	4369	6550

0.5	0.5	2404	5816	0.5	0.5	3651	6018	0.5	0.5	4377	6570

19	199	2419	5819	49	229	3670	6022	79	259	4385	6592

0.5	0.5	2434	5822	0.5	0.5	3689	6027	0.5	0.5	4394	6614

E-3	IT 0306

SITE C - Continued

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

20	200	2449	5825	50	230	3708	6031	80	260	4402	6637

0.5	0.5	2464	5827	0.5	0.5	3726	6036	0.5	0.5	4410	6662

21	201	2479	5830	51	231	3744	6041	81	261	4418	6688

0.5	0.5	2494	5833	0.5	0.5	3761	6046	0.5	0.5	4425	6715

22	202	2510	5836	52	232	3778	6051	82	262	4433	6744

0.5	0.5	2525	5839	0.5	0.5	3794	6056	0.5	0.5	4441	6774

23	203	2540	5842	53	233	3811	6061	83	263	4449	6807

0.5	0.5	2556	5844	0.5	0.5	3827	6066	0.5	0.5	4456	6841

24	204	2571	5847	54	234	3842	6071	84	264	4464	6877

0.5	0.5	2587	5850	0.5	0.5	3858	6076	0.5	0.5	4472	6915

25	205	2602	5853	55	235	3873	6082	85	265	4479	6957

0.5	0.5	2618	5856	0.5	0.5	3887	6087	0.5	0.5	4487	7001

26	206	2634	5859	56	236	3902	6093	86	266	4494	7048

0.5	0.5	2650	5862	0.5	0.5	3916	6099	0.5	0.5	4502	7099

27	207	2666	5865	57	237	3930	6105	87	267	4509	7153

0.5	0.5	2682	5868	0.5	0.5	3944	6110	0.5	0.5	4516	7212

28	208	2698	5871	58	238	3957	6117	88	268	4524	7277

0.5	0.5	2714	5874	0.5	0.5	3970	6123	0.5	0.5	4531	7346

29	209	2731	5877	59	239	3983	6129	89	269	4538	7423

0.5	0.5	2747	5880	0.5	0.5	3996	6135	0.5	0.5	4546	7507

90	270	4553	7599	120	300	4976	9131	150	330	758	5512

0.5	0.5	4560	7702	0.5	0.5	4984	9152	0.5	0.5	783	5516

91	271	4567	7816	121	301	4991	9174	151	331	807	5521

0.5	0.5	4574	7944	0.5	0.5	4999	9196	0.5	0.5	831	5525

92	272	4581	8012	122	302	5017	9218	152	332	854	5530

0.5	0.5	4588	8032	0.5	0.5	5035	9240	0.5	0.5	877	5534

93	273	4595	8052	123	303	5053	9262	153	333	900	5538

0.5	0.5	4603	8072	0.5	0.5	5070	9284	0.5	0.5	922	5542

94	274	4610	8092	124	304	5086	9307	154	334	945	5546

0.5	0.5	4617	8112	0.5	0.5	5101	9330	0.5	0.5	966	5550

95	275	4624	8131	125	305	5116	9352	155	335	988	5554

0.5	0.5	4631	8151	0.5	0.5	5131	9375	0.5	0.5	1010	5558

96	276	4673	8171	126	306	5145	9399	156	336	1031	5562

0.5	0.5	4644	8191	0.5	0.5	5158	9422	0.5	0.5	1052	5566

97	277	4651	8210	127	307	5171	9446	157	337	1072	5570

0.5	0.5	4658	8230	0.5	0.5	5183	9469	0.5	0.5	1093	5574

98	278	4665	8249	128	308	5196	9493	158	338	1113	5577

0.5	0.5	4672	8269	0.5	0.5	5207	9517	0.5	0.5	1133	5581

99	279	4679	8289	129	309	5219	9542	159	339	1153	5585

0.5	0.5	4686	8308	0.5	0.5	5230	9566	0.5	0.5	1172	5588

IT 0306	E-4

SITE C - Continued

BEARING	COORDINATES	BEARING	COORDINATES	BEARING	COORDINATES

100	280	4693	8328	130	310	5240	9591	160	340	1192	5592

0.5	0.5	4700	8347	0.5	0.5	5251	9616	0.5	0.5	1211	5596

101	281	4707	8367	131	311	5261	9641	161	341	1230	5599

0.5	0.5	4713	8386	0.5	0.5	5271	9667	0.5	0.5	1249	5603

102	282	4720	8406	132	312	5280	9693	162	342	1268	5606

0.5	0.5	4727	8425	0.5	0.5	5290	9719	0.5	0.5	1286	5610

103	283	4734	8445	133	313	5299	9745	163	343	1304	5613

0.5	0.5	4741	8464	0.5	0.5	5307	9772	0.5	0.5	1323	5616

104	284	4748	8484	134	314	5316	9798	164	344	1341	5620

0.5	0.5	4755	8503	0.5	0.5	5324	9826	0.5	0.5	1358	5623

105	285	4762	8523	135	315	5332	9853	165	345	1376	5626

0.5	0.5	4768	8542	0.5	0.5	5340	9881	0.5	0.5	1394	5629

106	286	4775	8562	136	316	5348	9909	166	346	1411	5633

0.5	0.5	4782	8581	0.5	0.5	5356	9937	0.5	0.5	1428	5636

107	287	4789	8601	137	317	5363	9966	167	347	1446	5639

0.5	0.5	4796	8621	0.5	0.5	5370	9995	0.5	0.5	1463	5642

108	288	4803	8640	138	318	32	5377	168	348	1480	5645

0.5	0.5	4810	8660	0.5	0.5	69	5384	0.5	0.5	1469	5648

109	289	4817	8680	139	319	106	5391	169	349	1513	5652

0.5	0.5	4824	8700	0.5	0.5	142	5398	0.5	0.5	1530	5655

110	290	4831	8719	140	320	177	5404	170	350	1546	5658

0.5	0.5	4838	8739	0.5	0.5	211	5411	0.5	0.5	1563	5661

11	291	4845	8759	141	321	245	5417	171	351	1579	5664

0.5	0.5	4852	8779	0.5	0.5	278	5423	0.5	0.5	1595	5667

112	292	4859	8799	142	322	310	5429	172	352	1611	5670

0.5	0.5	4866	8819	0.5	0.5	342	5425	0.5	0.5	1627	5673

113	293	4873	8840	143	323	373	5441	173	353	1643	5676

0.5	0.5	4880	8860	0.5	0.5	404	5446	0.5	0.5	1659	5679

114	294	4888	8880	144	324	434	5452	174	354	1675	5681

0.5	0.5	4895	8900	0.5	0.5	464	5457	0.5	0.5	1691	5684

115	295	4902	8921	145	325	493	5463	175	355	1706	5687

0.5	0.5	4909	8941	0.5	0.5	521	5468	0.5	0.5	1722	5690

116	296	4917	8962	146	326	549	5473	176	356	1737	5693

0.5	0.5	4924	8983	0.5	0.5	577	5478	0.5	0.5	1753	5696

117	297	4931	9004	147	327	604	5483	177	357	1768	5699

0.5	0.5	4939	9025	0.5	0.5	631	5488	0.5	0.5	1783	5702

118	298	4946	9046	148	328	657	5493	178	358	1799	5704

0.5	0.5	4953	9067	0.5	0.5	683	5498	0.5	0.5	1814	5707

119	299	4961	9088	149	329	708	5503	179	359	1829	5710

0.5	0.5	4968	9109	0.5	0.5	734	5507	0.5	0.5	1844	5713

E-5	IT 0306