Buffalopumps                                                                     Manual 3353E-1

VERTICALLY SUBMERGED

LUBE OIL PUMPS

# CLASS VCRE

VERTICALLY SUBMERGED

## LUBE OIL PUMPS

## CLASS VCRE

### EXTERNAL GREASE CUP CONVERSION KIT

#### These instructions are written to complete

field installation of an external grease cup

in conjunction with Service Manual 3353E.

Refer to Service Manual 3353E for motor

and coupling information.

Buffalopumps

874 Oliver St.  North Tonawanda, NY 14120

phone (716) 693-1850    fax (716) 693-6303

Instructions for installation of the external grease cup conversion kit.

Refer to Figure #1 for LO1/LO2 shaft group and Figure #2 for LO3 shaft group

NOTE:

This Buffalo Pumps conversion kit is offered for the expressed purpose of re-locating the grease cup from inside of the coupling guard to now be located outside of the coupling guard.   This new location provides easier access to the grease cup to perform the required thrust bearing lubrication.  The required replenishment of lubricant to the thrust bearing, including the replenishment periods and the re-lubrication of the thrust bearing, including the specified re-lubrication period remain as stated in Service Manual 3353E.   Proper thrust bearing lubrication is critical and all instructions on page 2 and page 3 of Service Manual 3353E must be followed.

Refer to Service Manual 3353E before proceeding, (copy is enclosed with kit).

- Lockout the equipment to assure that it cannot be operated during installation of this kit.

- Remove the coupling guard (125) and discard.

- Remove the original grease cup, item 32A.

- If required, rotate the bearing cover, item #24, to locate the grease cup so it clears the opening in the motor stand.

- Remove the extension pipe, item a, from the kit box and assemble the pipe coupling, item b, to one end of this extension pipe.

- Remove the grease cup, item 32A from the kit.  Separate the cup from the body.  Assemble the body of the grease cup to the pipe coupling of the extension pipe.

- Completely fill the body of the grease cup with Mobilgrease 28 or equal.

- Fill the grease cup with Mobilgrease 28 or equal

- Thread the grease cup to the grease cup body.  Fully thread the cup onto the body fully discharging the grease into the pipe extension.

- Repeat steps 7, 8 and 9 until the extension pipe is fully filled with grease.  The cup now should be fully engaged to its body and empty of grease.

- Assemble the mounting bracket, item c, to the motor stand following the drawing

CD 16881.

- Insert the pipe extension thru the mounting bracket and thread the extension pipe into the bearing cover.

- Assemble the new coupling guard (125) to the motor stand properly locating the grease cup to the opening in the guard.  (If your kit has two coupling guards, use the guard that is the same height as the motor stand.  Discard the unused guard.)

- Assembly the coupling guard clamps (61H) to the guard and fasten together drawing the coupling guard tight and securely to the motor stand.

- The relocation of the grease cup is complete.

- If the thrust bearing requires grease replenishment, repeat step 8 and 9.

Refer to page 4 for a table listing the components contained in your kit.

3

The following parts are included in the External Grease Cup Conversion Kit.

| for LO-1 and LO-2 shaft group |  |  |
| --- | --- | --- |
| Piece Number | Quantity | Description |
| A | 1 | 1/8-27 NPT pipe x 8 1/2 long, threaded both ends |
| B | 1 | 1/8-27 NPT female coupling |
| C | 1 | grease tubing support bracket |
| 32A | 1 | Essex grease cup, 1/2 ounce |
| 61H | 2 | coupling guard clamps |
| 125 | 1 | coupling guard 6 inch high |
| 125 | 1 | coupling guard 7 1/4 inch high |
| for LO-3 shaft group |  |  |
| Piece Number | Quantity | Description |
| A | 1 | 1/8-27 NPT pipe x 8 1/2 long, threaded both ends |
| B | 1 | 1/8-27 NPT female 45 degree elbow |
| C | 1 | grease tubing support bracket |
| 32A | 1 | Essex grease cup, 1/2 ounce |
| 61H | 2 | coupling guard clamps |
| 125 | 1 | coupling guard |

Buffalopumps

874 Oliver St.  North Tonawanda, NY 14120

phone (716) 693-1850    fax (716) 693-6303