Section 3.0	Division of Responsibility and Equipment Supply Matrix

Material/Responsibility		Qty	Description

Owner                                  2	GE Frame VII FA Gas Turbine Generators with associated equipment as outlined by GE Scope of Supply

2	GE Frame VII FA Exhaust Stacks

2	100% Capacity Fuel Gas Compressors

1	Raw Water Storage & supply to Plant Boundary Limits

1	Lot Water for construction, commissioning, and start up

1	Lot Permits for Environmental, Building, Construction, etc.

1	Lot access roads to site

1	Lot of Fuel Gas for commissioning and start up

1 Lot    230 KV Substation including 18 KV/230KV GSU’s, etc.

1	Lot of Electrical Power for construction, commissioning and start-up

1	Construction Lay Down Area

1	Lot Import Duties and Taxes

Contractor

Civil / Structural

- Lot Site Preparation, Rough Grading, and final grading

- Lot Concrete Foundations

- Lot Structural Steel Pipe Racks, Stairs, Walkways, and Bus              Supports

Buildings

- Control / Office  Building

- Water Treatment / Firewater Pump/Warehouse Building

1         Electric Utility Building

- Guard House

Mechanical Equipment and Materials

- Fuel Gas Filter Regulator Skids

2        Duplex Filter Coalescer Skids

1	Fuel Gas ESD Valve

2         Fuel Gas metering flow transmitter

1	Fuel Gas Scrubber

1	Fuel Gas Electronic Check Meter

1         Fuel Gas Compressor Cooling Tower (LO & Fuel Gas Cooling)

3         50% capacity Cooling Water circulation pumps

1	Water Treatment System w/ RO and EDI

1	Duplex Raw Water Forwarding Pump Skid

1	Duplex Demin Water Forwarding Pump Skid

2	Duplex Demin Filter Skids

1	RO Water Tank

1	Demin Water Tank

1	Fire Water Skid w/ three pumps

1	Lot underground Fire Water Piping, Monitors, Hydrants, etc.

1	Duplex Instrument / Service Air Compressor with Dryers, Tanks etc.

1	Oily Water Drain System

1	Oily Water Separator

1	Waste Oil Tank

1	Waste Oily Water Tank

1	Waste Oil Pump

1	Waste Oily Water Pump

1	Diesel Fuel Tank

1	Emergency Diesel Generator 500 KW

Electrical Equipment and Materials

2ea	18KV Generator Circuit Breakers

2ea	Sets 18KV ISO Phase Bus

2ea	Aux Transformers 18KV/4.16KV 15 MVA

2ea	4.16 KV Switchgear

1ea	4.16 KV MCC Gas Compressors

4ea	Aux Transformer 4.16 KV/480 volt

3ea	DP Panels 480 volt

2ea	BOP MCCs 480 volt

- Lot Area Lighting

- Lot Plant Grounding

- Lot Cathodic Protection

- Lot Lightning Protection

- Set BOP 125 VDC Batteries and Charger

- 125 VDC DP Panel

- Lot 15KV, 4.16KV, 480V Cabling, Conduit, Tray and Duct Banks

Instrumentation and Controls

- Lot Plant Instrumentation

- Lot Fiber Optic Communication Cabling System to interconnect  Telephones and DCS Systems

- Plant DCS System

Construction

- Lot Construction Tools, Rental Equipment, and Cranes

- Lot Local Subcontractors Civil, Mechanical, Electrical

- Lot Temporary Electrical Power Distribution

- Lot Transportation of Contractor Supply Equipment and Materials

- Lot Lubricants, Chemicals, and  Filters plant commissioning

- Lot BOP Start Up Spares

- This Technical Scope Document will become an exhibit to the EPC Contract.

THIS DOCUMENT IS CONFIDENTIAL.  IT IS DESIGNED AND INTENDED FOR efacec’S USE.  THIS DOCUMENT IS FOR THE SOLE PURPOSE OF EVALUATING CONTRACTOR’S PROPOSAL FOR THE (2) GE Frame 7FA ANACO POWER PROJECT.