CADAFE

Frame 7EA Power Project

Technical Scope Document

Section 4.0	Design Basis and Interconnect Points

4.1	Design Basis

Design Conditions

Site Elevation 403 feet (310.5 meters) Air Temperature, High 95°F? Air Temperature, Low 60 °F? Design 86°F Relative Humidity 60% Wind Speed 100 mph? Gas Turbine Power 75 MW net Total Plant Fuel Consumption Rate, 21,000 MCFD Natural Gas High Voltage Interconnect 115 KV Demin Water Storage 10,000 Gallon Fiber Glass Tanks by Contractor Instrument Air System 85 SCFM by Contractor Waste Oil Storage 10,000 Gallons by Contractor Raw Water Storage 10,000 gallon CS Tank by Contractor Treated Water (Water Wash) Storage 10,000 gallon CS Tank by Contractor

  - Interconnect Points

| Natural Gas at min. 450 psig | Owner to provide interconnection point outside of the Plant Battery Limits. |
| --- | --- |
| Plant Waste Water | Plant Battery Limits. |
| Plant Waste Oil | Plant Waste Oil Tank. |
| 115 KV | Connection to Utility via dead end structure inside Substation. |
| Telephone | Plant Battery Limits. |
| Raw Water Supply | Plant Battery Limits. |
| Sanitary Sewer | Plant Septic System |
| Raw/Firewater Supply | Plant Battery Limits. |