PROPOSAL

Presented To:

Electricidad de Caracas

for

LM6000 Gas Turbine Equipment

Prepared By

Proposal No. 710-3241

February 18, 2010

This document is privileged and contains confidential information intended for use only by

Electricidad de Caracas.

Table of Contents

1.0	Introduction	3

2.0	Gas Turbine Generator Set Scope of Supply	3

3.0	Limits of Seller Scope & Exclusions	5

3.1	Exclusions	6

4.0	Pricing	7

4.1	Equipment Pricing	7

4.2	Payment	7

4.3	Taxes, Duties and Fees	7

5.0	Schedule	7

6.0	Warranty	8

7.0	Terms & Conditions	8

8.0	Site Services	8

9.0	Follow Up	8

Attachment A	Standard LM6000 Generator Package

- Introduction

Derwick Associates (“Derwick”) is pleased to provide this proposal to Electricidad de Caracas (“EDC”) for three (03) new (never use) GE LM6000PC Gas Turbine.

The GE LM6000PC is a highly reliable, mid-sized packaged power plant developed for either 50 or 60 hertz applications with design emphasis placed on energy efficiency, availability, performance and maintainability.

- Gas Turbine Generator Set Scope of Supply

Derwick Associates is offering a LM6000 PC ISO rated at 48 MW which includes the following scope of supply:

- One (1) Gas Turbine Generator, GE Aero, LM 6000 PC-NGW, Dual Fuel, Sprint Package

- Air Cooled GE Generator

- Lifting/ spreader beams available for lifting the GT/Gen package (beams only)

- Inlet Anti-Ice Coil for glycol circulation (Coils are located on the air inlet for the turbine only)

- Inlet Anti Icing System (Steam/Glycol Heat Exchangers/circulation skid module)

- Lube Oil Systems with shell and tube coolers (Lube Oil enclosure contains only the hydraulic jacking pump and hydraulic oil reservoir, turbine lube oil reservoir and tube and shell coolers and the water wash heating tank)

- GT 1 Control Cubicle, which contains the GT 1 Turbine Control Panel which consists of the following controls:

  - Synchroscope with pistol grip controller for Generator with manual-off

generator auto

  - PM 295 Power Analyzer for the generator

  - Excitation Null Meter

  - Generator Breaker Control pistol grip controller with Trip/Close

  - PF/VAR pistol grip controller with Voltage-PF-VAR

  - Manual Regulator Voltage pistol grip controller with Lower-Normal-Raise

  - Generator Exciter Voltage Regulator with pistol grip controller with

  - Manual-Normal-Auto

  - Automatic Regulator Voltage pistol grip controller with Lower-Normal-

Raise

  - Brush Prismic A30 Excitation Control, HMI into the GE Series 6 and

FANUC 90/70

Emergency Stop Button

  - Control Selector Switch with pistol grip controller with Local-Remote

  - Governor pistol grip controller with Lower-Normal-Raise

  - Beckwith Electric Company Generator Integrated Protection System

- GT 1 termination Cubicle which consists of the following, termination points for inputs into the GT 1 Turbine Control Panel and into the Allestec Corporation Model 800 natural gas and fire monitoring system for the Gas Turbine and Generator Enclosures and the Bently Nevada 3500 Vibration Monitoring System for the Turbine and Generator.

- Remote work station consisting of the following: A Dell PC and keyboard with a 29 inch Samsung flat screen monitor.

- Neutral Cubicle with ground resistor located next to the Generator Enclosure.

- Noise reduction enclosures:

  - Auxiliary Module (located above the lube oil enclosure)

  - Sprint Skid

  - Water Injection Skid (two (2) such enclosure, one for each NOX reduction

  - Pumps

- Pulse Air Filter/Pre Filter (The Equipment has an air intake filter self supporting housing for the turbine and generator with local control mounted on the structure for the pulse air system)

- Sprint Power Boost (The Equipment is equipped with a GE supplied water Sprint System)

- Standard Documentation consisting only of the following:

  - GE Aero Energy Products, A GE Power Systems Business, LM 6000,

  - Gas Turbine-Generator Set 60 Hz, Installation Manual

  - GE Aero Energy Products, A GE Power Systems Business, LM 6000 Gas

Turbine-generator Set 60Hz, Installation Manual, July 2002,

  - GE Aero Energy Products, A GE Power Systems Business, Gas Turbine-

  - Generator Set 60 Hz, Basic Operator’s Course, March 2003

  - GE Aero Energy Products, A GE Power Systems Business, LM 6000,

  - Gas Turbine-Generator Set 60 Hz, Book 1 of 2 and Book 2 of 2, Basic

  - Operator’s Course and Package Maintenance Course, April 2003

  - GE Industrial Aero Derivative, Gas Turbines, LM 6000, SAC, GEK

105059, Model-PC, Volume 1 &11, On-Site Operation and Maintenance

Manual

  - GE Aero Energy Products, A GE Power Systems Business, LM 6000,

  - Gas Turbine-Generator Set 60 Hz, Advanced Systems & Controls,

Training Course (Millennium-MARK VI / FANUC 90/70) April 2003

  - GE Aero Energy Products, A GE Power Systems Business, LM 6000 Gas

Turbine-Generator Set 60 Hz, Operational and Maintenance, Manual,

Volume I, II, III, IV, V, VI, VII, VIII, IX, X, XI, XII, XII, XIV, XV. August 2002

- Standard Enclosure for Turbine and Generator

  - Ventilation System

  - CO2 Fire Suppression System mounted in a separate enclosure next to

The generator

- Water Injection for NOX Reduction (Duplex Rotojet Pumps, separate enclosures)

- Limits of Seller Scope & Exclusions

Listed below are the limits/exclusions to the Seller standard Scope of Supply. All piping, wiring, cables, ducts, etc. connecting to these points is furnished by Purchaser (others) unless modified by specification agreement.

| Equipment System | Limits of Seller Scope |
| --- | --- |
| All piping, including Fuel Gas, Fuel Oil, Steam, Cooling Water, Heating Water, Demineralized Water, Lube Oil, Compressed Air, Instrument Air, Hydraulic Start Oil | Flanged or threaded connection on Seller baseplate. |
| Inlet Air-to-Filter | Atmosphere (non-standard duct by others) |
| Turbine Package Ventilation/Cooling Air | Atmosphere (non-standard duct by others) |
| Turbine Exhaust | Exhaust flange on main baseplate |
| Instruments on Seller’s Baseplate | Terminal box on baseplate |
| Instrument wiring in Turbine Control Panel | Wiring Terminal block in Turbine Control Panel |
| High Voltage Connections | Bus bar in Seller Line side cubicle |
| Generator Ground Connections | Seller Neutral cubicle |
| Electric Motors | Terminal box on individual motor |
| Ladders and Platforms for Air Filter | Ladders and Platforms for Inlet Air Filter maintenance only |
| 24 V DC Batteries and Chargers for Control System and Fire and Gas Systems | Battery terminals to baseplate (if supplied loose) |

  - Exclusions

Derwick Associates has excluded the items listed below from our offering.  Any other equipment or service not described in our proposal is also excluded.

- Balance of plant and energy optimization controls

- Buildings, foundations, anchor bolts, embedments and grouting

- Bus bars and bus duct beyond generator lineside and neutral enclosures

- Distributed plant control

- Filter house support structure – other than standard

- Field Supervision

- Fuel, fluids and chemicals

- Fuel storage tanks, forwarding equipment and primary fuel filter

- Gas compression, filtration and separation or regulation equipment

- High voltage transformer(s), cables, switchgear and associated equipment

- Interconnecting piping, conduit, and wiring between equipment modules (site layout is unknown at this time)

- Plant utilities

- Power plant calibration tools and ordinary hand tools

- Spare parts (quoted separately)

- Transportation to job site and off loading of equipment

- Water injection pressurization equipment

- Water treatment and purification equipment

- Yard light and fences

- Pricing

Pricing references the scope of equipment and service work described in this proposal:

- Three (03) GE LM6000 gas turbine generator

- GE OEM certified depot overhauled and tested to GE Standards

Equipment is subject to prior sale until down payment is received.

  - Equipment Pricing

- $

- $

All pricing is based on ex-works Sedalia, Missouri delivery unless otherwise stated.

  - Payment

This proposal and pricing is based upon receipt of the progress payments shown below:

- Down Payment:  30% to take the units off the market.  Non-refundable

- Second Payment 65% 15 days after contract signing.

- Balance:  5% upon notice of readiness to ship.

| Wire Transfer Instructions: |  |  |  |
| --- | --- | --- | --- |
| Intermediary Bank: |  | Beneficiary: |  |
| Citibank, N.A. |  | International Union Bank, S.A. |  |
| ABA: 021000089 |  | Account: 36246731 |  |
| Branch: New York |  | Address: |  |
| Address: |  | Ave. Samuel Lewis, Edif. Omega Piso 5, |  |
| 111 Wall Street, New York, NY 10043 |  | Apartado 0391 Wtc Panamá, Republica de Panamá |  |
|  | FFC: Davos International Bank/000767-224-001 |  |  |
|  | Ref: Derwick Associates/ 13102362 |  |  |

  - Taxes, Duties and Fees

No sales or use taxes have been included in this quotation.  The prices quoted exclude any federal, state or local taxes or fees which may be associated with the export, import or purchase of equipment and/or services.

- Schedule

Derwick Associates expects to prepare the equipment for shipment Ex-works within ninety (90) days after receipt of down payment.

- Warranty

Derwick Associates will provide a one (1) year warranty on the entire gas turbine generator package and any other balance of plant equipment provided.

- Terms & Conditions

This proposal shall be valid for thirty (30) days; provided, however, the obligation to treat this proposal as confidential, and that it cannot be shared with any third party without the prior written consent of Derwick Associates shall survive.

Derwick Associates and EDC will negotiate in good faith to establish general terms and conditions that are usual and customary of the sale of used equipment.

- Site Services

Derwick Associates would be pleased to also provide a proposal for the installation, startup and commissioning of the facility.  This would include providing construction supervision as well as startup engineers for all equipment provided.

Derwick Associates can also provide an experienced service representative to assist the operating personnel during the first two (2) months after the equipment goes online.

- Follow Up

Please contact the following person at Derwick Associates for information regarding this proposal:

Joaquin Mavares

jmavares@Derwick Associatesservices.com

Office:  660.829.5100

Cell:  	 660.281.8588

Omar Petit

opetit@Derwick Associatesservices.com

Office:  660.829.5100

Cell:  	 660.281.8588

- Attachment A

- Standard LM6000 Generator Package

Gas Turbine

General Electric gas turbine model LM6000 is a two-shaft/two-spool engine consisting of a five-stage low pressure compressor, a fourteen-stage high pressure compressor, a two-stage high pressure turbine, and a five-stage low pressure turbine. The engine is equipped with a stainless steel mesh screen in the inlet air stream for "last chance" protection against foreign object damage. The engine is shock mounted and shipped in position, with the exception of the coupling spacer, which is removed and shipped in a separate container.

Generator

Air cooled, 2-pole generator operating at 13.8 kV, 60 Hz. Generator is capable of handling Purchaser power requirement throughout a wide ambient temperature range. A cooling water loop and its associated fans and pumps are not required. The generator includes a brushless excitation system with permanent magnet generator. Neutral and line side cubicles are included.

Unit Enclosure

The basic equipment package is supplied with weatherproof acoustic enclosures with sound attenuation to an average of 85 dB(A) at 3ft 3 in (1 m) from the face of the equipment at 4 ft 11 in (1.5 m) above ground. The enclosures are completely assembled and mounted over the equipment prior to testing and shipment. The turbine and generator compartment is fully ventilated with belt driven fans. Explosion-proof lighting is provided in both compartments.

Gas Turbine / Generator Baseplate

The basic equipment package is supplied with the support structures for the gas turbine generator set consisting of a two-piece skid assembly, which is sectioned between the gas turbine and the generator. The full depth, bolted section is designed to provide the full structural properties of the wide flange I-beams. Full depth crossmembers are utilized to provide for a rigid design that is suitable for installation in earthquake areas (U.S. Seismic Zone 4) as well as providing a convenient structure for transportation. The baseplate support system is enhanced by the installation of a heavy-duty, welded superstructure, which utilize structural tubing for wall columns and roof beams.

Air Inlet System

The basic equipment package is supplied with a modular, multi-stage filtration system consisting of inlet screens, a prefilter and a final barrier filter. All air for ventilation systems is filtered to the same level as turbine combustion air. An evaporative cooling system is included in the basic equipment package scope. Filtered air is silenced before entering the turbine plenum. This design results in a compact arrangement and eliminates the need for Purchaser supplied inlet ducting when the standard design is utilized. Internal lighting of the filter house is provided to facilitate inspection and service.

Package is also supplied with platforms and ladders to service the inlet filter. These items are packaged separately for shipment. Ladders, platforms and stairways to service other portions of the gas turbine generator enclosure are not included. Special or customized filter arrangements can be supplied, and they are quoted on an individual basis.

Turbine Exhaust

The basic equipment package is supplied with a circular, axial exhaust outlet with connection flange to facilitate in-line mounting of an HRSG or simple cycle exhaust stack.

Fuel System

The basic equipment package is supplied with a natural gas fuel system using an electronically controlled fuel-metering valve. All necessary shutoff valves, piping and instruments between the auxiliary skid connection and the turbine are included. For full-load operation, the gaseous fuel must be supplied to the baseplate at 675 psig±20 (4,658 ±138 kPag). All necessary shutoff valves, piping and instruments between the baseplate connection and the turbine are included. Gas fuel must meet General Electric specification MID-TD-0000-1.

Lube Oil Systems

The basic equipment package is supplied with two separate lube oil systems: one for the gas turbine (synthetic oil) and one for the generator (mineral oil). The oil reservoirs and piping are all stainless steel, and the lube oil system valves have stainless steel trim. Each lube oil system has duplex filters, duplex shell and tube coolers, and thermostatically-controlled electric heaters. The coolers, oil reservoir, and filters for each system are mounted on an auxiliary equipment module located next to the gas turbine baseplate.    The auxiliary equipment module provides simplified piping connections and reduces Purchaser's installation time and costs. Purchaser must supply cooling water to the shell and tube coolers. Turbine lube oil must meet MID-TD-0000-6.

Electro-Hydraulic Start System

The basic equipment package is supplied with an electric motor driven hydraulic pump assembly, filters, cooler and controls, mounted on the auxiliary equipment module. A hydraulic motor is also mounted on the gas turbine accessory gearbox. Hydraulic hoses are furnished to connect the auxiliary equipment module and the main baseplate.

Fire Protection System

The basic equipment package is supplied with a factory installed fire protection system complete with optical flame detection, hydrocarbon sensing and thermal detectors, piping and nozzles in both generator and engine compartments. The fire protection system includes cylinders containing CO2 mounted on a separate skid. A 24 V DC battery and charger to power the fire protection system is also included. All alarms and shutdowns are annunciated at the turbine control panel (TCP). An alarm sounds at the turbine if the gas detectors detect high gas levels, or if the system is preparing to release the CO2. When the system is activated, the package shuts down, and the primary CO2 cylinders are discharged into the turbine and generator compartments via multiple nozzles, and the ventilation dampers automatically close. After a time delay and if required, the reserve supply of CO2 is discharged.

Digital Control System

The basic equipment package is supplied with a free-standing control panel suitable for mounting in an indoor, non-hazardous area. The control system features an integrated Woodward MicroNet Plus turbine control system, vibration monitor, digital meter, digital generator protective relay module and an HMI (human machine interface) display of key discrete and analog data. The operator selects HMI displays with convenient touch screen control. Alarm and shutdown events are displayed on the HMI automatically. An Ethernet TCP/IP EGD or RS485 Modbus Port is provided to transmit unit conditions (status, pressures, temperature, etc.) to the Purchaser's distributed control system. Power for the control panel is provided by a dedicated 24V DC battery system with dual 100% capacity chargers, which are shipped separately for installation by others.

Generator Protective Relays

The basic equipment package is supplied with a microprocessor-based generator protective relay module, mounted in the TCP. The protective relay system includes functions necessary for protection of the generator.

Soak Wash System

The basic equipment package is supplied with a turbine cleaning system, which allows Purchasers to clean the compressor section of the turbine during full power operation. The same system reservoir and piping are utilized for off-line soak washing. Auxiliary skid connections are provided for Purchaser supplied purified water at a maximum of 50 psig (345 kPag) and air at 100 – 120 psig (689 – 827 kPag). Purchaser is required to provide purified water meeting MID-TD-0000-4, detergent meeting MID-TD-0000-5, and air filtered to ISA S7.3 standards.

  - Component Testing and Package Full Load Test

Every new gas turbine is performance tested under load in a GE Test Cell, using procedures developed for flight turbine reliability. The generator is tested to ANSI C50.14 or IEC 34.3 standards at its factory of manufacture.

All gas turbine generator sets receive a rigorous 400-point static test including:

- Switch State (N.O. or N.C., actuation, wiring, and setpoint)

- Temperature element output, and wiring

- Transmitter range, output, and wiring

- Solenoid operation

- Control valve torque motor, excitation, and return signal

- Fire system continuity, and device actuation

- System flushing verification

- Tubing integrity

Drawings, Data and Manuals

The basic equipment package is supplied with a Purchaser drawing package that includes general arrangement drawings, flow and instrument diagrams, electrical one-line drawings and interconnection plan drawings. Additional electrical schematic diagrams and logic drawings are provided for record. Maintenance manuals are provided and are printed in English. The manuals cover operating concepts for power generating equipment, guides to troubleshooting, basic information on components, and equipment within the turbine generator set.

Training (Optional)

The base scope of supply can includes as an option hands-on training for up to 10 operators and supervisors, where students are assumed to have at least a journeyman's knowledge of electrical generating plant operation and to be proficient in reading piping flow and instrument drawings, mechanical drawings, and have a working knowledge of electrical generators, and gas turbines. The course is designed around an eight-hour day, five consecutive day schedule with an hour lunch break and fifteen-minute breaks every one and one half hours. Experienced instructors, using specially developed training materials, provide a firm groundwork of basic theory, plus advanced concepts with classroom and hands-on training. Training includes Gas Turbine Familiarization plus System Design & Operations and Maintenance.

The trainer conducts the course in a lecture/seminar format where each major topic is supported by literature with detailed descriptions and associated engineering drawings. A student-training manual is given to each student and the client’s turbine-generator system is used for hands-on training to supplement the classroom instruction. At the completion of several related topics the students are given a progressive examination to measure the effectiveness of the presentation and as a tool to identify if any student has not grasped the material. At the completion of the course a final examination is given which covers the entire course material and students are given a certificate of completion.

Improvements and Changes

It is understood that the Seller has the right to make changes in product design and add improvements to products or services at any time without incurring any obligations to install the same on or in connection with the Equipment or Services provided hereunder.

Factory Options Descriptions

Option A        SPRINT® Power Augmentation

SPRINT® boosts engine performance using a demineralized water spray intercooling design that significantly increases the mass flow by cooling the air during the compression process. The system is based on an atomized water spray injected through spray nozzles placed at two locations, one between the high pressure and low-pressure compressors, and the second at inlet bellmouth. Water is atomized using high-pressure air taken off of the eighth stage bleed. The water flow rate is metered, using the appropriate engine control schedules and at the inlet bellmouth. Bellmouth and inter-stage portions on SPRINT® alternate operation based on turbine inlet temperature. Purchaser supplies 30 gpm (114 lpm) of demineralized water to the connection on the unit. Water must meet GE specification MID-TD-0000-3

Option B        NOx Control - Water Injection System

A water injection system for control of NOx emissions shall be provided. The demineralized water injection system consists of inlet strainer, pump, valves, piping and controls for use with a gaseous fuel, liquid fuel or dual fuel system. Water injection shall reduce NOx emissions to 25 ppm (51 mg/N m3) (Ref. 15% O2) on gaseous fuel. For gaseous fuel applications, Purchaser must provide a demineralized water supply of up to 55 gpm (208 lpm) and at 20-40 psig (138-276 kPag). Water must meet GE specification MID-TD-0000-3. The minimum Purchaser supplied pressure and temperature is determined by the water injection rate required and the type of fuel nozzle utilized.

Option C    Transportation Services

Seller arranges for shipment on behalf of the Buyer. The Buyer pays the Seller for all fees and expenses including, but not limited to, those covering preparation of consular documents, freight, loading fees at storage, storage, transit insurance and warehouse-to-warehouse insurance.