Derwick Associates, S.A. is pleased to provide the following pricing information in response to your request for three (3) new FT8 SWIFTPAC™ gas turbine generators for Venezuela.

Introduction

The FT8® SWIFTPAC® Unit configurations are completely self-contained; gas turbine powered electric generating units containing all the equipment necessary for local automatic operation. The configuration consists of four (4) primary units’ two (2) gas turbines units, the electric generator unit, and the electrical/control unit. The SWIFTPAC turbine and generator units consist of two opposed Industrial Gas Turbines directly connected through a diaphragm coupling to a single double-ended electric generator.

The SWIFTPAC power plants are designed to provide quick, reliable power. The plug and play packaging allows for reduced site setup time by delivering factory-assembled modules, quick disconnect cables and prefabricated field piping. Factory flushing and checkout is used where ever possible.

The electrical/control unit includes the 15 kV switchgear and all of the controls and instruments necessary for operation. The 15 kV switchgear is connected to the generator by prefabricated, totally enclosed, weatherproof, 15 kV class bus duct. The turbine/generator and electrical control units are housed in all weather, prime-painted steel enclosures including lighting and electrical services. Fire protection equipment is provided in the gas turbine enclosure.

The generating plant, as erected on a site, is ready for connection to the Customer’s distribution system and water and fuel supply systems. The package concept of these types of power plants, coupled with their relatively small size, allows them to be readily installed in almost any location and conveniently relocated or combined with other units, or other equipment such as waste heat boilers, to meet changing load requirements

Gas Turbine

The FT8 Industrial Gas Turbine consists of a GG8 gas generator and a PT8 power turbine (also referred to as a "free" turbine). The gas generator provides high energy gas to the power turbine, where this gas performs useful work when mechanically coupled to a driven load through a flexible coupling.

Gas Generator

The GG8 gas generator is based on the latest version of Pratt & Whitney's JT8D turbo fan aero engine, the JT8D-219. The JT8D is the most often used engine in the world's commercial aircraft fleet with more than 14,000 units accumulating over 400 million operating hours. The JT8D aero engine provides a total compression ratio of 20:1 and a thrust rating of 21,700 lbs. utilizing dual spool compressors and turbines.

The GG8 gas generator is an axial flow, gas turbine engine. The major components are two compressor modules, a combustion section, and two turbine modules. Inlet air passes through the low compressor and is then further compressed through the high compressor. The high pressure air is then diffused to a low mach number where it enters a large plenum consisting of nine fuel nozzles and combustion cans. Most of the air enters the combustion cans through the fuel nozzles and through the combustion can walls, which also cool the combustion chamber walls.

A small portion of the air bypasses the combustion cans and is used to cool the turbine section. Two independent spark ignitors provide ignition for starting. Thereafter, combustion is self-sustaining. The hot gases pass through the single-stage high pressure turbine and the two-stage low pressure turbine, which extract energy to drive the two (2) compressor sections. The remainder of the hot gas energy is used for driving the power turbine.

Controlling the flow of fuel to the combustion chambers regulates the output of hot gases. The GG8 gas generator uses modulated cooling air to the Low Pressure Turbine (LPT) to maintain optimum clearances throughout the operating profile of the machine. This system includes cooling air valves on the gas generator that are controlled by signals from the electronic engine control.

Figure 1: Major sections of the Gas Generator

Inlet Case Group

This group includes the inlet case, Number 1 bearing, and the Number 1 bearing housing. The case incorporates variable inlet guide vanes for directing air into the Low Pressure Compressor (LPC). The case also provides support for the Number 1 bearing. The front accessory drive houses the NL speed sensors.

Low Pressure Compressor

In the GG8 LPC the fan stage used in the JT8D is replaced with three booster compressor stages at the GG8 inlet. The inlet guide vanes plus the first two stages have been fitted with variable geometry. Variable vane movement provides optimum efficiency for the compressor in the complete speed range, and excellent part-load efficiency.

The LPC has 8 blade stages and 7 stator vane stages. The first two stator vane stages are variable. The LPC gas path is connected to the High Pressure Compressor (HPC) by the intermediate case. The intermediate case also provides a structural connection between the two compressor sections, provides support for the Number 2 and Number 3 bearings, is the location of the gas generator front mounts and is the stator vane stage for the last low compressor blade stage.

The LPC rotor is physically connected to the LPT rotor by a shaft that passes through the HPC and turbine

High Pressure Compressor

The HPC consists of seven blade stages and seven stator vane stages, and is driven by the High Pressure Turbine (HPT) through a connecting shaft. The HPC operates at a higher rotational speed than the LPC. The gearbox, attached to the intermediate case, is driven through gearing from the front of the HPC section.

Diffuser Group

The diffuser case houses the Number 4 bearing that support the HPC rotor. The case also houses the nine fuel nozzle and support assemblies that provide fuel to the combustion chambers. The flow path of the diffuser case reduces the velocity and increases the static pressure of the compressed air before entering the combustion chambers.

Combustion Section

The combustion section has nine chambers arranged in an annulus around the turbine shafts and positioned between the HPC and the HPT. The combustion chambers are enclosed by inner and outer cases. The outer case can be unbolted and moved rearward to allow for inspection or removal of the combustion chambers and fuel nozzles.

Combustion chambers Nos. 4 and 7 each accommodate an igniter plug. During initial ignition, flame is propagated from these two chambers to the remaining chambers through integral flame crossover tubes, which interconnect all nine chambers.

The combustion chambers and nozzles were modified to allow liquid fuel, gaseous fuel, or both simultaneously. Water injection can also be utilized with these combinations. The combustion chamber outlet duct consists of inner and outer annular burner liners with a front bulkhead for support of the nine cans. This outlet duct directs the hot air to the high turbine nozzle guide vanes.

Turbine Nozzle Group

This group includes the turbine inlet nozzle guide vanes that direct the hot gas flow from the combustion chambers to the HPT blades. Parts in this group also provide support for the outlet end of the combustion chambers.

The inner support for the nozzle guide vanes includes the tangential on-board injection (TOBI) system which provides 13th stage compressor air for cooling of the HPT blades and attachments.

High Pressure Turbine

The HPT is a single stage turbine with shroudless, air-cooled turbine blades. The HPT is supported by the Number 5 bearing. The HPT is physically connected to the HPC by the HPT shaft. The turbine blades are cooled by 13th stage compressor air that flows through passages within each blade. Air from the TOBI duct enters the blades at the root attachment area after passing through passages at the front of the 1st stage turbine rotor. The HPT converts thermal and kinetic energy from the gas path into shaft horsepower to drive the HPC.

The combustion chambers and nozzles were modified to allow liquid fuel, gaseous fuel, or both simultaneously. Water injection can also be utilized with these combinations. The combustion chamber outlet duct consists of inner and outer annular burner liners with a front bulkhead for support of the nine cans. This outlet duct directs the hot air to the high turbine nozzle guide vanes.

Turbine Nozzle Group

This group includes the turbine inlet nozzle guide vanes that direct the hot gas flow from the combustion chambers to the HPT blades. Parts in this group also provide support for the outlet end of the combustion chambers.

The inner support for the nozzle guide vanes includes the tangential on-board injection (TOBI) system which provides 13th stage compressor air for cooling of the HPT blades and attachments.

High Pressure Turbine

The HPT is a single stage turbine with shroudless, air-cooled turbine blades. The HPT is supported by the Number 5 bearing. The HPT is physically connected to the HPC by the HPT shaft. The turbine blades are cooled by 13th stage compressor air that flows through passages within each blade. Air from the TOBI duct enters the blades at the root attachment area after passing through passages at the front of the 1st stage turbine rotor.

The HPT converts thermal and kinetic energy from the gas path into shaft horsepower to drive the HPC.

Low Pressure Turbine

The LPT has two turbine rotor stages and two stages of turbine vanes. The front blade and vane rows are air cooled, and both blade rows utilize shrouded tips. These turbine stages use tip shrouded blades. The last stage vanes are clustered. Each vane cluster consists of three vane airfoils. The LPT shaft connects to the LPC. The LPT converts kinetic energy from the gas path into shaft horsepower to drive the LPC.

Turbine Exhaust Case

The Turbine Exhaust Case (TEC) forms the discharge path for the exhaust gas flow into the power turbine. The case also provides physical connection to the power turbine and houses the Number 6 bearing, which supports the rear of the LPT rotor. Pressure oil supplied to the TEC provides lubrication for the Number 6 bearing and for the Number 4½ bearing.

Bearings

The FT8 lube oil system is a combined system, containing the gas generator lube oil system and the power turbine lube oil system, together with necessary filters, cooler, and reservoir.  Anti-friction ball and roller bearings are used throughout the gas turbine since they absorb about 50% less power than sleeve or hydrodynamic bearings and require a less expensive and less complex lubrication system. They can also withstand intermittent losses of lubricant flow for time periods that would be catastrophic to sleeve bearings. Main bearings carry the radial and axial thrust loads of the rotors.

The No. 3 bearing (ball bearing) is a thrust bearing for the main accessory drive bevel gear. It provides radial support for the front of the High Pressure Compressor rotor. It is supported by the Intermediate Case.

The No. 4 bearing (duplex ball bearing) is the thrust bearing for the High Pressure Compressor rotor, and supports the rear hub of the High Pressure Compressor.

The No. 4-1/2 bearing (roller bearing) provides intershaft support between the Low Pressure Turbine shaft and the High Pressure Turbine shaft.

The No. 5 bearing (oil damped roller bearing) provides radial support for the High Pressure Turbine. It is supported by the Combustion Chamber inner case and the Turbine Nozzle group.

The No. 6 bearing (oil damped roller bearing) provides radial support for the rear of the Low Pressure Turbine.

The oil damped bearings have a layer of pressurized oil around the bearing outer diameter to absorb rotor-induced vibration during engine operation.

The main oil seals are fabricated from specially treated carbon, and are of the face type or ring type configuration. Ring type seals are used only where assembly procedures preclude use of face type seals.

POWER TURBINE

Development of the PT8 Power Turbine focused on high efficiency and extended service life. The power turbine consists of an annular, transition duct connected to the gas generator; a single four-stage, axial flow, reaction turbine; and an exhaust casing consisting of exit guide vanes and rear bearing support that connects to the diffuser section and an exhaust collector. The power turbine is supported by two anti-friction roller-type journal bearings and one ball-type thrust bearing. This bearing configuration is similar to the field-proven FT4.

In addition to the speed alternatives of 3,000, or 3,600 rpm, the turbines are available with either direction of rotation. This allows for a Twin Pac configuration with twice the power capacity of the Power Pac. In Twin Pac installations, the generator is driven from both sides.

The power turbine converts the thermal and kinetic energy in the exhaust of the gas generator into a rotational force. This shaft horsepower is transmitted to the driven equipment via two flexible couplings and drive shaft.

### INLET SECTION

### TURBINE CASE AND VANE SECTION

### ROTOR ASSEMBLY

### EXHAUST CASE SECTION

INLET SECTION

The inlet case and inner and outer ducts of the power turbine form an annulus, which directs the gas generator exhaust hot gases to the power turbine. The inlet case, the only power turbine part physically attached to the gas generator, carries the structural load of the gas generator to the power turbine supports.

TURBINE CASE & VANE SECTION

The vanes direct the hot gases to the turbine blades at the proper angle for maximum rotor efficiency. They form an assembly with the split turbine case.

ROTOR ASSEMBLY

The power turbine rotor is composed of a four stage disk and blade assembly, supported by anti-friction bearings on each end. The hot gases directed against the blades turn the rotor, creating the mechanical energy to drive the output shaft.

Exhaust Case Section

The exhaust case section turns the exhaust gases via exit guide vanes to axial flow before entering the diffuser section. The case structure supports the rear bearings and rear engine mounts. The shaded area in the figure below depicts the exhaust case section.

Diffuser and Turning Vane Assembly

The diffuser and turning vane assembly reduces the velocity and increases the static pressure of the exhaust gas flow. The diffuser directs the gas flow from the power turbine through a conical annulus surrounding the power turbine shaft. The conical annulus ends in three annular, flared exit baffles, the diffuser turning vanes. The vanes redirect the gas flow 90 degrees from all circumferential locations into the collector box for exhaust, in simple cycle configurations, to the stack acoustical system and then to the atmosphere. In combined cycle configurations, the gas flow would be exhausted to the heat recovery steam

generator inlet ducting rather than the stack acoustical system. The diffuser and turning vane design permits the smoothest possible movement of exhaust gas through the exhaust system, minimizing backpressure and maximizing efficiency.

Output Drive Shaft

The output drive shaft extends through a cavity in the exhaust diffuser, and exits throughthe rear of the collector box. It transmits mechanical energy to the driven equipment (such as an electrical generator). The front of the shaft is connected to the rotor with a flexible coupling. The rear of the shaft is connected to the driven equipment with a flexible coupling. The output drive shaft is shaded in the figure below.

Collector Box

The collector is suspended from the exhaust enclosure. It receives exhaust gases from the diffuser and turning vanes. The gases are then exhausted through the silencer sections to the atmosphere, or through an optional heat recovery system.

Mounts

The mounts, located at the 3 and 9 o'clock positions on the power turbine exhaust case, provide the rear support for the gas generator/power turbine system. The mounts are adjustable vertically and horizontally. There is also a bottom mount pin, which positions the gas generator/power turbine system in the axial and lateral directions. This allows radial thermal growth.

Cooling Air and Thrust Balance System

One cooling air tube from the gas generator HPC case and two tubes from the LPT exhaust case provide thrust balance and cooling air to the power turbine. The two tubes from the LPT provide cooling air to the first stage disk and blade attachment as well as providing cooling air to the No. 7 bearing support and first vane attachment. The air tube from the HPC provides thrust balancing air to the backside of the rotor to offset the gas path air loads and also provides cooling air to the rotor bore and stages 2-4 disk and blade attachments. This air is also providing cool buffer air to the Nos. 7, 8 and 9 bearing compartment seals.

BEARINGS

The No. 7 bearing (roller bearing) provides radial support for the front of the rotor assembly. The No. 8 (ball bearing) provides axial support and the No. 9 bearing (roller bearing) provides radial support for the rear of the rotor assembly.

#### NO. 7 BEARING

#### NO. 8 BEARING

#### NO. 9 BEARING

##### CROSS SECTIONAL VIEW OF POWER TURBINE WITH MAIN BEARINGS

Ancillary Systems

Lubrication/Hydraulic Control

The FT8 lube oil system is a combined system, containing the gas generator lube oil system and the power turbine lube oil system. Components not integral to the gas turbine are mounted as a pre-engineered package inside the GT enclosure. This package contains the combined reservoir, duplex filters, chip detectors, air-to-oil cooler, and the power turbine supply and scavenger pumps.

The gas generator (GG)supply and return lines terminate at the accessory drive engine mounted gearbox. Since both the GG supply and scavenge pumps are hard coupled to the low-pressure and high-pressure compressor shafts, positive pressures are generated by shaft rotation. This system also supplies oil to the hydraulic control system pump suction port.  The gearbox driven hydraulic high pressure pump and servo valve supply control oil to the IGV/VSV compressor actuators.

The power turbine lube oil system lubricates and cools the main bearings and bearing seal assemblies. Lubrication is provided by identical three-element pumps, which provide primary and auxiliary flow functions. The primary and auxiliary pumps are AC motor driven while the emergency pump is a DC motor driven. The auxiliary pump automatically starts in case of trouble with the primary pump. The DC pump is used for emergency shutdown when AC

power is lost.

FUEL SYSTEM

The Customer furnished fuel gas supply is provided at a minimum pressure of 445 psig (30 bar). After flowing through a fire valve, the gas flows through two shutoff valves and the modulating valve to the gas manifold, where it is injected through the nine gas fuel nozzles. The modulating valve meters fuel in response to signals from the electronic gas turbine control.

# GAS FUEL SYSTEM

# LIQUID FUEL SYSTEM

In the liquid system, fuel in a flooded suction line from the fuel storage tanks is pumped by the customer supplied fuel forwarding system to the filtering skid. After flowing through a fire stop valve, the fuel is boosted by the gas generator mounted fuel pump, and sent to the modulating valve. The modulating valve meters fuel in response to a signal from the electronic gas turbine control. The fuel then flows through two fuel shutoff valves and a flow divider, into the manifold and through the nine liquid fuel nozzles.

###### LIQUID FUEL SYSTEM

Dual Fuel System (when specified)

The dual fuel system utilizes the same components as the gas and liquid fuel systems plus a liquid fuel pump clutch and additional valves to allow switching from one fuel to the other. At startup with liquid fuel selected, the fuel pump is engaged and liquid fuel flows through the liquid modulating valve and liquid shutoff valve into the liquid fuel manifold to the nozzles. At shutdown, the pump is disengaged, the liquid modulating valve and liquid shutoff valves are

closed, the liquid drain valves are opened to prevent pressure buildup downstream of the shutoff, and the manifold drain valve is opened to drain the manifold.

When gas fuel operation is selected, gas flows from the supply through two shutoff valves, through the modulating valve to the gas manifold. A small amount is sent through the liquid system purge valve to the liquid fuel manifold for continuous purging of all liquid fuel from the manifold and nozzles cool and prevent choking of the liquid fuel section of the nozzles. At shutdown, the shutoff valves are closed and the vent line between shutoff valves is opened to

relieve pressure.

The gas turbine has the capability to transfer from one fuel to the other during operation.

Water Injection System (when specified)

The water injection system introduces demineralized water with the fuel into the nozzles of the gas turbine to assist in achieving required exhaust gas emission levels. The system operation is automatically controlled by the electronic gas turbine control.

The demineralized water is Customer supplied to the water injection skid located just outside the gas turbine enclosure. The skid contains a particulate filter, VFD motor driven pump to boost the water to the required pressure, coriolis flow meter and necessary valving.

When operating on liquid fuel, the water is mixed with the fuel and sent through the liquid fuel nozzles. When running on gas fuel, the water is sent through the liquid fuel nozzles and the gas through the gas fuel nozzles. During fuel transfer, the water is mixed with the liquid fuel and injected through the nozzles via the liquid fuel manifold, while the gas flows through the gas fuel nozzles.

Starting System

The starting system consists of a hydraulic starter mounted on the gas turbine and a skidmounted hydraulic start pac. When the operator initiates a start, the hydraulic start pac provides high-pressure fluid to the starter motor geared to the high compressor rotor shaft of the gas generator. The starter converts the fluid pressure to shaft torque and rotates the highspeed rotor to ignition speed in approximately 17 seconds.

After fuel is admitted to the gas turbine combustion section and ignition is achieved, the gas turbine accelerates to its self-sustaining speed and the starter is disengaged from the shaft. If any problems are detected during the start cycle, the control shuts off fuel flow and hydraulic fluid flow, causing the starter to disengage and the gas turbine to coast down.

The starting system is also used to rotate the gas turbine rotors for water wash and/or gas path purge. When water wash is selected, the hydraulic start system is used to motor the gas generator while water, with or without detergent, is sprayed into the bellmouth. During a purge operation, gases accumulated in the gas turbine are purged by motoring the gas generator with the starting system to above 1500 rpm with the ignition and fuel systems off per requirements.

FIRE PROTECTION SYSTEM

The fire protection system provides independent fire detection and CO2 total flooding fire suppression systems for the gas turbine enclosure and is available as an option for the generator and control enclosures.

Automatic fire detection is provided by rate compensated thermal detectors. Facilities for manual (electric and mechanical) initiation of the fire systems are also provided. The CO2 tanks, solenoids, and manifold are located outside the enclosures, with the system’s control module located in the control room.

The control system monitors and displays the status of all fire system inputs and provides outputs to activate audible and visual alarms, discharge CO2, close fuel fire safety valves, and signal turbine and unit control systems for required responses. The system operates on 24volts DC and contains its own internal power supply and battery backup.

Immediately upon actuation of the system, the turbine enclosure secondary air supply fans are deenergized and the fuel supply is shut off. A 20-second time delay permits rundown of the gas turbine and generator before a solenoid valve releases the pressurized CO2 into its distribution manifold. A pneumatic cylinder, actuated by the pressurized CO2, releases a pair of guillotine type dampers (just forward of the fan blades) closing off the secondary air path.

Simultaneously, a series of nozzles floods the enclosures to a 34% CO2 concentration, sufficient for inerting the combustion process.

The CO2 supply to the manifold is fed from pressurized tanks. The first tank is quick emptying, while the second slow-emptying tank maintains the 5% level required to overcome dilution from air leakage. This CO2 concentration is maintained for approximately 30 minutes, sufficient time to allow combustibles to cool below their auto ignition temperatures. A CO2 status display board is provided near each protected enclosure entry to visually indicate the

status of the fire protection system (i.e. CO2 armed or disarmed).

Disarming may be accomplished by disabling the CO2 discharge system, either electronically by means of a key switch and/or blocking the flow of CO2 by a manually activated safety block valve in the CO2 piping discharge system. When disarmed the detection alarm system will remain active while the CO2 discharge capability will be disabled. Continuous alarm signals are sent to the monitoring system notifying the operator while the system is disarmed.

Additional safety features include a suppressant release delay and audible and visual alarms inside and outside the enclosure.

Gas Detection System

A resistance type combustible gas sensor provides gas detection in the gas turbine

enclosure. When the gas concentration reaches 20% lower flammable level (LFL), the gas hazard alarm will be displayed in the fire control panel and warning alarms will be activated at the enclosure. When a 60% LFL level is reached an automatic trip of the fuel and gas turbine will be initiated. The enclosures ventilation system will remain operational to reduce the gas hazard.

Engine Heating (Dehumidification) System

The engine heating system maintains the internal parts of the gas turbine at a temperature above the dew point of the ambient air during non-operation periods. Hot air forced into the compressor section and out through the exhaust and inlet ducts prevents condensation, which can cause corrosion. The system is fully automatic.

Offline Water Wash System

The gas turbine requires periodic washing of its aerodynamic components that have accumulated deposits which could affect performance.

A water wash circuit, when activated by the operator, energizes a solenoid valve in the water wash line. The water flows through a nozzle located inside the inlet air plenum, directed toward the gas turbine’s inlet. While the gas turbine is rotated on the starter, the sprayed water is pulled through the compressor section of the gas turbine and is drained through drain valves. After washing, the gas turbine is started to dry out any remaining water.

The water wash skid is optional.

Electric Generator

The electric generator is an open ventilated, two-pole, air-cooled unit rated to BS/IEC standards with Class “F” insulation but limited to Class “B” temperature rises on a total temperature basis.

It includes shaft mounted overhung main and pilot brushless exciters complete with rotating fused diodes, and all required support auxiliaries, instrumentation, protective devices, and controls.

Stator

The stator frame is fabricated from mild-steel plate, forming a rigid structure. The stator coreis built up from laminations of low loss, high permeability, and high silicon content electrical steel. Radial ventilating ducts are formed at intervals along the core by “H” section steel spacers. The core is hydraulically pressed at several stages during the building operation to ensure uniform compaction. When finished it is clamped between heavy ribbed steel end plates split on the bearing lines. The stator winding is of the two-layer diamond type half coilsbeing used for ease of handling during manufacture and winding.

The insulation system is based on a resin rich thermosetting mica glass tape which, when processed results in a dimensionally stable high performance insulation capable of continuous operation at temperatures up to 155ºC (Class F).

The half coils are placed in the stator slots in two layers, wedged securely in position by synthetic resin bonded wedges, and joined by brazing the copper laminations. The end winding is securely braced to insulated brackets supported from the stator frame. The

The AVR, which controls the excitation of the generator, is housed in a 19-inch rack assembly that has the advantages of compact modular construction, enabling a wide range of optional features to be readily incorporated into the excitation system. Power to operate the unit is supplied from a permanent magnet pilot exciter mounted on the generator shaft.

The unit includes the following solid state plug-in modules and facilities:

- Hand control bridge

- Auto control bridge

- Voltage control card including over flux limiter

- Diode failure detector and fast acting current limiter

- Excitation limiter card including over and underexcitation limiters with ambient temperature compensation

- Falling frequency protection

- "Soft start" circuit to minimize voltage buildup overshoot

- Power factor card for constant power factor or constant reactive power control

The AVR has a voltage adjustment range of + 10% and accuracy of regulation of no load to full load.

Generator Data

Manufacturer

Brush Electrical Machines, Ltd. or equivalent

Type

Synchronous, direct air-cooled, cylindrical rotor, and two-bracket bearing AC generator with brushless exciter and permanent magnet pilot exciter. Class F insulated rotor stator.

Note: Rating details and performance curves are found in the attached

Manufacturer’s Data Sheets.

Electrical Control Center Enclosure

The electrical/control center enclosure contains all of the equipment necessary for local control of the FT8, together with the switchgear and generator protectives. The enclosure is air-conditioned and heated for operating personnel comfort and is designed for global installation.

The enclosure contains the gas turbine and generator controls, motor control center, low voltage AC and DC distribution, station batteries, metal-clad switchgear, station auxiliary transformer, protectives relaying, and master terminal board. These controls and instruments are mounted on the following cabinets:

-	Protective Relay Cabinet

-	Instrument Cabinet

-	Operator Cabinet

-	Motor Control Center
	-	Monitoring Cabinet

- Master Terminal Board Cabinets

MONITORING CABINET (MAIN OPERATOR INTERFACE)

This cabinet houses equipment used by the operator to interface with the control system for data monitoring, trending and event history. It is located next to the operator cabinet for ease of access to the manual controls.

Housed in this cabinet is the Industrial Control Engine (ICE) monitoring system computer, which is utilized by the operator for gas turbine data acquisition and operating parameter adjustments. It consists of a 20" touch screen monitor, CPU, keyboard and trackball. The CPU interfaces with the controller via an ethernet link.

A printer is also provided to print current screen information, event log, plots and trends.

Synchronizer - This Westinghouse XMC synchronizer is a microprocessor based unit that synchronizes the generator to the bus when running the unit in automatic operating mode. Power is turned on to the XMC by the unit control system once synchronous speed is reached and remains on for one minute after breaker closure. This time delay allows breaker closure time, shown on the front LCD display, to be recorded if desired.

Static Inverter - The inverter converts DC power from the batteries into AC power to be used by the auxiliary AC instruments and equipment. It contains inputs from both the AC and DC buses and can provide 120 VAC output power from either source. The inverter monitors the AC bus at all times. If power supplied from the AC bus, the inverter will automatically transfer to the DC source upon loss of AC power.

Operator Cabinet

This cabinet contains panel-mounted switches and instruments

used to manually control and monitor the generator performance.

Manual voltage/VAR control, speed/load control, synchronization,

and generating mode selection are all performed from this panel.

Following is a list of components installed on the panel.

Alarm Horn

Synchronizing Lights

Synchroscope and Syncroscope Switch

Bus and Generator Voltmeters

Generator Wattmeter

Generator Var Meter

Power Turbine/Generator Tachometer

Generator Circuit Breaker Control Switch

Speed/Load Control Switch

Volts/Var Switch

Lockout/Operate Switch Engine A&B

Emergency Stop.

64F Test Switch

Timer – Running Hours, Liquid Hours, and Gas Hours

Counters – Total Starts and Cycles

Speed Relay Reset Button

Panel Back Wall Devices)

Overspeed Switch

SEL 2401 – Satellite Clock

Protective Relay Cabinet

The equipment in this cabinet provides hardwired protection for the gas turbine, generator and bus equipment. The panel contains nine (9) lockout relays that are used to trip the unit or a single gas turbine in response to signals from the various electrical protective devices and the unit control system. Each lockout is a high speed, multi-contact, hand-reset relay provided with a mechanical target. Health monitoring relays are also provided for each lockout and are mounted on the panel back wall.

GEN WHM - Generator Watt-Hour Meter

DBR – Dead Bus Relay

25SC - Sync Check Relay

TSB-1, TSB-9 - Test Blocks

86BF – Breaker Failure Lockout Relay

86BF-R – Redundant Breaker Failure Lockout Relay

86G1 – Generator 52G Trip Lockout Relay

86G2 - Generator 52G Trip + Controlled Shutdown

Lockout Relay

86E – Engine Lockout Relay

86AT – Auxiliary Transformer Lockout Relay

86EA – Engine A Trip Lockout Relay

86EB – Engine B Trip Lockout Relay

86BG – Generator Breaker Lockout Relay

Panel Back Wall Devices

WT3801, 3802 – Watt Transducers

Speed Switch - Provides generator and power turbine overspeed protection.

SCB1,SCB10 – Short Circuit Blocks

Instrument Cabinet

This cabinet holds the accessory controls such as voltage regulator, synchronizer, vibration monitors, and generator protection relay. Each piece of equipment operates independently to control a specific aspect of plant operation. The unit control system coordinates the activity of these controllers through both hardwired and communication signals.

Auto Voltage Regulator (AVR) -Primarily designed to control the excitation of the generator. The AVR allows both manual and automatic control of generator

excitation.

Synchronizer – Consists of three rack mounted modules that determine the proper time to initiate closing of the breaker to parallel the generator to the bus.

Generator Protective Relay – Beckwith M-3425A is a microprocessor based unit that uses digital signal processing technology to provide up to thirty four

protective relaying functions for generator protection.

Vibration Monitors - Bentley Nevada Vibration Monitoring System.

• Electric Generator - The monitor interfaces with two embedded proximity probes on each generator bearing. Displays of each bearing vibration level

amplitude are shown on the front of the monitor along with lamp indication of system malfunction, alarm and shutdown vibration levels.

• Gas Turbine - The monitor interfaces with velocity transducers mounted on the GG8 inlet case (flange A) mid-engine case (flange K) and the PT8 exhaust

case (flange S). Vibration amplitudes are displayed on the front of the monitor for each of the three channels. Lamps on the front of the monitor indicate a faulty transducer or monitor malfunction and vibration level alarm and vibration level

shutdown. A reset button must be depressed to rearm visual alarms after alarm indication

Master Terminal Boards

These cabinets terminate the majority of field control wiring at one central location. They contain terminal strips that connect the instruments to monitor and control the gas turbine and support systems to the CPU. One cabinet also houses the unit control system.

The unit control is a Woodward Governor Company digital microprocessor. It uses data from instruments throughout the plant to control unit operation. This data is also passed on to the monitoring system where it is accessible to the operator. The system is composed of a main chassis and one expansion chassis. The cabinet holds the main chassis, which contains the CPU, communication hardware, and input/output cards for the generator and control house equipment.

Motor Control Center (MCC)

The Motor Control Center contains all the 3-phase motor starters and contactors and required 480V - 3-phase feeder circuit breakers for the plant. Additionally, the MCC contains the AC and DC distribution panels and the transfer switch. The front panel of each motor starter bucket has a red and green indicating light to identify motor ON/OFF operation.

Power to the MCC is supplied by the 300 kVA, 13.8 kV / 480 V, 3-phase, 3-wire auxiliary transformer that distributes the power to the MCC through a 480 V, 3-pole, 600 amp circuit breaker. The auxiliary transformer and circuit breaker are housed in a cubicle next to the main circuit breaker. The 125 VDC and 24 VDC distribution panels in the MCC are supplied power from the 125 VDC and 24 VDC batteries and associated 125 VDC and 24 VDC battery chargers that are located close to the MCC.

DC Power Supply

One hundred twenty five (125) VDC and 24 VDC batteries are rack mounted in a ventilated enclosure. They supply all necessary DC power for safe shutdown of the FT8 and Emergency

blackout conditions. Battery chargers are furnished and properly rated to supply the FT8’s DC loads and also have capacity to charge the batteries. Chargers are supplied power from the MCCs 480 VAC distribution panel that is fed from the 13.8 kV / 480 VAC, 3-phase, auxiliary transformer.

The auxiliary transformer is connected to the load side, not the generator side, of the 13.8 kV switchgear circuit breaker. Thus, when the FT8 is in the standby mode and not generating power, the auxiliary transformer derives its power via the back-energized main step-up transformer.

Unless there is a blackout, this is a very reliable source of power. However, when the FT8 is started and synchronized to the 13.8 kV side of the main-step-up transformer and the main breaker is closed, the FT8 generator supplies power to the 13.8 kV / 480 VAC auxiliary transformer.

15 kV Class, Metal-Clad Switchgear Module

The 15 kV class switchgear is installed in the control center enclosure, and is connected to the generator by a totally enclosed, non-segregated phase, 3-phase bus duct system rated at 3000 amp. The switchgear module is of the metal-clad construction and consists of three cubicles installed on one end of the electrical control center enclosure and described as follows:

The first cubicle contains the dry-type, auxiliary transformer that is rated at 300

KVA, 13.8 kV / 480 VAC, 3-phase, 3-wire, 60 Hz. The 13.8 kV primary side of the

Transformer is connected to the load side of the main circuit breaker located in the adjacent second cubicle. The auxiliary transformer supplies the necessary 480

VAC power to the FT8 MCC. A 480 VAC, 3-pole, 600 amp, 60 Hz, circuit breaker

connected to the secondary side of the auxiliary transformer is also contained in

this cubicle.

The second cubicle contains the main circuit breaker and necessary current

transformers for metering and relaying. The main switching and interrupting

device (vacuum circuit breaker) is of the removable draw-out type arranged with a mechanism for moving it physically between connected and disconnected

positions and equipped with self-aligning and self-coupling primary disconnecting

devices and control wiring connections. The main generator circuit breaker is a 750 MVA interrupting, 3000 amp, 15 kV air circuit breaker featuring 125 VDC close and trip.

The third cubicle contains the potential transformers, lightning arrestors, and the

surge capacitors as required for metering and equipment protection.

Control System

The FT8 control system contains an integrated Woodward gas turbine and unit control. This integrated controller acts as a central processing point for all I/O, serial and Ethernet communications associated with the FT8. Data from this control is sent to a user-friendly operator interface to display pertinent information.

Integrated Gas Turbine and Unit Control

The Woodward MicroNet Control System performs both fuel control and sequencing functions. This system incorporates 32-bit microprocessor based digital controllers, which optimizes turbine safety and efficiency. The programmable features of this control enhance the ability of the end user to incorporate the latest features in gas turbine technology.

The MicroNet provides a flexible system to control associated processes such as high speed control functions, system sequencing, auxiliary system control, surge control, monitoring and alarming, and station control. Communications with the MicroNet platform are available to program and service the control as well as to interface with other systems (Plant DCS, HMI, etc.). A service interface allows the user to view and tune system variables.

This integrated control performs all of the gas turbine control functions including:

Speed Control

Temperature Protection and Control

Acceleration and Deceleration Limiting

Fuel Valve Control

Inlet Guide Vane Control

Variable Stator Vane Control

Water Injection

Gas Turbine Operational Monitoring

Start Sequencing

Unit Synchronization

Alarm and Shutdown Protection

Monitoring System

The monitoring system package operates on an industrial PC and serves as the operator interface for the FT8. This system interfaces to the gas turbine and unit control via an Ethernet local area network (LAN). The monitoring system performs such functions as:

Data Logging and Trending

Alarm Monitoring

Alarm and Event Logging

Sequence-of-Events Recording

First-Out Alarm Indication

X-Y Plotting

Calculation Functions

Event Storage, Archiving, and Redisplay

Operator Control Functions

Process Animation

Control System Diagnostics

Gas Turbine Enclosure

The gas turbine enclosure protects the gas turbine and other ancillaries from the environment and provides noise attenuation. When the unit is operating the enclosure protects outside workers from the high component temperatures. When the unit is not operating it provides a protected working environment and inside lighting for maintenance.

The gas turbine enclosure includes the inlet plenum and the exhaust enclosure. The inlet plenum channels the air from the inlet air silencer to the gas turbine bellmouth to minimize the inlet air pressure loss. The exhaust enclosure houses the exhaust diffuser/collector, which delivers the power turbine exhaust gases to the exhaust transition duct and silencer. The enclosure serves as a mounting structure for the inlet and exhaust air systems, and as a duct

for passing secondary air over the gas turbine for cooling. The enclosure also provides a limited containment volume for the fire protection system.

Gas Turbine Enclosure Secondary Air System

The gas turbine enclosure secondary air system provides cooling air flow throughout the length of the enclosure. Air is drawn into the intake assembly, through the silencer chamber, and blown into the gas turbine enclosure. It passes along the length of the gas generator and -power turbine, around the exhaust collector and up into the exhaust air silencer, where it joins with the gas turbine exhaust.  A pair of AC motor driven fans, rated at 22,000 scfm (10.3 m3/sec), are mounted low on the front bulkhead of the enclosure to take in atmospheric air through a louvered opening with a 2x 2-inch (5 x 5 cm) mesh screen and a silencer module. If a gas turbine enclosure fire is detected, the fans are de-energized and a pair of guillotine type gates are released to block the inlet air supply ports.

Inlet Air Filtration

The primary inlet filtration system has a filtration efficiency of 99.7% for particles down to 5 microns, and 95% for particles down to 2 microns. Clean elements flow about 190 pounds (86 kg) of air per second with a maximum pressure drop of approximately one inch (2.54 cm) of water.

The filter house weighs approximately 32,500 lbs (14,800 Kg) and is 20 feet (6.1 m) long, 12 feet (3.6 m) wide and 13 feet (4.0 m) high. It is constructed of mild steel and rests atop (with its flange bolted to) the inlet air silencer module. Air enters both end walls of the filter house. Each end wall is configured to have 30 two ft2 filter elements and 6000 half-sized elements in individual galvanized frames.

The high efficiency, replaceable fiberglass filters are preceded by a pre-filter. The two filters are held together by a pair of extended spring clips. Filter elements can be replaced from inside the filter house. The doors can be accessed from a full-length external catwalk with safety railing and fixed-end ladder.

Inlet Air Silencer (when specified)

The inlet air silencer module attenuates noise over a broad band of frequencies. It is 10 feet (3 m) wide by 11.5 feet (3.5 m) long. The box configuration housing is four feet high and flange-bolted on top of the forward end of the gas turbine enclosure. Air flows through this module directly into the contoured inlet plenum.

A similar flange ringing the top of the housing is bolted to a mating flange in the floor of the inlet air filter house. The silencer outer wall is 3/16-inch (4.75 mm) steel plate and the inner wall is 16 gauge perforated galvanized steel, with four inches of mineral wool fill between them.

Module walls, configured for sound attenuation, consist of 17 equally spaced, transversely mounted vertical acoustic baffles. The baffles, which have a 16-gauge perforated galvanized steel shell and are filled with three inches of mineral wool, may be removed for servicing.

Exhaust Silencer (when specified)

The exhaust silencer is composed of two major modules, the transition module and the silencer module. The transition module provides a transition for the gas turbine exhaust gases and surrounding secondary cooling air. The silencer module is flange-mounted to the exhaust silencer transition-housing module. The number of the exhaust silencer modules is shown on the general arrangement drawings, and is also indicated in the Scope of Supply.

Construction of the transition module and the silencer module is similar; both employ double wall construction. Mineral wool is used in the outer wall for sound absorption and fiberglass wool is used in the inner wall for heat insulation and sound absorption. Four sound attenuating baffles are contained in the silencer module

Each consists of perforated stainless steel channel shapes filled with fiberglass wrapped by stainless steel wire mesh. The baffles slide into module sidewall channels. The exhaust silencer modules are stacked for maximum sound attenuation. Surfaces exposed to hot exhaust gas are made from 409 stainless steel.

ESTIMATED TWIN PAC WEIGHTS AND SIZES

Quantity	Description	Weight, lb/kg (ea)	Length, ft/m	Width, ft/m	Height, ft/m

2	Gas turbine assembly on base	53,500/24,300	27.5/8.4	12/3.7	11/3.3
inside enclosure

2	Engine heater	950/430	5/1.5	4/1.2	3.5/1.1

2	Exhaust enclosure	8,700/3,950	15/4.6	10.5/3.2	11/3.3

2	Collector box *	4,400/2,000	14/4.3	6/1.8	10/3.2

2	Secondary inlet silencer	6,200/2,800	11.5/3.5	10/3.2	5/1.5
with inlet louver

2	Inlet air filter assembly,	32,500/14,800	20/6.0	13/4.1	14/4.2
ladders and walkways

2	Diffuser inner/outer cylinder, 	6,200/2,800	8/2.5	8/2.5	5/1.6
tunnel and turning vanes

2	Inlet silencer		9,900/4,500	11.5/3.5	10/3.1	5/1.6

2	Exhaust transition piece	9,900/4,500	15/4.6	10/3.1	5/1.6

4	Exhaust silencer	16,000/7,300	15/4.6	10/3.1	9/2.7

2	Lube oil system inside enclosure	19,800/9,900	20/6.0	11.5/3.5	10.5/3.2

2	Water injection skid	2,500/1,100	8/2.4	5.5/1.7	6/1.8

1	Hydraulic start pac	9,900/4,500	7.5/2.3	7/2.2	8/2.4

1	Electric generator *	180,600/82,100	23/7.1	13/3.9	11/3.3
with lube oil module

1	Recirculation damper	2,500/1,100	9/2.8	10/3.2	4/1.2

1	Generator enclosure *	14,500/6,600	22/6.7	12/3.6	6/1.8
		14,500/6,600	22/6.7	12/3.6	6/1.8

11,000/5,000	24.5/7.5	6.5/2.0	7/2.2

1	Generator silencer	15,000/6,800	24.5/7.5	10.5/3.2	6/1.9

1	Control/switchgear unit	71,000/32,300	42/12.8	11.5/3.5	12/3.7

1	Batteries/remote panel	1,900/850	3/1	3/1	8/2.5

1	Bus duct		5,100/2,300	18/5.5	4/1.3	6.5/2

*  Requires field assembly on site

COMMERCIAL TERMS

Basis of Pricing

Validity

This proposal is valid until February 13, 2010. All unit are subject to prior Sale.

Three (3) new FT8 SWIFTAPAC™ Gas generators………….….…$97,500,000

Delivery is ex-works, Inco terms 2000 and subject to prior sale as follows…

- Units 1: September,   2010

- Units 2: October,  2010

- Units 3: November. 2010

Dual fuel, water injected, 60Hz, 13.8kV electric generator.

- Guarantees and warrantees to include delivery, power output, heat rate, and emissions along with a material and workmanship warranty 12 months from commercial operations or 18 months after equipment shipment. Additional details provided in the enclosed Turbine Purchase Agreement Terms & Conditions.

- The guarantees and warrantees can be transferred to the end user in Venezuela.

- Installation of the equipment is the responsibility of the customer.

Payment Schedule

Purchaser will demonstrate its financial capability to continue to carry out its obligations under this Contract. This demonstration may require that Purchaser furnish adequate payment security. This proposal is based upon receipt of a letter of credit.

Payment Event

Signature of the Contract Agreement

by both Parties, paid against Seller’s Invoice 		30%

Upon notice of Ready to Ship for the Gas Turbines

Turbines and Auxiliaries 				                       70%

## Wire Information

| BENEFICIARY: | DERWICK ASSOCIATES CORP. |
| --- | --- |
| BANK NAME: | JPMorgan Chase Bank N.A. |
| BANK ADDRESS: | 345 Park Avenue |
| CITY, STATE, ZIP: | New York, NY 10154 |
| COUNTRY: | United States of America |
| SWIFT CODE: | CHASUS33 |
| Routing Number / ABA Code: (For USA Vendors) | 021000021 |
| ACCOUNT NUMBER: (without IBAN details) | 811156397 |

Taxes/Duties/Bank Expenses

All taxes, VAT, custom duties, and bank expenses due are for the purchaser’s account.

Proposal Validity

This budgetary proposal is valid for thirty (30) days.  The equipment described herein is subject to prior sale.

Terms and Conditions

This proposal is based on Derwick Associates Corp. standard Terms and Conditions.

Confidentiality

This Proposal is submitted in confidence for evaluation by Buyer.  Its contents are proprietary to Seller.  By taking receipt of this Proposal, Buyer agrees not to reveal its contents in whole or in part beyond those persons in its own organization necessary to properly evaluate this Proposal or to perform any resulting contract.  Buyer shall not reveal the contents of this Proposal to a third party or make copies of this Proposal without the prior written consent of Seller.  Buyer shall return this entire Proposal to the undersigned, if Buyer does not accept this Proposal.

We sincerely appreciate this opportunity to provide you with this offer and are willing to discuss any issues and resolve them on a mutually acceptable basis as you progress through your overall proposal preparation.

- Follow Up

Please contact the following person at Derwick Corporation for information regarding this proposal:

Pedro Trebbau

Director

ptrebbau@derwickassociates.com

58-412-3007470

Scope of Supply