Turbo Fume Exhauster®

# MANUAL NO. 8881-0127

# REV. D

##### WARNING

Read the safety & set-up information sections in this manual thoroughly before installing and using your fume extraction system.

## Safety

PACE Fume Extraction systems are designed to reduce the level of harmful contaminants from the work environment and to assist in the achievement of recommended health and safety requirements for local exhaust ventilation and contaminant removal.  Be sure to follow all safety guidelines contained herein and precautions contained in other relevant product safety literature (i.e., Material Safety Data Sheets) provided with the substances and equipment producing the fumes to be extracted.

Disclaimer

PACE hereby disclaims all responsibility for any personal injury, property damage, fine, citation or penalty imposed by any government or private entity which results from any use, misuse or mis-application of this product, failure of the user to regularly maintain the product according to the recommended guidelines, or failure to adequately monitor fume extraction exhaust air and the ambient workplace air for the presence of harmful levels of gases, fumes and particulates.

Compliance with all applicable environmental and personal safety regulations is the sole responsibility of the user and adequate self-monitoring of exhaust air released into the atmosphere or the workplace as well as monitoring of the ambient workplace air is strongly recommended.  To insure continued effective performance, the following guidelines must be followed.

Dangers

- PACE Fume Extractors are not intended to be used as a substitute for devices such as personal protective respirators which are designed to remove poorly adsorbed substances including carbon monoxide, methane, hydrogen, acetylene and other gases.  Risk of serious injury or death, fire or explosion may result from improper use.  If in doubt, contact your industrial hygienist or PACE.

- Never use PACE Fume Extraction Systems to extract fumes from highly flammable liquids and gases such as Ether, Gasoline (Petrol) and others.  Risk of serious injury, death, fire or explosion may result.

- High concentrations or airborne contaminants such as Methyl Ethyl Ketone, Cyclohexone, Ozone and others when contacted with activated carbon, can undergo oxidation, decomposition or polymerization resulting in exothermic reactions or heat generation.  PACE Fume Extraction Systems must not be used to extract fumes from these substances when high concentrations are present.

- Failure to comply with the application and maintenance guidelines, filter replacement schedules, monitoring recommendations and safety guidelines contained herein and in other relevant product safety literature (i.e., Material Safety Data Sheets) provided with the substances and equipment producing the fumes to be extracted could result in risk of serious injury, fire or explosion.

Specifications

Turbo Fume Exhauster Part Numbers				8888-0600 220/240V 50hz

8888-0605 110V, 60hz

Flow Rate.	128m³/h (75cfm)

Noise Level*							54dbA at 1 metre

Weight								7.51Kg (16.54lbs)

Size H x W x D							550 x 240 x 89mm

(21.5” x 9.5” x 3.5”)

Housing	18 gauge Steel

Standard Filter							Activated carbon impregnated

foam.

For Replacement filters, package of ten. P/N 8883-1311-P10

###### Environmental Specifications

## Ambient operating temperature					0ºC to 50º (32ºF to 122ºF)

Storage temperature						-40ºC to 125ºC (-40ºF to 257ºF)

Ambient operating humidity	90% relative humidity max non-condensing

Storage humidity	90% relative humidity max non-condensing

*Airflow and noise levels are nominal figures and will vary based on voltage

## Features

Turbo Fume Exhauster

      - Fitted power cord.

      - Fuse holder.

      - On/Off switch.

1		2	3

Fig.1	Turbo Fume Exhauster.

###### Set up

Position the Extractor in a convenient location near an appropriate electrical outlet. Remove items which may block the airflow from the working area to the extractor. Locate the extractor as close to the fume source as possible.

###### Operation

###### Position the extractor so the front grill faces the operator. Turn on the fume extractor. For maximum extractor performance, use the adjustable feet (Located on back side) to tilt down the extractor towards the tabletop.

###### Maintenance

In general, the Extractor filter must be changed on a monthly basis. However, depending on your particular process, the filter may need to be changed more or less frequently. Replacement can be performed by unscrewing the filter grill retention screws, and removing the filter grill. Remove the old filter and replace it with a new filter. Reinstall the filter grill and filter grill retention screws.

Warranty

PACE warrants that this equipment will be free of defects in materials and workmanship for a period of one (1) year from the date of receipt by the first user.

This warranty does not cover repair of replacement required as a result of misuse, mishandling or improper storage.  Failure to perform recommended routine maintenance, alterations or repairs made other than in accordance with PACE’s directions, or removal or alteration of identification plates in any way will void this warranty.  This warranty is available only to the first user, but the exclusions and limitations therein apply to all persons and entities.

This warranty does not apply to consumable items such as filters.

PACE MAKES NO OTHER WARRANTY, EXPRESSED OR IMPLIED, AND MAKES NO WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

PACE will, at its option, repair or replace and defective equipment or parts at its facility or other location approved by it at no charge to the user, or provide parts without charge for installation by the user in the field at the user’s expense and risk.  User will be responsible for all costs of shipping equipment to PACE or other warranty location for warranty service.

EXCEPT FOR THE REMEDY ABOVE DESCRIBED, UNLESS OTHERWISE REQUIRED BY APPLICABLE LAW, PACE WILL HAVE NO OTHER OBLIGATION WITH REGARD TO ANY BREACH OF WARRANTY OR OTHER CLAIM WITH RESPECT TO THE EQUIPMENT, OR LIABILITY FOR ANY DIRECT, INDIRECT, CONSEQUENTIAL, OR INCIDENTAL LOSS OR DAMAGE CAUSED BY OR OCCURRING IN CONNECTION WITH ANY OF THE EQUIPMENT.

To obtain warranty service, contact the appropriate PACE company listed below

PACE Inc, 9020 Junction Drive

Annapolis Junction, Maryland 20701

Tel. (888) 535-7223 (toll free) Warranty Service FAX 301 483 7030

PACE Europe Ltd. Sherbourne House, Sherbourne Drive, Tilbrook, Milton Keynes

United Kingdom, MK7 8HX

Tel. (44) 01908 277 666 Warranty Service FAX (44) 01908 277 777

Do NOT return defective equipment or parts to PACE without obtaining prior authorization.

Any warranty or other claim with respect to the equipment must be made in writing and delivered to PACE (or local authorized PACE Distributor outside the U.S.) within a reasonable time of the expiration date of this warranty.  Sufficient evidence of purchase and date of receipt must also be included, otherwise user’s rights under this warranty shall be deemed waived.

To register your purchase with PACE, fill in the form below and mail or FAX to the applicable PACE address listed above.

EQUIPMENT WARRANTY REGISTRATION CARD

Date

Model No.			 	Serial No.			Of Receipt

Purchased from

Company Name

Mailing Address

Telephone number with area code

Fax Number with area code

Internet Address

Full Name

Authorized Signature, Title

www.paceworldwide.com