TRAY NOT OPENING.

SYMPTOMS

Tray will not open.

CAUSE

NO OPEN OPERATION DUE TO SHORT ED S001 ON MB-84 BOARD. ERROR CODE 85 WITH 19 HOURS COMBINED CD/DVD USAGE.

RESOLUTION

REPLACED SWITCH (S001) FOR PROPER TRAY OPERATI ON.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 177156211; S001; CHUCK SENSOR; 161B; NA; MS48; Q; A.

No video output.

SYMPTOMS

No video output

CAUSE

no video, very intermittent. .

RESOLUTION

Found cold solder joints at ref. RV501 on MB86, resoldered, test ran ok

DVD LOCKING CODE.

SYMPTOMS

DVD players had the ability to electronically lock the disc drawer using a locking and unlocking sequence. The new models also have this ability but it is different. The sequence is as follows.

RESOLUTION

To lock AND unlock (same sequence),In stand-by mode (Power light is red)press this button sequence on the remote 1 ANGLE SUBTITLE 9 POWER (The POWER key on the front panel does not work for this purpose.)

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; MDP. DVPS560D; MDP.

FUSE OPEN, NO DISC READ.

SYMPTOMS

Will not READ DISCS or C13:00 .

RESOLUTION

REPLACED SHORTED OPTICAL ASSY AND F402 with components listed in service bulletin MDP068.

See Service Bulletin MDP068 for more details and part numbers.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPC660; 0158681; 0158681; MDP. DVPC670D; 0158681; 0158681; MDP. DVPS360; 0158681; 0158681; MDP. DVPS560D; 0158681; 0158681; MDP. DVPS570D; 0158681; 0158681; MDP.

JIG FOR TESTING MAIN BOARDS.

SYMPTOMS

Service kit for the DVPs360/ 560D mechs used for troubleshooting MCB. Tokyo was not include in Manual. A correction may be issued. However the service kit exists Order the part number of it is J-6090-100-A. All service kits what you need are included in  the above part number, so you can get those kits if you would order the above part number. If you look on 5 and 6 of Service Manual, you will see where this item is used.

RESOLUTION

We continue to investigate this issue

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; MDP. DVPS560D; MDP. DVPS560D; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. J6090100A.

NO BOOT UP, NO DISC READ, SHUTS OFF.

SYMPTOMS

INTERMITTENT SHUT DOWN.

RESOLUTION

REPLACED MICRO (IC102) FOR PROPER OP ERATION. REPLACED OPTIC DUE TO MINIMALTORN FLEX AND PERFORMED DRIVE ADJUSTMENTS.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; 878163; 878163; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 875966392; IC102; ICMB91107PFV-G-BND; MAIN.

NO CHROMA PRESENT IN VIDEO OUTPUT SIGNAL.

SYMPTOMS

No chroma present while observing menu video or dvd playback video, or the dvd playback video. burst signal is ok. luma level ok, s video output missing chroma also. check ic502 pin 77 on mb86 board, this test point is accessible without deck removeal,  use a test disc with split field color bar pattern. if chroma is present at pin 77 of ic502,it is likely that ic101 has failed onbottom al 17 board. video buffer ic101 part 875966763.

RESOLUTION

Replacement of IC is not confirmed by AS, has only given some good reference points.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; 846304; 846304; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 875966763; IC101; AI-17.

NO LISTING IN S/M FOR SW101.

SYMPTOMS

The power switch - SW101 - has no part number listed in the manual.

RESOLUTION

Tokyo reports they don't have a part number for SW101 (switch) on the Power Block in DVP-S560D, because it is not the service part for this model. That's why it's not listed in the service manual to show a part number. Please exchange whole power  block to the new one if you would need to repair it. Part# 1468-505-11

NO POWER.

SYMPTOMS

NO POWER. POOR CONNECTION FROM THE MAIN BOARD R.# 112 TO THE BOTTOM BOARD R. # 115.

RESOLUTION

RESEAT THE BOARD UNIT BACK ON NORMAL OPERATION.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; 911379; 911379; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. A6065460A; MB-86; MAIN BOARD; MAIN.

NO POWER, MECHANICAL SWITCH DISLODGE.

SYMPTOMS

Switch lever will dislodge from this joint. The switch must be set back into the claw.

RESOLUTION

Realign THE switch assy AND use GLUE TO HOLD BETTER.

Tokyo has told us that countermeasure will be applied after s/n 690000. A change in claw design is expected. See technical webpage in Aug 2000 for pictures. These pictures are on website for a limited time

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; 0178526; 0178526; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 305932001; 107; JOINT (P); CASE.

POWERS ON, BUT SHOWS STANDBY.

SYMPTOMS

Unit appears to be dead after power switch is pressed.

RESOLUTION

Start sequence is as follows, Power switch pressed, red led, green led, standby appears then LCD goes dark. You must hit a function button (for example open/close or play)to keep unit out of standby. This activate the display again and unit is ready to  use. This is normal operation of the unit.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; MDP. DVPS560D; MDP.

THE PART FOR D105 (PGKE120) IN POWER BLOCK.

SYMPTOMS

Part number not listed in S/M

CAUSE

The part number is not listed in S/M.

RESOLUTION

The part # for D105 (PGKE120) in Power Block PART NUMBER : 9-885-005-98

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; all; all; MDP. DVPS560D; all; all; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 988500598; D105; PGKE120; 118Z; N/A; PWR; A; A.

VIDEO HAS FAINT LINE IN PICTURE.

SYMPTOMS

THIN VERTICAL LINE IN CENTER OF PICTURE DURING LOGO AND DVD PLAYBACK. CLEAN PICTURE IN TEST MODE.

CAUSE

THIN VERTICAL LINE IN CENTER OF PICTURE DURING LOGO AND DVD PLAYBACK. CLEAN PICTURE IN TEST MODE.

RESOLUTION

REPLACED IC502 (AV DECODER) TO RESTORE PICTURE QUALITY.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; 836020; 836020; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 875239955; IC502; IC CXD1932Q; 1334; VPD; MAIN; D; A.

WILL NOT POWER UP.

SYMPTOMS

WHEN POWER TURNED ON LED WOULD TURN RED THEN GREEN STAY GREEN AND UNIT HAD NO FUNCTION

CAUSE

WHEN POWER TURNED ON LED WOULD TURN RED THEN GREEN STAY GREEN AND UNIT HAD NO FUNCTION.

RESOLUTION

REPLACED IC404 IN THE SYSTEM CONTROL CIRCUIT. IC404 FIXED UNIT.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; n/a; n/a; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 875966995; IC404; C M38B57M6-130; 111A; IFC; AI-17; A; A.

no part number listed for ic101.

SYMPTOMS

Missing Part Number

CAUSE

no part number is listed for ic1010 in s/m for board srv940juc. schematic is also incorrect in calling ic101 an - on 3131. an - on 3131 is an opto isolator. ic101 is drawn as a power control fet type integrated circuit.

RESOLUTION

IC101 Part# 9-885-005-99

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS560D; 888105; VID.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 9-885-005-99; ic101; srv940juc.

MB86 Board modification and Service Bulletin #68.

SYMPTOMS

There are 2(two) versions of MB86 board available.

CAUSE

The MB86 Board has two versions present in the field at this time. The newest version of the board has had F402 removed from the board. This effects the implementation of service bulletin #68. The new version of MB86 does not require the update  changing the resistors.

The new board can be identified by veiwing the B side of the board. There are no indications for F402. The older MB86 has the F402 location stenciled on the board.

RESOLUTION

Please examine the board to determine which version you are working with before you start the service bulletin update. If it is a newer MB86 board, do not perform the update. SB#68 will be updated.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; na; na; MDP. DVPS560D; na; na; MDP. DVPS570D; na; na; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. A6065437A; 112; MB-86 Board, Complete; X185; SYS; MB-86; Y; I. A6065510A; 112; MB-86 Board, Complete; X185; SYS; MB-86; Y; I. A6065472A; 112; MB-86 Board, Complete; X185; SYS; MB-86; Y; I.

Syscon ROM update.

SYMPTOMS

Intermittent Disc reading or No Disc errors.

CAUSE

1. When playing Dual Layer Disc, the video sometimes may stop or may not play normally if Dual Layer Disc is not compatible. 

2. When playing a DVD Disc of LPCM after playing CDDA, the sound sometimes may disappear.

*CDDA.- Normal music CD  
*LPCM.- Linear-PCM (Standard Audio of DVD) 

3. Sometimes, CD/DVD/Video-CD will not play. In these cases, No Disc or C:13:00 appear on display.

RESOLUTION

1. Replace the SYSCON ROM IC 106 on the MB86 board to the latest version one.

Model IC# Part Number Description
DVP-S360 IC106 PN 875982872 IC MR27V1602E-39TPZ060
DVP-S560D IC106 PN 680010101 IC  MR27V3202E-A4TPZ060
DVP-S570D IC106 PN 680010101 IC MR27V3202E-A5TPZ060
These parts are in stock
Note : In Symptom No.3, it is necessary to take the actions speciiefed in Service Bulletin 68 if they have not already been  performe.

Service Bulliten to follow.

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; all; all; MDP. DVPS560D; all; all; MDP. DVPS570D; all; all; MDP.

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 875982872; IC106; IC MR27V1602E-39TPZ060; 1185; SYS; MB86; 1; A. 680010101; IC106; IC MR27V3202E-A4TPZ060; 1185; SYS; MB86; 1; A. 680010101; IC106; IC MR27V3202E-A5TPZ060; 1185; SYD; MB86; 1; A.

Error codes DC and D8..

SYMPTOMS

Shuts off intermittently.

CAUSE

Error codes DC and D8 in Emergency History. DC means that related software has some kind of problem. D8 means AVDEC has problem.

RESOLUTION

Perform SYSCON Diagnosis using the service manual and if you can find the defective part from it, take appropriate measures.

If the SYSCON Diagnostic goes through to the end and you don't find any problems, check the SDRAM (IC 504 and IC505), which  are placed around AVDEC. There are 2 possibilities, 

1. That the SDRAM is defective. 

2. There is a cold solder joint on theSDRAM.

If you still cannot find any problem on the SDRAM, exchange AVDEC.

MORE INFORMATION

  - Unit/Model Details

  - Links

Unit/Model Detail

Unit(s):; SN Start; SN End; Factory; Family Name; Product Category. DVPS360; NA; NA; MDP. DVPS560D; NA; NA; MDP. DVPS570D; NA; NA; MDP.